3GPP TSG RAN WG1 #109-e		R1-2203581

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on PDCCH reception on CRS symbol

Agenda Item:	9.9.1

Document for:	Discussion and Decision

# Introduction

The following objectives are included for improvement of NR spectrum efficiency for LTE NR co-existence (RAN1):

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

  - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

- Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

In this contribution, we discuss the first objective and provide evaluation results on different options of PDCCH puncturing and DMRS receptions when NR PDCCH shares the CRS symbol.

# Discussion

## PDCCH reception on CRS symbol

Currently, UEs are not expected to decode NR PDCCH candidate that overlaps with LTE CRS. Therefore, gNB is not allowed to configure NR PDCCH overlapping with CRS REs.

38.213: If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate.

To improve the resource utilization as well as the NR PDCCH capacity for DSS in the first 3 symbols, it was suggested to study NR PDCCH reception in symbols overlapping with LTE CRS.

To evaluate the performance of this feature, we have performed LLS for the following cases:

- Baseline1: 1 NR PDCCH symbol without collision

- Baseline2: 2 NR PDCCH symbol without collision

- Case1: 2 NR PDCCH symbol with CRS collision

In case1, there are two ways for NR PDCCH puncturing:

  - Case1-1 TX puncturing: gNB punctures NR PDCCH REs overlapped with CRS REs while UE still assumes the PDCCH present on these REs during NR PDCCH decoding.

    - For example, idle UEs without CRS pattern information, or UEs reluctant to change their NR PDCCH decoding module may still use the RE overlapping with CRS for PDCCH decoding.

  - Case1-2 RX puncturing: gNB punctures NR PDCCH REs overlapped with CRS REs and UE also assumes that NR PDCCH REs are punctured according to the CRS REs.

    - For connected UE with information of CRS pattern, it may not use the RE overlapping with CRS for PDCCH decoding.

BLER curves of baseline1/baseline2/case1-1/case1-2 with different DCI payload sizes are shown in the following figures.

| Figure 1 Case1-1 TX puncturing: DCI payload size=41bits excluding CRC | Figure 2 Case1-1 TX puncturing: DCI payload size=60bits excluding CRC |
| --- | --- |
| Figure 3 Case1-2 RX puncturing: DCI payload size=41bits excluding CRC | Figure 4 Case1-2 RX puncturing: DCI payload size=60bits excluding CRC |

From Figure 1 and Figure 2, it can be observed that it is difficult for case1-1 TX puncturing to meet the 1% BLER requirement if PDCCH is transmitted using AL=1/2/4/8 for DCI size=60 bits. When DCI size becomes as small as 41 bits (i.e., the size of DCI format 1-0 in 2GHz with BW=20MHz), PDCCH with AL=8 may meet the BLER requirement at SNR>=-2.5. Therefore, if no modification is made to the UE implementation, the PDCCH performance is unacceptable unless the gNB can guarantee the use of AL=16 for PDCCH only.

Observation 1. For TX puncturing, 1% PDCCH BLER cannot be met for AL=1/2/4/8 when SNR<0.

In case1-2 RX puncturing, it is assumed that UE has changed its implementation of PDCCH decoding and channel estimation according to the information of CRS pattern. From Figure 3 and Figure 4, Case1-2 with AL=2*X provides better performance than baseline1 or baseline2 with AL=X. The corresponding ratio of AL for different DCI sizes can be derived and the average number of CCEs occupied by a DCI is shown in Table 1.

Observation 2. RX puncturing can provide better performance for PDCCH using AL=2*X than baseline1 or baseline2 for PDCCH using AL=X.

Table 1 Average number of CCEs

| DCI size | Baseline1 | Baseline2 | Case1-1 TX puncturing | Case1-2 RX puncturing |
| --- | --- | --- | --- | --- |
| 41 bits | 3.6232 | 3.4638 | 8.7273 | 4.1818 |
| 60 bits | 3.7910 | 3.6119 | 9.8065 | 4.8750 |

It can be observed that the average CCE of case1-2 is <1.5 times of baseline1 and thus the PDCCH capacity of case1-1 should be larger than baseline1. There are two ways for comparison:

- Option1. The available PDCCH symbol in case1-2 is 2 times of baseline1, therefore the PDCCH capacity of case1-2 is 2*3.6232/4.1818=173.28% and 2*4.8750/3.7910=155.53%

- Option2. The available PDCCH RE in case1-2 is 1.67 times of baseline1, therefore the PDCCH capacity of case1-2 is 1.67*3.6232/4.1818=144.40% and 1.67*4.8750/3.7910=129.61%

Since the CRS REs are not excluded when calculating the average CCE, option1 should be used for comparison.

| Figure 5 Option1 | Figure 6 Option2 |
| --- | --- |

However, the PDCCH capacity of case1-2 is much less than baseline2 due to the impact of PDCCH puncturing.

Observation 3. From the perspective of PDCCH capacity, 2 NR PDCCH symbols overlapped with 1 CRS symbol with RX puncturing improves the PDCCH capacity compared with 1 NR PDCCH symbol.

Observation 4. From the perspective of PDCCH capacity, 2 NR PDCCH symbols overlapped with 1 CRS symbol with RX puncturing decreases the PDCCH capacity compared with 2 NR PDCCH symbols without CRS collision.

One typical scenario in NR is that idle UE and connected UE share CORESET(s) for PDCCH monitoring. To avoid negative effects on the PDCCH decoding performance of idle UE, gNB shall either ensure that any PDCCH candidates in a CORESET shared by idle UE and connected UE are transmitted in a large AL (e.g. AL = 8/16).

Observation 5. To avoid negative effects on the PDCCH decoding performance of idle UE, gNB shall ensure that any PDCCH candidates in a CORESET shared by idle UE and connected UE are transmitted in a large AL (e.g. AL = 8/16).

## Interaction with NR PDCCH DMRS

If NR PDCCH reception on CRS symbol is performed, how to handle NR PDCCH DMRS and the impact of puncturing on DMRS should be investigated. In this section, several options to handle this issue are further evaluated.

- Case A (i.e., case1-2) DMRS is punctured by LTE CRS

- Case B. DMRS is shifted to another RE(e.g, RE#10) to avoid collision

- Case C. new DMRS pattern without collision is applied

- Case D. DMRS is not transmitted on the PDCCH symbol with LTE CRS

Figure 7 Example of DMRS pattern

For case A, case B and case C, extra efforts on channel estimation for irregular PDCCH DMRS patterns or new DMRS patterns are inevitable. Consequently, the UE must implement channel estimations for all possible DMRS patterns, taking into account all possible CRS patterns, which would highly increase the complexity of implementation. In case D, UE may simply use the estimated channel on the 2nd PDCCH symbol for decoding on the 1st PDCCH symbol

Observation 6. Regardless of whether simply performing DMRS puncturing or introducing a new DMRS pattern to handle the collision between DMRS and CRS, extra efforts on channel estimation are inevitable for the UE side. UE must implement channel estimations for all possible DMRS patterns, taking into account all possible CRS patterns, which would highly increase the complexity of implementation.

Figure 8. Performance of caseA/B/C/D

For case A, AL=1 cannot be used due to the impact of PDCCH puncturing and DMRS puncturing. Case B provides worse performance than case A because of the unequally spanned DMRS pattern and less available PDCCH resources. In case C and case D, the gain of more resource allocated to PDCCH compensates for the loss brought by less DMRS RE for channel estimation when AL is small, thus AL=1 can be used to meet the 1% BLER requirement. When the AL becomes larger, the gap in the performance of the four cases decreases.

Observation 7. If PDCCH reception on the CRS symbol is supported and if DMRS is punctured by CRS, AL=1 cannot be used for NR PDCCH due to the performance degradation.

Proposal 1: In case of NR PDCCH reception on the CRS symbol is supported, RAN1 should discuss how to resolve performance loss on channel estimation of NR PDCCH DMRS, taking into account the UE implementation complexity.

# Conclusion

In this contribution, we focus on NR PDCCH reception on CRS symbol, and have the following observations and proposals:

Observation 1. For TX puncturing, 1% PDCCH BLER cannot be met for AL=1/2/4/8 when SNR<0.

Observation 2. RX puncturing can provide better performance for PDCCH using AL=2*X than baseline1 or baseline2 for PDCCH using AL=X.

Observation 3. From the perspective of PDCCH capacity, 2 NR PDCCH symbols overlapped with 1 CRS symbol with RX puncturing improves the PDCCH capacity compared with 1 NR PDCCH symbol.

Observation 4. From the perspective of PDCCH capacity, 2 NR PDCCH symbols overlapped with 1 CRS symbol with RX puncturing decreases the PDCCH capacity compared with 2 NR PDCCH symbols without CRS collision.

Observation 5. To avoid negative effects on the PDCCH decoding performance of idle UE, gNB shall ensure that any PDCCH candidates in a CORESET shared by idle UE and connected UE are transmitted in a large AL (e.g. AL = 8/16).

Observation 6. Regardless of whether simply performing DMRS puncturing or introducing a new DMRS pattern to handle the collision between DMRS and CRS, extra efforts on channel estimation are inevitable for the UE side. UE must implement channel estimations for all possible DMRS patterns, taking into account all possible CRS patterns, which would highly increase the complexity of implementation.

Observation 7. If PDCCH reception on the CRS symbol is supported and if DMRS is punctured by CRS, AL=1 cannot be used for NR PDCCH due to the performance degradation.

Proposal 1: In case of NR PDCCH reception on the CRS symbol is supported, RAN1 should discuss how to resolve performance loss on channel estimation of NR PDCCH DMRS, taking into account the UE implementation complexity.

# Appendix

| LLS simulation assumption |  |
| --- | --- |
| Frequency | 2 GHz, 15 kHz SCS, 20 MHz(106RB) carrier BW |
| Channel model | TDL-C |
| Delay spread | 100ns |
| No of BS antenna | 2TX |
| No of UE antenna | 2RX |
| UE speed | 3km/h |
| DCI payload | 41/60 bits excluding CRC |
| CCE-to-REG mapping | interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |
| Tx Diversity | One port precoder cycling |
| LTE CRS | 4 port |

Table 2. SNR CDF table

| Outage | SNR | Outage | SNR | Outage | SNR | Outage | SNR | Outage | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [%] | [dB] | [%] | [dB] | [%] | [dB] | [%] | [dB] | [%] | [dB] |
| 0 | -42.0009 | 20 | -11.4379 | 40 | -1.8138 | 60 | 7.37009 | 80 | 21.8427 |
| 1 | -31.8849 | 21 | -10.8611 | 41 | -1.30838 | 61 | 7.83212 | 81 | 22.857 |
| 2 | -29.0244 | 22 | -10.309 | 42 | -0.987306 | 62 | 8.52468 | 82 | 23.8359 |
| 3 | -26.7069 | 23 | -9.80395 | 43 | -0.539956 | 63 | 9.01432 | 83 | 25.2484 |
| 4 | -25.1853 | 24 | -9.22335 | 44 | -0.02455 | 64 | 9.64745 | 84 | 26.1092 |
| 5 | -24.1001 | 25 | -8.73453 | 45 | 0.514781 | 65 | 10.3223 | 85 | 27.1961 |
| 6 | -22.4849 | 26 | -8.23876 | 46 | 0.914871 | 66 | 10.8492 | 86 | 28.2274 |
| 7 | -21.4003 | 27 | -7.71898 | 47 | 1.38251 | 67 | 11.4266 | 87 | 29.2534 |
| 8 | -20.0501 | 28 | -7.35025 | 48 | 1.76021 | 68 | 11.8898 | 88 | 30.3302 |
| 9 | -19.2108 | 29 | -7.00459 | 49 | 2.3121 | 69 | 12.6519 | 89 | 31.511 |
| 10 | -18.2579 | 30 | -6.4603 | 50 | 2.6275 | 70 | 13.3364 | 90 | 32.6716 |
| 11 | -17.3824 | 31 | -6.04018 | 51 | 3.04551 | 71 | 14.3154 | 91 | 34.1882 |
| 12 | -16.6241 | 32 | -5.57099 | 52 | 3.65251 | 72 | 15.3801 | 92 | 35.9803 |
| 13 | -15.9984 | 33 | -5.04452 | 53 | 4.09493 | 73 | 16.2055 | 93 | 37.8353 |
| 14 | -15.3292 | 34 | -4.59313 | 54 | 4.49795 | 74 | 16.8765 | 94 | 39.505 |
| 15 | -14.6555 | 35 | -3.9164 | 55 | 4.92715 | 75 | 17.6576 | 95 | 41.4667 |
| 16 | -14.0759 | 36 | -3.53971 | 56 | 5.51118 | 76 | 18.3134 | 96 | 43.9856 |
| 17 | -13.1798 | 37 | -3.10979 | 57 | 5.93663 | 77 | 19.0396 | 97 | 47.1326 |
| 18 | -12.5796 | 38 | -2.58348 | 58 | 6.38761 | 78 | 19.8443 | 98 | 52.0502 |
| 19 | -12.0032 | 39 | -2.15193 | 59 | 6.96155 | 79 | 20.9638 | 99 | 56.7553 |
|  |  |  |  |  |  |  |  | 100 | 75.8838 |