3GPP TSG RAN WG1 #109-e		R1-2203583

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on multi-cell scheduling

Agenda Item:	9.10.1

Document for:	Discussion and Decision

# Introduction

It was agreed to support the following feature:

- Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Identify the maximum number of cells that can be scheduled simultaneously

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In the contribution, we provide our views on scheduling PDSCH or PUSCH on multi-cells by a single DCI. For simplicity, ‘mc-DCI’ and ‘sc-DCI’ are used to represent DCI for multi-cell scheduling and DCI for single-cell scheduling in this contribution respectively.

# Discussion

## Scope

Before delving into the details of this feature, it is suggested to align the understanding of the scope and principles for the discussion first. The following aspects are suggested for consideration:

- A multi-cell scheduling grant is for unicast DL scheduling only, or for unicast UL scheduling only.

There is no clear motivation to support mixed DL/UL scheduling. For simplicity, it is suggested to focus on the DL-only grant and UL-only grant.

- The scheduled cells can be associated with the same/different TDD configurations/Numerologies.

This feature should be applicable for intra-band CA and inter-band CA cases in both FR1/FR2, therefore the scheduled cells may be configured with the same/different TDD configurations and Numerologies.

- Either Pcell or a Scell can be configured as a scheduling cell for joint multi-cell scheduling, but for a scheduled cell, there is only one scheduling cell.

Either a Pcell or a Scell can be configured as a scheduling cell. In Rel-16/Rel-15, there is only one scheduling cell for each serving cell. In Rel-17, some special treatments were introduced for Pcell to offload PDCCH from Pcell, that is, a sScell can be configured to cross-carrier schedule Pcell on top of Pcell self-scheduling, in which case Pcell enjoys two scheduling cells. Consequently, it is unclear whether a Pcell can be scheduled together with other Scell by a mc-DCI transmitted on the sScell when sScell scheduling Pcell is configured.

First, there is no clear use case or motivation for combining mc-DCI with sScell scheduling Pcell. Mc-DCI is primarily used to relieve the PDCCH burden as well as reduce PDCCH blocking rate on the scheduling cell, but if the sScell is suffering some degree of PDCCH blocking, why is it configured to offload PDCCH from Pcell?

Second, greater specification effort can be expected if sScell is allowed to send mc-DCI to schedule Pcell. Specifically, when sScell scheduling the Pcell is configured, not only is the total BD limits for Pcell self-scheduling and sScell-scheduling Pcell bounded by a joint upper limit, but also each of them is bounded by scaling factors α and 1-α, respectively. If mc-DCI is further configured on the sScell to schedule Pcell, PDCCH monitoring/BD budget for multi-cell scheduling would be constrained by the scaling factors, whereas for other scheduling cells with mc-DCI, the corresponding PDCCH monitoring/BD budget is fairly clean and independent because they involve neither the scaling factor nor a joint BD budget shared by multiple scheduling cells. The combination of sScell scheduling Pcell and multi-cell scheduling would ultimately require separate PDCCH monitoring/BD budget solutions for sScell and other scheduling cells respectively. Thus, it is suggested to follow the single-scheduling cell principle.

- The scheduled PDSCHs or PUSCHs correspond to different TBs.

Currently, HARQ entities are maintained per cell. If the scheduled PDSCHs or PUSCHs correspond to the same TB, HARQ entities should be managed jointly for scheduled cells, which inevitably requires significant RAN2 efforts. Since multi-cell scheduling is a RAN1-dominated function and RAN2 has very limited TUs, designs requiring significant RAN2 impact should be avoided.

- Changes or extensions to the legacy PDCCH channel coding/mapping procedure should be avoided.

The size of the mc-DCI can be quite large as the number of increased cells increases. However, the maximum DCI size without CRC supported by polar codes in 38.212 is 140 bits. To support DCI sizes above 140 bits would require at least modification of the interleaving sequence of polar codes, which is highly undesirable. Therefore, RAN1 should ensure that the mc-DCI size without CRC is no larger than 140 bits. In addition, a large AL may be required to ensure PDCCH decoding performance for large DCI payload sizes, however, any extension to the supported AL (e.g., AL >16) should be avoided.

- The number of cells that can be scheduled by a single DCI is no larger than 8 and is configurable.

One of the objectives of the WID is to determine the maximum number of cells that can be scheduled by a mc-DCI. On one hand, the number of scheduled cells is closely related to the deployment (e.g. SCS, FR, TDD configuration, scheduling constraints, etc.); on the other hand, it is not realistic to align the simulation assumptions and determine the maximum number of cells through a fair comparison of simulations results within 3 TUs. A simple approach is to refer to the limit in CCS where a scheduling cell has up to 8 scheduled cells (including itself) – that is, the maximum number of scheduled cells by a mc-DCI is no more than 8. Further, the number of scheduled cells can be up to gNB’s configuration.

Proposal 1. For multi-cell scheduling, the following principles should be taken into account:

- The multi-cell scheduling grant is for unicast DL scheduling only or unicast UL scheduling only.

- The scheduled cells can be associated with the same/different TDD configurations/numerologies.

- Either Pcell or a Scell can be configured as a scheduling cell for joint multi-cell scheduling.

- For a scheduled cell, there is only one scheduling cell.

- The scheduled PDSCHs or PUSCHs correspond to different TBs.

- Changes or extensions to the legacy PDCCH coding/mapping procedure, including the maximum DCI size=140 bits excluding CRC and supported ALs, should be avoided.

- The number of cells that can be scheduled by a single DCI is no larger than 8 and is configurable.

## Potential works

### Framework

First of all, it should be clarified whether multi-cell scheduling should reuse the framework of cross-carrier scheduling. If CCS and multi-cell scheduling are coupled, only UEs supporting cross-carrier scheduling can be configured with multi-cell scheduling, and the linkage between jointly scheduled cell and scheduling cell can be configured and derived from RRC IE CrossCarrierSchedulingConfig. However, this could introduce unnecessary mutual restrictions between single-cell scheduling and multi-cell scheduling, e.g., a cell can only be scheduled by sc-DCI or mc-DCI, but not both.

Alternatively, multi-cell scheduling can be designed as an independent feature from cross-carrier scheduling. Considering that UL-DL traffic patterns across scheduled cells may be time-varying and highly unbalanced, independent multi-cell scheduling and single-cell scheduling operations are more desirable so as to better accommodate traffic loads.

Proposal 2. For a scheduled cell, both multi-cell scheduling and single-cell scheduling can be configured at the same time.

### DCI format

The main benefit of multi-cell scheduling comes from the potential PDCCH overhead reduction when using a mc-DCI compared with using multiple sc-DCIs, so how to achieve a good trade-off between mc-DCI with small size and scheduling flexibility becomes crucial for achieving performance gain.

The DCI size is highly related to the field type (i.e., shared by cells or not) of each field in a mc-DCI. On one hand, not all fields can be shared directly among cells, instead, most of the information fields in the DCI depend on the configuration of the scheduling cells. On the other hand, if each function has a separate indication for each scheduled cell in a mc-DCI, it would be difficult to maintain the DCI size at an acceptable level. According to the study of 2-cell scheduling in R17, there would be no overall performance gain with a large DCI size for inter-band CA.

It is not realistic to discuss field types of the fields one by one due to the limited TU. One way is to leave all the fields to gNB configuration. For example, gNB informs UE whether a field is shared or not by RRC configuration. but it would be difficult for gNB to determine if such configuration could bring gains eventually. Another way is to first have a quick check on companies’ views on the field types, then focus on those fields that are more important or less controversial, while for the ones that are less important or more controversial, the corresponding the field types can be pre-defined or up to gNB configuration.

Proposal 3. Regarding whether a DCI field should be shared among the scheduled cells or split into separate indications:

- Some fields (e.g., CIF/BWP id/identifier DCI) can be shared by the scheduled cells.

- Discussion for some of the fields (e.g., HARQ process/FDRA/TDRA/PUCCH related field) is needed

- Other fields can be up to gNB configuration

Even with the aforementioned guidelines to save the DCI size by sharing some indications between cells, it would still be challenging to maintain an acceptable DCI payload size in some cases, e.g., when the number of scheduled cells is large, or when the resource allocation field consumes dozens of bits for each scheduled cell. Thus, some fields such as FDRA/TDRA with high overheads should be jointly encoded, or further compressed.

For TDRA, a shared indication, such as jointly coded SLIV, can be considered at least for the same SCS cases. For FDRA, the RBG size can be jointly determined based on the total RBs/BW of the multi-scheduled cells. Alternatively, a scaled RBG size can be used if the DCI size would become too large due to that there are a large number of RBs for scheduling. Although the scheduling flexibility would be somehow restricted, the overall spectrum efficiency may increase thanks to the reduced PDCCH blocking rate and the PDCCH reduction gain achieved by mc-DCI with a smaller size. We provide some preliminary simulations on the following schemes for multi-cell scheduling in intra-band CA cases where the BW/SCS of the scheduled cells are the same:

- sc-DCI(baseline): sc-DCI with payload size=60bits (excluding CRC), the RBG size for single-cell scheduling is N

- mc-DCI1: mc-DCI with separate FDRA indications for each scheduled cell, RBG size is N. For 2GHz case, mc-DCI1 payload sizes are 100 and 140 bits for 2-cell scheduling and 3-cell scheduling respectively.

- mc-DCI2: mc-DCI with separate FDRA indication for all scheduled cells, but the RBG size is determined based on the total RBs of the scheduled cells, thus RBG size = 2*N. For 2GHz case, mc-DCI2 payload sizes are 86 and 118 bits for 2-cell scheduling and 3-cell scheduling respectively.

Assumptions about the DCI field type for 2-cell scheduling and 3-cell scheduling of the above schemes are provided in the appendix.

According to the simulation results, PDCCH blocking rate decreases with the mc-DCI. If the number of cells increases, more PDCCH blockage can be reduced by mc-DCI. From the perspective of throughput, it can be seen that around 13~14% throughput gains can be achieved by mc-DCI2 with compressed FDRA indication for 2GHz case. When it comes to 700MHz suffering from more severe PDCCH blocking problems, the gain achieved by mc-DCI2 with the compressed FDRA is increased to 20+%.

Throughput gains can also be seen for mc-DCI1 with RA indicated separately for each cell without compression, but mc-DCI1 requires 140bits for the scheduling of 3 cells. Thus, it is difficult for mc-DCI1 to support more scheduled cells while keeping a DCI size below 140 bits.

| Fig 1. 2GHz, 2-cell scheduling | Fig 2. 2G Hz, 3-cell scheduling |
| --- | --- |
| Fig 3. 700MHz, 2-cell scheduling | Fig 4. 700M Hz, 3-cell scheduling |

Observation 1. It would be difficult to schedule more than 3 cells by a mc-DCI with payload size <=140 bits if RA fields are split into separate indications for scheduled cells without compression or joint coding for intra-band CA.

Observation 2. Mc-DCI with compressed/joint FDRA and other shared fields brings 13~27% throughput compared with sc-DCI for intra-band CA.

Proposal 4. For FDRA in mc-DCI, the FDRA granularity may be scaled or determined considering the BW of all the scheduled cells to reduce DCI size.

Another aspect that needs to be specified is the DCI format for multi-cell scheduling. If an existing DCI format such as DCI format 1-1 is used for multi-cell scheduling purposes, the DCI for single-cell scheduling case should always perform zero-padding to keep alignment with the DCI size of the case where multi-cell scheduling is enabled, which would lead to worse performance for single-cell scheduling. By contrast, introducing a new DCI format for multi-cell scheduling would be more desirable as it can not only ensure a high degree of scheduling flexibility but also avoids negative impacts on the performance of the existing DCI. Nevertheless, in this way, whether/how to maintain the DCI size budget should be considered.

Proposal 5. Introduce new DCI format(s) for mc-DCI.

### PDCCH monitoring

For the multi-cell scheduling, it should be discussed how to handle the PDCCH BD budget across the scheduled cells. If a mc-DCI is received, they are two options:

- Option1. this DCI is still counted in the BD budget for each scheduled cell following the legacy BD framework, which means, it is duplicated counted for N times by the N scheduled cells.

- Option2. it is only counted as part of the BD budget of the scheduling cell.

Option1 reuses the existing BD framework of single-cell scheduling, but it has the disadvantage that the opportunities for single-cell scheduling on all scheduled cells would be decreased due to a single mc-DCI as the mc-DCI consumes BDs in all the scheduled cells. Option2. is similar to the way of handling type3 CCS on a scheduling cell, thus saving more BDs than option1.

Proposal 6. The mc-DCI should be counted as part of the BD budget of the scheduling cell instead of the BD budget of each scheduled cell.

### HARQ-ACK codebook

In Rel-16/17 cross-carrier scheduling, in case the UE is configured with two PUCCH groups, the scheduling cell and the scheduled cell are within the same PUCCH group. For the multi-cell scheduling, it should be clarified whether the scheduling cell and the scheduled cells are within the same PUCCH group or not. As we discussed in section 2.2.1, if multi-cell scheduling relies on RRC IE CrossCarrierSchedulingConfig, it naturally follows the same rule in cross-carrier scheduling. Alternatively, multi-cell scheduling is designed as an independent feature of cross-carrier scheduling, it is suggested that the Rel-16/17 principle is reused, otherwise, the complexity of HARQ-ACK generation and PUCCH resource determination would increase.

Proposal 7: For multi-cell scheduling, the scheduling cell and scheduled cells should belong to the same PUCCH cell group.

In Rel-17, the K1 set configured by RRC IE PUCCH-config is extended according to the TDRA Table configured by RRC IE pdsch-TimeDomainAllocationListForMultiPDSCH-r17 during type 1 HARQ-ACK codebook generation procedure when multi-PDSCH scheduling is configured.

For multi-cell scheduling, the type 1 HARQ-ACK codebook generation may also need to extend K1 set depending on TDRA indication. For example, the TDRAs of the multiple PDSCHs in the multiple cells are jointly coded. However, in this case, the K1 extension method in Rel-17 is not suitable because type 1 HARQ-ACK codebook is per serving cell-independent. In addition, when different SCS are applied to the multiple scheduled cells, the type 1 codebook determination needs further modifications.

Proposal 8. For type 1 HARQ-ACK codebook, an extension depending on the TDRA indication for multi-cell scheduling to the K1 set should be considered. Moreover, further enhancement on top of the legacy K1 set extension may be needed.

For legacy type 2 HARQ-ACK codebook, there are at most two HARQ-ACK sub-codebooks. The second HARQ-ACK sub-codebook contains either the HARQ-ACK for CBG-based PDSCH or HARQ-ACK for multi-PDSCH scheduling in one cell, as CBG-based PDSCH and multi-PDSCH cannot be configured simultaneously in a PUCCH cell group. For simplicity, the multi-cell scheduling should not be configured with CBG-based or single-cell multi-PDSCH scheduling simultaneously. Then, the type 2 HARQ-ACK for multi-cell scheduling can be contained in the second sub-codebook. Furthermore, the counter DAI (C-DAI) value and the total DAI(T-DAI) value apply separately for each HARQ-ACK sub-codebook, so that for the type 2 HARQ-ACK codebook, the C-DAI/T-DAI can also be counted per mc-DCI.

Proposal 9. For type 2 HARQ-ACK codebook, the multi-cell scheduling is not expected to be configured with CBG-based or multi-PDSCH scheduling simultaneously for any serving cell within a same PUCCH cell group.

Proposal 10. For the type 2 HARQ-ACK codebook, HARQ-ACK bits corresponding to mc-DCI should be contained in a separate sub-codebook apart from the sub-codebook for sc-DCI.

Proposal 11. For the type 2 HARQ-ACK sub-codebook, the C-DAI/T-DAI can be counted per mc-DCI.

# Conclusion

In this contribution, we discuss some issues on multi-cell scheduling and have the following observations and proposals:

Observation 1. It would be difficult to schedule more than 3 cells by a mc-DCI with payload size <=140 bits if RA fields are split into separate indications for scheduled cells without compression or joint coding for intra-band CA.

Observation 2. Mc-DCI with compressed/joint FDRA and other shared fields brings 13~27% throughput compared with sc-DCI for intra-band CA.

Proposal 1. For multi-cell scheduling, the following principles should be taken into account:

- The multi-cell scheduling grant is for unicast DL scheduling only or unicast UL scheduling only.

- The scheduled cells can be associated with the same/different TDD configurations/numerologies.

- Either Pcell or a Scell can be configured as a scheduling cell for joint multi-cell scheduling.

- For a scheduled cell, there is only one scheduling cell.

- The scheduled PDSCHs or PUSCHs correspond to different TBs.

- Changes or extensions to the legacy PDCCH coding/mapping procedure, including the maximum DCI size=140 bits excluding CRC and supported ALs, should be avoided.

- The number of cells that can be scheduled by a single DCI is no larger than 8 and is configurable.

Proposal 2. For a scheduled cell, both multi-cell scheduling and single-cell scheduling can be configured at the same time.

Proposal 3. Regarding whether a DCI field should be shared among the scheduled cells or split into separate indications:

- Some fields (e.g., CIF/BWP id/identifier DCI) can be shared by the scheduled cells.

- Discussion for some of the fields (e.g., HARQ process/FDRA/TDRA/PUCCH related field) is needed

- Other fields can be up to gNB configuration

Proposal 4. For FDRA in mc-DCI, the FDRA granularity may be scaled or determined considering the BW of all the scheduled cells to reduce DCI size.

Proposal 5. Introduce new DCI format(s) for mc-DCI.

Proposal 6. The mc-DCI should be counted as part of the BD budget of the scheduling cell instead of the BD budget of each scheduled cell.

Proposal 7: For multi-cell scheduling, the scheduling cell and scheduled cells should belong to the same PUCCH cell group.

Proposal 8. For type 1 HARQ-ACK codebook, an extension depending on the TDRA indication for multi-cell scheduling to the K1 set should be considered. Moreover, further enhancement on top of the legacy K1 set extension may be needed.

Proposal 9. For type 2 HARQ-ACK codebook, the multi-cell scheduling is not expected to be configured with CBG-based or multi-PDSCH scheduling simultaneously for any serving cell within a same PUCCH cell group.

Proposal 10. For the type 2 HARQ-ACK codebook, HARQ-ACK bits corresponding to mc-DCI should be contained in a separate sub-codebook apart from the sub-codebook for sc-DCI.

Proposal 11. For the type 2 HARQ-ACK sub-codebook, the C-DAI/T-DAI can be counted per mc-DCI.

# Appendix

Table 1. Assumption of mc-DCI @ 2GHz

| Sc-DCI | Field type | 2-cell DCI1 | 2-cell DCI2 | 3-cell DCI1 | 3-cell DCI2 |
| --- | --- | --- | --- | --- | --- |
| Identifier for DCI formats – 1 bit | Shared field | 1 | 1 | 1 | 1 |
| Carrier indicator –3 bits | Shared field | 3 | 3 | 3 | 3 |
| Bandwidth part indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| FDRA – 14 bits for a cell | Separate field | 28 | 14 | 42 | 20 |
| TDRA –2 bits | Separate field | 4 | 4 | 6 | 6 |
| VRB-to-PRB mapping – 1 bit | Shared field | 1 | 1 | 1 | 1 |
| PRB bundling size indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| Rate matching indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| ZP CSI-RS trigger –1 bit | Separate field | 2 | 2 | 3 | 3 |
| Modulation and coding scheme – 5 bits | Separate field | 10 | 10 | 15 | 15 |
| New data indicator – 1 bit | Separate field | 2 | 2 | 3 | 3 |
| Redundancy version – 2 bits | Separate field | 4 | 4 | 6 | 6 |
| HARQ process number – 4 bits | Separate field | 8 | 8 | 12 | 12 |
| Downlink assignment index – 4 bits | Shared field | 4 | 4 | 4 | 4 |
| TPC command for scheduled PUCCH – 2 bits | Shared field | 2 | 2 | 2 | 2 |
| PUCCH resource indicator – 3 bits | Shared field | 3 | 3 | 3 | 3 |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field | 3 | 3 | 3 | 3 |
| Antenna port(s) –5 bits | Separate field | 10 | 10 | 15 | 15 |
| Transmission configuration indication 3 bits | Shared field | 3 | 3 | 3 | 3 |
| SRS request – 2 bits | Separate field | 4 | 4 | 6 | 6 |
| DMRS sequence initialization – 1 bit. | Separate field | 2 | 2 | 3 | 3 |
| Total size (excluding CRC) |  | 100 | 86 | 140 | 118 |

Table 2. Assumption of mc-DCI @ 700MHz

| Sc-DCI | Field type | 2-cell DCI1 | 2-cell DCI2 | 3-cell DCI1 | 3-cell DCI2 |
| --- | --- | --- | --- | --- | --- |
| Identifier for DCI formats – 1 bit | Shared field | 1 | 1 | 1 | 1 |
| Carrier indicator –3 bits | Shared field | 3 | 3 | 3 | 3 |
| Bandwidth part indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| FDRA – 13 bits for a cell | Separate field | 26 | 13 | 39 | 20 |
| TDRA –2 bits | Separate field | 4 | 4 | 6 | 6 |
| VRB-to-PRB mapping – 1 bit | Shared field | 1 | 1 | 1 | 1 |
| PRB bundling size indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| Rate matching indicator –1 bit | Separate field | 2 | 2 | 3 | 3 |
| ZP CSI-RS trigger –1 bit | Separate field | 2 | 2 | 3 | 3 |
| Modulation and coding scheme – 5 bits | Separate field | 10 | 10 | 15 | 15 |
| New data indicator – 1 bit | Separate field | 2 | 2 | 3 | 3 |
| Redundancy version – 2 bits | Separate field | 4 | 4 | 6 | 6 |
| HARQ process number – 4 bits | Separate field | 8 | 8 | 12 | 12 |
| Downlink assignment index – 4 bits | Shared field | 4 | 4 | 4 | 4 |
| TPC command for scheduled PUCCH – 2 bits | Shared field | 2 | 2 | 2 | 2 |
| PUCCH resource indicator – 3 bits | Shared field | 3 | 3 | 3 | 3 |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field | 3 | 3 | 3 | 3 |
| Antenna port(s) –5 bits | Separate field | 10 | 10 | 15 | 15 |
| Transmission configuration indication 3 bits | Shared field | 3 | 3 | 3 | 3 |
| SRS request – 2 bits | Separate field | 4 | 4 | 6 | 6 |
| DMRS sequence initialization – 1 bit. | Separate field | 2 | 2 | 3 | 3 |
| Total size (excluding CRC) |  | 98 | 85 | 137 | 118 |

# Simulation assumption

Table 3 SLS Simulation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier frequency | Configuration1: 2 GHz, 15 kHz SCS,20 MHz(106RB/1272 SC) carrier BW, 2-symbol CORESET with 96RBs/1152 SC Configuration2: 700MHz, 15 kHz SCS, 10 MHz(52RB/624 SC) carrier BW, 3-symbol CORESET with 48RBs/576 SC |
| SCS |  |
| Simulation bandwidth |  |
| CA | intra-band CA |
| Number of cells | 2 or 3 cell |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds of interest | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Number of UEs per cell | 15UEs |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |