3GPP TSG RAN WG1 #109-e	                                                                                R1-2203585

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on XR specific power saving enhancements

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In RAN plenary #94e meeting, the WID for R18 XR enhancement was approved [1], and the objectives as follows:

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution, we will discuss the potential techniques for XR specific power saving enhancements, followed by our initial evaluation results.

- Challenges

- XR traffic characteristics

In R17 XR SI, we have already done deep survey on the XR traffic characteristics, which are listed as follows for review.

- Non-integer periodicity (e.g., 33.33, 16.67, 11.11, 8.33ms)

- Jitter (i.e., jitter range for each frame can be from -4ms to +4ms) as shown in Figure 1

- Low latency (e.g., PDB=10ms for DL VR/AR video flow)

- High reliability (e.g., 10e-3, 10e-4)

- High data rate (e.g., 30Mbps, 45Mbps)

- Variable packet size

- Multiple flows

Figure 1 Example of non-integer periodicity and jitter distribution

On account of all of above characteristics, XR services generally can be equivalent to combining all the requirements of eMBB and URLLC services. In result, XR services will bring greater challenges on UE power consumption. Moreover, among these characteristics, for power saving purpose, non-integer traffic periodicity and jitter will be the most critical two, which is the biggest difference compared to the other traffic models discussed in 3GPP. In the following, we will focus on these two XR characteristics to drive power saving enhancements.

- Analysis of the existing power saving schemes

First of all, according to the normative work for R15/16 power saving schemes in RRC connection mode, many power saving technologies have been widely investigated, e.g., DRX mechanism, wake up indication based on DCI format 2-6 (including dormancy mechanism which is specified in Rel-16 MR-DC WI), cross-slot scheduling, MIMO layer adaptation and RRM relaxation. Going through the existing power saving techniques, all of them are designed in accordance with traditional traffic models such as FTP 3, instant messaging and VoIP, without taking XR traffic into consideration. Regarding DRX mechanism, when UE steps into the DRX off state, it will suspend for PDCCH monitoring and can go to sleeping power state to reduce power consumption, but at the same time it will incur additional scheduling latency. The increased latency has a marginal negative impact on the eMBB traffic, however, for delay-sensitive services, the impact on packet loss due to exceed delay bound will be intolerable. On the other hand, power saving will be sacrificed to make sure no latency increase, which obeys the intention of power saving. Therefore, more suitable DRX enhancements tailored to the XR are needed. In addition, due to short XR traffic periodicity, if near 90% of DRX cycles have data transmission, DCI format 2_6 based wake-up indication will become meaningless for power saving toward XR traffic. Other schemes such as MIMO layer adaption and dormancy mechanismcan be regarded as adaptation in frequency domain (rather than adaptation in time domain) and can be applied naturally no matter what the traffic model is. Thus, for adaptation in time domain, the current R15/16 DRX schemes are not fully suitable for XR traffic.

Besides, R17 PDCCH monitoring adaptation scheme, i.e., PDCCH skipping indication and/or search space set group switching, has been already agreed in the on-going R17 power saving WI for RRC connected mode. Although R17 PDCCH monitoring adaptation based on L1 signalling is flexible and prompt enough for UE power saving, without comprehensively considering XR traffic, it could be doubtful to be completely applicable to XR traffic. As XR traffic periodicity is much shorter than that for typical eMBB traffic, signalling resource overhead and PDCCH blocking rate caused by R17 PDCCH monitoring adaptation scheme would not be neglectable. Moreover, whether R17 power saving techniques are able to make a better trade-off between capacity and power saving is questionable due to the unpredicted packets jitter and non-integer periodicity.

In summary, as per TR 38.838 [2], there are at least two issues related to XR specific power saving enhancements which have been pointed out unanimously by most companies. One is the mismatch between DRX cycle and traffic periodicity due to non-integer periodicity. The other critical issue is the unpredicted traffic burst arrival time caused by jitter.

- C-DRX enhancements

  - Enhanced C-DRX scheme to align with non-integer XR traffic periodicity

Considering DRX is a very basic configuration in the current network deployment, the study on enhancing DRX mechanism to address XR specific issues should be prioritized. The values of R15/16 DRX cycle are all integers, e.g., long DRX cycle values: 10, 20, 32, 40ms etc. and short cycle values: 2, 3, 5, 6, 7, 8, 10, 14, 16, 20, 30, 32, 35ms etc. However, in the RAN1 #104e meeting, it was agreed that the FPS of DL video stream for a single UE can be 30, 60 (baseline), 90, 120 FPS corresponding to 33.33, 16.67, 11.11, 8.33ms, respetively. It can be found that the current DRX cycle values cannot match with the non-integer XR traffic periodicity with time shift and this mismatch will either lead to unacceptable transmission delay as shown in Figure 2, and also unnecessary power consumption (causing by configuring with long DRX timers). Note that the purpose of aligning DRX cycle with traffic periodicity is aimed at letting DRX onduration cover the XR traffic burst arrival time.

Observation 1: The R15/16 DRX cycle values cannot match with the non-integer XR traffic arrival interval and this mismatch may change over time.

Figure 2 Illustration of the mismatch isse caused by non-integer traffic periodicity

To align DRX cycle with non-integer traffic periodicity, a straightforward way is to extend the candidate values of DRX cycles to include non-integer value(s). However, when non-integer DRX cycle(s) would be introduced, new issues may be raised, e.g. due to different SCS configurations, resulting in more complexity for MAC implementation. Hence, to align DRX ondurations with traffic arrival as mentioned above, some potential solutions are provided in the following, which can be divided into two categories based on signalling of solutions. It should be noted that the following DRX enhancements need to be studied in conjunction with RAN2.

Figure 3 DRX enhancement by align DRX cycle with non-integer traffic periodicity

- Higher layer based DRX enhancement

It is much straightforward to enhance DRX based on higher layer configuration, especially considering there is a fixed XR traffic pattern with the time shift.

According to the XR traffic frame rate, we can convert the unit of FPS into the unit of millisecond. Taking the typical 60FPS as an example, by simplifying the numerator and denominator to interprime, there appears an integer value which is multiple of traffic periodicity, i.e., 50ms. During the 50ms, three traffic bursts (e.g., frame) will arrive.

60 frames1000 ms=3 frames50 ms for 60FPS

Leveraging the integer value, we can set a integer DRX cycle length, i.e., 50ms and then provide three DRX ondurations respectively to match every three frames’ arrival. However, as per the R15/16 DRX mechanism, there is only one DRX onduration within one DRX cycle, so it is not allowed to configure more than one DRX ondurations within one DRX cycle. One way to avoid breaking this rule is to configure three DRX configurations, as depicted in Figure 4 of approach 1. For 60FPS(e.g., with periodicity of 16.67ms) XR traffic, three DRX configurations can be configured by the gNB. And all DRX parameters of them are the same except for the drx-startoffset which decides the start time of DRX onduration, for the purpose of matching every three frames’ arrival time. Note that these DRX cycles have the same length, i.e., 50ms. In Figure 4 of approach 2, another approach is to re-define that there can be more than one DRX ondurations within one DRX cycle so as to match every three frames’ arrival. Actually, these two approaches can both deal with the mismatch issue by aligning the DRX cycle with traffic periodicity.

Figure 4 Two DRX configuration approaches to solve the mismatch issue

Observation 2: It is beneficial to introduce higher layer based DRX enhancement to align DRX cycle with XR traffic periodicity with the following approaches:

- Approach 1: multiple DRX configurations with different drx-StartOffset values

- Approach 2: single DRX configuration with multiple DRX ondurations

- DCI based DRX enhancement

In addition to the higher layer indication, another more flexible solution is DCI based DRX enhancement, i.e., DCI-based DRX start-offset indication. Its working principle is that if the network predicts that the DRX onduration of the next DRX cycle can no longer cover the arriving frame, it could indicate a reasonable new DRX start-offset so as to the next long and short DRX cycle can align with the following traffic burst. Therefore, the network should guarantee pre-sending the indication to UE in advance. Also, UE will calculate the remaining sleep length according to the updated DRX start-offset. As such, the mismatch issue aforementioned will be readily solved. However, the potential drawback of DCI based solution is the signalling overhead. When we adopt DCI to bear the DRX start-offset indication, the network needs to send the DCI frequently.

Despite DCI-based DRX start-offset indication will increase the network overhead, in some cases, for example, at the beginning stage of the XR traffic transmission, the network may not be aware of the ideal traffic arrival time, i.e., the position of jitter=0 for each traffic burst for the reason of unpredicted jitter. Hence, it is better to do some quick adjustment according to DCI-based DRX start-offset indication. Additionally, by jointly considering the mismatch and jitter issues, it will be more appropriate to adopt DCI to indicate a proper DRX start-offset of the next DRX onduration for UE, under the condition that the network can predict the jitter based on statistic. On this account, DCI based DRX enhancement is more flexible and efficient to deal with both the mismatch and jitter issues.

Observation 3: It is beneficial to introduce DCI based DRX enhancement (i.e., adjusting drx-startOffset by a DCI) to align DRX cycle with XR traffic periodicity.

- Initial evaluation for R15/16 DRX and enhanced DRX schemes

As we analyzed in above, no matter which concrete DRX enhancement approach is adopted, the same power saving gain will be obtained. And based on the evaluation assumptions which are detailly provided in Appendix, some partial simulation results are extracted here for intuitive comparison. As shown in Figure 5, compared to R15/16 DRX scheme, enhanced DRX scheme can achieve an additional {19.78%~24.71%} power saving gain which is drived from alignment between DRX cycle and non-integer XR traffic periodicity.

Figure 5 Power saving gain of R15/16 DRX and Enhanced DRX

Observation 4: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R15/16 DRX power saving scheme, enhanced DRX scheme can achieve additional {19.78%~24.71%} power saving gain.

Proposal 1: Study DRX enhancements for XR traffic in conjunction with RAN2.

  - Handling of CG transmission outside C-DRX active time

In accordance with the existing power saving mechanisms, a CG transmission will always be performed regardless of whether it falls outside DRX active time or falls into PDCCH skipping duration. If the CG transmission can still be performed outside DRX active time, then there will be less time for the UE to go to sleep for power saving. Therefore, it may be beneficial to skip the CG transmission  when it falls outside DRX active time, as well as falls into PDCCH skipping duration, to achieved more power saving gain.

Proposal 2: Study skipping a CG transmission if it falls outside DRX active time or falls into PDCCH skipping duration for XR traffic.

  - UL slot handling for C-DRX timers

For DRX configuration in LTE  [3], the following DRX related timers are defined as below:

drx-InactivityTimer: Except for NB-IoT UEs, BL UEs or UEs in enhanced coverage, it specifies the number of consecutive PDCCH-subframe(s) after the subframe in which a PDCCH indicates an initial UL, DL or SL user data transmission for this MAC entity. For NB-IoT UEs, it specifies the number of consecutive PDCCH-subframe(s) after the subframe in which the HARQ RTT timer or UL HARQ RTT timer expires. For BL UEs or UEs in enhanced coverage, it specifies the number of consecutive PDCCH-subframe(s) following the subframe containing the last repetition of the PDCCH reception that indicates an initial UL or DL user data transmission for this MAC entity. drx-RetransmissionTimer: Specifies the maximum number of consecutive PDCCH-subframe(s) until a DL retransmission is received. drx-ULRetransmissionTimer: Specifies the maximum number of consecutive PDCCH-subframe(s) until a grant for UL retransmission or the HARQ feedback is received. onDurationTimer: Specifies the number of consecutive PDCCH-subframe(s) at the beginning of a DRX Cycle.

However, in NR, these DRX timers are defined by the duration with the unit of ms or slots in [4]. With the unit of ms or slot, the  timers are controlled by absolute time duration, without considering TDD pattern configuration. There may be no issue for non-delay-sensitive traffic, where only long DRX cycle is actually configured. But for XR service, which has requirements of stringent latency and high data rate, much short DRX timers need to be configured for seeking the balance between power saving and capacity.

Taking drx-onDurationTimer as the example, in order to achieve power saving gain, shorter drx-onDurationTimer will be configured. In case based on the TDD pattern configuration several slots within a periodicity are configured as UL slots, there may be the case that the duration determined by a running DRX timer may partially or fully fall into UL slot(s). Hence, UE will have less or even no opportunity to monitor PDCCH during the duration actually while the timer is running. In this way, the latency requirement for the traffic may not be guaranteed. On the contrary, if longer drx-onDurationTimer can be configured to allow UE have enough PDCCH monitoring occasions, it would result in increased power consumption.

Take TDD frame structure DDDUU and SCS=30KHz as an example, we assume the DRX cycle is 16ms and drx-onDurationTimer is configured as 2ms as shown in Figure 6. Following the current NR spec for DRX timers, the decrement of DRX timer will regardless of DL or UL slot.When a drx-onDurationTimer runs, some of the slots during the drx-onDurationTimer are overlapping with the UL slots. Therefore, the UE have less PDSCH scheduling opportunities, which will lead to larger scheduling delay.

To address this problem, we need to enhance specification so that DRX timers do not decrement during UL slots.

Figure 6 DRX configuration in TDD deployment

Proposal 3: Study enhanced DRX scheme to prevent DRX timers decrementing over UL slots.

- PDCCH monitoring enhancement

  - PDCCH monitoring adaptation

Since PDCCH skipping indication and/or search space set group switching has already been supported in R17, it is straightforward to study how to address non-integer traffic periodicity and jitter issues for XR traffic by starting from the existing PDCCH monitoring adaptation mechanisms.

Firstly, without considering jitter, R17 PDCCH skipping indication can be used to indicate PDCCH monitoring skipping considering the non-integer traffic periodicity. The network can dynamically indicate a PDCCH skipping duration to match the traffic arrival occasions, from up to three RRC configured candidates of skipping duration.  However, due to dynamic change of the interval between the end time of a previous frame transmission and the arrival time of the next frame, it may be difficult to indicate an accurate skipping duration by one PDCCH.

For example, as illustrated in Figure 7 (a), we assume the selected three skipping durations are 6ms, 4ms and 2ms. When finishing the previous frame transmission, gNB determines the time gap to the target position, e.g., the arrival time of the next frame, is greater than 10 ms. Then gNB indicates the a skipping duration with 6ms to UE such that UE can enter light sleep once receciving the PDCCH skipping indication. After that, since the remaining time gap is still more than 3ms, gNB can send another PDCCH skipping duration indication with 2ms to UE. However, UE can only enter micro sleep mode after receiving the second PDCCH skipping indication. For the UE, because of the discontinuity of sleep, the segmented PDCCH skipping duration will cause some waste of power consumption.

Hence, enhancement for the indication of PDCCH skipping can be further studied, so that UE can skip PDCCH monitoring until the target position by one indication. An example is shown in Figure 7 (b). After the finishing of transmission, the network will indicate UE to the target position , which can realize one shot skipping.

Figure 7 PDCCH skipping method with different skipping duration configuration.

Figure 8 SSSG switching and PDCCH skipping methods.

Secondly, regarding jitter, the exact jitter is difficult to predict. It would be very helpful for RAN to obtain some statistics information about jitter, e.g. jitter range from CN, which will be discussed in SA2 and RAN2. Based on this assumption, enhancement for PDCCH monitoring considering the jitter range can be further discussed.

Leveraging R17 search space set group (SSSG) switching indication, as depicted in Figure 8 (a) and Figure 8 (b), before a reference time, i.e., the -jitter boundary of the next traffic burst, e.g., frame, UE switch to the default SSSG with sparse PDCCH monitoring periodicity via the expiry of searchSpaceSwitchTimer-r17 or another explicitly indication. Once the UE detect the scheduling PDCCH, it can accordingly switch to a dense SSSG. And with the successful transmission of the last packet of the frame, a PDCCH skipping indication will be sent to UE to skip the remaining PDCCH monitoring until the -jitter boundary of the next frame.

Observation 5: SSSG switching mechnasim can be used for jitter handling.

Besides, similar as the discussion in section 3, suspending CG transmission during PDCCH skipping duration can also be considered. Flexible DG based UL and DL transmission in one slot would be also benefit from UE power saving perspective. However, how it is implemented and whether it has any specification impact need to be further studied.

Initial evaluation for PDCCH monitoring adaption related schemes

As analyzed in section 4.1, in Figure 9, we give two comparisons based on the evaluation assumption detailly provided in Appendix:

- Without jitter handling, PDCCH skipping indication scheme (configured with three skipping indications) vs. enhanced PDCCH skipping indication (configured with additional skipping indication which can achieve one-shot skipping to the -jitter boundary of the next frame).

- With jitter handling, PDCCH skipping and SSSG swiching indication scheme vs. enhanced PDCCH skipping and SSSG indication scheme.

Compared to PDCCH skipping scheme, enhanced PDCCH skipping scheme can achieve additional {4.83%~7.15%} power saving gain by reducing unnecessary PDCCH monitoring due to one-shot skipping instead of segmented skippings. Similarly, compared to PDCCH skipping and SSSG switching scheme, enhanced PDCCH skipping and SSSG switching scheme can also achieve additional {6.31%~8.14%} power saving gain. Note that for PDCCH skipping and SSSG switching scheme, there can be only two skipping durations which will have a negative impact on power saving gain.

Figure 9 Power saving gain of PDCCH skipping

Observation 6: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping scheme, enhanced PDCCH skipping scheme can achieve an additional {4.83%~7.15%} power saving gain.

Observation 7: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping with SSSG switching scheme, enhanced PDCCH skipping with SSSG switching scheme can achieve an additional {6.31%~8.14%} power saving gain.

Proposal 4: Study XR specific PDCCH skipping enhancement, e.g. extention of skipping duration.

  - Low power-wake up signal-based jitter handling

To be more efficient for jitter handling, as given in Figure 10, low power wake-up signal (WUS) monitoring can be considered, instead of monitoring PDCCH with certain SSSG. Here, the total power consumption of LP-WUS monitoring will be much lower than that of PDCCH monitoring. The working principle can be that: firstly, LP-WUS will be monitored by UE at the beginning of the DRX onduration. And the UE switches to dense PDCCH monitoring after the LP-WUS is detected. Note that the LP-WUS can be receiverd via another receiver, e.g., almost zero power receiver.

For LP-WUS based jitter handling, some open issues should be further studied, including the feasibility of applying LP-WUS in RRC connected mode and the impact on transmission delay considering the transition from receiving LP-WUS to be ready for PDCCH monitoring. For example, by enabling main radio to be always ‘standby’ (i.e., in a micro/light sleep state) and to be waked up by the LP-WUS radio, as such, the delay caused by LP-WUS will be acceptable.

Figure 10 Jitter handling method based on LP-WUS

Proposal 5: Study LP-WUS for PDCCH monitoring reduction.

  - The leftover of R17 power saving WI

In R17 power saving WI, due to time limitation, the following power saving enhancements are not supported, such as

- Non-scheduling DCI based PDCCH monitoring adaptation

- Single DCI indicates monitoring adaptation for multiple cells

- PDCCH monitoring adaptation applying for Type 0-2 CSS sets (depending on the outcome of R17 Power Saving WI)

- PDCCH skipping and interaction with HARQ retransmission (depending on the outcome of R17 Power Saving WI)

These leftover enhancements are also beneficial to XR traffic, we can re-consider them in R18 XR.

Proposal 6: The following leftover enhancements of R17 power saving WI can also be further studied for XR traffic, if needed:

- Non-scheduling DCI based PDCCH monitoring adaptation

- Single DCI indicates monitoring adaptation for multiple cells

- PDCCH monitoring adaptation applying for Type 0-2 CSS sets (depending on the outcome of R17 Power Saving WI)

- PDCCH skipping and interaction with HARQ retransmission (depending on the outcome of R17 Power Saving WI)

- Conclusion

In this contribution, we provide our views on XR specific power saving enhancements with the following observations and proposals:

Observation 1: The R15/16 DRX cycle values cannot match with the non-integer XR traffic arrival interval and this mismatch may change over time.

Observation 2: It is beneficial to introduce higher layer based DRX enhancement to align DRX cycle with XR traffic periodicity with the following approaches:

- Approach 1: multiple DRX configurations with different drx-StartOffset values

- Approach 2: single DRX configuration with multiple DRX ondurations

Observation 3: It is beneficial to introduce DCI based DRX enhancement (i.e., adjusting drx-startOffset by a DCI) to align DRX cycle with XR traffic periodicity.

Observation 4: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R15/16 DRX power saving scheme, enhanced DRX scheme can achieve additional {19.78%~24.71%} power saving gain.

Observation 5: SSSG switching mechnasim can be used for jitter handling.

Observation 6: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping scheme, enhanced PDCCH skipping scheme can achieve an additional {4.83%~7.15%} power saving gain.

Observation 7: For DL XR traffic for FR1 in Indoor Hotspot scenario, compared to R17 PDCCH skipping with SSSG switching scheme, enhanced PDCCH skipping with SSSG switching scheme can achieve an additional {6.31%~8.14%} power saving gain.

Proposal 1: Study DRX enhancements for XR traffic in conjunction with RAN2.

Proposal 2: Study skipping a CG transmission if it falls outside DRX active time or falls into PDCCH skipping duration for XR traffic.

Proposal 3: Study enhanced DRX scheme to prevent DRX timers decrementing over UL slots.

Proposal 4: Study XR specific PDCCH skipping enhancement, e.g. extention of skipping duration.

Proposal 5: Study LP-WUS for PDCCH monitoring reduction.

Proposal 6: The following leftover enhancements of R17 power saving WI can also be further studied for XR traffic, if needed:

- Non-scheduling DCI based PDCCH monitoring adaptation

- Single DCI indicates monitoring adaptation for multiple cells

- PDCCH monitoring adaptation applying for Type 0-2 CSS sets (depending on the outcome of R17 Power Saving WI)

- PDCCH skipping and interaction with HARQ retransmission (depending on the outcome of R17 Power Saving WI)

References

- RP-213587, Study on XR Enhancements for NR, RAN #94e, Dec. 6 - 17, 2021.

- 3GPP TR 38.838 V17.0.0 (2021-12), Study on XR (Extended Reality) Evaluations for NR (Release 17)

- TS 36.321, “Medium Access Control (MAC) protocol specification”, V17.0.0 (2022-03).

- TS 38.321, “Medium Access Control (MAC) protocol specification”, V17.0.0 (2022-03).

Appendix A – Initial evaluation results for power saving schemes

Based on the power saving schemes analyized above, we further provide our initial evaluation for some of them. And The system-level simulation results are presented to analyze the power saving gains achieved by adopting power saving schemes while ensuring the system capacity. For the evaluation of the power saving echanism, the power saving gain and the system capability should be jointly taken into consideration, and the detailed evaluation methodology and traffic model are defined in TR38.838 [2]. The DL single-stream video of VR/AR application with 60FPS, 30Mbps/45Mbps is evaluated for the FR1 indoor hotspot scenario. The overall simulation assumptions and traffic models used can be found in Appendix B.

AlwaysOn (i.e., without adopting any power saving schemes) is assumed as the baseline scheme. Besides, the R15/16 DRX configuration (i.e., R15/16 DRX) is also evaluated for reference. Due to non-integer XR traffic periodicity, to guarantee system capacity performance, the extremely long length of drx-onDurationTimer should be configured. As we mentioned before, to solve the impact of jitter and align DRX cycle with traffic arrival, the enhanced DRX scheme is evaluated as well. For enhanced DRX scheme, we assume the start time of DRX onduration is align to the ideal arrival time of traffic burst (e.g., frame). Table 1 provides the sets of DRX related configuration for both R15/16 DRX and enhanced DRX scheme.

Table 1. The configuration for R15/16 DRX and enhanced DRX schemes

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 DRX | 16 | 14 | 4 |
| enhanced DRX aligning to ideal arrival time of frame | 16 | 4 | 4 |

Regarding PDCCH monitoring enhancement, the R17 PDCCH skipping indication by using scheduling DCI to indicate a maximum of three PDCCH skipping durations is evaluated for reference. Besides, since these candidate skipping durations can not realize to skip to the accurate position by one shot. On this account, enhanced-R17 PDCCH skipping indication, i.e., configuring with additional PDCCH skipping duration value to make UE can skip to target position (i.e., the -jitter boundary of the next packet) need to be evaluated. Moreover, to further reduce unnecessary PDCCH monitoring, we simulate the R17 PDCCH skipping and SSSG switching scheme. Note that in R17 PDCCH skipping and SSSG switching scheme, only two skipping durations are supported. And different PDCCH monitoring periodicitys for the sparse SSSG are configured in our system-level simulation.

For convenience, all power saving schemes evaluated in this contribution are summarized as below.

Case 0：baseline

- AlwaysOn (Baseline): without adopting any power saving mechanism.

Case 1：DRX related

- R15/16 DRX: adopting R15/R16 DRX configuration.

- Enhanced DRX scheme: aligning the start time of DRX ON Duration to a target position (e.g., the ideal arrival time of frame, i.e., the position of jitter=0).

Case 2：PDCCH monitoring adaptation related

- R17 PDCCH skipping: after finishing the transmission, the network will indicate UE to skip the remaining PDCCH monitoring to a position by one or more  PDCCH skipping indication. Repeat the above actions until skip to the nearest -jitter boundry of next frame. Three candidate PDCCH skipping durations are configured with different values.

- R17 PDCCH skipping and SSSG switching: Based on R17 PDCCH skipping scheme, UE is indicated to switch to a default search space set group (SSSG) with sparse PDCCH monitoring periodicity before the next traffic burst transmission. And UE will switch to a dense SSSG for data transmission once the scheduling PDCCH is detected. Two candidate PDCCH skipping durations are configured.

- Enhanced PDCCH skipping : after finishing the transmission, the network will indicate UE to skip the remaining PDCCH monitoring to a target position by adding additional skipping duration (e.g., the -jitter boundary of the next frame if gNB can be aware of jitter range).

- Enhanced PDCCH skipping and SSSG switching : Based on enhanced PDCCH skipping scheme, UE is indicated to switch to a default search space set group (SSSG) with sparse PDCCH monitoring periodicity before the next traffic burst transmission. And UE will switch to a dense SSSG for data transmission once the scheduling PDCCH is detected. Besides, adopt a new skipping duration value (e.g., skip to the -jitter boundary of the next frame).

Table 2 Power consumption results in FR1 DL Indoor Hotspot with 30Mbps traffic model

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 5 | 10 | 100% | - | - | - |
|  | R15/16 DRX | 5 | 10 | 100% | 3.67% | 3.67% | - |
|  | Enhanced DRX | 5 | 10 | 100% | 28.38% | 28.38% | - |
|  | R17 PDCCH skipping | 5 | 10 | 100% | 18.15% | 18.15% | Note 1 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 22.04% | 22.04% | Note 2,4 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 23.73% | 23.73% | Note 2,5 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 10 | 100% | 24.60% | 24.60% | Note 2,6 |
|  | Enhanced PDCCH skipping | 5 |  | 100% | 25.30% | 25.30% | Note 3,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 30.73% | 30.73% | Note 4,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 32.55% | 32.55% | Note 5,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 10 | 100% | 33.44% | 33.44% | Note 6,7 |
|  | AlwaysOn - baseline | 10 | 10 | 92.50% | - | - | - |
|  | R15/16 DRX | 10 | 10 | 91.81% | 3.24% | 3.46% | - |
|  | Enhanced DRX | 10 | 10 | 91.25% | 23.84% | 24.11% | - |
|  | R17 PDCCH skipping | 10 | 10 | 92.50% | 15.48% | 15.67% | Note 1 |
|  | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 19.17% | 19.38% | Note 2,4 |
|  | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 20.78% | 20.99% | Note 2,5 |
|  | R17 PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 21.58% | 21.83% | Note 2,6 |
|  | Enhanced PDCCH skipping | 10 | 10 | 92.50% | 21.01% | 21.27% | Note 3,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 26.10% | 26.38% | Note 4,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 27.80% | 28.09% | Note 5,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 10 | 10 | 92.50% | 28.65% | 28.94% | Note 6,7 |
| Note 1: three candidate PDCCH skipping durations are configured {6ms 4ms 2ms} Note 2: two candidate PDCCH skipping durations are configured {6ms 4ms} Note 3: PDCCH monitoring periodicity in the sparse SSSG is 1 slot Note 4: PDCCH monitoring periodicity in the sparse SSSG is 2 slot Note 5: PDCCH monitoring periodicity in the sparse SSSG is 3 slot Note 6: PDCCH monitoring periodicity in the sparse SSSG is 4 slot Note 7: skip to the -jitter boundary of the next frame |  |  |  |  |  |  |  |

Table 3 Power consumption results in FR1 DL Indoor Hotspot with 45Mbps traffic model

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (45Mbps) | AlwaysOn - baseline | 3 | 5 | 100% | - | - | - |
|  | R15/16 DRX | 3 | 5 | 100% | 3.46% | 3.46% | - |
|  | Enhanced DRX | 3 | 5 | 100% | 26.74% | 26.74% | - |
|  | R17 PDCCH skipping | 3 | 5 | 100% | 16.47% | 16.47% | Note 1 |
|  | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 20.28% | 20.28% | Note 2,4 |
|  | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 21.91% | 21.91% | Note 2,5 |
|  | R17 PDCCH skipping and SSSG switching | 3 | 5 | 100% | 22.73% | 22.73% | Note 2,6 |
|  | Enhanced PDCCH skipping | 3 | 5 | 100% | 22.59% | 22.59% | Note 3,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 27.79% | 27.79% | Note 4,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 29.52% | 29.52% | Note 5,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 3 | 5 | 100% | 30.38% | 30.38% | Note 6,7 |
|  | AlwaysOn - baseline | 5 | 5 | 96.67% | - | - | - |
|  | R15/16 DRX | 5 | 5 | 94.44% | 2.83% | 3.14% | - |
|  | Enhanced DRX | 5 | 5 | 93.89% | 22.61% | 23.62% | - |
|  | R17 PDCCH skipping | 5 | 5 | 96.67% | 14.40% | 14.62% | Note 1 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 18.01% | 18.23% | Note 2,4 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 19.55% | 19.78% | Note 2,5 |
|  | R17 PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 20.38% | 20.58% | Note 2,6 |
|  | Enhanced PDCCH skipping | 5 | 5 | 96.67% | 19.20% | 19.45% | Note 3,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 24.14% | 24.41% | Note 4,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 25.80% | 26.08% | Note 5,7 |
|  | Enhanced PDCCH skipping and SSSG switching | 5 | 5 | 96.67% | 26.60% | 26.89% | Note 6,7 |
| Note 1: three candidate PDCCH skipping durations are configured {6ms 4ms 2ms} Note 2: two candidate PDCCH skipping durations are configured {6ms 4ms} Note 3: PDCCH monitoring periodicity in the sparse SSSG is 1 slot Note 4: PDCCH monitoring periodicity in the sparse SSSG is 2 slot Note 5: PDCCH monitoring periodicity in the sparse SSSG is 3 slot Note 6: PDCCH monitoring periodicity in the sparse SSSG is 4 slot Note 7: skip to the -jitter boundary of the next frame |  |  |  |  |  |  |  |

The power saving gain and system capacity performance of low load and high load are provided in Table 2 and Table 3 for DL XR traffic with 30Mbps and 45Mbps, respectively. Based on the simulation results given above, by using R15/16 DRX, to ensure achieving the requirement of capacity, the values of drx-onDurationTimer and drx-InactivityTimer should be larger enough to provide sufficient scheduling, transmission opportunities and cover the jitter range (i.e. [-4, 4]ms). Thus, only about 2.83%~3.67% PSG can be obtained by DRX with 16ms DRX cycle. Compared to the R15/R16 DRX configuration, a shorter drx-onDurationTimer can be configured for enhanced DRX scheme by aligning start offset to the ideal traffic burst arrival time without damaging system capacity performance. With a conservative target position configuration, 22.61%~28.38% PSG can be observed for enhanced DRX with only 0.00%~2.78% of satisfied UE loss.

Compared to DRX or enhanced DRX scheme, R17 PDCCH skipping scheme can better match the transmission of variable packet size of XR services by flexible PDCCH skipping indication. As shown in Table 2 and Table 3, 14.40%~18.15% PSG can be achieved by R17 PDCCH skipping with no capacity loss, where 6ms, 4ms, and 2ms are configured as three candidate PDCCH skipping durations in the simulation. However, the segmented PDCCH skipping duration caused by fixed pre-configured skipping duration also limits its power saving gain. The enhanced PDCCH monitoring adaptation scheme can maximize UE sleep duration at once by directly indicating skipping PDCCH monitoring to the target position, which can increase the UE chance to enter a more low-power state and achieve 19.20%~25.30% PSG without satisfied UE loss. A further 2.46%~7.25% power saving gain can be achieved by switching to sparser SSSG before monitoring the scheduled PDCCH on top of enhanced PDCCH monitoring adaptation. Under the current simulation configuration, the sparser PDCCH listening periodicity leads to larger PSG but has negligible impact on system capacity performance.

# Appendix B – Simulation assumptions

Table 4. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 5.  DL traffic models of XR with 60 FPS

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| Packet delay budget (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 |  |
| Jitter Range (ms) | [-4, 4] |  |