3GPP TSG RAN WG1 #109-e		    R1-2203596

e-Meeting, May 9th–20th, 2022

Source: 	ZTE, Sanechips

Title:	Remaining issues of PDCCH monitoring adaptation

Agenda item:	8.7.2

Document for:	Discussion and decision

Introduction

In 3GPP TSG RAN1#108-e meeting, the agreements about PDCCH monitoring adaptation were reached as follows [1].

Working Assumption (Excerpted from RAN#107bis-e) - The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS Agreement - The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation. Agreement - It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values. Agreement For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2, - 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured - 1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2; - 2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2; Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates. Agreement Answer 1 for R1-2200884 (R2-2201960): Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e. Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. - It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated ). Agreement Answer 4 for R1-2200884 (R2-2201960): - It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration. Agreement It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time. Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0 Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running Agreement When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot - no earlier than [application delay] after the slot where timer expires and - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires. Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

In this contribution, the remaining issues of PDCCH monitoring adaptation including application delay, UE behavior after a BWP switch, etc. are discussed.

PDCCH monitoring adaptation

PDCCH skipping restriction

In RAN1 #108-e meeting, the following was discussed.

- [Medium] proposal 2-1 (v4) – skipping C-RNTI/... in Type 0/0A/1/2 CSS - UE also skips monitoring of PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in any CSS and USS when UE is indicated skipping PDCCH monitoring for a duration. Note: it is previously agreed that PDCCH based monitoring adaptation is applied to USS and type-3 CSS.

If UE continues monitoring C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS, the power consumption will increase. Skipping monitoring of DCI scrambled by C-RNTI, MCS-C-RNTI, or the CS-RNTI is helpful to achieve more power saving gain. Meanwhile, we think it is unnecessary to emphasize the DCI format, the DCI format in the proposal can be removed.

When the UE is indicated to skip PDCCH monitoring for a duration on a serving cell, the UE skips monitoring of PDCCH candidates for DCI with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in any CSS and USS.

PDCCH skipping for SR and RACH procedure

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated ).

It has been agreed that the PDCCH skipping is not applied during RAR window/MsgB window/contention resolution timer or when SR is pending. The corresponding text proposals of RAN1 specification are provided in this section hereafter.

Adopt the following Text Proposal on PDCCH skipping for SR and RACH procedure

<Unchanged parts are omitted> A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires. PDCCH skipping is not applied(i.e., stop PDCCH skipping), - when UE monitors PDCCH within a ra-ResponseWindow, as described in clause 8.2, - when UE monitors PDCCH within a MsgB-ResponseWindow, as described in clause 8.2A, - if contention resolution timer is running [38.321]. If a UE is skipping PDCCH monitoring for a duration and transmits a positive SR in PUCCH, the UE stops PDCCH skipping at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH. <Unchanged parts are omitted>

UE behavior during outside active time

In RAN1 #108-e meeting, whether or not PDCCH skipping applies only in active time and the UE behavior during outside active time(if PDCCH skipping happens) were discussed, and no conclusion was reached.

[High] Proposal 7-3a (v1):  The PDCCH skipping applies only in active time  Down-select from the following options:  Option 1a: the PDCCH skipping duration is considered irrespective the slot(s) are in active time or not,   Option 1b: PDCCH skipping is terminated when UE enters outside active time

It is clear that the PDCCH skipping applies only in active time. If a PDCCH skipping is not finished when UE enters outside active time, the UE behavior during outside active time should be clarified.

DRX is performed in MAC layer, and the PDCCH skipping technique is performed in physical layer It is preferred that these two procedures are operated without close dependency. Moreover, terminating the PDCCH skipping when UE enters outside active time may decrease the power saving gain since one PDCCH skipping indication cannot be valid across DRX cycle regardless of the indicated PDCCH skipping duration. Therefore, Option 1a is preferred.

The PDCCH skipping duration is considered irrespective the slot(s) are in active time or not.

Application delay for PDCCH monitoring adaptation

The application delay for PDCCH monitoring adaptation was discussed in previous meetings. The following agreement was reached in RAN1 #107b-e meeting.

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

First of all, whether or not UE monitors PDCCH when DRX Retransmission timer is running should be discussed.

In fact, the latency impact caused by retransmission can be solved by NW implementation. That is, gNB can indicate a suitable PDCCH monitoring behavior or schedule a PDSCH with a lower MCS to UE based on the information of traffic QoS.

It is noticed that DRX UL retransmission timer will be started after each UL transmission since there is no HARQ-ACK feedback for PUSCH transmission. Moreover, the DRX Retransmission timer is maintained per DL/UL HARQ process. Hence, it will increase UE power consumption if UE needs to monitor PDCCH when DRX DL/UL retransmission timer is running.

If UE monitors PDCCH when DRX Retransmission timer is running, UE monitoring behavior may change frequently before the HARQ-ACK is transmitted (i.e., during an interval of K1), which will increase the complexity of UE implementation. An example of UE behavior transition is provided in [2].

An additional UE behavior is not needed to handle the case of PDCCH skipping interaction with HARQ retransmission.

W.r.t the impact of minimum scheduling offset, a sufficient time for UE to decode the DCI should be ensured. Thus the following is proposed,

Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping,

  - UE applies PDCCH skipping on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

  - otherwise, UE applies PDCCH skipping on the BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission.

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching,

  - UE applies SSSG switching on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

  - otherwise, UE applies SSSG switching on the BWP of the serving cell at the first slot that is at least Pswitch symbol after the last OFDM symbol of the PDCCH transmission.

PDCCH monitoring adaptation in a BWP

PDCCH monitoring adaptation indication by cross-BWP DCI

The cross-BWP DCI is a scheduling DCI indicating BWP switching. If PDCCH monitoring adaptation is indicated in a cross-BWP DCI, whether and when to apply the PDCCH monitoring adaptation in the new BWP should be discussed. Then the following alternatives are provided.

Alt 1: UE applies the PDCCH monitoring adaptation after BWP switching.

Alt 2: UE applies the PDCCH monitoring adaptation after the application delay.

In TS 38.133, the BWP switching delay is defined from RF requirement perspective. Meanwhile, in TS 38.213, the BWP switching delay is defined from data transmission perspective. For example, if a UE detects a DCI format indicating an active BWP switching for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot when the UE detects BWP switching until the beginning of a slot indicated by the slot offset value (i.e., K0/K2) in the DCI format. Therefore, the indicated PDCCH monitoring adaptation should be applied at least after BWP switching. And Alt 1 is preferred.

When UE receives a DCI indicating a PDCCH monitoring adaptation and a BWP switching, UE applies the PDCCH monitoring adaptation after BWP switching.

PDCCH monitoring behavior adaptation due to BWP timer expiration

If UE is in a PDCCH skipping duration and a BWP switch occurs at the same time due to BWP inactivity timer expiration, the PDCCH monitoring behavior after BWP switch should be clarified [3].

If only PDCCH skipping duration list is configured in default BWP, UE should monitor PDCCH after BWP switch for potential data transmission. For example, BWP inactivity timer may expire due to no packet arrival for a long time. And when BWP inactivity timer expires, UE switches to the default BWP. The default BWP can be used to re-align the understanding of active BWP between gNB and UE. Therefore, it is better to maintain PDCCH monitoring after the BWP switch.

If SSSG list is configured in default BWP, it is proposed that PDCCH monitoring behavior is performed according to search space sets with group index 0.

If the active BWP is switched due to the expiration of bwp-InactivityTimer before the end of the PDCCH skipping duration,

- UE does not perform PDCCH skipping before receiving a DCI indicates PDCCH skipping, if only PDCCH skipping duration list is configured in the new BWP.

- UE performs PDCCH monitoring behavior according to search space sets with group index 0, if SSSG list is configured in the new BWP.

Consideration on Rel-17 PDCCH monitoring adaptation and NRU search space set group switching

In TS 38.213, the following paragraph was captured in the beginning of the section 10.4.

TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set.

According to the above paragraph, the procedures in section 10.4 are not applicable if searchSpaceGroupIdList is not configured. However, the description of Rel-17 PDCCH monitoring adaptation is also captured in section 10.4. Therefore, the procedures of Rel-17 PDCCH monitoring adaptation will not be valid because of the above restriction. To enable the Rel-17 PDCCH monitoring adaptation, the following proposal is proposed.

The following text proposal is proposed:

TS 38.213 <Unchanged parts are omitted> 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for PDCCH monitoring on a serving cell. And a UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for a search space set, and the UE is not provided PDCCHSkippingDurationList for PDCCH monitoring on a serving cell, the following procedures are not applicable for PDCCH monitoring according to the search space set. <Unchanged parts are omitted>

Conclusion

In this contribution, we discuss the remaining issues of PDCCH monitoring adaptation and have the following proposals.

Proposal 1: When the UE is indicated skipping PDCCH monitoring for a duration on a serving cell, the UE skips monitoring of PDCCH candidates for DCI with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in any CSS and USS.

Proposal 2: Adopt the following Text Proposal on PDCCH skipping for SR and RACH procedure

<Unchanged parts are omitted> A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires. PDCCH skipping is not applied(i.e., stop PDCCH skipping), - when UE monitors PDCCH within a ra-ResponseWindow, as described in clause 8.2, - when UE monitors PDCCH within a MsgB-ResponseWindow, as described in clause 8.2A, - if contention resolution timer is running [38.321]. If a UE is skipping PDCCH monitoring for a duration and transmits a positive SR in PUCCH, the UE stops PDCCH skipping at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH. <Unchanged parts are omitted>

Proposal 3: The PDCCH skipping duration is considered irrespective the slot(s) are in active time or not.

Proposal 4: An additional UE behavior is not needed to handle the case of PDCCH skipping interaction with HARQ retransmission.

Proposal 5: Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping,

• UE applies PDCCH skipping on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

• otherwise, UE applies PDCCH skipping on the BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission.

Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching,

• UE applies SSSG switching on an active BWP of the serving cell at the first slot after the application delay of minimum applicable scheduling offset if a minimum applicable scheduling offset is configured in the BWP,

• otherwise, UE applies SSSG switching on the BWP of the serving cell at the first slot that is at least Pswitch symbol after the last OFDM symbol of the PDCCH transmission.

Proposal 6: When UE receives a DCI indicating a PDCCH monitoring adaptation and a BWP switching, UE applies the PDCCH monitoring adaptation after BWP switching.

Proposal 7: If the active BWP is switched due to the expiration of bwp-InactivityTimer before the end of the PDCCH skipping duration,

UE does not perform PDCCH skipping before receiving a DCI indicates PDCCH skipping, if only PDCCH skipping duration list is configured in the new BWP.

UE performs PDCCH monitoring behavior according to search space sets with group index 0, if SSSG list is configured in the new BWP.

Proposal 8: The following text proposal is proposed:

TS 38.213 <Unchanged parts are omitted> 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for PDCCH monitoring on a serving cell. And a UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList_r17 for a search space set, and the UE is not provided PDCCHSkippingDurationList for PDCCH monitoring on a serving cell, the following procedures are not applicable for PDCCH monitoring according to the search space set. <Unchanged parts are omitted>

References

Draft Report of 3GPP TSG RAN WG1 #108-e v0.1.0, RAN1#108e

R1-2203597, Discussion on power saving for RRC connected UEs, ZTE, Sanechips, RAN1#109e

R1-2200946, Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP, Huawei, HiSilicon, RAN1#108e