3GPP TSG RAN WG1 #109-e	   	                 R1-2203599

e-Meeting, May 9th–20th, 2022

Source: 	ZTE, Sanechips

Title:	Remaining issues of UE feature for UE power saving enhancements

Agenda item:	8.16.7

Document for:	Discussion and decision

Introduction

The UE feature list for UE power saving enhancement can be found in the appendix according to the agreements in previous meetings [1-3]. In this contribution, more details are discussed.

Discussion

- 29-2 TRS

##### TS 38.212 ##### 7.3.1.3.8 Format 2_7 ……… The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – bit(s), where …. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise.

The dynamic indication of TRS availability indication can be conveyed by paging DCI and PDCCH-based PEI.

In RAN1#108-e meeting, a note ‘Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7’ was discussed for FG 29-2. However, this wording is not clear considering there is no FG indicates whether or not a UE supports receiving DCI format 2_7. Therefore, the following is proposed

Modify the note of FG 29-2 as “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1”.

Conclusion

In this contribution, we discuss the UE features for power saving enhancements. We have the following proposals.

Proposal 1: Modify the note of FG 29-2 as “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1”.

References

- Draft_Minutes_report_RAN1#107

- Draft_Minutes_report_RAN1#107bis

- Draft Report of 3GPP TSG RAN WG1 #108-e v0.1.0

- 38.212-h10

Appendix

- Agreements in RAN1#107

Agreement

- FG 29-1 is kept as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Component 2 in FG 29-2 for support receiving L1 indication for TRS availability is kept

Agreement

- FG 29-3 is split into multiple FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: 2 search space sets group switching

  - [FG 29-3c]: 3 search space sets group switching

  - [FG 29-3d]: 2 search space sets group switching with PDCCH skipping

  - FFS: A UE reporting 29-3a and 29-3b does not implicitly mean the UE supports FG 29-3d

  - FFS: FG for 3 search space sets group switching with PDCCH skipping

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of PDCCH skipping Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 2 search space sets grouping switching Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 3 search space sets grouping switching Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2 search space sets grouping switching with PDCCH skipping Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

- Agreements in RAN1#107bis

Agreement

- FG 29-3c is confirmed as “3 search space sets group switching” as follows

| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 29-3d is confirmed as “2 search space sets group switching with PDCCH skipping” as follows

| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-23b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Agreements in RAN1#108

Agreement

- FG 29-2 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- The type of FG 29-1 is per band

Agreement

- The type of FGs 29-3a to 29-3d is per band

Agreement

- FG 29-1 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Revise component of 29-3a as: Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured

- Revise component of FG 29-3b as: Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3c as: Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3d as: Support of 2-bit indication of SSSG switching between 2 SSSGs, with PDCCH skipping by scheduling DCI, and timer based SSSG switching