3GPP TSG RAN WG1 meeting #109-e                                 R1-2203603

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on NW energy saving performance evaluation

Source: 	ZTE, Sanechips

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

Network energy saving is of great importance for environmental sustainability, greenhouse gas emission reduction, and operational cost savings. As agreed in RAN #94e meeting, the study item description of network energy saving is shown as follows [1]:

The objectives of the study are the following: - Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study. The following example scenarios are listed in no particular order. - Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) - FR2 beam-based scenarios (note: this scenario can also model small cells) - Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) - EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

To compare the benefits and impacts of potential techniques of network energy saving, the energy consumption model and evaluation methodology are discussed in this contribution.

Energy consumption model

The network power consumption model is beneficial for the provision of in-depth understanding of different network energy saving techniques. And a unified relative power consumption model avoids dependency on implementation details, which is also simpler for standardization work and evaluations of network energy saving techniques. Hence, a generic network power consumption model should be determined first. According to [1], it was agreed to adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 [2] to the base station side.

In TR38.840, it can be observed that the power consumption values of different states are derived from a reference configuration, which comprises key parameters, such as bandwidth, number of Rx and Tx, etc. Furthermore, multiple scaling factors are introduced considering adaptation in frequency, spatial and power domains.

Therefore, similar with the UE power consumption model in TR38.840, aspects such as reference configuration, power states, relative power of each state and power consumption scaling for adaptation should be considered in the relative network energy consumption model.

To derive a relative network energy consumption model, the following factors need to be considered

- a reference configuration;

- different power states and relative power;

- power consumption scaling factors for adaptation.

To simplify the evaluation workload, the power consumption value for different states should be defined per-slot. If a finer granularity, for example, symbol-level power consumption is considered, more efforts are required by both power consumption model and evaluation.

To simplify the evaluation workload, the power consumption value for different states should be defined per-slot.

Reference configuration

To align the evaluation results of different companies, a reference configuration needs to be determined when establishing a base station power consumption model. For the reference configuration, all configurations in time, frequency, power and spatial domains should be considered. For example, parameters including the system bandwidth, the number of carriers, the sub-carrier spacing, and the antenna configuration of the base station, etc. should be clarified. Considering the current 5G base station deployment status, FR1 should be prioritized for the study of network energy saving.

For the UE power consumption model in TR38.840, the power consumption value for UL transmission is defined by the transmission power level. The similar rule is also observed at network side, i.e., DL transmission power is critical in the determination of network power consumption. Meanwhile, the maximum transmission power varies with different base station products. Therefore, it should be clarified in the reference configuration that the maximum transmission power is assumed for the full-bandwidth transmission.

According to above analysis, reference configuration for FR1 is recommended as the following.

- Carrier configuration: 1CC;

- Sub-carrier spacing (SCS): 30KHz;

- System Bandwidth: 100MHz;

- Antenna configuration: 64T64R;

- Note: the maximum transmission power is assumed.

Adopt the following reference configuration for FR1:

- Carrier configuration: 1CC;

- Sub-carrier spacing (SCS): 30KHz;

- System Bandwidth: 100MHz;

- Antenna configuration: 64T64R;

- Note: the maximum transmission power is assumed.

- FFS: reference configuration for FR2.

Power state and relative power

Similar with UE, the base station also has sleep states and active states. For active states, the DL transmission and UL reception should be separately discussed. The downlink transmission is the main component of network power consumption. Compared with downlink transmission, the uplink transmission consumes a smaller proportion of network energy.

Downlink transmission is the main component of the network power consumption, which should be the focus of the power consumption model.

The following power states should be considered in the network power consumption model:

- Sleep states;

- DL transmission states;

- UL reception state.

For the determination of sleep states, the required transition time, power consumption values, and power consumed by the ramp- down and ramp-up are needed, as shown in Figure 1.

Figure 1 Transition between active and sleep states

According to the current base station implementation, it can switch in and out of symbol-level sleep (which is called micro sleep) with a microsecond level transition time, which can be ignored in the slot-level power consumption. For a deeper-level sleep, it takes network a longer transition time between the sleep state and the active state. For light sleep, it can be defined in a level of hundreds of millisecond. While for deep sleep, transition time in a level of second can be considered.

Table 1 can be taken as a starting point for power consumption and transition time of network sleep states.

Table 1 The power consumption and transition time of network sleep states

| Sleep mode | Relative power of sleep state | Total transition time | Additional transition energy |
| --- | --- | --- | --- |
| Deep sleep | P1 | several seconds | E1 |
| Light sleep | P2 | hundred milliseconds | E2 |
| Micro sleep | P2 | 0 ms | 0 |
| *Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |  |

Power consumption scaling for adaptation

The power consumption value defined by the reference parameters is only applicable to the specific configuration. When the base station configuration is changed, the power consumption of each power state should also be changed accordingly. Therefore, scaling rules when the configuration of base station changes relative to the reference configuration should be formulated.

Power consumption scaling for adaptation should be formulated.

As it is mentioned above, the DL power consumption is the main contributor to base station energy consumption. For the active states of the network power consumption model, it is more important to distinguish power values of different DL transmission states. In TR38.840, the DL states of UE power consumption model are classified by PDCCH/PDSCH reception, and reference signal detection since UE’s DL reception behaviors are simple and clear based on the configuration and scheduling grant. Moreover, considering blind decoding is involved in PDCCH monitoring at UE side, it seems reasonable to differentiate the power state of PDCCH detection from PDSCH reception.

However, as shown in Figure 2, network has to simultaneously transmit multiple DL data and/or reference signals to one or more UEs in the cells based on a proper scheduling strategy. And from power consumption perspective, there is little difference among Polar Code encoding for PDCCH, LDPC encoding for PDSCH and DL reference signal generation. Hence, using a specific transmission state, such as PDCCH or PDSCH transmission, cannot accurately reflect network DL power state. When other factors such as bandwidth remain the same, what matters for DL power consumption is the symbol occupation. Therefore, it is suggested to set the DL power consumption value as a function of the number of DL symbol occupation. As for the function, a linear formula is simple and sufficient for the evaluation purpose.

Figure 2 DL transmission at NW side

In addition to symbol occupation, the power consumption of different DL services also depends on other configurations, such as the number of RBs in the frequency domain, transmission power and the antenna configuration.

The following factors should be considered for the scaling of DL network power consumption:

- Transmission bandwidth;

- Occupied symbols within a slot;

- Antenna scaling;

- Transmission power.

As to UL reception state, it is observed in the current base station products that the power consumption don’t differ greatly in different uplink services as DL transmission. Therefore, a simple alternative of the model of UL power state is to use one power consumption value for all the potential UL reception. Another alternative is the same with DL power state, i.e., defining a reference power consumption value along with multiple scaling factors. And the first alternative is preferred from our perspective.

For UL reception, the following two options can be considered

- Alt-1: one power consumption value for all the UL reception

- Alt-2: reference power consumption value with multiple scaling factor

To sum up, Table 2 can be considered as a starting point of the scaling factors of network power consumption model.

Table 2 can be considered as a starting point of the scaling factors of network power consumption model.

Table 2 Scaling factors of network power consumption model

| Adaptation domain | Scaling factors |
| --- | --- |
| DL bandwidth | Linear formula ( different coefficients for the adaptation in different domain) |
| Occupied DL symbols within a slot |  |
| Transmission antenna |  |
| Transmission power |  |
| FFS: scaling factors for UL reception |  |

Evaluation methodology

Scenarios and evaluation assumptions

Scenarios and evaluation assumptions are important for the evaluation of performance gains and identification of efficient network energy saving techniques. In this section, scenarios and evaluation assumptions for network energy saving are discussed.

- Scenario

Urban Macro and Dense Urban are the important scenarios for network energy saving considering the types of base stations deployed in these scenarios.

Considering the current 5G base station deployment, scenarios for FR1 should have higher priority than that for FR2. Therefore, Urban Macro and Urban Micro which are the important scenarios of FR1 should be prioritized in this study. Moreover, the scenarios for FR2 should also be considered with the increased demand for high throughput. Therefore, the scenarios including Dense Urban can be considered.

Urban Macro and Urban Micro should be prioritized for the study of network energy saving, and Dense Urban for FR2 can be also considered but as low priority.

According to the SID, this study should prioritize idle/empty and low/medium load scenarios, while the definition of the load needs further discussion. According to [3], the different traffic loads are defined by PRB utilization. To be more specific, the medium load is defined as 30 % PRB utilization, while the busy hour is defined as 50 % PRB utilization. We think the similar definition can be reused. For the high load with 50 % PRB utilization, we think the impact of network energy saving techniques can be also evaluated with low priority due to the workload.

The traffic load can be defined as following

- idle/ empty load: only common signal/channel, such as SSB and SIB is transmitted in DL;

- low load: 5%~15% PRB utilization;

- medium load: 20%~30 % PRB utilization.

- Traffic model

Except for scenario, the load condition is also related to UE traffic model and other aspects. In our opinion, FTP3 traffic models which have been used in TR38.840 can be considered as a high priority. More details are shown in Table 3. Other traffic models, such as XR traffic model in TR 38.838 [4], can be also considered for further studied.

Table 3 UE traffic model in TR 38.840

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 s |  |

The traffic model of FTP traffic, Instant message, and VoIP in TR 38.840 can be considered for network energy savings.

- Carrier frequency and common DL transmission

In addition to single carrier, the power consumption in multi-carrier, such CA, also needs to be considered for network energy saving evaluation.

Both single carrier and multi-carrier, such CA, should be considered for network energy saving evaluation.

For simulation assumptions of other configurations, Table 4 can be considered.

For simulation assumptions of other configurations, Table 4 can be considered.

Table 4 Simulation assumption for other configurations

| Configuration | Simulation assumption |
| --- | --- |
| Carrier frequency | 3.5GHz, 2.6GHz and 2.3GHz |
| SSB | 20ms, 8 beams |
| SIB1 | 40ms, 8 beams |

- Evaluation assumptions

The system-level simulation can provide more performance, e.g. capacity, UPT, handover and mobility, for both network and UE side. However, the workload required by system-level simulation is much larger than that for numerical and link-level simulation. Even less performance can be provided by numerical simulation, one can obtain network energy saving gain by numerical simulation. Therefore, the numerical simulation can be considered in the evaluation of certain network energy saving techniques.

The numerical simulation can be considered in the evaluation of network energy saving techniques due to the low workload.

For system level simulation, the evaluation assumptions including the gNB and UE antenna configuration, frequency band, number of layers, slot format, UE distribution, Tx and Rx power, etc. for UMa scenario in TR 38.838 [4] and TR 38.901 [5] are shown in Table A1 in appendix. It can be considered as a start point for the system level simulation assumption for network energy savings.

In Table A1, 64Tx antenna port is assumed. For the evaluation of network energy saving techniques in spatial domain, other antenna configurations, e.g. 32Tx, also can be considered. Moreover, the other values of the basic simulation parameters can be further discussed or reported by companies for evaluating different network energy saving techniques.

The simulation assumption in Table A1 in appendix can be considered as a start point for the system level simulation assumption for network energy savings.

KPIs

According to the objectives of network energy saving SI, the evaluation methodology should target for evaluating network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. In our opinion, the purpose of network energy saving study is to identify potential network energy saving techniques in time, frequency, spatial and power domains, and assess the impact to network and UE performance. Therefore, the network energy saving gain, UPT and latency should be mainly considered as the KPI. The UE power consumption can be also considered as the KPI to assess the impact to UE performance.

The network energy saving gain, UPT and latency can be mainly considered as the KPI.

The UE power consumption can be considered to assess the impact to UE performance.

Conclusion

In this contribution, we discuss the energy consumption model, the evaluation methodology. We have the following observations and proposals

Observation 1: Downlink transmission is the main component of the network power consumption, which should be the focus of the power consumption model.

Proposal 1: To derive a relative network energy consumption model, the following factors need to be considered

 a reference configuration;

 different power states and relative power;

 power consumption scaling factors for adaptation.

Proposal 2: To simplify the evaluation workload, the power consumption value for different states should be defined per-slot.

Proposal 3: Adopt the following reference configuration for FR1:

 Carrier configuration: 1CC;

 Sub-carrier spacing (SCS): 30KHz;

 System Bandwidth: 100MHz;

 Antenna configuration: 64T64R;

 Note: the maximum transmission power is assumed.

 FFS: reference configuration for FR2.

Proposal 4: The following power states should be considered in the network power consumption model:

 Sleep states;

 DL transmission states;

 UL reception state.

Proposal 5: Table 1 can be taken as a starting point for power consumption and transition time of network sleep states.

Table 1 The power consumption and transition time of network sleep states

| Sleep mode | Relative power of sleep state | Total transition time | Additional transition energy |
| --- | --- | --- | --- |
| Deep sleep | P1 | several seconds | E1 |
| Light sleep | P2 | hundred milliseconds | E2 |
| Micro sleep | P2 | 0 ms | 0 |
| *Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |  |

Proposal 6: Power consumption scaling for adaptation should be formulated.

Proposal 7: The following factors should be considered for the scaling of DL network power consumption:

 Transmission bandwidth;

 Occupied symbols within a slot;

 Antenna scaling;

 Transmission power.

Proposal 8: For UL reception, the following two options can be considered

 Alt-1: one power consumption value for all the UL reception

 Alt-2: reference power consumption value with multiple scaling factor

Proposal 9: Table 2 can be considered as a starting point of the scaling factors of network power consumption model.

Table 2 Scaling factors of network power consumption model

| Adaptation domain | Scaling factors |
| --- | --- |
| DL bandwidth | Linear formula ( different coefficients for the adaptation in different domain) |
| Occupied DL symbols within a slot |  |
| Transmission antenna |  |
| Transmission power |  |
| FFS: scaling factors for UL reception |  |

Proposal 10: Urban Macro and Urban Micro should be prioritized for the study of network energy saving, and Dense Urban for FR2 can be also considered but as low priority.

Proposal 11: The traffic load can be defined as following

 idle/ empty load: only common signal/channel, such as SSB and SIB is transmitted in DL;

 low load: 5%~15% PRB utilization;

 medium load: 20%~30 % PRB utilization.

Proposal 12: The traffic model of FTP traffic, Instant message, and VoIP in TR 38.840 can be considered for network energy savings.

Proposal 13: Both single carrier and multi-carrier, such CA, should be considered for network energy saving evaluation.

Proposal 14: For simulation assumptions of other configurations, Table 4 can be considered.

Table 4 Simulation assumption for other configurations

| Configuration | Simulation assumption |
| --- | --- |
| Carrier frequency | 3.5GHz, 2.6GHz and 2.3GHz |
| SSB | 20ms, 8 beams |
| SIB1 | 40ms, 8 beams |

Proposal 15: The numerical simulation can be considered in the evaluation of network energy saving techniques due to its low workload.

Proposal 16: The simulation assumption in Table A1 in appendix can be considered as a start point for the system level simulation assumption for network energy savings.

Proposal 17: The network energy saving gain, UPT and latency can be mainly considered as the KPI.

Proposal 18: The UE power consumption can be considered to assess the impact to UE performance.

References

RP-213554, study on network energy savings for NR.

3GPP TR 38.840, V16.0.0, 2019-06.

ETSI ES 202 706-1 V1.6.1, 2021-01.

3GPP TR 38.838, V17.0.0, 2021-12.

3GPP TR 38.901, V17.0.0, 2022-03.

Appendix

Table A1 Evaluation assumptions on basic simulation parameters for UMa scenario

| Parameters | Value |
| --- | --- |
| Scenario | Urban Macro; Hexagonal grid, 19 macro sites, 3 sectors per site |
| Inter-BS distance | 500m |
| Min. BS - UE distance | 35m |
| Carrier frequency | 2.3G, 2.6 GHz; or 3.5 GHz; |
| Duplex Mode / Simulation bandwidth | TDD/100 MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 64Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64Tx: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1; 4,8) The antenna tilt is 12 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Transmit Power | 49 dBm for 20MHz at TRP, and 23 dBm at UE |
| Antenna Height | 25 m for BS and 1.5 m for UE |
| Antenna Element Gain | 8dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE distribution | Uniform in cell; 80% of users are indoor, 20% of users are outdoor; Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 10 |
| Scheduling Algorithm | SU-MIMO + PF; or MU-MIMO + PF |
| HARQ/repetition | HARQ retransmission |
| Channel estimation | Realist |
| PDCCH overhead | 1/7 (2symbols per 14symbol) |
| DMRS overhead | 1 symbol per 14symbol |
| BS receiver | MMSE-IRC |