# 3GPP TSG RAN WG1 #109-e			R1-2203606

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE, Sanechips

Title:	Discussion on XR specific power saving techniques

Agenda item:	9.11.1

Document for:	Discussion

Introduction

The SID of XR was approved in RAN #94. The objectives include CDRX enhancement and PDCCH monitoring enhancement for power saving purpose. In this contribution, potential issues/enhancements are discussed to support 5G-New Radio based XR traffic.

Discussion

In this section, CDRX technique is enhanced to accommodate the XR traffic properties such as pseudo-periodicity, aiming at achieving reasonable power saving gain with acceptable capacity loss. Meanwhile, corresponding evaluation results are provided to justify the potential enhancements.

CDRX alignment

Background

DRX technique is widely applied for UE power saving in LTE as well as NR networks. In fact, it controls UE’s PDCCH monitoring activity with a set of DRX configuration, e.g., drx-onDurationTimer, drx-LongCycleStartOffset, etc. According to the common understanding in [1], DRX technique is supposed to be applied to the XR application to save UE power consumption.

More specifically, the periodicity of XR traffic is determined by frames per second (fps), which is a non-integer. In that case, current DRX configurations cannot match the non-integer periodicity, as shown in Figure 1. It is probable that UE is within a DRX-off period while XR downlink packet arrives at gNB. Thus the packet would be postponed to next on-duration time. Moreover, that problem may occur periodically along the time. As a consequence of mismatched periodicities, the scheduling latency would increase.

Figure 1 Mismatch between XR traffic and DRX

Other impacts, e.g., power saving gains, should also be investigated. For example, in the aforementioned case, the power consumption was wasted due to vain attempt of PDCCH decoding during onDurationTimer if the data packets arrives outside the onDurationTimer.

Due to the mismatch between XR traffic periodicity and CDRX cycle, XR data packets arrive outside onDurationTimer frequently.

Solutions

In order to achieve an aligned periodicity between CDRX configuration and XR service, potential CDRX enhancements can be considered in two directions, i.e., via dynamic change of CDRX configuration or via semi-static RRC configuration of CDRX.

For the first solution, more flexibility is provided but with additional signaling overhead. For example, dynamic CDRX configuration update can be implemented by adjusting the StartOffset over a period of time via dynamic indication to ensure that the start of next DRX cycle is aligned with the arrival of XR data packet. However, since UE does not know when the CDRX configuration should be updated to adapt to the traffic, UE needs to monitor the dynamic indication to obtain the update information, it will cause extra power consumption on PDCCH monitoring.

Dynamic change of CDRX configuration will provide additional signaling overhead and may cause more power consumption on PDCCH monitoring.

For semi-static RRC configuration of eCDRX, less spec impact is observed. For example, a set of CDRX configurations can be applied for the traffic transmissions in a specific XR scenario. The problem of semi-static RRC configuration is that the latency for RRC re-configuration, if needed, is large. However, the periodicities of XR traffics should not be changed once the service starts. Hence semi-static RRC configuration is considered as proper to align the CDRX cycle and XR traffic periodicity.

In additional, for both alternatives, traffic periodicity and jitter range information are needed for proper CDRX configuration and dynamic indication [2].

Semi-static RRC configuration for CDRX enhancement can be considered to align the CDRX cycle and XR traffic periodicity.

  - FFS: dynamic update of the CDRX configuration.

More specifically, multiple implementations of periodicities match via RRC configuration are elaborated as follows:

- Multiple CDRX configurations

In [3, 4], multiple CDRX configurations are proposed for multiple XR flows. It can also be used to align the XR traffic and CDRX cycle. Compared to the legacy dual CDRX configurations, the potential enhancement is to allow multiple CDRX configurations with staggered start of onDurationTimer, so that similar effect with different drx-cyle sets, e.g., (17,17,16 ), can be realized.

The advantages of multiple configurations of CDRX are clear, e.g, the legacy CDRX operation can be reused to the most extent. In addition, the increased complexity for both network and UE is minimal. On the other hand, more signaling overhead is required, and the handling of multiple CDRX configurations at the same time needs to be considered.

- Non-integer CDRX cycle

Given the requirement of non-integer periodicity of the service, a non-integer value may be introduced as the CDRX cycle. This method seems to be the most straightforward solution to address the issue. However, the issue is how to use the non-integer value and how to determine the position where the drx-onDurationTimer starts.

- CDRX resynchronization

In this case, multiple small CDRX cycles are used to compose a longer CDRX cycle. For example, XR traffic periodicity is 1000/120 =25/3ms. A CDRX cycle set with three small CDRX cycles, e.g., (9ms, 8ms, 8ms) or (8ms,8ms,9ms), can obtain a long CDRX cycle of 25ms and the alignment between XR traffic and CDRX is achieved, as shown in Figure 2.

Figure 2 Semi-static CDRX periodicity adaptation

To summarize, semi-static RRC configuration for CDRX enhancement can be achieved by multiple methods and can be studied. In additional, other methods to reach matched periodicities of traffic and CDRX can also be considered.

RAN 1 to study solutions to match periodicities of XR traffic and CDRX via semi-static RRC configuration, e.g., multiple CDRX configurations, non-integer CDRX cycle, CDRX periodicity resynchronization.

Jitter handling

Background

It is elaborated that the varying frame encoding delay and network transfer time introduces jitter (e.g.,for DL video stream). And the jitter is modeled to follow truncated Gaussian distribution with a range of [-4ms~4ms] or [-5ms~5ms] [1]. For a particular CDRX configuration, the data packet may arrive at gNB at the time when UE is in inactive time because of the uncertainty of jitter. In this regard, the packet would be postponed to next on-duration time. Firstly, jitter is more likely follow a real Gaussian distribution in realistic network, which indicates the jitter range may be extended further, and probably bring the arrival packet out of active time. Then if the exceeding happens with an extent of probability, the delay of the postponed packet may not be within the requirement of PDB.

In our understanding, the UE’s experience may decrease when the delay exceeds a threshold, because in that case the packet can’t be successfully received at UE side within the required PDB.

The performance would decrease when the delay caused by jitter exceeded a threshold.

Solutions

In order to decrease the jitter impact, one simple way is to increasing PDCCH monitor occasions, e.g.,via increasing the drx-onDurationTimer to reduce the latency caused by jitter. However, in this regard, the power saving gain will also be decreased. Thus a technique which has less capacity impact and large power saving gain is needed. Two solutions are provided as following:

- Enhanced CDRX with flexible additional PDCCH monitoring.

- Fast/dense WUS for jitter handling with eCDRX.

For the first solution, considering that there is always a packet during one XR traffic cycle, an additional PDCCH monitoring time can be considered to monitor the schedule for the late arrival data packet as shown in Figure 3. That is, if a data packet arrives earlier than the DRX On because of jitter, the data packet can be scheduled immediately when UE starts drx-OndurationTimer. If a data packet arrives later than the end of DRX On, an additional active time can be activated for this data packet. Since the additional active time will not always be activated, the power consumption caused by the additional active time will not increase obviously.

Figure 3 Flexible additional PDCCH monitoring

For the second solution, UE needs to monitor multiple WUS during outside active time and be wakened up if a packet arrives e.g., at gNB side. The latency and power saving gain are highly dependent on the design of WUS (e.g., the periodicity of WUS, the signal of the WUS) and would be critical in this regard. To decrease the latency, the start position of a drx-onDurationTimer needs to be changed by WUS which is different from current CDRX procedure.

Enhancements of handling jitter through introducing flexible additional PDCCH monitoring, or introducing a new wake up signal can be studied.

UL power saving scheme

The impact of UL signaling on power consumption should also be considered except the DL power saving scheme. If the UL signaling transmitted irrelevant with the DL signaling, the sum of power consumption will be increased. For example, if UE transmits a SR outside DRX active time, UE will wake up and monitor PDCCH for potential UL grant, and increase the monitoring occasion of PDCCH. Therefore, UL power saving scheme should also be considered, especially regarding DL & UL alignment schemes. Some UL power saving schemes and corresponding evaluation results are provided in our companion contribution [5].

For UL, power saving schemes and corresponding evaluations should be considered.

Evaluation results

In this section, the evaluation results of the potential enhancements are provided. Following assumptions are adopted in the evaluation of power consumption, which is in line with TR 38.838.

- The power consumption results for both all UEs and satisfied UEs are counted to comprehensively evaluate XR power consumption performance.

- Step function with only two transmission power values is used for UL power consumption.

  - Step function: power number is given as A for X= [0, 20]dBm and B for X =(20, 23] dBm, where A and B (defined in 38.840) correspond to power consumption value for a given uplink slot for 0dBm and 23dBm, respectively

- The same number of UEs per cell is used in baseline. And power saving schemes along with the metric of capacity are reported.

- More details about simulation parameters and power models are shown in Appendix.

For the evaluation of power consumption of the above schemes, the results of joint UL&DL and DL only are provided. The traffic models used for power consumption evaluation are presented in Table 1.

Table 1 Traffic models for power consumption evaluation

| Traffic Model No. | Mean (Byte) | STD (Byte) | Maximum (Byte) | Minimum (Byte) | PDB(ms) |
| --- | --- | --- | --- | --- | --- |
| VR30M | 62500 | 6563 | 93750 | 31250 | 10 |
| CG30M | 62500 | 6563 | 93750 | 31250 | 15 |
| VR45M | 93750 | 9844 | 140625 | 46875 | 10 |

Next, following schemes are evaluated:

- Baseline (Always On): No power saving technique is applied, that is, UE is always in Active Time.

- R15 CDRX

- CDRX alignment

  - CDRX can align with the XR traffic every 50ms.

- Enhanced CDRX with flexible additional PDCCH monitoring.

  - It allows UE to wake up at a flexible additional PDCCH monitoring period during DRX off state in addition to the CDRX alignment. The length of the additional PDCCH monitoring period is 4ms in the simulation.

FR1 InH DL

Only DL states are evaluated in this situation, and the state of UL slots is set to “sleep state”. The power consumption results of DL VR30M, CG30M, VR45M traffic are shown in Table 2~Table 4.

Table 2 FR1 power consumption results in Indoor Hotspot scenario for VR30M

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 11 | 11 | 93.18% | / | / |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 6.2% | 7.1% |
| CDRX alignment | 16 | 6 | 4 | 11 | 11 | 90.15% | 33.8% | 35.4% |
| eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | 11 | 11 | 91.67% | 29.8% | 31.78% |

In FR1, DL only, InH, high load, VR30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {6.2%,7.1%} with 3.03% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {33.8%-35.4%} with 3.03% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {29.8%,31.78%} with 1.51% of satisfied UE loss.

Table 3 FR1 power consumption results in Indoor Hotspot scenario for CG30M

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of DL satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 12 | 12 | 96.53% | / | / |
| R15 CDRX | 10 | 8 | 4 | 12 | 12 | 88.89% | 4.2% | 6.1% |
| CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 30.9% | 34% |
|  | 16 | 8 | 6 | 12 | 12 | 88.89% | 20.6% | 23.1% |
| eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | 12 | 12 | 88.19% | 32.4% | 34.5% |

In FR1, DL only, InH, high load, CG30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {4.2%,6.1%} with 7.64% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {20.6%,34.5%} with {7.64%-12.56%} of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {32.4%,34.5%} with 8.34% of satisfied UE loss.

Table 4 FR1 power consumption results in Indoor Hotspot scenario for VR45M

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of DL satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 7 | 7 | 91% | / | / |
| R15 CDRX | 10 | 8 | 4 | 7 | 7 | 87% | 7.4% | 8.1% |
| CDRX alignment | 16 | 8 | 6 | 7 | 7 | 89% | 22..4% | 23.5% |

In FR1, DL only, InH, high load, VR45, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {7.4%,8.1%} with 4% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {22.4%-23.5%} with 2% of satisfied UE loss.

For CDRX alignment power saving scheme, a larger drx-onDurationTimer value will lead to a larger capacity, while the power saving gain will be decreased.

Compared with CDRX alignment power saving scheme, eCDRX with flexible additional PDCCH monitoring power saving scheme can provide larger capacity under similar power saving gain.

FR1 InH DL + UL

In this section, simulation results of joint power consumption by independently evaluating DL and UL power consumption are provided. The joint power consumption results are evaluated by:

- Evaluating DL and UL power consumption independently;

- Collecting DL and UL slot states respectively;

- Recombining these slot states in a single timeline;

- Calculating overall power consumption according to the recombined timeline.

Single stream of video with jitter is used in DL and pose/control traffic model is used for UL evaluation. UE can transmit UL data in UL slots even if UE is in “DRX off” state. The power consumption results are shown in Table 5 ~ Table 7.

Table 5 FR1 power consumption results in Indoor Hotspot scenario (VR30M + pose/control)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 11 | 11 | 93.18% | / | / |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 6% | 6.65% |
| CDRX alignment | 16 | 6 | 4 | 11 | 11 | 90.15% | 23.2% | 24.6% |
| eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | 11 | 11 | 91.67% | 20.5% | 21.6% |

In FR1, DL+UL, InH, high load, VR30, ccomparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {6%,6.65%} with 3.03% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {23.2%,24.6%} with 3.03% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {20.5%,21.6%} with 1.51% of satisfied UE loss.

Table 6 FR1 power consumption results in Indoor Hotspot scenario (CG30M + pose/control)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 12 | 12 | 96.53% | / | / |
| R15 CDRX | 10 | 8 | 4 | 12 | 12 | 88.89% | 4.5% | 6.3% |
| CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 21.2% | 23.7% |
|  | 16 | 8 | 6 | 12 | 12 | 88.89% | 14.7% | 16.6% |
| eCDRX with flexible additional PDCCH monitoring | 16 | 6 | 4 | 12 | 12 | 88.19% | 21.3% | 22.7% |

In FR1, DL+UL, InH, high load, CG30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {4.5%,6.3%} with 7.64% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {14.7%,23.7%} with {7.64%-12.53%}% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {21.3%,22.7%} with 8.34% of satisfied UE loss.

Table 7 FR1 power consumption results in Indoor Hotspot scenario (VR45M + pose/control)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) | Mean PSG of satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always On |  |  |  | 7 | 7 | 91% | / | / |
| R15 CDRX | 10 | 8 | 4 | 7 | 7 | 87% | 7.22% | 7.82% |
| CDRX alignment | 16 | 8 | 6 | 7 | 7 | 89% | 19.6% | 20.5% |

In FR1, DL+UL, InH, high load, VR45, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {7.22%,7.83%} with 4% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {19.6%,20.5%} with 2% of satisfied UE loss.

Conclusion

In this contribution, we discuss the XR-specific power saving techniques and we have the following observations and proposals.

Observation 1: Due to the mismatch between XR traffic periodicity and CDRX cycle, XR data packets arrive outside onDurationTimer frequently.

Observation 2: Dynamic change of CDRX configuration will provide additional signaling overhead and may cause more power consumption on PDCCH monitoring.

Observation 3: The performance would decrease when the delay caused by jitter exceeded a threshold.

Observation 4: In FR1, DL only, InH, high load, VR30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {6.2%,7.1%} with 3.03% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {33.8%-35.4%} with 3.03% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {29.8%,31.78%} with 1.51% of satisfied UE loss.

Observation 5: In FR1, DL only, InH, high load, CG30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {4.2%,6.1%} with 7.64% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {20.6%,34.5%} with {7.64%-12.56%} of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {32.4%,34.5%} with 8.34% of satisfied UE loss.

Observation 6: In FR1, DL only, InH, high load, VR45, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {7.4%,8.1%} with 4% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {22.4%-23.5%} with 2% of satisfied UE loss.

Observation 7: For CDRX alignment power saving scheme, a larger drx-onDurationTimer value will lead to a larger capacity, while the power saving gain will be decreased.

Observation 8: Compared with CDRX alignment power saving scheme, eCDRX with flexible additional PDCCH monitoring power saving scheme can provide larger capacity under similar power saving gain.

Observation 9: In FR1, DL+UL, InH, high load, VR30, ccomparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {6%,6.65%} with 3.03% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {23.2%,24.6%} with 3.03% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {20.5%,21.6%} with 1.51% of satisfied UE loss.

Observation 10: In FR1, DL+UL, InH, high load, CG30, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {4.5%,6.3%} with 7.64% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {14.7%,23.7%} with {7.64%-12.53%}% of satisfied UE loss.

- for eCDRX with flexible additional PDCCH monitoring, the power saving gain is in the range of {21.3%,22.7%} with 8.34% of satisfied UE loss.

Observation 11: In FR1, DL+UL, InH, high load, VR45, comparing to the scheme(UE always on), following is observed:

- for R15 CDRX power saving scheme, the power saving gain is in the range of {7.22%,7.83%} with 4% of satisfied UE loss.

- for CDRX alignment power saving scheme, the power saving gain is in the range of {19.6%,20.5%} with 2% of satisfied UE loss.

Proposal 1: Semi-static RRC configuration for CDRX enhancement can be considered to align the CDRX cycle and XR traffic periodicity.

• FFS: dynamic update of the CDRX configuration.

Proposal 2: RAN 1 to study solutions to match periodicities of XR traffic and CDRX via semi-static RRC configuration, e.g., multiple CDRX configurations, non-integer CDRX cycle, CDRX periodicity resynchronization.

Proposal 3: Enhancements of handling jitter through introducing flexible additional PDCCH monitoring, or introducing a new wake up signal can be studied.

Proposal 4: For UL, power saving schemes and corresponding evaluations should be considered.

References

3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

R1-2203591, Discussion on UE power saving for XR and media services, ZTE, Sanechips, RAN1#109-e

R1-2111787, Discussion on enhancements for XR, Ericsson, RAN1#107-e

R1-2111831, Discussion on remaining issues and enhancements for XR evaluations, InterDigital Inc, RAN1#107-e

R1-2203608, Consideration about XR services, ZTE, Sanechips, RAN1#109-e

- Appendix

- A Simulation Parameters

Table A.1 simulation parameters for FR1 DL

| Parameters | Value |
| --- | --- |
| Scenario | Indoor Hotspot(12 nodes in 50 m x 120 m) |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| Duplex Mode / Simulation bandwidth | 100 MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 32TX:(M, N, P, Mg, Ng; Mp, Np) = (4,4, 2, 1, 1; 4, 4); The antenna tilt is 90 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Transmit Power | 24 dBm per 20M at TRP, and 23 dBm at UE |
| BS Height | 3 m |
| UE Height | 1.5m |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 15 |
| Scheduling Algorithm | MU-MIMO+ PF |
| HARQ/repetition | HARQ retransmission |
| Channel estimation | Realist |
| Target BLER | 10% for first transmission |
| PHY processing delay | UE PUSCH processing Capability #1,N1=10 |
| PDCCH overhead | 1/7(2symbols per 14symbol) |
| DMRS overhead | 1 symbol per 14symbol |
| BS receiver | MMSE-IRC |

Table A.2 Simulation assumption for FR1 UL

| Parameter | value |
| --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |
| Power control | P0 = -80 dBm, alpha = 0.8 |
| Noise Figure | 5dB for BS and 9dB for UE |
| Scheduler | MU-MIMO+PF |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% for first transmission |
| UE speed | 3 km/h |
| HARQ/repetition | support |
| PHY processing delay | UE PUSCH processing Capability #1,N2=12 |
| DMRS overhead | 4RE |

- B Power model

Table B.1 Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power(1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

- Note: The power of “PDCCH+PDSCH+PUSCH” slot, “PDSCH+PUSCH” slot and “PDCCH+PUSCH” slot is the sum of corresponding DL power consumption and PUSCH power consumption. For example, the power value of “PDCCH+PUSCH” is equal to the sum of power of “PDCCH-only” and power of “PUSCH”.