3GPP TSG RAN WG1 #109-e	                         	R1-2203608

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE, Sanechips

Title:	Consideration about XR services

Agenda item:	9.11.3

Document for:	Discussion and decision

Introduction

The SID of XR was approved in RAN #94. The objectives include enhancement techniques for capacity and power saving. In this contribution, more potential issues/enhancements are discussed to support eXtended Reality traffic based on 5G-New Radio.

Potential enhancements

UE assistant information

One of the most distinctive XR traffic is VR 360 video, which tile-based approach can be utilized for delivery [1]. The VR 360 video consists of original 8K 360 panoramic video, most likely the combination of tiles, and a low resolution 4K 360 panoramic video. And this kind of traffic is identified as a multi-stream model, FoV/non-FoV streaming in RAN1’s perspective, wherein FoV streaming consist of multiple 8K 360 video tiles and non-FoV streaming is a low resolution 4K 360 panoramic video. It is obvious that the FoV streaming is characterized by high real-time requirement and strong user behavior correlation. Therefore, higher real-time requirements for 5G-New Radio has been put forward for supporting this kind of traffic.

According to parameters of FoV/non-FoV streaming [1], base station should transmit both FoV streaming and non-FoV streaming to UE, where the FoV streaming is the 18 tiles based on the user’s viewpoint generating the corresponding FoV picture. According to [2], the FoV/non-FoV multi-stream model has the following types as shown in Figure 1 and Figure 2.

Figure. 1 Type-1 FoV/non-FoV multi-stream traffic model

Figure. 2 Type-2 FoV/non-FoV multi-stream traffic model

In order to meet real-time requirements, UE should inform some of assistance information of user’s viewpoint to gNB for delivering the corresponding FoV picture/message.

For example, the location/ orientation /viewpoint information may be needed. Besides, other assistance information including e.g., data amount, periodicity and priority are also considered [2].

UE assistance information like, orientation information, data amount, periodicity and priority can be considered.

In our view, enhanced SR/BSR can be considered to convey the assistance information without introducing additional reporting procedure/or signal. For instance, UEs can provide orientation information through enhanced SR/BSR to assist DL transmission.

Enhanced SR/BSR can be considered to convey the assistance information

UL power saving schemes

Issue

The UL signals/data (e.g., CSI report, HARQ ACK, UL data, etc) consume power continuously with UE’s modems on, if the transmission occasions of mentioned UL signals/data are TDM-ed.

Solutions

- UL active time

If the transmission of UL signals/data can be confined within a period of time, the UL transmission occasion may be reduced and UE is more likely to go to a deeper sleep. Hence, an UL active time similar as the configuration of DRX is proposed. The UL active time stipulates a time duration when UE can transmit signals/data. And UE is not expected to perform UL transmission outside the active time. The UL active time can also be configured and associated with DRX, for example, the UL active time is configured near to the DRX active time. This is helpful to achieve DL and UL alignment.

- SR group switching

The method similar as SSSG switching can be considered to reduce the transmission occasion of SR and without increasing much latency.

The SR transmission occasion is determined by SR resource configuration which includes a periodicity and an offset. To align the DL packet transmission, a dense SR resource can be used during DL packet burst time and a sparse SR resource can be used after DL packet burst as shown in Figure 3.

Figure. 3 Switch SR group according to DL packet burst

An UL active time used to confine the UL transmission can be considered for UL power saving.

SR resource switching can be considered for uplink power saving.

Evaluation results

In this section, some UL power saving schemes are evaluated. Only uplink traffic is considered.

In line with the previous meetings’ agreement, the following assumptions are adopted in the evaluation of power consumption.

- Both linear interpolation method and step function with only two transmission power values are used to count UL power consumption.

  - Linear interpolation method: the power of UL slot is calculated according to the following function:

Tx_power is the Tx power values in one slot.

  - Step function: power number is given as A for X= [0, M]dBm and B for X =[M, 23]dBm, where A and B (defined in 38.840) correspond to power consumption numbers for a given uplink slot for 0dBm and 23dBm respectively, M=20dBm

- Using the power model of 0 dBm for UE with transmission power less than 0 dBm.

- The same number of UE per cell is used in baseline and power saving schemes, and the results for baseline capacity are reported.

- The power consumption of CSI reporting and SRS transmission is not considered. The simulation parameter and power models are shown in Appendix.

UL active time

In this section, pose/control signal is transmitted according to configured grant (CG) scheduling. The periodicity of configured grant is set as 2.5ms. The following schemes are evaluated:

- Baseline: UL is set as always on. It means UE can transmit PUSCH at every CG occasion if needed. DL slots are considered as sleeping slots.

- UL active time: The pose/control signal can be transmitted within a UL active time. The configuration of UL active time is shown in Figure 4. The cycle of UL active time is 8ms and the duration of UL active time is 4ms/3ms. DL slots are considered as sleeping slots.

Figure 4 Illustration of UL active time

FR1 InH UL

Table 1 FR1 power consumption results in Indoor Hotspot scenario (pose/control)

| Application | Scheme | avg # UEs/ cell = N1 | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function |
| UL: pose/control | Baseline | 11 | - | 100% | / | / |
|  | UL active time (cycle=8, duration=4) | 11 | - | 100% | 13.7% | 13.67% |
|  | UL active time (cycle=8, duration=3) | 3 | - | 100% | 18.67% | 19% |

For FR1 InH UL,low load, compared to scheme that UE can always transmit UL signal,the power saving gain of proposed scheme(UL active time) is in the range of {13.67%, 19%} without satisfied UE loss.

FR1 DU UL

Table 2 FR1 power consumption results in Dense Urban scenario (pose/control)

| Application | Scheme | avg # UEs/ cell = N1 | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function |
| UL:pose/control | Baseline | 11 | - | 100% | / | / |
|  | UL active time | 11 | - | 100% | 13.2% | 14.3% |

For FR1 DU UL,low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(UL active time) is in the range of {13.2%, 14.3%} without satisfied UE loss.

SR group switching

In this section, the case UL pose/control traffic: 0.2Mbps@250fps, 10ms is evaluated. Dynamic grant scheduling is used for the traffic streams. That is, if there is traffic to be transmitted, SR will be transmitted firstly, and then UE will detect PDCCH for dynamic UL grant from gNB. In this section, the following schemes are simulated:

- Baseline: The UL is set as always-on as the baseline. It means UE can perform UL transmission at every UL slots/symbols if needed. If UE transmits SR, UE will monitor PDCCH for the subsequent 2.5ms, otherwise, UE does not monitor PDCCH.

- SR group switching: Two SR groups are assumed, where the periodicities of SR group #1 and SR group #2 are 1 slot and 15 slots respectively. The cycle of SR group switching is 16ms, where SR group #1 is active and SR group #2 is disable in the first 5ms, and SR group #2 is active and SR group #1 is disable in the next 11ms as Figure 5 shown.

Figure 5 Illustration of SR group switching

FR1 InH UL

Table 3 FR1 power consumption results in Indoor Hotspot scenario (pose/control)

(Note: the power consumption of SR, PDCCH and PUSCH is considered.)

| application | Scheme | avg # UEs/ cell = N1 | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function |
| UL: pose/control | Baseline | 11 | - | 100% | / | / |
|  | SR group switching | 11 | - | 100% | 12% | 12.1% |

For FR1 InH UL, low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(SR group switching) is in the range of {12%, 12.1%} without satisfied UE loss.

FR1 DU UL

Table 4 FR1 power consumption results in dense urban scenario (pose/control)

| Application | Scheme | avg # UEs/ cell = N1 | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function |
| UL: pose/control | Baseline | 11 | - | 100% | / | / |
|  | SR group switching | 11 | - | 100% | 11.4% | 11.37% |

For FR1 DU UL, low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(SR group switching) is in the range of {11.37%, 11.4%} without satisfied UE loss.

Conclusion

In this contribution, more considerations about XR-specific techniques are provided; we have the following observations and proposals.

Observation 1: For FR1 InH UL,low load, compared to scheme that UE can always transmit UL signal,the power saving gain of proposed scheme(UL active time) is in the range of {13.67%, 19%} without satisfied UE loss.

Observation 2: For FR1 DU UL,low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(UL active time) is in the range of {13.2%, 14.3%} without satisfied UE loss.

Observation 3: For FR1 InH UL, low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(SR group switching) is in the range of {12%, 12.1%} without satisfied UE loss.

Observation 4: For FR1 DU UL, low load, compared to scheme that UE can always transmit UL signal, the power saving gain of proposed scheme(SR group switching) is in the range of {11.37%, 11.4%} without satisfied UE loss.

Proposal 1: UE assistance information like, orientation information, data amount, periodicity and priority can be considered.

Proposal 2: Enhanced SR/BSR can be considered to convey the assistance information

Proposal 3: An UL active time used to confine the UL transmission can be considered for UL power saving.

Proposal 4: SR resource switching can be considered for uplink power saving.

References

- S4aV200614. “Proposed Updated Modelling for Viewport Dependent 3DoF Streaming” ModellingHiSilicon, 3GPP SA4-Post-111e, 22 December 2020.

- R1-2203591. “Discussion on UE Power Saving for XR and Media Services.”, ZTE, Sanechips, 3GPP TSG RAN WG1 #109-e May 16th-23th, 2022.

- Appendix

- A Simulation Parameters

Table A Simulation assumption for FR1 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2 : Dense Urban |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64R: (8,8, 2, 1, 1; 4, 8) (dH,dV) = (0.5λ, 0.5λ); |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | Ceiling-mount pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |  |
| Power control | P0 = -80 dBm, alpha = 0.8 |  |
| Noise Figure | 5dB for BS and 9dB for UE |  |
| Scheduler | PF (support MU-MIMO) |  |
| Device deployment | 100% indoor | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Down-tilt | 90 degrees | 12 degrees |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% for first transmission |  |
| UE speed | 3 km/h |  |
| HARQ/repetition | Support |  |
| PHY processing delay | UE PUSCH processing Capability #1, N2=12 |  |
| DMRS overhead | 4RE |  |

- B Power model

Table B.1 Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power(1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by linear function or step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

Note: The power of “PDCCH+PDSCH+PUSCH”, “PDSCH+PUSCH” and “PDCCH+PUSCH” is the sum of corresponding DL power consumption and PUSCH power consumption. For example, the power value of “PDCCH+PUSCH” is equal to the sum of power of “PDCCH-only” and power of “PUSCH”.