3GPP TSG RAN WG1 #109	R1-2203610

e-Meeting, May 09 – 20, 2022

Agenda item:		8.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on PUSCH coverage enhancements

Document for:		Discussion and Decision

# Introduction

In Rel-17 coverage enhancements, the following features have been specified:

- PUSCH repetition type A enhancements

- TB processing over multi-slot PUSCH

- Joint channel estimation for PUSCH and PUCCH

- Inter-slot frequency hopping for joint channel estimation

- Dynamic indication of repetition factor PUCCH

- Type A PUSCH repetitions for Msg3

In this contribution, we discuss the remaining issues on Rel-17 coverage enhancements for PUSCH.

# Discussion

## Remaining issues on PUSCH repetition type A enhancements

### Slot indicated by K2 offset for DG-PUSCH when K>1

One question that was brought up during RAN1#107-bis-e meeting was whether the slot indicated by K2 offset should always be an available slot (by adding restriction on the scheduler) or not. The following agreement was then made in RAN1#108-e on this aspect:

Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.

From the above agreement, 3 scenarios have been clarified including CG-PUSCH with K = 1 and K>1 and DG-PUSCH with K>1. However, the scenario of DG-PUSCH with K>1 was not concluded in RAN1#108-e meeting. Indeed, the following two options are still open for discussions:

For DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enables, and for K>1, - Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s). - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

It’s worth noting that, in Rel-15/16, K2 offset was specified in TS 38.214 Clause 6.1.2.1 and the corresponding RRC parameter (k2) configured in PUSCH-TimeDomainResourceAllocation was specified in TS 38.331. In both specifications, no restriction on K2 offset was specified aside from its value range. The motivation for adding further restriction on K2 offset in Rel-17 due to available slot counting is unclear. Indeed, the Rel-17 2-step available slot counting procedure needs only three inputs: the starting slot to apply the procedure, the number of repetitions, and the colliding slots. It can be observed that the procedure works without any constraint on whether the first slot indicated by K2 offset is available slot or not.

Observation 1. Rel-17 2-step available slot counting procedure works without any constraint on whether the first slot indicated by K2 offset is available slot or not. Specification change is not needed.

From the above observation, our view is that, for DG-PUSCH, when AvailableSlotCounting is enabled, and for K>1: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s) (Option 2).

However, it’s worth noting that the above behaviour has been covered by the current specification. Since the current procedure works, further discussion on modifying the current specification is not justified.

Proposal 1. Further restriction on K2 offset for available slot counting in Rel-17 is not supported for DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enables, and for K>1.

### TP for TS 38.214 Clause 6.3.1 on FH for single-slot PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI

In RAN1#107-bis-e and RAN1#108-e meetings, it was identified that a correction on frequency hopping configuration for TS 38.214 Clause 6.3.1 is needed. Specifically, frequency hopping configuration for single-slot PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI was not included in the latest specification due to changes while capturing Rel-17 agreements.

In addition, there was the similar discussion in RAN1#101-e and the conclusion form the past discussion was as follows:

R1-2003594 was rejected for Rel-15. Conclusion Draft CR in R1-2003594 is rejected for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in referredGrantConfig is not provided or set to interSlot.

In our views, the above conclusion should be respected and RAN1 should avoid reopening the discussion again, since it is not directly related to the Rel-17 enhancements. Therefore, TP for TS 38.214 Clause 6.3.1 should be the following.

Proposal 2. RAN1 to adopt the following TP for Clause 6.3.1 of TS 38.214 (changes are highlighted in red).

| Reason for change: | Correction of specificaiton changes for adopting enhancements of PUSCH repetition type A for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Add descriptions for frequency hopping configuration in case of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. |  |  |  |
| Consequences if not approved: | Frequency hopping configuration in case of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI is not defined in NR. |  |  |  |
| Clauses affected: | 6.3.1 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38.211, TS 38.212, TS 38.214 |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

***Unchanged text is omitted *** ### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. ***Unchanged text is omitted ***

## Remaining issues on TB processing over multi-slot PUSCH

The following can be noted from the latest TS 38.214, after RAN#95-e meeting:

<omitted text> ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <omitted text> For TB processing over multiple slots: - For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. - For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot. - For the case of reduced capability half-duplex UE, [and when AvailableSlotCounting is enabled,] the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. <omitted text>

It can be observed from the above specification excerpt that:

- The highlighted paragraph above for reduced capability half-duplex UE should be moved one bullet-level up to align with the other bullets of unpaired and paired spectrum.

- Text in square backets of the highlighted paragraph above for HD-FDD UE should be removed. Indeed, TBoMS only supports counting on available slots for the number of slots allocated for TBoMS. This is the reason why the term “AvailableSlotCounting is enabled” is not needed in the previous two bullets for unpaired and paired spectrum. The same logic should be applied for reduced capability half-duplex UE.

From the above observations, we propose the following:

Proposal 3. RAN1 to adopt the following changes for Section 6.1.2.1 of TS 38.214 (changes are highlighted in red):

| Reason for change: | Completion of TBoMS feature for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Complete the descriptions of resource allocation in case reduced capability half-duplex UEs support TBoMS |  |  |  |
| Consequences if not approved: | Potential confusion on the descriptions of resource allocation in case reduced capability half-duplex UEs support TBoMS. |  |  |  |
| Clauses affected: | 6.1.2.1 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

<omitted text> ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <omitted text> For TB processing over multiple slots: - For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. - For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot. - For the case of reduced capability half-duplex UE, [and when AvailableSlotCounting is enabled,] the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. <omitted text>

## Remaining issues on type A PUSCH repetitions for Msg3

### Msg3 PUSCH repetition for CFRA PUSCH scheduled by UL RAR

In RAN1#102-e, the following agreement was made concerning for CFRA PUSCH to imply that PUSCH repetition can be supported for CFRA:

Agreements: Enhancement to PUSCH scheduled by RAR UL grant will not consider the optimization specific for CFRA case in NR coverage SI.

In RAN1#107-e, above agreement was followed by a Working Assumption as provided below:

Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

In RAN1#107-bis-e, concerns were raised by some companies on interpreting the information field applied for CFRA prior to confirming the Working Assumption. As mentioned by many companies during RAN1#107-bis-e, this Working Assumption can be confirmed only if no additional RAN1 specification impact nor specific optimization for CFRA PUSCH is considered. In contrast, the Working Assumption should not be confirmed if additional optimizations are needed.

Furthermore, there were concerns from some companies during RAN1#108-e discussion on whether there will be RAN1 impact by using separate preambles/ROs for requesting Msg3 repetition. Obviously, if RAN2 decides to define RACH preambles/ROs configured for CFRA, a UE may request CFRA repetition by using separate preambles/ROs when the measured RSRP is lower than a configured RSRP threshold. This is like a UE requesting Msg3 repetitions using separate preambles/ROs. In other words, the same condition agreed for CBRA can also be applied to CFRA.

As a matter of fact, UE can trigger CFRA PUSCH only when it is in RRC_CONNECTED state. The UE will be indicated a PUSCH transmission by UL RAR grant and a dedicated resource can only be configured to a specific UE only if C-RNTI for that specific UE is known to gNB. Knowing that, corresponding CFRA resources can be scheduled at the gNB and UE can make use of it. The significance of adding the first note in the Working Assumption was for gNB to allocate resources for a UE triggering CFRA PUSCH and selecting dedicated preambles for Msg3 repetition. gNB must know in advance whether the corresponding UE is capable of repeating Msg3 or not. Otherwise, if gNB does not know that UE is capable of Msg3 repetition, configuring the CFRA resources with Msg3 repetition will result in additional redundancies.  Furthermore, gNB will be aware of UE capabilities and can take all relevant decisions after initial access. In this context, the ambiguities and scenarios proposed by few companies on different understanding of MCS field between Rel-17 new UE and Rel-15/16 gNB has no impact in RAN1 since UE would always be able to understand the indication by looking at the content of higher-layer signaling. Eventually, we do not see any possible misinterpretation of UL grants or UE capability reports which should be captured in RAN1 specification. For these reasons, we prefer again to propose confirming the WA and we think that no additional solutions in RAN1 is required.

Observation 2. The Working Assumption on supporting PUSCH repetition for CFRA scheduled by UL RAR grant can be confirmed only if no additional RAN1 specification impact nor optimization specific CFRA PUSCH is employed.

Observation 3. No further understanding between new CE UE and legacy gNB is required from RAN1 perspective.

Proposal 4. RAN1 to confirm the current WA for PUSCH repetition for CFRA scheduled by UL RAR grant, if no additional RAN1 specification impact nor optimization specific CFRA PUSCH is employed.

# Conclusion

The following observations can be noted:

Observation 1. Rel-17 2-step available slot counting procedure works without any constraint on whether the first slot indicated by K2 offset is available slot or not. Specification change is not needed.

Observation 2. The Working Assumption on supporting PUSCH repetition for CFRA scheduled by UL RAR grant can be confirmed only if no additional RAN1 specification impact nor optimization specific CFRA PUSCH is employed.

Observation 3. No further understanding between new CE UE and legacy gNB is required from RAN1 perspective.

In addition, the following were proposed:

Proposal 1. Further restriction on K2 offset for available slot counting in Rel-17 is not supported for DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enables, and for K>1.

Proposal 2. RAN1 to adopt the following TP for Clause 6.3.1 of TS 38.214 (changes are highlighted in red).

| Reason for change: | Correction of specificaiton changes for adopting enhancements of PUSCH repetition type A for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Add descriptions for frequency hopping configuration in case of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. |  |  |  |
| Consequences if not approved: | Frequency hopping configuration in case of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI is not defined in NR. |  |  |  |
| Clauses affected: | 6.3.1 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38.211, TS 38.212, TS 38.214 |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

***Unchanged text is omitted *** ### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. ***Unchanged text is omitted ***

Proposal 3. RAN1 to adopt the following changes for Section 6.1.2.1 of TS 38.214 (changes are highlighted in red):

| Reason for change: | Completion of TBoMS feature for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Complete the descriptions of resource allocation in case reduced capability half-duplex UEs support TBoMS |  |  |  |
| Consequences if not approved: | Potential confusion on the descriptions of resource allocation in case reduced capability half-duplex UEs support TBoMS. |  |  |  |
| Clauses affected: | 6.1.2.1 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

<omitted text> ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <omitted text> For TB processing over multiple slots: - For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. - For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot. - For the case of reduced capability half-duplex UE, [and when AvailableSlotCounting is enabled,] the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. <omitted text>

Proposal 4. RAN1 to confirm the current WA for Msg3 PUSCH repetition for CFRA scheduled by UL RAR grant, if no additional RAN1 specification impact nor optimization specific CFRA PUSCH is employed.

# References

- R1-2202569		FL Summary #4 on Enhancements on PUSCH repetition type A, Sharp (moderator), March 2022.

- R1-2202640		Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2), Nokia/NSB (moderator), March 2022.

- R1-2202594 	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3, ZTE (moderator), March 2022.

- R1-2202829, 	Reply LS on UL BWP with PRACH resources only for RACH with Msg3 repetition, ZTE (moderator), March 2022