3GPP TSG RAN WG1 #109-e	R1-2203611

e-Meeting, May 9 – 20, 2022

Agenda item:		8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on joint channel estimation for PUSCH and PUCCH

Document for:		Discussion and Decision

# Introduction

This contribution discusses the remaining issues on joint channel estimation for PUSCH and PUCCH, stemming from discussions and agreements made in RAN1 #108-e.

# Discussion and Text Proposal

## Restarting of actual time domain window (TDW)

In RAN1#108-e, discussion continued on whether a UE is expected to restart an actual TDW after a semi-static event according to the two scenarios below:

- Scenario 1: Within a nominal TDW, a semi-static event happens after one or multiple dynamic events.

- Scenario 2: Within a nominal TDW, a semi-static event overlaps with dynamic event within a slot.

For both scenarios, companies seem to have different understandings on whether a new actual TDW is created after the semi-static event or not, when it occurs after or overlap with a dynamic event. It seems the ambiguity comes at least from the specification text “When PUCCH-Window-Restart is enabled, the start of a new actual TDW is the first symbol of the PUCCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUCCH transmissions of PUCCH repetition within the nominal TDW, and the PUCCH transmission is in a slot determined for transmission of the PUCCH.”, and in particular from the yellow highlighted part.

This specification text is specific for the case in which PUCCH-Window-Restart or PUSCH-Window-Restart is enabled. We believe that emphasis should be put on the green highlighted text rather than the yellow highlighted text above. The green highlighted text explicitly says that a UE is expected to start a new actual TDW with the first symbol after an event that breaks power and phase continuity, when PUCCH-Window-Restart is enabled. The yellow highlighted text represents only a definition of an event as causing power and phase continuity not to be maintained across all the PUCCH repetitions within a nominal TDW, but still allows power and phase continuity to be maintained across sub-sets of PUCCH transmissions within the nominal TDW, namely the sub-set of PUCCH transmissions before the event and the one after the event.

For the cases in which PUCCH-Window-Restart or PUSCH-Window-Restart is not enabled, UE behavior in case a semi-static event happens according the two scenarios above is also well described in Clause 6.1.7 of TS 38.214, as quoted below. Therefore, even if the semi-static event occurs after or overlaps with a dynamic event, UE shall create a new actual TDW after the semi-static event, even if capability for window restart is not enabled. No specification change is needed to account for this behavior.

The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability.

From the above observations, we propose the following:

Proposal 1. For UE not capable of restarting DM-RS bundling in response to dynamic events,

- If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the semi-static event.

- If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the semi-static event.

Note: No specification impact is expected.

## TPC command handling

In RAN1#107-e meeting, the following working assumption was made for the handling of TPC commands.

Working assumption:​ - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.​ - If UE is configured to accumulate TPC commands,​ - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.​ - If UE is not configured to accumulate TPC commands​ - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. ​ FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

The above working assumption not only provides a clear UE behavior, but also well captures the majority’s view on the handling of TPC commands with format 2_2 in case of joint channel estimation. Although the concept in the above working assumption is clear and helps improving joint channel estimation performance (and hence improving coverage) by avoiding the segmentation of nominal TDW due to TPC commands, capturing the concept in specification is not straightforward and has been extensively discussed in RAN1#108-e.

In particular, the following options for capturing the working assumption were discussed:

- Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2a: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - Option 3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after are received between KPUSCHi1 symbols before transmission occasion i1 and no later than KPUSCHk symbols before transmission occasion k (i.e. including occasion k itself). - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n. - Combined Option 1&3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, KPUSCHi for PUSCH transmissions occasion i from K KPUSCHi1 symbols before the start a first symbol of the transmission occasion i1 before to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. - The definition of set for in Rel-15/16 TS 38.213 is reused. For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that are received between KPUSCHi1 symbols before transmission occasion i1 and KPUSCHk symbols before transmission occasion k. Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n.

Option 2a and Option 3a were introduced after concerns with original Option 2 and Option 3 were raised. Indeed, in the case of Figure 1, both original options would modify the transmit power of PUSCH5 compared to legacy behavior, even if PUSCH5 was not subject to a power consistency requirement (i.e. does not occur within a TDW). It was further highlighted that Option 1 would not present any issue in this scenario, and would preserve the transmission power of PUSCH5 compared to legacy behavior.

Figure 1. Dynamic PUSCH scheduled within the nominal TDW but occurring after the TDW

Modified Option 3 (i.e. Option 3a without the note) would finally have no issues with a scenario like the one presented in Figure 1. At this point, another scenario as the one in Figure 2 was presented and agreed as problematic for Option 3a. A UE following Option 3a (without the note) would not be able to account for any group common TPC command received between KPUSCH(3) symbols before PUSCH3 and KPUSCH(1) symbols before PUSCH1, as δE in the example of Figure 2.

Figure 2. PUSCH after nominal TDW scheduled by a DCI arriving before the nominal TDW

For this reason, the note in Option 3a was finally added to regard the scenario in Figure 2 as an error case. It was however observed again that Option 1 would not have such problems, and that definition of an error case would not be necessary.

Based on these considerations, the combined Option 1&3a was proposed. In our understanding, this combined Option 1&3a is tightly aligned with Option 1 while clarifying that the first and i-th transmission occasion within a nominal TDW carry the same transport block as in the case of PUSCH repetitions. Overall, we believe that the combined Option 1&3a represents a good compromise among the proposed options, and therefore it is our preferred option for integration of the working assumption in the specifications.

Proposal 2. RAN1 to agree on Combined Option 1&3a for capturing the working assumption on group common TPC command with format 2_2:

For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling,

    - For a transmission occasion i that occurs within a nominal time domain window, KPUSCHi for PUSCH transmissions occasion i spans from KPUSCHi1 symbols before a first symbol of the transmission occasion i1 to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx.

The definition of set  for  in Rel-15/16 TS 38.213 is reused.

Considering that no consensus was being reached in RAN1#108-e on how to capture the working assumption in the specifications, a proposal was made by the FL to define a behavior for the case of group common TPC commands (Proposal 9 below):

Proposal 9: - The action of group common TPC commands with format 2_2 [indicating different values of power control adjustment] is regarded as an event. - The following working assumption is not confirmed. Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW..

This proposal was thought mostly as a way out in the case companies could not converge, and in order to still have a UE behavior for the action of group common TPC commands with format 2_2. In case this proposal would be agreed, the working assumption would not be confirmed.

However, after careful analysis, this proposal may lead itself to additional complications and possible ambiguities between UE and gNB operations:

- A timeline detailing when the event would actually happen would need to be discussed and agreed. Group common TPC commands are in general not applied right away by a UE, but only if the TPC command is received in a specific time interval for the particular transmission occasion. Hence, even if the TPC command is received before transmission occasion i, a UE will apply it to transmission occasion i only if it occurs KPUSCH(i) symbols before transmission occasion i, else it would be applied at transmission occasion j where j>i is the smallest integer for which the TPC command is received KPUSCH(j) symbols before transmission occasion j. Even if this is generally feasible, it would complicate gNB operation. Additionally, it is worth noticing that gNB always knows the time when a group common TPC command is sent but may not always know when this is received by a UE, and especially if it occurs KPUSCH(i) symbols before a generic transmission occasion i. Indeed, a certain group common TPC command may be sent earlier than KPUSCH(i) symbols before receiving (at gNB) a generic transmission occasion i, but may be seen and received at UE less than KPUSCHi symbols before the generic transmission occasion i.

- Such a behavior is subject to the problem of missed DCI. If a UE misses a DCI 2_2 carrying a group common TPC command that should be received during a nominal TDW, a UE will continue maintaining power consistency and phase continuity without any benefit in UL performance.

Given the above considerations, defining the group common TPC command with format 2_2 as an event is not our preference and we believe that RAN1 should strive to agree on a way to capture the working assumption in the specifications, by down-selection of the available options that have been extensively discussed so far.

Proposal 3. Do not consider group common TPC command with format 2_2 as an event and strive to agree on a way to capture the working assumption in the specifications, by down-selection of the available options that have been extensively discussed so far.

## Maximum duration for nominal TDW

### Candidate values for maximum duration

Concerning the candidate values for maximum duration, the following can be noted from the recent reply LSs from RAN4:

For FDD [6]:

Additionally, RAN4 has discussed the maximum duration and from RAN4 perspective, it is concluded that for per band UE capability on length of maximum duration, the set of values of duration lengths for FDD are {4, 8, 16, 32} slots. Meanwhile, RAN4 will not define the requirements with 32 slots for FDD in Rel-17. In RAN4 understanding, the final conclusion on the set of values for UE capability reporting is up to RAN1 to decide.

For TDD [7]:

For the length of maximum duration for TDD: - Regarding the RAN1 request for maximum duration, RAN4 understands that the UE capability for TDD refers to the number of consecutively transmitted UL slots over which the UE can meet the phase consistency requirement, assuming no phase/power consistency violating events in between. - For the above UE capability (as defined), RAN4 has further concluded that the set of values of duration lengths should at least include {2, 4, 8} slots, and whether 16 slots can be included in the set of values for UE capability reporting is up to RAN1 to decide, but RAN4 does not intend to define requirements for 16 slots for TDD in Rel-17.

It can be observed that:

- For FDD, the set of candidate values for maximum duration is clear from RAN4 and needs confirmation from RAN1.

- For TDD, RAN1 needs to decide on whether 16 slots can also be included in the set of candidate values for maximum duration.

Given that UE will report only one value in the set of candidate values, we do not see the need to preclude any (upper bound) value in this set. Let’s not forget that with flexible duplexing (currently studied in Rel-18), the maximum number of consecutive slots in TDD could be higher in the upcoming releases. In addition, in some specific TDD pattern, e.g., 7D1S2U, considering maximum 8 slots for maximum duration will not allow us to test the case of restarting DMRS bundling, since a nominal TDW cannot cover two bundles of U slots. Therefore, we propose the following:

Proposal 4. Concerning the set of candidate values for maximum duration, RAN1 to confirm the following:

- The set of candidate values for maximum duration in FDD is {4, 8, 16, 32} slots.

- The set of candidate values for maximum duration in TDD is {2, 4, 8, 16} slots.

RAN1 to send an LS to RAN4 regarding the above confirmation.

### Clarification on the description of maximum duration in RAN1 and RAN2 specifications

In RAN1#107-e meeting, the following agreements can be noted:

Agreement - If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements. - If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions) Agreement The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

In RAN1#108-e meeting, the following was agreed:

Agreement - The value range of PUSCH-TimeDomainWindowLength is INTEGER (2 .. 32). - The value range of PUCCH-TimeDomainWindowLength is INTEGER (2 .. 8). - Note: the value shall not exceed the maximum duration.

The above agreements were captured in TS 38.214 as follows:

<omitted text> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. <omitted text>

The following can be noted in the description of PUSCH-TimeDomainWindowLength and PUCCH-TimeDomainWindowLength in the endorsed CR for TS 38.331 in [8]:

pusch-TimeDomainWindowLength Configures the length of a nominal time domain window in number of consecutive slots for DMRS bundling for PUSCH. The value shall not exceed the maximum duration defined in TS 38.101-1 [15] and TS 38.101-2 [39]. For PUSCH repetition type A/B, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUSCH repetitions and the maximum duration defined in TS 38.101-1 [15] and TS 38.101-2 [39]. For TBoMS, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the duration of TBoMS transmission (including repetition of TBoMS) and the maximum duration defined in in TS 38.101-1 [15] and TS 38.101-2 [39].

pucch-TimeDomainWindowLength Configures the length of a nominal time domain window in slots for DMRS bundling for PUCCH. The value shall not exceed the maximum duration defined in TS 38.101-1 [15] and TS 38.101-2 [39]. If this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUCCH repetitions and the maximum duration defined in TS 38.101-1 [15] and TS 38.101-2 [39].

In addition, the following can be noted from the endorsed CRs for TS 38.101-1 and TS 38.101-2 in [9] and [10], respectively:

#### 6.4.2.5 Phase continuity requirements for DMRS bundling For bands that UE indicates the support of DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 for the measurement conditions defined in Table 6.4.2.5-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDMRS-BundlingDuration], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms.

#### 6.4.2.6 Phase continuity requirements for DMRS bundling For bands that UE indicates the support of DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.6-1 for the measurement conditions defined in Table 6.4.2.6-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDMRS-BundlingDuration], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.8. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms.

Concerning [maxDMRS-BundlingDuration]:

It can be observed from the above specification excerpts that the current text in both TS 38.214 and TS 38.331 did not well capture the agreements about the maximum duration should be reported as UE capability. Indeed, maxDMRS-BundlingDuration was introduced and kept in bracket in TS 38.214, i.e., “[maxDMRS-BundlingDuration]”. From our understanding, the intention was to have a placeholder for RRC parameter of the UE capability of maximum duration. In that case, the description of that parameter (i.e., maxDMRS-BundlingDuration) should clarify that the parameter represents the maximum duration reported as a UE capability, and the set of candidate values for the parameter. Note that this reporting should be about which number is supported by the UE in a set of candidate numbers for maximum duration, but not about reporting whether a UE supports the feature or not. Although this can be combined, e.g., value 0 can be added in the list of candidate values such that a UE can report value 0 and hence implicitly indicating that it does not support the whole DM-RS bundling feature. However, such indication of supporting whole feature may not be needed since we already have separate sub-features for indicating whether the UE supports DMRS bundling for PUSCH repetition type A, PUSCH repetition type B, TBoMS and PUCCH repetitions or not.

Observation 1. The UE capability reporting of maximum duration should be about which number is supported by the UE in a set of candidate numbers for maximum duration, but not about reporting whether a UE supports the feature or not.

A clarification in TS 38.214 is needed anyway to clarify that maxDMRS-BundlingDuration is subject to UE capability.

Observation 2. A clarification in TS 38.214 is needed for the definition of maximum duration which is subject to UE capability.

Concerning pusch-TimeDomainWindowLength/ pucch-TimeDomainWindowLength:

The descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength are misleading because the maximum duration is not defined in RAN4 specifications (TS 38.101-1 and TS 38.101-2). As discussed in Section 2.3.1, RAN4 only identifies a set of candidate values for maximum duration, namely, {4, 8, 16, 32} slots for FDD and {2, 4, 8, [16]} slots for TDD, but the final decision on the sets of candidate values is up to RAN1. The exact value in this list is subject to UE capability and should be reported by the UE according to the above RAN1 agreements. Indeed, in the above RAN4 endorsed CRs for TS 38.101-1 and TS 38.101-2, there is no notion of defined value for maximum duration. Therefore, modifications on the descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength are needed to avoid confusion. A draft LS to RAN2 concerning this aspect can be found in our companion contribution in [11]. In addition, RAN1 can proactively clarify this in RAN1 specification.

Observation 3. The current descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength in the approved CR in RAN2 for TS 38.331 are misleading because the maximum duration is not defined in RAN4 specifications. The maximum duration is subject to UE capability and reported by UE. An LS to RAN2 is needed for clarification. In addition, RAN1 can proactively clarify this in RAN1 specification.

From the above observations, we propose the following:

Proposal 5. RAN1 to send an LS to RAN2 for clarifying the descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength.

Proposal 6. RAN1 to adopt the following changes for Section 6.1.7 f TS 38.214 (changes are highlighted in red):

| Reason for change: | Completion of joint channel estimation feature for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Complete the descriptions of the maximum duration for nominal time domain window. |  |  |  |
| Consequences if not approved: | Potential confusion on the descriptions of the maximum duration for nominal time domain window. |  |  |  |
| Clauses affected: | 6.1.7 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

<omitted text> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. The UE does not expect PUSCH-TimeDomainWindowLength to be greater than [maxDMRS-BundlingDuration], which is maximum duration for a nominal TDW subject to UE capability. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where [maxDMRS-BundlingDuration] is maximum duration for a nominal TDW subject to UE capability, M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. The UE does not expect PUSCH-TimeDomainWindowLength to be greater than [maxDMRS-BundlingDuration], which is maximum duration for a nominal TDW subject to UE capability. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where [maxDMRS-BundlingDuration] is maximum duration for a nominal TDW subject to UE capability, M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. <omitted text>

# Conclusion

In this contribution we have discussed the remaining issues on joint channel estimation for PUSCH and PUCCH, and made the following observations and proposals:

Observation 1. The UE capability reporting of maximum duration should be about which number is supported by the UE in a set of candidate numbers for maximum duration, but not about reporting whether a UE supports the feature or not.

Observation 2. A clarification in TS 38.214 is needed for the definition of maximum duration which is subject to UE capability.

Observation 3. The current descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength in the approved CR in RAN2 for TS 38.331 are misleading because the maximum duration is not defined in RAN4 specifications. The maximum duration is subject to UE capability and reported by UE. An LS to RAN2 is needed for clarification. In addition, RAN1 can proactively clarify this in RAN1 specification.

Proposal 1. For UE not capable of restarting DM-RS bundling in response to dynamic events,

- If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the semi-static event.

- If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the semi-static event.

Note: No specification impact is expected.

Proposal 2. RAN1 to agree on Combined Option 1&3a for capturing the working assumption on group common TPC command with format 2_2:

For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i that occurs within a nominal time domain window, KPUSCHi for PUSCH transmissions occasion i spans from KPUSCHi1 symbols before a first symbol of the transmission occasion i1 to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. The definition of set for in Rel-15/16 TS 38.213 is reused.

Proposal 3. Do not consider group common TPC command with format 2_2 as an event and strive to agree on a way to capture the working assumption in the specifications, by down-selection of the available options that have been extensively discussed so far.

Proposal 4. Concerning the set of candidate values for maximum duration, RAN1 to confirm the following:

- The set of candidate values for maximum duration in FDD is {4, 8, 16, 32} slots.

- The set of candidate values for maximum duration in TDD is {2, 4, 8, 16} slots.

RAN1 to send an LS to RAN4 regarding the above confirmation.

Proposal 5. RAN1 to send an LS to RAN2 for clarifying the descriptions of pusch-TimeDomainWindowLength and pucch-TimeDomainWindowLength.

Proposal 6. RAN1 to adopt the following changes for Section 6.1.7 f TS 38.214 (changes are highlighted in red):

| Reason for change: | Completion of joint channel estimation feature for coverage enhancement in NR. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Complete the descriptions of the maximum duration for nominal time domain window. |  |  |  |
| Consequences if not approved: | Potential confusion on the descriptions of the maximum duration for nominal time domain window. |  |  |  |
| Clauses affected: | 6.1.7 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

<omitted text> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. The UE does not expect PUSCH-TimeDomainWindowLength to be greater than [maxDMRS-BundlingDuration], which is maximum duration for a nominal TDW subject to UE capability. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where [maxDMRS-BundlingDuration] is maximum duration for a nominal TDW subject to UE capability, M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. The UE does not expect PUSCH-TimeDomainWindowLength to be greater than [maxDMRS-BundlingDuration], which is maximum duration for a nominal TDW subject to UE capability. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where [maxDMRS-BundlingDuration] is maximum duration for a nominal TDW subject to UE capability, M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. <omitted text>

# References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R1-2200790	Summary of email discussion on joint channel estimation for PUSCH, China Telecom (moderator), January 2022.

- R1-2201029	Absolute TPC timeline and support of absolute TPC with DCI format 2_2, Nokia, Nokia Shanghai Bell, February 2022.

- R1-2201030	TPC accumulation, interpretation of KPUSCH(i), Nokia, Nokia Shanghai Bell, February 2022.

- R4-2202368	Reply LS on Maximum duration for DMRS bundling, Qualcomm (moderator), January 2022.

- R4-2206537	Reply LS on Maximum duration for DMRS bundling, Qualcomm (moderator), March 2022.

- R4-2206580	Reply LS on Length of Maximum duration, China Telecom (moderator), March 2022.

- R2-2204037	Introduction of NR coverage enhancements in RRC, Huawei/HiSilicon, March 2022

- R4-2206538	CR on UE RF requirements for DMRS bundling in TS 38.101-1, Huawei/HiSilicon, March 2022.

- R4-2206539	CR on UE RF requirements for DMRS bundling in TS 38.101-2, Huawei/HiSilicon, March 2022

- R1-2203612	Draft LS on description of RRC parameters for nominal time domain window length for PUSCH and PUCCH DMRS bundling, Nokia/NSB, May 2022.