3GPP TSG RAN WG1 #109-e                                              R1-2203619

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Remaining issues on accuracy improvement for Rel-17 positioning

Agenda item:    8.5.1

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on remaining issues for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays.

Multiple reference signals to determine the same Rx timing

According to the definitions in TS 38.215, UE/TRP may use multiple reference signals to determine the same Rx timing for DL RSTD measurement. In addition, we made following agreement in previous meeting. UE will report only a UE Rx TEG ID for the RSTD reference time and another UE Rx TEG ID for timing determined from neighbor TRP to construct a RSTD measurement. Therefore, if multiple DL PRS resources are used to determine a start of one subframe from a TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID. Otherwise the Rx timing may be biased due to different Rx timing errors from different TEGs, which breaks the basis that the Rx timing is associated with a single TEG.

Agreement: - Subject to UE capability, support a UE to include one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report. These UE Rx TEG IDs can be the same or different. - Note: RSTD reference time is related to the DL_PRS_Reference_Info IE

Proposal 1: For DL RSTD measurement, if multiple DL PRS resources are used to determine a start of one subframe from a TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID.

Based on the proposal, we provide a text proposal for TS 38.215 as follows

<Unchanged parts are omitted>

5.1.29 DL reference signal time difference (DL RSTD)

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources associated with a same UE Rx TEG ID if provided can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

<Unchanged parts are omitted>

Similarly, for UE Rx-Tx time difference measurement, if multiple DL PRS resources are used to determine a start of one subframe of the first arrival path of the TP, the multiple DL PRS resources should also be associated with a same UE Rx TEG ID. Furthermore, for UL RTOA measurement, if multiple SRS resources are used to determine a beginning of one subframe containing SRS received at a RP, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID. For gNB Rx–Tx time difference measurement, if multiple SRS resources for positioning are used to determine a start of one subframe containing SRS, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID.

TEG valid time

In RAN1#108-e meeting, some proposals were mentioned to support UE to report a UE RX temporal timing error index (TTEI), where the state of the UE RX TTEI at the instance of DL PRS reception for an RSTD or UE Rx-Tx time difference measurement should be reported together with UE RX TEG association, timestamp and RSTD/UE Rx-Tx time difference measurement in the DL-TDOA/multi-RTT measurement report. The argument is that the correspondence between TEG and group delay is not always valid, i.e. may be changed because UE may be configured with a long periodicity for a measurement report. In this case, the TTEI can be used to indicate the validity of a TEG.

However, we don't see the need to introduce this new feature as it has been agreed that UE can report multiple measurement instances in a measurement report as shown in the following agreement, i.e. a UE reports in a single NR-XXX-ProvideLocationInformation, multiple NR-XXX-SignalMeasurementInformation elements.

- Agreement - The association between measurement instances and UE measurements in the report to LMF should be defined as follows: - For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method. - E.g., a UE reports in a single NR-XXX-ProvideLocationInformation, multiple NR-XXX-SignalMeasurementInformation elements for UE assisted positioning, and NR-XXX-LocationInformation for UE-based positioning. - It is up to RAN2 on how to implement above agreement - It is up to RAN3 to implement the association between measurement instances and gNB measurements in the report to LMF - Send an LS to RAN2/RAN3, asking them to take above information into account in their signalling work.

Meanwhile, in RAN4#102-e meeting, the applicability of reported UE Rx TEG is agreed to be limited to the measurements contained within the measurement report in which the Rx TEG information is provided.

- Agreements: - The framework of UE/TRP Rx TEG - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part.

Hence, the more appropriate assumption to align with above RAN4’s agreement is that no group delay change for a specific TEG ID within a single measurement instance in a measurement report. Time span for a measurement instance can be decided by UE itself and reported together with the TEG ID since only UE can monitor the group delay change over time and know when the correspondence between TEG and group delay is no longer valid.

Specifically, in each measurement instance, the starting time and the ending time of the time span is reported together with TEG ID and the measurement results, where no group delay change is assumed during the time span.

Taking UE Rx-Tx timing measurement as an example as shown in the following Table 1, where the association between SRS resource 1 and TEG ID is changed in t3 and t5. Specifically, the SRS resource#1 is associated with TEG#1 during time t1~t2, associated with TEG#2 during time t3~t4, and associated with TEG#1 again during time t5. It is noted that the timing error may be different between t5 and t2 even though the same TEG ID is used. If UE can report the time span and the association between SRS and TEG ID for each measurement instance, there will not be any ambiguity issues.

Table 1 Three measurement instances in a single measurement

| Time | t1 | t2 | t3 | t4 | t5 |
| --- | --- | --- | --- | --- | --- |
| SRS#1 | TEG#1 | TEG#1 | TEG#2 | TEG#2 | TEG#1 (delay error may not be the same as t1~t2) |
| Measurement instance | Instance#1 |  | Instance#2 |  | Instance#3 |

Proposal 2: In each measurement instance, a time span including starting time and ending time of the measurement instance is reported together with the measurement results, where no group delay change is assumed during the time span.

Then, for UE side multi-RTT measurement, we have the following proposal

Proposal 3: For each UE Rx-Tx time difference measurement instance, if a Tx TEG ID is reported, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID during the time span of the measurement instance.

Timing error margin

In RAN4#102-e meeting, RAN4 worked on the UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation in R17 ePOS WI and discussed TEG framework including the timing error margin, applicability and RRM requirements impact etc., and the following agreements are made.

Then RAN4 sent an LS R1-2203024(2206998) to ask RAN1/2 to take the information into account in the following work on NR positioning enhancements, and design the necessary signaling support for the TEG framework.

Agreements: - The framework of UE/TRP Rx TEG - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part. - The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised Agreements: - The same timing margin is used for all Rx TEGs per UE/TRP Agreements: - The timing error margin value is decided by UE/TRP among the candidate values defined in TS 38.133 based on its implementation. - UE/TRP will report the selected timing error margin value to LMF. - No NW recommendation on the timing error margin value is needed in R17.

Based on the above agreements, multiple candidate timing error margin values {TE1, TE2, …, TEN} will be defined in the TS 38.133, then UE/TRP will report the selected timing error margin value to LMF. However, it is unclear whether the selected timing error margin value can be changed dynamically. If not, UE will report the timing error margin value by UE capability signaling, otherwise, by some other signaling, e.g. ProvideLocationInformation. Hence, we think a reply LS to RAN4 is needed for clarification.

Proposal 4: Send a reply LS to RAN4 to ask whether timing error margin value is reported to LMF by UE capability signaling or other signaling, e.g. by ProvideLocationInformation.

Conclusions

In this contribution, we discuss remaining issues for mitigating timing delay. Based on the discussion, we have the following proposals:

Proposal 1: For DL RSTD measurement, if multiple DL PRS resources are used to determine a start of one subframe from a TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID.

Proposal 2: In each measurement instance, a time span including starting time and ending time of the measurement instance is reported together with the measurement results, where no group delay change is assumed during the time span.

Proposal 3: For each UE Rx-Tx time difference measurement instance, if a Tx TEG ID is reported, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) that have already been transmitted and are associated with the Tx TEG ID during the time span of the measurement instance.

Proposal 4: Send a reply LS to RAN4 to ask whether timing error margin value is reported to LMF by UE capability signaling or other signaling, e.g. by ProvideLocationInformation.

References

- Draft Report of 3GPP TSG RAN WG1 #108-e, RAN1 Chairman Notes