3GPP TSG RAN WG1 #109-e                                              R1-2203620

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Remaining issues other than accuracy improvement for Rel-17 Positioning

Agenda item:    8.5.2

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on some remaining issues for AoD, latency reduction and multi-path RSRP report.

- DL PRS Rx beam index for AoD

In current NR DL-AoD positioning method, when the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same DL PRS Rx beam index have been performed using the same spatial domain filter for reception. However, as cited texts from TS 38.214 in the following box, this mechanism only restricts to DL PRS-RSRP measurements from the same DL PRS resource set. For example, LMF would not understand how to interpret the case when two DL PRS-RSRP measurements associated with the same DL PRS Rx beam index are derived from different DL PRS resource sets. In this case, LMF can’t assume that the two DL PRS-RSRP measurements are associated with the same spatial domain filter for receiving corresponding DL PRS resources.

Clause 5.1.6.5 of TS 38.214: The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set.

Figure 1. An example of transmitting and receiving DL PRS

As shown in Figure 1, two DL PRS resource sets transmitted from the same TRP of the same positioning frequency layer can be implemented to jointly estimate DL-AOD, so it’s possible that two DL PRS resources from different DL PRS resource sets are received by the same spatial domain filter (i.e. the same DL PRS Rx beam index). With this information being reported to LMF, LMF can determine that whether the two different DL PRS-RSRP measurements associated with the two DL PRS resources are due to different TRP transmission beams or different UE Rx beams when conducting angle calculation.

Proposal 1: When the UE reports DL PRS-RSRP measurements from DL PRS resource sets associated with the same positioning frequency layer, the DL PRS-RSRP measurements associated with the same DL PRS Rx beam index have been performed using the same spatial domain filter for reception.

PRS processing capability enhancements in PPW

In Rel-17, it has been agreed that PRS can be processed in PRS processing window outside measurement gap for the purpose of latency reduction. Hence, we think the location information report should be ready right after the end of the PRS processing window. That is, UE has to finish all the DL PRS receiving and computation in the PRS processing window to make full use of its hardware resources. A PRS processing capability shown in the Figure 2 below, a PRS processing window is divided into a PRS buffering window and a PRS computation window. UE is only expected to receive the DL PRS in the PRS buffering window. Then, based on the buffered DL PRS, UE can compute/process the DL PRS in the PRS computation window to get ready for a location information report by the end of PRS processing window. According to this understanding, UE has to report its capability with at least one combination of {N, T} under the following interpretations,

- During the first part of the window with duration of L-(T-N) msec, up to N msec of PRS symbols are expected to be buffered, where L is the duration of the PRS processing window, and (N,T) is the reported capability for MG-less PRS processing.

- The UE is expected to be capable of reporting measurements derived on the PRS measured in the first window after T msec from the end of first part of the PRS processing window

- UE is not expected to be configured a PRS processing window with duration smaller than (T-N) msec

Figure 2. PRS processing capability

Proposal 2: For the PRS processing capability in a PRS processing window, UE has to report its capability with at least of the combination {N, T},

- During the first part of the window with duration of L-(T-N) msec, up to N msec of PRS symbols are expected to be buffered, where L is the duration of the PRS processing window, and (N,T) is the reported capability for MG-less PRS processing.

- The UE is expected to be capable of reporting measurements derived on the PRS measured in the first window after T-N msec from the end of first part of the PRS processing window

- UE is not expected to be configured a PRS processing window with duration smaller than (T-N) msec

LS on lower Rx beam sweeping factor for latency improvement

In Rel-16, RAN4 defined the positioning measurement requirement assuming Rx beam sweeping factor is 8 in FR2. Hence, sweeping factor 8 is supported by all Rel-16 UEs by default.

In Rel-17, a new UE capability (UE FG 27-9) with a reduced Rx beam sweeping factor has been agreed for latency improvement in FR2 where the candidate value of the reduced Rx beam sweeping factor is { 1, 2, 4, 6}. Then, one LS R1-2203022 (R4-2206980) is sent from RAN4 to ask whether UE needs to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

It is common understanding that a Rel-17 UE supporting the new FG 27-9 should also support sweeping factor 8 as Rel-16 positioning is the prerequisite of Rel-17. Hence, we think a Rel-17 UE needs to be indicated whether sweeping factor 8 or a reduced value reported by FG 27-9 is used to perform PRS measurement in FR2.

Proposal 3: A UE should be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

Report of PRS-RSRPP

Based the previous agreements, maximum N=8 additional paths have been introduced for DL-TDOA, UL-TDOA and Multi-RTT reporting enhancement. Further, LMF can request RSRPP together with timing measurement where the RSRPP reporting is applied to both the first path and additional paths.

However, it is still not decided yet in RAN1 whether the RSRPP reporting for additional paths should be an absolute power or relative power.

Based on the LS R1-2200905 (R4-2202780) from RAN4 in last meeting, PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 thinks it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting.

- Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TDoA and multi-RTT for at least the additional paths. - Alt. 1: The reference, for calculation of differential RSRPP, for all the additional paths PRS-RSRPPs is the PRS-RSRPP of the first path associated with the nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16 measurement. - Alt. 2: The reference, for calculation of differential RSRPP, for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

According to the discussion in RAN1#108-e meeting, there are two candidate solutions as shown in the above box. In our view, reporting PRS-RSRPP for the first path should be the prerequisite of PRS-RSRPP for the additional paths. That is, if PRS-RSRPP for the additional paths exists, it should be reported together with PRS-RSRPP for the first path. Hence, for the sake of lower signalling overhead, we think PRS-RSRPP for the additional paths can always be differential RSRP measurement report with respect to the PRS-RSRPP for the first path where the additional paths and the first path should be derived from the same PRS resource. Regarding the PRS-RSRPP for the first path, it is better to support the same mechanism as for DL-AOD.

In short, we have the following suggestion:

Proposal 4: Support Alt. 2, i.e. the reference, for calculation of differential RSRPP, for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

Conclusions

In this contribution, for Rel-17 positioning, we discussed remaining issues other than mitigating timing delay. Based on the discussion, we have the following proposals:

Proposal 1: When the UE reports DL PRS-RSRP measurements from DL PRS resource sets associated with the same positioning frequency layer, the DL PRS-RSRP measurements associated with the same DL PRS Rx beam index have been performed using the same spatial domain filter for reception.

Proposal 2: For the PRS processing capability in a PRS processing window, UE has to report its capability with at least of the combination {N, T},

- During the first part of the window with duration of L-(T-N) msec, up to N msec of PRS symbols are expected to be buffered, where L is the duration of the PRS processing window, and (N,T) is the reported capability for MG-less PRS processing.

- The UE is expected to be capable of reporting measurements derived on the PRS measured in the first window after T-N msec from the end of first part of the PRS processing window

- UE is not expected to be configured a PRS processing window with duration smaller than (T-N) msec

Proposal 3: A UE should be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

Proposal 4: Support Alt. 2, i.e. the reference, for calculation of differential RSRPP, for the additional path PRS-RSRPP is the PRS-RSRPP of the first path of the same PRS resource as the additional path PRS-RSRPP.

References

- Draft Report of 3GPP TSG RAN WG1 #108-e, RAN1 Chairman Notes