3GPP TSG RAN WG1 #109-e                                               R1-2203623

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Discussion on evaluation of SL positioning

Agenda item:    9.5.1.2

Document for:	Discussion and Decision

- Introduction

In RAN#94e meeting, the study item on Rel-18 NR positioning was approved [1], where one of the potential enhancements is for sidelink positioning. As shown in the SID, some bullets for sidelink positioning are to define evaluation methodology and evaluate performance.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we provide our views on the evaluation methodology and scenarios to evaluate SL positioning solutions. In addition, SL positioning simulation results are also presented.

- Discussion

In SL positioning, three network coverage scenarios are considered as shown in Figure 1, these are in-coverage, partial coverage and out-of-coverage.

Figure 1. Coverage scenarios for sidelink positioning

In-coverage refers to the case where all the UEs are inside the network. Partial coverage means that some UEs are inside the network and the others are outside the network. Out-of-coverage refers to the case where all the UEs are outside the network. When UEs are in-coverage or partial coverage, absolute positioning can be obtained. BSs and anchor UEs (e.g. RSU) know their own location information in in-coverage case, then the absolute location information of target UEs can be estimated by using BSs and anchor UEs. Also, relative positioning can be obtained in in-coverage or partial coverage, where the relative location information between target UEs and anchor UEs can be estimated. When UEs are out-of-coverage, anchor UEs don’t know their location information, and then target UEs cannot obtain their own absolute location information. Thus, relative positioning is required in out-of-coverage case. Since the simulation implementation of partial coverage is similar to that of in-coverage and out-of-coverage, we prefer to mainly consider the coverage case of in-coverage and out-of-coverage for SL positioning simulation.

Proposal 1: Only in-coverage and out-of-coverage scenarios are considered for SL positioning simulation purpose where both absolute positioning and relative positioning can be considered for in-coverage case, while only relative positioning is considered for out-of-coverage case.

Positioning techniques based on time difference and angle are supported for R16/17 NR positioning. However, massive antennas are required for high positioning accuracy by angle based positioning methods, which is expensive cost for SL devices. Therefore, we mainly consider to reuse the positioning techniques based on time difference for absolute positioning in in-coverage case, such as TDOA and multi-RTT. For relative positioning between two UEs in out-of-coverage, to obtain the accuracy relative location information of target UEs, we suggest to adopt RTT and AOA positioning methods for simplicity. That’s because TDOA may not be workable for UE-to-UE relative positioning, and AOD may be more suitable for FR2 in which sidelink may not work well for beam management based on the existing specification.

Proposal 2: For absolute positioning, TDOA or RTT are supported. For relative positioning, RTT and AOA are supported.

Based on agreements, Rel-16/17 sidelink mainly focused on FR1, and it may not work well in FR2 due to lack of basic FR2 functionalities, e.g. beam management. In our view, the same beam management mechanism should be used for both regular SL operation and SL positioning in FR2. So, to avoid overlapping discussion between Rel-18 SL and positioning group, we suggest to discuss SL positioning in FR2 after completing specifying regular SL communication. Therefore, for SL positioning, we suggest focusing on SL positioning evaluation in FR1 only.

Proposal 3: SL positioning evaluation work should focus on FR1.

For evaluation scenarios, we think at least V2X and IIOT use cases should be considered, the detailed rationale can be found in our companion contribution [6]. For V2X, the typical simulation scenarios contain highway and urban scenarios as described in TR 37.885 [2], where the most simulation assumption, parameters and methodologies can be reused for SL positioning. For IIOT use case, we suggest to reuse Rel-17 positioning scenarios, i.e., indoor factory. To mitigate simulation workload, we suggest focusing on one of InF scenarios, e.g., InF-SH.

-  Scenario 1: highway scenario

-  Scenario 2: urban scenario

-  Scenario 3: InF scenario, e.g., InF-SH

Proposal 4: For evaluation of SL positioning, three scenarios are suggested, including highway, urban and indoor factory.

  - Evaluation Scenarios and Assumptions

In this subsection, we present the evaluation scenarios and assumptions for the three following scenarios, e.g., highway, urban and indoor factory.

### 2.1.1 Scenario 1— Highway

Highway scenario for sidelink has been specified in TR 37.885 [2]. Most parameters and procedures in TR 37.885 [2], e.g., layout of highway, RSU location, UE type, UE drop procedure, SL channel model and channel model between BS and UE can be reused for SL positioning simulation. Here is our suggestion:

- Layout of highway: According to TR 37.885 [2], there are two directions and three lanes in each direction. The lane width is 4m and highway length is not less than 2000m.

- RSU location: According to TR 37.885 [2], for absolute positioning in in-coverage case, RSU are uniformly allocated with 100m spacing in the middle of highway.

- UE type: There are three types defined for UE in TR 37.885 [2]. We use Type 2 with length 5m, width 2m, height 1.6m, antenna height 1.6m, speed 140km/h.

- UE drop procedure: According to TR 37.885 [2], the distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. All the vehicles in the same lane have the same speed. And Option A is used in SL positioning simulation.

- SL channel model: For absolute positioning, the channel model between RSU and UE is UE2UE described in TR 37.885 [2]. For relative positioning, the channel model between two UEs (i.e. anchor UE and target UE) is UE2UE described in TR 37.885 [2].

- Channel model between BS and UE: According to section 6.2.1 of TR 37.885 [2], RMa described in TR 38.901 [3] is suggested for absolute positioning in in-coverage case, where LOS propagation type is used for V2B links in the high way scenarios. That’s because the coverage of highway scenario is large for supporting high speed vehicles, whose key characteristics is similar to that of rural scenario.

However, to achieve absolute positioning based on the combination of SL positioning measurements and Uu based measurements, there are something new need to be discussed, like BS location, anchors selection, bandwidth, antenna models and performance metrics.

- BS location: For absolute positioning in in-coverage case, considering to improve positioning accuracy for SL positioning, we suggest to modify Option 1 which is described in section A.1.3 TR 36.885 [8] to make that BS are deployed on both side of the road where inter-site distance is 500m as shown in Table 1 below. Alternatively, Option 2 described in section A.1.3 TR 36.885 [8] can also be considered, i.e. wrap around method of 19*3 hexagonal cells with 500m ISD. However, in our view, it is not preferable to completely reuse Option 1 described in section A.1.3 TR 36.885, that’s because TRPs are located only at single side of the highway which cannot provide high accuracy positioning.

- Anchors selection:

- For absolute positioning, all BSs and RSUs can be used as anchors or TRPs.

- For relative positioning between two UEs (an anchor UE and a target UE), a relative distance threshold should be set, which is defined as required positioning distance between two UEs. That is, relative positioning is only performed between two UEs of which distance is less than the threshold. In highway scenario, we think the relative distance threshold should be no less than 100m for safety.

- Bandwidth: 20MHz, 40MHz and 100MHz are used for evaluation of SL positioning, where 20MHz and 40MHz are typically used for ITS band, and 100MHz are typically used for licensed band in FR1.

- Antenna models: Parameters listed in section 6.1.4 of TR 37.885 [2] can be reused. We suggest to make Option 1 (single panel) as the baseline and Option 2 (two panels) as optional for antenna element pattern and antenna array configuration for vehicle UE.

- Performance metrics: On top of Rel-17 performance metrics, we suggest to at least further include 95% of UEs positioning accuracy.

Proposal 5: The parameters listed in Table 1 are supported for evaluating SL positioning in highway scenario.

Table 1. Parameters in highway for evaluation of SL positioning

|  |  | Absolute positioning | Relative positioning |
| --- | --- | --- | --- |
| Layout | Layout of highway | According to TR 37.885, 3 lanes in each direction (6 lanes in total), lane width 4m, highway length is not less than 2000m |  |
|  | RSU location | According to TR 37.885, uniformly allocated with 100m spacing in the middle of highway | RSUs are disabled |
|  | BS location | Allocated on both side of the road, inter-site distance is 500m | BSs are disabled |
|  | Illustration of layout |  |  |
| UE drop |  | According to TR 37.885, Type 2 UEs and Option A dropping are used. |  |
| Anchors selection |  | All BSs and RSUs can be used | Relative positioning is performed for two Ues within 100m |
| SL channel model |  | UE2UE described in TR 37.885 |  |
| Channel model between BS and UE |  | Rma described in TR 37.885, LOS propagation type is used for V2B links | No model |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |  |
| Antenna model |  | According to TR 37.885, Option 1 (single panel) as the baseline and Option 2 (two panels) as optional |  |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |  |

### 2.1.2 Scenario 2— Urban

Urban scenario for sidelink has been specified in TR 37.885 [2]. The parameters in TR 37.885 [2], e.g., layout of road, UE type and UE drop procedure, can be reused for SL positioning simulation. Due to the large pathloss and very low LOS probability, the absolute positioning accuracy may not be good enough in the urban scenario. Therefore, we suggest to evaluate SL positioning in relative positioning case only in urban scenario. Parameters in urban scenario for evaluating SL relative positioning are listed in the following.

- Layout of road in urban scenario: According to TR 37.885 [2], there are two directions in each street and two lanes in each direction. The lane width is 3.5m and road grid size by the distance between intersections is 433m*250m.

- UE type: There are three types defined for UE in TR 37.885 [2]. We use Type 2 with length 5m, width 2m, height 1.6m, antenna height 1.6m, speed 60km/h.

- UE drop procedure: According to TR 37.885 [2], all the vehicles in the same lane have the same speed. And Option A is used.

- Anchors selection: For relative positioning between two UEs (an anchor UE and a target UE), a relative distance threshold should be set, which is defined as required positioning distance between two UEs. That is, relative positioning is only performed between two UEs of which distance is less than the threshold. In Urban scenario, we think the relative distance threshold can be 100m.

- SL channel model: For relative positioning, the channel model between anchor UE and target UE is UE2UE described in TR 37.885 [2].

- Bandwidth: 20MHz, 40MHz and 100MHz are used for evaluation of SL positioning.

- Antenna models: Parameters listed in section 6.1.4 of TR 37.885 [2] can be reused. We suggest to make Option 1 (single panel) as the baseline and Option 2 (two panels) as optional for antenna element pattern and antenna array configuration for vehicle UE.

- Performance metrics: On top of Rel-17 performance metrics, we suggest to at least further include 95% of UEs positioning accuracy

Proposal 6: In urban scenario, SL positioning simulation should be focused on relative positioning case. The parameters listed in Table 2 are supported for evaluating SL relative positioning in urban scenario.

Table 2. Parameters in urban for evaluation of SL positioning

|  |  | Relative positioning |
| --- | --- | --- |
| Layout | Layout of urban | According to TR 37.885, 2 lanes in each direction (4 lanes in total), lane width 3.5m, road grid size by the distance between intersection 433m*250m |
|  | Illustration of layout |  |
| UE drop |  | According to TR 37.885, Type 2 UEs and Option A dropping are used. |
| Anchors selection |  | UEs within 100m of target UE, i.e. relative positioning is only performed between two UEs within 100m |
| SL channel model |  | UE2UE described in TR 37.885 |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |
| Antenna model |  | According to TR 37.885, Option 1 (single panel) as the baseline and Option 2 (two panels) as optional |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |

### 2.1.3 Scenario 3— InF-SH

Indoor factory for Rel-17 positioning has been specified in TR 38.857 [4]. The parameters in TR 38.857 [4], e.g., layout of hall, BS location, UE type, UE drop procedure, and antenna model, can be reused for SL positioning simulation. For achieving absolute positioning based on the combination results of SL positioning measurements and Uu based measurements, there are some parameters need to be newly defined, like anchors selection, SL channel model, bandwidth, and performance metrics. As a typical scenario of InF, InF-SH are suggested. Parameters in InF scenario for evaluating SL positioning are listed in the following.

- Layout of hall: According to TR 38.857 [4], the hall size is set to be 300m*150m, and room height is 10m.

- BS location: According to TR 38.857 [4], for absolute positioning in in-coverage case, 18 BSs are set on a square lattice with spacing 50m, located 25m from the wall.

- UE type: According to TR 38.857 [4], antenna height 1.5m, speed 3km/h.

- UE drop procedure: According to TR 38.857 [4], uniformly distributed at the convex hull of the horizontal BS deployment.

- Antenna model: Based on TR 38.857 [4].

- Anchors selection:

- For absolute positioning, all BSs and 20 anchor UEs randomly selected are regarded as anchors or TRPs.

- For relative positioning between two UEs (an anchor UE and a target UE), a relative distance threshold should be set, which is defined as required positioning distance between two UEs. That is, relative positioning is only performed between two UEs of which distance is less than the threshold. In indoor factory, we think the relative distance threshold can be 10m.

- SL channel model: InF-SH described in TR 38.901 [3] are used for both absolute positioning and relative positioning. Since there is no channel model defined for sidelink in InF scenario, we consider to modify InF-SH channel model for sidelink. That is to replace BS with the anchor UE in the channel model of BS2UE, where anchor UE height, transmit power are used to replace gNB’s height and transmit power. It is noted that the similar simulation assumption was used before in Rel-16 NRU discussion [7].

Figure 2. Suggested SL channel model for indoor factory

- Channel model between BS and UE: InF-SH described in TR 38.901 [3] is adopted for absolute positioning.

- Bandwidth: 20MHz, 40MHz and 100MHz are used for evaluation of SL positioning.

- Performance metrics: On top of Rel-17 performance metrics, we suggest to at least further include 95% of UEs positioning accuracy.

Proposal 7: The parameters listed in Table 3 are supported for evaluating SL positioning in InF-SH scenario.

Table 3. Parameters in InF-SH for evaluation of SL positioning

|  |  | Absolute positioning | Relative positioning |
| --- | --- | --- | --- |
| Layout | Layout of hall | According to TR 38.857, hall size 300m*150m, room height 10m, |  |
|  | BS location | According to TR 38.857, 18 BSs are set on a square lattice with spacing 50m, located 25m from the wall. | BSs are disabled |
|  | Illustration of layout |  |  |
| UE drop |  | According to TR 38.857, antenna height 1.5m, speed 3km/h, uniformly distributed at the convex hull of the horizontal BS deployment |  |
| Anchors selection |  | All BSs and 20 anchor UEs randomly selected are used to locate target UE | Relative positioning is performed for two UEs within 10m |
| SL channel model |  | Modify InF-SH described in TR 38.901, i.e. replace BS with the anchor UE in the channel model of BS-2-UE, where anchor UE height, transmit power are used to replace gNB’s. |  |
| Channel model between BS and UE |  | InF-SH described in TR 38.901 | No model |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |  |
| Antenna model |  | According to TR 38.857 |  |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |  |

  - Evaluation Results

In this subsection, we provide the simulation results for SL positioning in the following three scenarios: highway, urban and InF, including both absolute positioning and relative positioning.

### 2.2.1 Scenario 1—Highway

Table 4 to Table 5 give the evaluation parameters for absolute positioning in highway scenario. Figure 3 and Table 6 show the CDF errors of simulation results for absolute positioning in highway scenario. Table 7 gives the evaluation parameters for relative positioning in highway scenario. Figure 4 and Table 8 show the CDF errors of simulation results for relative positioning in highway scenario.

Table 4. Evaluation parameters for absolute positioning in highway scenario

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 | Case 7 | Case 8 | Case 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | BSs and RSUs | Only BSs | Only RSUs | BSs and RSUs | Only BSs | Only RSUs | BSs and RSUs | Only BSs | Only RSUs |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Bandwidth | 100MHz | 100MHz | 100MHz | 40MHz | 40MHz | 40MHz | 20MHz | 20MHz | 20MHz |
| Number of BSs | 8 | 8 | 0 | 8 | 8 | 0 | 8 | 8 | 0 |
| Number of RSUs | 20 | 0 | 20 | 20 | 0 | 20 | 20 | 0 | 20 |
| Number of UEs | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm |
| Additional notes, if any | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel |
| Positioning mode | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning |

Table 5. Evaluation parameters for absolute positioning in highway scenario

| Parameters | Case 10 | Case 11 | Case 12 |
| --- | --- | --- | --- |
| Anchor nodes | BSs and RSUs | Only BSs | Only RSUs |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Bandwidth | 40MHz | 40MHz | 40MHz |
| Number of BSs | 8 | 8 | 0 |
| Number of RSUs | 20 | 0 | 20 |
| Number of UEs | 40 | 40 | 40 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm |
| Additional notes, if any | Ideal LOS path | Ideal LOS path | Ideal LOS path |
| Positioning mode | Absolute positioning | Absolute positioning | Absolute positioning |

Figure 3. Curves of CDF errors for absolute positioning in highway scenario

Table 6. CDF errors for absolute positioning in highway scenario

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 0.1835 | 0.2371 | 0.3026 | 0.3901 | 0.4719 |
| Case 2 | 0.2134 | 0.2791 | 0.339 | 0.4193 | 0.5118 |
| Case 3 | 4.374 | 11.68 | 14.44 | 20.04 | 20.68 |
| Case 4 | 0.5683 | 0.8355 | 1.077 | 1.423 | 1.791 |
| Case 5 | 0.7271 | 0.9827 | 1.252 | 1.571 | 1.961 |
| Case 6 | 7.587 | 12.76 | 18.94 | 21.15 | 23.13 |
| Case 7 | 0.9305 | 1.179 | 1.463 | 1.864 | 2.261 |
| Case 8 | 1.051 | 1.375 | 1.746 | 2.225 | 2.795 |
| Case 9 | 10.88 | 15.11 | 20.14 | 23.31 | 28.29 |
| Case 10 | 0.5202 | 0.7343 | 0.9984 | 1.296 | 1.637 |
| Case 11 | 0.7271 | 0.9827 | 1.252 | 1.571 | 1.961 |
| Case 12 | 6.234 | 12.35 | 17.73 | 20.63 | 22.08 |

Table 7. Evaluation parameters for relative positioning in highway scenario

| Parameters | Case 13 | Case 14 | Case 15 |
| --- | --- | --- | --- |
| Anchor nodes | Anchor UE | Anchor UE | Anchor UE |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Bandwidth | 100MHz | 40MHz | 20MHz |
| Number of target UEs | 20 | 20 | 20 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC |
| Additional notes, if any | Normal channel | Normal channel | Normal channel |
| Positioning mode | Relative positioning | Relative positioning | Relative positioning |

Figure 4. Curves of CDF errors for relative positioning in highway scenario

Table 8. CDF errors for relative positioning in highway scenario

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Case 13 | 0.1112 | 0.1611 | 0.2239 | 0.3187 | 0.4551 |
| Case 14 | 0.2477 | 0.362 | 0.5414 | 0.7814 | 0.996 |
| Case 15 | 0.4145 | 0.5466 | 0.8197 | 1.173 | 1.572 |

Observation 1: In highway scenario, all techniques can meet the requirement of Set 1 described in TR 38.845, i.e. positioning accuracy is less than 50m for 95% of UEs. If both TRPs and RSUs are used to locate target UEs, the requirement of Set 2 described in TR 38.845 can be satisfied in positioning accuracy less than 3m for 95% of UEs for absolute positioning and for relative positioning without bandwidth limitation.

### 2.2.2 Scenario 2—Urban

Table 9 gives the evaluation parameters for relative positioning in urban scenario. Figure 5 and Table 10 show the CDF errors of simulation results for relative positioning in urban scenario.

Table 9. Evaluation parameters for relative positioning in urban scenario

| Parameters | Case 16 | Case 17 | Case 18 |
| --- | --- | --- | --- |
| Anchor nodes | Anchor UE | Anchor UE | Anchor UE |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Bandwidth | 100MHz | 40MHz | 20MHz |
| Number of target UEs | 20 | 20 | 20 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC |
| Positioning mode | Relative positioning | Relative positioning | Relative positioning |

Figure 5. Curves of CDF errors for relative positioning in urban scenario

Table 10. CDF errors for relative positioning in urban scenario

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Case 16 | 0.1706 | 0.258 | 0.3614 | 0.5451 | 0.7794 |
| Case 17 | 0.4631 | 0.7135 | 1.024 | 1.493 | 2.075 |
| Case 18 | 0.7715 | 1.204 | 1.761 | 2.612 | 3.865 |

Observation 2: In urban scenario, relative positioning can meet the requirement of Set 1 described in TR 38.845 without bandwidth limitation, i.e. positioning accuracy less than 50m for 95% of UEs. The requirement of Set 2 described in TR 38.845 can be satisfied by relative positioning with sufficient bandwidth, i.e. positioning accuracy less than 3m for 95% of UEs for absolute positioning with bandwidth 100MHz and 40MHz.

### 2.2.3 Scenario 3—InF-SH

Table 11 to Table 12 give the evaluation parameters for absolute positioning in InF-SH scenario. Figure 6 and Table 13 show the CDF errors of simulation results for absolute positioning in InF-SH scenario. Table 14 gives the evaluation parameters for relative positioning in InF-SH scenario. Figure 7 and Table 15 show the CDF errors of simulation results for relative positioning in InF-SH scenario.

Table 11. Evaluation parameters for absolute positioning in indoor factory scenario

| Parameters | Case 19 | Case 20 | Case 21 | Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | BSs and anchor UEs | Only BSs | Only anchor UEs | BSs and UEs | Only BSs | Only anchor UEs | BSs and UEs | Only BSs | Only anchor UEs |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Bandwidth | 100MHz | 100MHz | 100MHz | 40MHz | 40MHz | 40MHz | 20MHz | 20MHz | 20MHz |
| Number of BSs | 18 | 18 | 0 | 18 | 18 | 0 | 18 | 18 | 0 |
| Number of anchor UEs | 20 | 0 | 20 | 20 | 0 | 20 | 20 | 0 | 20 |
| Number of target UEs | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm |
| Additional notes, if any | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel | Normal channel |
| Positioning mode | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning | Absolute positioning |

Table 12. Evaluation parameters for absolute positioning in indoor factory scenario

| Parameters | Case 28 | Case 29 | Case 30 |
| --- | --- | --- | --- |
| Anchor nodes | BSs and anchor UEs | Only BSs | Only anchor UEs |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Bandwidth | 40MHz | 40MHz | 40MHz |
| Number of BSs | 18 | 18 | 10 |
| Number of anchor UEs | 20 | 0 | 20 |
| Number of target UEs | 20 | 20 | 20 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm | TDOA, Gauss-Newton algorithm |
| Additional notes, if any | Ideal LOS path | Ideal LOS path | Ideal LOS path |
| Positioning mode | Absolute positioning | Absolute positioning | Absolute positioning |

Figure 6. Curves of CDF errors for absolute positioning in indoor factory scenario

Table 13. CDF errors for absolute positioning in InF-SH scenario

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Case 19 | 0.145 | 0.1807 | 0.2224 | 0.2823 | 0.3457 |
| Case 20 | 0.2084 | 0.2762 | 0.347 | 0.4907 | 0.5529 |
| Case 21 | 0.1688 | 0.2294 | 0.302 | 0.4154 | 0.5173 |
| Case 22 | 0.4283 | 0.5397 | 0.7021 | 0.8509 | 1.076 |
| Case 23 | 0.7401 | 1.002 | 1.291 | 1.595 | 2.054 |
| Case 24 | 0.4485 | 0.5785 | 0.8091 | 1.038 | 1.325 |
| Case 25 | 0.742 | 0.9484 | 1.19 | 1.452 | 1.667 |
| Case 26 | 1.445 | 1.808 | 2.38 | 3.075 | 3.768 |
| Case 27 | 0.7485 | 0.9504 | 1.267 | 1.675 | 1.973 |
| Case 28 | 0.3527 | 0.4837 | 0.6308 | 0.7306 | 0.7857 |
| Case 29 | 0.6745 | 0.9214 | 1.154 | 1.438 | 1.712 |
| Case 30 | 0.3905 | 0.5072 | 0.6911 | 0.8276 | 1.08 |

Table 14. Evaluation parameters for relative positioning in indoor factory scenario

| Parameters | Case 31 | Case 32 | Case 33 |
| --- | --- | --- | --- |
| Anchor nodes | Anchor UEs | Anchor UEs | Anchor UEs |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Bandwidth | 100MHz | 40MHz | 20MHz |
| Number of target UEs | 20 | 20 | 20 |
| Description of measurement algorithm | MUSIC | MUSIC | MUSIC |
| Additional notes, if any | Normal channel | Normal channel | Normal channel |
| Positioning mode | Relative positioning | Relative positioning | Relative positioning |

Figure 7. Curves of CDF errors for relative positioning in InF-SH scenario

Table 15. CDF errors for relative positioning in InF-SH scenario

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Case 31 | 0.1741 | 0.2776 | 0.3925 | 0.714 | 1.049 |
| Case 32 | 0.3589 | 0.5859 | 0.8929 | 0.974 | 1.314 |
| Case 33 | 0.6745 | 1.092 | 1.727 | 2.995 | 4.963 |

Observation 3: In indoor factory scenario, all techniques can meet the requirement of Set 1 described in TR 38.845, i.e. positioning accuracy less than 50m for 95% of UEs. If both TRPs and RSUs are used to locate target UEs, absolute positioning can meet the requirement of Set 2 described in TR 38.845 without bandwidth limitation, e.g., positioning accuracy less than 3m for 95% of UEs. Besides, relative positioning can meet the requirement of Set 2 described in TR 38.845 with sufficient bandwidth, i.e. positioning accuracy less than 3m for 95% of UEs with bandwidth larger than or equal to 40MHz.

- Conclusion

In this contribution, we provide the evaluation methodology and scenarios to evaluate SL positioning. In addition, SL positioning simulation results are showed.

Proposal 1: Only in-coverage and out-of-coverage scenarios are considered for SL positioning simulation purpose where both absolute positioning and relative positioning can be considered for in-coverage case, while only relative positioning is considered for out-of-coverage case.

Proposal 2: For absolute positioning, TDOA or RTT are supported. For relative positioning, RTT and AOA are supported.

Proposal 3: SL positioning evaluation work should focus on FR1.

Proposal 4: For evaluation of SL positioning, three scenarios are suggested, including highway, urban and indoor factory.

Proposal 5: The parameters listed in Table 1 are supported for evaluating SL positioning in highway scenario.

Table 1. Parameters in highway for evaluation of SL positioning

|  |  | Absolute positioning | Relative positioning |
| --- | --- | --- | --- |
| Layout | Layout of highway | According to TR 37.885, 3 lanes in each direction (6 lanes in total), lane width 4m, highway length is not less than 2000m |  |
|  | RSU location | According to TR 37.885, uniformly allocated with 100m spacing in the middle of highway | RSUs are disabled |
|  | BS location | Allocated on both side of the road, inter-site distance is 500m | BSs are disabled |
|  | Illustration of layout |  |  |
| UE drop |  | According to TR 37.885, Type 2 UEs and Option A dropping are used. |  |
| Anchors selection |  | All BSs and RSUs can be used | Relative positioning is performed for two Ues within 100m |
| SL channel model |  | UE2UE described in TR 37.885 |  |
| Channel model between BS and UE |  | Rma described in TR 37.885, LOS propagation type is used for V2B links | No model |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |  |
| Antenna model |  | According to TR 37.885, Option 1 (single panel) as the baseline and Option 2 (two panels) as optional |  |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |  |

Proposal 6: In urban scenario, SL positioning simulation should be focused on relative positioning case. The parameters listed in Table 2 are supported for evaluating SL relative positioning in urban scenario.

Table 2. Parameters in urban for evaluation of SL positioning

|  |  | Relative positioning |
| --- | --- | --- |
| Layout | Layout of urban | According to TR 37.885, 2 lanes in each direction (4 lanes in total), lane width 3.5m, road grid size by the distance between intersection 433m*250m |
|  | Illustration of layout |  |
| UE drop |  | According to TR 37.885, Type 2 UEs and Option A dropping are used. |
| Anchors selection |  | UEs within 100m of target UE, i.e. relative positioning is only performed between two UEs within 100m |
| SL channel model |  | UE2UE described in TR 37.885 |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |
| Antenna model |  | According to TR 37.885, Option 1 (single panel) as the baseline and Option 2 (two panels) as optional |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |

Proposal 7: The parameters listed in Table 3 are supported for evaluating SL positioning in InF-SH scenario.

Table 3. Parameters in InF-SH for evaluation of SL positioning

|  |  | Absolute positioning | Relative positioning |
| --- | --- | --- | --- |
| Layout | Layout of hall | According to TR 38.857, hall size 300m*150m, room height 10m, |  |
|  | BS location | According to TR 38.857, 18 BSs are set on a square lattice with spacing 50m, located 25m from the wall. | BSs are disabled |
|  | Illustration of layout |  |  |
| UE drop |  | According to TR 38.857, antenna height 1.5m, speed 3km/h, uniformly distributed at the convex hull of the horizontal BS deployment |  |
| Anchors selection |  | All BSs and 20 anchor UEs randomly selected are used to locate target UE | Relative positioning is performed for two UEs within 10m |
| SL channel model |  | Modify InF-SH described in TR 38.901, i.e. replace BS with the anchor UE in the channel model of BS-2-UE, where anchor UE height, transmit power are used to replace gNB’s. |  |
| Channel model between BS and UE |  | InF-SH described in TR 38.901 | No model |
| Bandwidth |  | 20MHz, 40MHz and 100MHz |  |
| Antenna model |  | According to TR 38.857 |  |
| Performance metrics |  | Percentiles of positioning error: 50%, 67%, 80%, 90% and 95% |  |

Observation 1: In highway scenario, all techniques can meet the requirement of Set 1 described in TR 38.845, i.e. positioning accuracy is less than 50m for 95% of UEs. If both TRPs and RSUs are used to locate target UEs, the requirement of Set 2 described in TR 38.845 can be satisfied in positioning accuracy less than 3m for 95% of UEs for absolute positioning and for relative positioning without bandwidth limitation.

Observation 2: In urban scenario, relative positioning can meet the requirement of Set 1 described in TR 38.845 without bandwidth limitation, i.e. positioning accuracy less than 50m for 95% of UEs. The requirement of Set 2 described in TR 38.845 can be satisfied by relative positioning with sufficient bandwidth, i.e. positioning accuracy less than 3m for 95% of UEs for absolute positioning with bandwidth 100MHz and 40MHz.

Observation 3: In indoor factory scenario, all techniques can meet the requirement of Set 1 described in TR 38.845, i.e. positioning accuracy less than 50m for 95% of UEs. If both TRPs and RSUs are used to locate target UEs, absolute positioning can meet the requirement of Set 2 described in TR 38.845 without bandwidth limitation, e.g., positioning accuracy less than 3m for 95% of UEs. Besides, relative positioning can meet the requirement of Set 2 described in TR 38.845 with sufficient bandwidth, i.e. positioning accuracy less than 3m for 95% of UEs with bandwidth larger than or equal to 40MHz.

- Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning, RAN#94e.

- 3GPP TR 37.885. “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”.

- 3GPP TR 38.901. “Study on channel model for frequencies from 0.5 to 100 GHz”.

- 3GPP TR 38.857. “Study on NR Positioning Enhancements”.

- 3GPP TS 38.845. “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”.

- R1-2203622 Discussion on scenarios and requirements for SL positioning ZTE.

- 3GPP TR 38.889-g00. “Study on NR-based access to unlicensed spectrum”.

- 3GPP TR 36.885-e00. “Study on LTE-based V2X services (Release 14)”