3GPP TSG RAN WG1 #109-e                                               R1-2203624

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Discussion on potential solutions for SL positioning

Agenda item:    9.5.1.3

Document for:	Discussion and Decision

Introduction

In RAN#94e meeting, an SID including sidelink positioning was approved for Rel-18 as follows [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we provide some views and considerations on potential solutions for SL positioning including the SL positioning structure, SL-PRS (sidelink positioning reference signal) design, SL positioning methods and frequency aspects.

Discussion

2.1 SL positioning structure

### 2.1.1 Positioning architecture in Rel-15/16

In Rel-15/16, in order to obtain the location information of a target UE, reference signals (downlink signals/uplink signals) transmitted between UEs and gNBs/ng-eNBs can be used to locate the target UE. Figure 2.1.1-1 specified in TS 38.305 [2] shows the architecture in 5GS applicable to positioning of a UE with NR or E-UTRA access. Among the elements related to UE positioning, the LMF may interact with a target UE (through LPP) in order to deliver assistance data or to obtain a location estimate, and the LMF may also interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, moreover, the LMF can determine the positioning methods based on UE positioning capabilities and gNB/ng-eNB information report and other factors.

Figure 2.1.1-1: UE Positioning Overall Architecture applicable to NG-RAN [2]

For the design of NG-RAN UE SL positioning architecture, in order to reduce the spec impact, we suggest to take the existing positioning architecture (including communications between UE and NG-RAN, UE and LMF, NG-RAN and LMF) as a baseline and introduce or update the UE-2-UE positioning module.

Observation 1: The existing NG-RAN positioning architecture is not applicable to SL positioning.

### 2.1.2 SL positioning structure

Figure 2.1.2-1 specified in TS 23.287 [3] shows the high level view of the roaming 5G System architectures for V2X communication over PC5 and Uu reference points.

Figure 2.1.2-1: Roaming 5G System architecture for V2X communication over PC5 and Uu reference points - Local breakout scenario [3]

The protocol stack of control plane for SCCH for RRC and for PC5-S (PC5 signaling) are shown in Figure 2.1.2-2 according to TS 38.300 [4]. The AS protocol stack for the control plane for SCCH for RRC in the PC5 interface consists of RRC, PDCP, RLC, MAC sublayers, and the physical layer. Also, for support of PC5-S protocol specified in TS 23.287 [3], PC5-S is located on top of PDCP, RLC, MAC sublayers, and the physical layer in the control plane protocol stack for SCCH.

| Figure 2.1.2-2.1: Control plane protocol stack for SCCH for RRC. | Figure 2.1.2-2.2: Control plane protocol stack for SCCH for PC5-S. |
| --- | --- |

Figure 2.1.2-2: Control plane protocol stack for SCCH [4]

In current sidelink communication, PC5-S signaling is used for SL unicast connection. There is a one-to-one correspondence between PC5-RRC connection and the PC5 unicast link. The PC5-RRC procedures and messages can be used for a UE to transfer UE capability and sidelink configurations to peer UEs where the UE may have multiple PC5-RRC connections with one or more UEs.

For the high layers structure of SL positioning, in order to reuse the current configuration as much as possible and reduce the specification complexity, UE may also firstly set up unicast link via PC5-S signaling which contains location request, and PC5-RRC signaling can be used for SL positioning capability transfer and SL positioning assistance data transfer. Furthermore, where to process the measurement request/report (e.g.PC5-S, PC5-RRC, or others) should also be considered. Figure 2.1.2-3 shows one of the SL positioning structure we considered where the measurement request/report can be delivered through PC5-S signaling, it is like sidelink LPP signaling.

Figure 2.1.2-3: SL positioning structure

Proposal 1: Consider the SL positioning structure design via PC5 interface including PC5-S and PC-5 RRC.

2.2 Reference signals for SL positioning

### 2.2.1 RS design

The design of SL-PRS (sidelink positioning reference signal) is one of an essential and key part of the NR sidelink positioning system. In order to meet the positioning accuracy requirement and reduce complexity, DL-PRS like signals can be considered. Specifically, the following aspects should be considered for SL-PRS design referring to the DL-PRS configuration [5]:

- Sequence: gold sequence or ZC (Zadoff-Chu) sequence

  - Gold sequence is preferable to align with sidelink CSI-RS design.

- SL-PRS Pattern

  - Comb size

  - Number of OFDM symbols within a slot

- Periodicity: specify the periodicity of SL-PRS and the modification based on the DL-PRS’s periodicity.

- Repetition

Proposal 2: For SL-PRS design, consider DL-PRS like signals to reduce the specification complexity.

Due to the half-duplex restriction, the UE cannot transmit and receive sidelink resource at the same time. Furthermore, like Rel-16/17 design, it is better not to support SL-PRS transmitted or received with SL-data (sidelink data) at the same time for the purpose of low latency, low UE complexity and high measurement accuracy. Figure 2.2.1-1 shows one example of FDM between SL-PRS and SL-data which we do not prefer, we suggest to only consider TDM between SL-PRS and SL-data.

Figure 2.2.1-1 FDM between SL-PRS and SL-data

Proposal 3: In Rel-18, TDM between SL-PRS and SL-data is only considered.

In addition, like other sidelink channel design, gap symbol(s) may be needed before and/or after SL-PRS symbols for Rx/Tx switch. Moreover, in comparison to the SL-data/PSSCH, the transmit power and power control of the SL-PRS may be different. Considering the AGC issue, one AGC symbol is also needed before the SL-PRS symbols and after the gap symbol before SL-PRS if configured to assist the UE receiver’s AGC tuning, this AGC symbol is a repetition of the first SL-PRS symbol which is the actual starting symbol. To sum up, we can consider adding extra symbols (AGC symbol and gap symbol) for SL-PRS. The number of SL-PRS symbols within a slot can be flexibly configured: {2+2/3, 4+2/3, 6+2/3, 12+2/3, ...}. One example of SL-PRS pattern considering comb size and symbol numbers are shown in Figure 2.2.1-2.

Figure 2.2.1-2: one example of SL-PRS pattern (comb-2; 2+3 symbols)

Proposal 4: For the number of OFDM symbols within a slot for SL-PRS, consider adding AGC symbol for power adjustment and gap symbol(s) for Rx/Tx switch.

### 2.2.2 SL-PRS configuration

It is well known that large bandwidth is required for high-accuracy positioning if timing based positioning methods are used, in other words, the larger the bandwidth, the higher the positioning accuracy. However, current sidelink RS in Rel-15/16 is always configured and limited within the bandwidth of PSSCH/SL-data resource pool. If the SL-PRS is configured similarly as current sidelink RS within the frequency range of PSSCH, it is difficult to satisfy the positioning requirement. Therefore, we need to consider solutions for large bandwidth SL-PRS configuration.

Observation 2: If the SL-PRS is configured within the bandwidth of PSSCH, it is difficult to satisfy the positioning requirement.

In Rel-15/16, NR sidelink resource pool is a set of time and frequency resources that can be used for sidelink transmission and/or reception. A UE can be (pre)configured by higher layers with one or more sidelink resource pool. The hierarchical sidelink resource configuration is SL Frequency --> SL BWP --> SL resource pool according to TS 38.331 [6]. Moreover, only one BWP is allowed to be configured on one carrier frequency for NR sidelink communication and this particular one BWP is used for both transmission and reception.

For SL-PRS resources (pre)configuration, in our understanding, there are two options as below.

- Dedicated SL-PRS configuration

In this option, SL-PRS resources can be (pre)configured separately from SL resource pool on the carrier frequency level or BWP level or resource pool level. For carrier frequency level or BWP level SL-PRS resource configuration, we can separately define carrier frequency for SL-PRS and BWP for SL-PRS like as Rel-16/17 DL-PRS frequency layer. This may not be necessary since currently there is only one carrier frequency and one BWP supported for data in order to avoid the situation that one UE has to transmit or receive sidelink data on multiple BWPs simultaneously since group-cast and broadcast is supported. For resource pool level configuration as shown in Figure 2.2.2-1, the SL-PRS and SL-data belong to the same BWP but different resource pool configuration. Each pool/configuration can occupy a part of the BWP resource including time and frequency resources.

Figure 2.2.2-1: Dedicated SL-PRS configuration - resource pool level

If dedicated SL-PRS configuration is applied, we can allocate larger frequency bandwidth for SL-PRS for positioning accuracy improvement. In this case, the frequency range of SL-PRS can be larger than PSSCH, one SL-PRS resource may be associated with one or more SL-data resource(s). There are some relationships between SL-PRS resource configuration and SL-data resource pools need further be considered as SL-PRS may be triggered by SCI.

- Configure SL-PRS in SL resource pool

SL-PRS resources can also be configured in SL resource pool, in other words, SL-PRS and SL-data transmission/reception share the same resource pools and same logic slots. However, as we clarified in the beginning of this section, large bandwidth is required for high-accuracy positioning. Therefore, extending SL-PRS outside the SL resource pool in terms of frequency domain should be considered to satisfy the positioning accuracy requirement. Figure 2.2.2-2 shows that even though the SL-PRS is configured in one SL resource pool, the frequency domain range of SL-PRS can be larger than two (or one or several) SL resource pools.

Figure 2.2.2-2: Extending SL-PRS outside the SL resource pool

Proposal 5: For SL-PRS resource configuration, we are open to study the following options:

- Alt 1: Dedicated SL-PRS configuration

  - SL-PRS configuration is separate from SL resource pool.

- Alt 2: SL-PRS is configured in SL resource pool

For the triggering of SL-PRS, there are also two ways: SL-PRS scheduled by SCI or standalone SL-PRS. If standalone SL-PRS similar as PSBCH is applied, the scheduling of SL-PRS is not flexible enough and standalone SL-PRS is more suitable for periodic SL-PRS. However, it may cause resource waste and resource collision especially for sidelink resource selection mode 2. Hence, we prefer to support SCI triggered SL-PRS. If sidelink resource collision happens, UEs need to be aware of other UE’s SL-PRS configuration and how to inform the information without SCI need further studied. It is noted that this issue won’t happen for S-SSB because the set of slots of the sidelink resource pool exclude slots in which S-SSB is configured.

Therefore, in our understanding, using SCI to schedule SL-PRS is preferred. A UE can use SCI to request or transmit SL-PRS configuration to other UE. A UE can detect SCI to further obtain the SL-PRS related information and one SCI may schedule multiple SL-PRS occasions for overhead reduction as shown in Figure 2.2.2-3. Also, we need consider 2 cases if SCI triggering SL-PRS is supported: SCI can schedule both SL-PRS and SL-data; SCI can either schedule SL-PRS or SL-data.

Figure 2.2.2-3 SCI triggers X SL-PRS occasions

Proposal 6: Support using SCI to trigger SL-PRS and consider the following 2 cases:

- SCI can schedule both SL-PRS and SL-data;

- SCI can either schedule SL-PRS or SL-data.

### 2.2.3 SL-PRS resource allocation

For current sidelink communication, gNB schedules sidelink resources for UEs in sidelink resource allocation mode 1, on the contrary, sidelink resource allocation mode 2 is a contention based scheme with Tx UE selecting sidelink control and data resources for its transmission.

According the revised SID approved in RAN#94e meeting, sidelink positioning coverage scenarios include in-coverage, partial-coverage and out-of-coverage. For in-coverage and partial-coverage scenarios, at least one UE is inside the network coverage so both model 1 and mode 2 resource allocation can be applied. For out-of-coverage scenario, the network cannot provide/determine the set of resources used for SL-PRS transmission, then resource allocation mode 2 can be applied. Therefore, we support both mode 1 and mode 2 resource allocation for SL positioning.

Proposal 7: Support both Mode 1 and Mode 2 resource allocation for SL positioning.

In sidelink resource allocation mode 1 according to TS 38.214 [4], for SL data transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported.

- For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

- For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0.

- For sidelink configured grant type 1, the PSSCH transmission follows the higher layer configuration.

If mode 1 resource allocation for SL positioning is supported, DCI can be used to dynamically or semi-persistently schedule the SL-PRS. We should consider whether a new DCI format is needed: (1) we can reuse the DCI format 3_0 and add some fields related to SL-PRS transmission. (2)We can also introduce a new DCI format exclusively for SL-PRS.

Proposal 8: Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format other than DCI format 3_0 is needed.

### 2.2.4 Power control for SL-PRS

In TS 38.213 [8], power control has been formulated for physical sidelink control channel (PSCCH) transmission and physical sidelink shared channel (PSSCH) transmission based on UE’s capability and pathloss. For example, the formula for a UE determines a power for a PSSCH transmission is as follows:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

For positioning purpose, anchor UEs transmit SL-PRS to a target UE, the target UE obtains the positioning measurements by measuring SL-PRS. Then, wireless dependent positioning methods can be used to calculate the target UE’s location. However, the transmit power and power control of the SL-PRS is still an open research. The power of SL-PRS transmission may be different from S-SSB block, PSSCH, PSCCH or PSFCH, then the related discussion is needed for a specific power control formula and related parameters for SL-PRS transmission.

Proposal 9: Study power control for SL-PRS transmission.

2.3 SL positioning methods

For Rel-18 NR sidelink positioning, in order to obtain the location information of a target UE, positioning methods make use of reference signals (e.g. SL-PRS) transmitted between UEs and the resulting measurements can be used to locate the target UE.

In the scenarios of in-coverage and partial-coverage, absolute positioning can be achieved based on TRP’s location. In such case, the positioning measurement results from both Uu and PC5 interface can be reported to LMF or UE. Especially in some scenarios with anchor UEs, e.g. RSUs which can be utilized as TRPs for positioning purpose, then all legacy positioning methods are applicable in our view including RTT, TDOA and angle based positioning methods. Also, relative positioning is applicable as two UEs can directly perform sidelink positioning measurement and location computation.

Figure 2.3-1 RTT and TDOA for sidelink positioning

In out-of-coverage scenarios, only relative positioning, i.e. UE-2UE location may be achieved. In such case, TDOA may not work anymore. That is, only RTT and angle based positioning methods are applicable.

Proposal 10: From standard perspective, support all in-coverage, partial-coverage and out-of-coverage scenarios. The legacy positioning methods should be reused.

- For in-coverage and partial-coverage, support both absolute positioning and relative positioning.

- For out-of-coverage, support relative positioning which TDOA may not be applicable.

For timing-based methods as shown in Figure 2.3-1:

- In TDOA positioning method, the target UE position is estimated based on SL-PRS-RSRP/SL-PRS RSTD (or maybe PRS-RSRPP introduced in Rel-17) measurements taken at the UE of sidelink signals from multiple peer/neighbour UEs. The legacy DL-TDOA/UL-TDOA positioning methods need time synchronization information between a reference TRP and a list of neighbour TRPs in Rel-15/16 to meet the positioning accuracy requirements. Therefore, time synchronization requirement need to be considered for TDOA method of SL positioning and several issues are related to the requirement: the time synchronization reference’s restriction, selection and determination and its signaling procedure and parameters.

- In RTT positioning method, the UE location is estimated based on measurements performed between two UEs (Tx UE and Rx UE). RTT does not require accurate synchronization between UEs.

Observation 3: Time synchronization between anchor UEs need to be considered for SL TDOA positioning.

2.4 Views on frequency aspects of SL positioning

For Rel-18 NR positioning, unlicensed spectrum was suggested by some companies for the sake of larger bandwidth of PRS/SRS and higher positioning accuracy for timing based positioning methods. However, LBT operation and collision nature of unlicensed band operation could seriously impact the positioning signaling (e.g., reference signal, measurement, report etc.) transmission and reception, it could further influence the accuracy and the latency which is important for some sidelink use cases. Hence, we suggest focusing on ITS and licensed spectrum in Rel-18.

Moreover, Rel-16/17 sidelink does not support unlicensed spectrum. To include unlicensed spectrum in Rel-18 NR positioning, fundamental mechanism of sidelink in unlicensed spectrum, like LBT, shall be specified. In our view, the fundamental mechanism should not be made only for SL positioning but also for SL communication. Hence, we suggest to specify SL positioning in unlicensed spectrum after completing specifying SL communication in unlicensed spectrum.

Proposal 11: Rel-18 NR sidelink positioning can focus on ITS and licensed spectrum first.

In addition, Rel-16/17 sidelink mainly focused on FR1, and it may not work well in FR2 due to lack of basic FR2 functionalities, e.g. beam management. There is a study bullet point of Rel-18 NR sidelink enhancement (excluding positioning and relaying) on FR2 licensed spectrum. In our view, the same beam management mechanism should be used for both regular SL operation and SL positioning in FR2. So, to avoid overlapping discussion between Rel-18 SL and positioning group, we suggest to specify SL positioning in FR2 after completing specifying regular SL communication. That is, to deprioritize FR2 enhancement for SL positioning in Rel-18.

Proposal 12: Deprioritize Rel-18 NR sidelink positioning in FR2.

Conclusion

In this contribution, we discuss potential solutions for SL positioning, and we have the following observations and proposals:

Observation 1: The existing NG-RAN positioning architecture is not applicable to SL positioning.

Observation 2: If the SL-PRS is configured within the bandwidth of PSSCH, it is difficult to satisfy the positioning requirement.

Observation 3: Time synchronization between anchor UEs need to be considered for SL TDOA positioning.

Proposal 1: Consider the SL positioning structure design via PC5 interface including PC5-S and PC-5 RRC.

Proposal 2: For SL-PRS design, consider DL-PRS like signals to reduce the specification complexity.

Proposal 3: In Rel-18, TDM between SL-PRS and SL-data is only considered.

Proposal 4: For the number of OFDM symbols within a slot for SL-PRS, consider adding AGC symbol for power adjustment and gap symbol(s) for Rx/Tx switch.

Proposal 5: For SL-PRS resource configuration, we are open to study the following options:

- Alt 1: Dedicated SL-PRS configuration

  - SL-PRS configuration is separate from SL resource pool.

- Alt 2: SL-PRS is configured in SL resource pool

Proposal 6: Support using SCI to trigger SL-PRS and consider the following 2 cases:

- SCI can schedule both SL-PRS and SL-data;

- SCI can either schedule SL-PRS or SL-data.

Proposal 7: Support both Mode 1 and Mode 2 resource allocation for SL positioning.

Proposal 8: Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format other than DCI format 3_0 is needed.

Proposal 9: Study power control for SL-PRS transmission.

Proposal 10: From standard perspective, support all in-coverage, partial-coverage and out-of-coverage scenarios. The legacy positioning methods should be reused.

- For in-coverage and partial-coverage, support both absolute positioning and relative positioning.

- For out-of-coverage, support relative positioning which TDOA may not be applicable

Proposal 11: Rel-18 NR sidelink positioning can focus on ITS and licensed spectrum first.

Proposal 12: Deprioritize Rel-18 NR sidelink positioning in FR2.

Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94e.

- 3GPP TS 38.305 h00: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RANPhysical layer procedures for control".

- 3GPP TS 23.287 h20: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

- 3GPP TS 38.300 h00: "NR; NR and NG-RAN Overall Description; Stage 2".

- 3GPP TS 37.355 h00: "LTE Positioning Protocol (LPP) (Release 17)".

- 3GPP TS 38.331 g80: "Radio Resource Control (RRC) protocol specification (Release 16)".

- 3GPP TS 38.214 h00: "NR; Physical layer procedures for data (Release 17)".

- 3GPP TS 38.213 h00: "NR; Physical layer procedures for control (Release 17)".

- 3GPP TS 22.261 i60: "Service requirements for the 5G system; Stage 1 (Release 17)".