3GPP TSG RAN WG1 #109-e                                               R1-2203627

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Discussion on low power high accuracy positioning(LPHAP)

Agenda item:    9.5.2.3

Document for:	Discussion and Decision

Introduction

According to the Rel-18 positioning SID[1], the low power high accuracy positioning(LPHAP) is described as below:

- Improved accuracy, integrity, and power efficiency: - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

This contribution shares some views about LPHAP.

Discussion

2.1 LPHAP use cases

The SA1#94 meeting in May 2021 approved the use cases and requirements of LPHAP [2], and nine use cases have been adopted in section A.7.2 of [3] as follows:

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

It can be seen that different use cases have different requirements of accuracy, interval and power consumption. Among which, use case 5 and 6 are both balanced in the KPIs: neither too restrict nor too loose on the power consumption but also has a pretty good accuracy requirement of less than 1m. Furthermore, due to the consideration of the workload and the indication of SID, it is better to select only one use case as the goal of Rel-18 and for evaluation work. Therefore, we suggest to evaluate use case 5 or use case 6 for the Rel-18 LPHAP.

Proposal 1: Support use case 5 or use case 6 for the Rel-18 LPHAP.

2.2 Power consumption model

In the past, positioning technology only focused on the accuracy and latency enhancements. For Rel-18 LPHAP, one significant difference is to treat low power consumption as equal important as high accuracy, in order to suit the large amount of low-power UEs in the market. Therefore, how to evaluate the power consumption of different technical enhancements/approaches and further weigh the performance becomes a critical issue. To accomplish this, a unified power consumption model should be applied among companies for evaluation.

In TS 38.840[4] section 8.1, the UE power consumption model (including relative power of different RS/channels sending/receiving and state transaction) is specified as below:

Table 18: UE power consumption model for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 19: UE power consumption during the state transition

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

Table 21: UE power consumption scaling for adaptation

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (DL) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. | For 10MHz BW, only AL up to 8 can be used for PDCCH The transition time is the same as DCI-based BWP switching delay for Rel-15. If the power after scaling is smaller than the BWP transition power, assume the BWP transition power as the output of scaling unless otherwise justified. |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. |  |
| CA (DL) | 2CC is 1.7x1CC 4CC is 3.4x1CC (i.e. 2x 2CC) Above refers to the worst case CA configuration in terms of power consumption. | Activation/deactivation delay follows RAN4 specification; FFS transition energy Applicable for FR1 and FR2 |
| CA (UL) | Same as downlink at 0dBm. No scaling at 23dBm 2CC is 1.2x1CC at 23dBm Limit scaling up to 2CC. | Applicable for FR1 and FR2 |
| Antenna scaling (DL) | 2Rx power is 0.7x 4Rx power for FR1 1Rx power is 0.7x 2Rx power for FR2 | Assume same number of antenna elements per Rx chain |
| Antenna scaling (UL) | 2Tx power is 1.4x 1Tx power at 0dBm. 1.2x.at 23dBm FR1 only | 2Tx support is not considered for FR2. |
| PDCCH-only | Power of cross-slot scheduling is 0.7x same-slot scheduling | Applicable for FR1 and FR2 |
| SSB | One SSB power is 0.75 of two SSB power, i.e. 75 power units |  |
| PDSCH-only slot | 280 for FR1 325 for FR2 | This assumes the same number of PDSCH symbols as in the PDCCH+PDSCH case. |
| CSI-RS | FFS for scaling w.r.t. # of symbols for CSI-RS |  |
| Short PUCCH | Short PUCCH power = 0.3 x uplink power Reference config consists of 1-symbol PUCCH | Applicable for FR1 and FR2. |
| SRS | SRS power = 0.3 x uplink power | Applicable for FR1 and FR2. |

The set of tables specifies the complete relative power/power scaling factor of almost all the channel/reference signals. We see it as a feasible unified assumption to evaluate the power consumption of different technical approaches. Therefore, the UE power consumption model in TS 38.840 can be the start point of Rel-18 LPHAP power evaluation. For example, the highlight parts in above tables may be helpful to define power consumption for positioning.

Proposal 2: Support to utilize the UE power consumption model in TS 38.840 as a starting point to evaluate the power consumption for positioning.

2.3 Rel-18 technical approaches

In Rel-17, we have agreed to support deferred MT-LR in RRC_INACTIVE, that is, firstly UE receives the deferred MT-LR request in RRC_CONNECTED period, and then UE keeps detecting events and may trigger event report in the subsequent RRC_INACTIVE period. According to the following agreement RAN2 made at RAN2#114:

Agreement of RAN2#114: - Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective. - Follow Rel-17 SDT framework for INACTIVE UL and DL positioning: - If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP message and RRC message (e.g. to configure SRS (TBD on what message is used), if UL positioning supported) to the UE. - Otherwise, if UE did not initiate UL SDT, rely on legacy operation, i.e. the network shall transition the UE to RRC_CONNECTED, e.g. based on RAN paging.

It can be seen that in Rel-17, only if UE initiates UL SDT can UE receive LCS message smoothly in RRC_INACTIVE without transmitting to RRC_CONNECTED. That is to say, the prerequisite of supporting MT-LR in RRC_INACTIVE is UE should firstly initiated UL SDT. However, positioning service from LCS client will trigger at any time, in the meanwhile core network does not know UE’s RRC state and UE’s SDT status. Note that usually SDT active period takes small percentage of the whole RRC_INACTIVE period, random LCS/LPP messages related positioning may lead to frequent RAN paging and UE has to wake up frequently from RRC_INACTIVE. Thus, it is rather a waste of power.

To reduce the power consumption of the network-initiated positioning request, Rel-18 should further support MT-LR in RRC_INACTIVE. Below shows the Rel-18 WID[5] of Mobile Terminated-Small Data Transmission (MT-SDT):

- Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. - Note: Data transmission in DL within paging message is not in scope of this WI.

MT-SDT is an approach to trigger DL message transmission by the NG-RAN node in RRC_INACTIVE, where the DL message can also convey the LCS/LPP message generated at core network. That is to say, MT-SDT facilitates the enhancement of MT-LR in RRC_INACTIVE.

Proposal 3: Support MT-LR via MT-SDT in RRC_INACTIVE in Rel-18.

As the Rel-18 SID indicates, the LPHAP study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state. In the past several months, Rel-17 has accomplished a pretty good start on the positioning in RRC_INACTIVE, especially for UL/DL+UL positioning by supporting the periodic/semi-persistent positioning SRS in RRC_INACITVE. However, there are still some room to further reduce UE power consumption for UL positioning in RRC_INACTIVE. Therefore, we suggest to continue the study of the UL/DL+UL positioning in RRC_INACTIVE/RRC_IDLE in Rel-18.

Proposal 4: In Rel-18, continue the study of the UL/DL+UL positioning in RRC_INACTIVE/RRC_IDLE for lower power consumption.

In Rel-17, the following agreements are achieved:

Agreement of RAN2#116-bis: - The SRSp configuration is considered as invalid if TA is not valid. - When cell reselection is performed and UE initiates RRC resume procedure to the cell which is different from the cell in which the SRSp is configured, the TA timer configuration for SRS should be released. - The SRSp configuration is released when the UE sends RRCResumeRequest to a cell other than the cell where it is released to RRC_INACTIVE state. Agreement of RAN2#117: - TA timer configuration of SRS for positioning (SRSp) is invalidated upon any cell reselection (i.e. even if the UE does not initiate the RRC resume procedure)

These agreements indicate that, the SRS configuration for RRC_INACTIVE is only restricted in the serving cell that the UE connects lastly. If cell re-selection is performed while UE still needs positioning (which is very likely to happen in wearable devices), UE has to reach to RRC_CONNECTED to receive new SRS configuration embedded in RRC reconfiguration. According to the power consumption model introduced above, this action will lead to a huge power waste. Therefore, Rel-18 should further enhance the UE mobility of UL positioning in RRC_INACTIVE/RRC_IDLE, in order to ensure the low power consumption.

Proposal 5: Rel-18 should further enhance the UE mobility of UL positioning in RRC_INACTIVE/RRC_IDLE, in order to ensure the low power consumption.

Conclusion

In this contribution, we provide our views about LPHAP as follows

Proposal 1: Support use case 5 or use case 6 for the Rel-18 LPHAP.

Proposal 2: Support to utilize the UE power consumption model in TS 38.840 as a starting point to evaluate the power consumption for positioning.

Proposal 3: Support MT-LR via MT-SDT in RRC_INACTIVE in Rel-18.

Proposal 4: In Rel-18, continue the study of the UL/DL+UL positioning in RRC_INACTIVE/RRC_IDLE for lower power consumption.

Proposal 5: Rel-18 should further enhance the UE mobility of UL positioning in RRC_INACTIVE/RRC_IDLE, in order to ensure the low power consumption.

Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning.

- S1-210366, Adding energy efficiency use cases for positioning to the ANNEX A, 2021-03.

- TS 22.104, Service requirements for cyber-physical control applications in vertical domains, 2021-03.

- TS 38.840, Study on User Equipment (UE) power saving in NR, 2019-06.

- RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, 2021-12