3GPP TSG RAN WG1 Meeting #109-e	R1-2203634

e-Meeting, May 9th – 20th , 2022

Agenda Item:	9.5.2.2  Improved accuracy based on NR carrier phase measurement

Source:	Locaila, Inc

Title:	Use cases and applications on Carrier Phase Based Positioning for NR

Document for:	Discussion & Decision

Introduction

Carrier phase positioning is one of the important objectives in the 5G NR Positioning WID [1]. When the carrier phase measurement method is applied in 5G NR positioning, great service improvements can be achieved. Examples of the use case are 1) urban macro LBS services, 2) vertical positioning for public safety, and 3) sidelink positioning for vehicles. In this contribution, we present some experiment results and an application method in the areas of 1) ~ 2)

Use Cases and Experiment Result

An outdoor test result of carrier/subcarrier-phase based positioning

Figures 1~3  below show the result of our field experiment of carrier phase based positioning conducted in May 2021. The test site is a normal soccer field 150m wide. We used both carrier wavelength and subcarrier wavelengths for measurement. Subcarriers are usually 100 ~ 100,000 times longer than carrier wavelength in FR1.

In the test field, we set up four 5G emulating gNodeBs and transmitted continuous PRS signals using a 1.2 GHz carrier frequency with a 30 MHz sample rate. The continuous PRS is a sequence of signals that allows the sinusoidal waveform to run smoothly across multiple symbols. Unlike the current staggered PRS pattern, the continuous PRS requires block type reference symbol arrangement, similar to PTRS (Phase Tracking Reference Signal). Such a sinusoidal waveform enables precise phase measurement. For more information, refer to R1-2203635 [6][2]. For the proof of the effect of continuous PRS, we show the measurement result of the carrier/subcarrier phase positioning compared to the RSTD measurement based TDoA method.

Figure 1 (a) below shows the result of location estimation when we measure the phase difference of the subcarriers of wavelength 41.23m. The emulated gNBs are synchronized using external GPSDO oscillators and additional phase tracking functions we developed proprietary. Four gNodeB stations (marked in red stars in the figure) transmit continuous PRSs periodically. The gNB to gNB distances (i.e. ISD) are 100m ~ 170m. The location of the UE was evaluated at every 10cm while moving the UE following the straight lines of 40m shown in the figure. Each colored dot represents the estimated positions. The CDF analysis in figure 2 (b) shows about 6m accuracy in 80% of probability when we use 41.23m length subcarriers. Since the size of the test area is relatively small compared to the wavelength of the subcarrier, no subcarrier phase jump from +π to -π was observed during the field measurement. In other words, integer ambiguity didn't occur in this case.

Figure 1 (b) shows the result of location estimation when a subcarrier of 6.87m wavelength was used. The figure shows about 4m accuracy in 80% of probability. The accuracy was improved with the shorter subcarrier length (=6.87m), however, the integer ambiguity occurred in this case. In other words, the phase was wrapped 2~3 times during the measurement, hence we had to compensate for the accumulated phase value and convert it to distance.

| - Subcarrier phase (41.23m wave) | - Subcarrier phase (6.87m wave) |
| --- | --- |

Figure 1.   Location estimation result when a subcarrier of lengths 41.23m and 6.87m was used

The wavelength of the subcarriers in the OFDM signal is 1/n times the symbol length. That is, the subcarriers near the center DC have a long wavelength, while the subcarriers near the outer guard band have a shorter wavelength. In the subcarrier phase measurement method we used here, the shorter the wavelength signal, the better the positioning accuracy could be. However, the issue of integer ambiguity becomes significant.

When we use a subcarrier signal with a length of hundreds of meters or more, the approximate location of the UE can be determined without the ambiguity of the wavenumber (i.e. the integer ambiguity). However, the positioning accuracy measured using long subcarriers may not be so satisfactory because the angular resolution is poor. Nevertheless, it provides the initial information on distance helpful for resolving the inter ambiguity.

Once the approximate position is known, it is possible to calculate the distance difference between the signal paths of neighboring gNBs. When we divide it by the wavelength of the subcarrier λc , we may obtain the integer number of the subcarrier phase. We can improve the positioning accuracy using the shorter subcarriers and the estimated integer wave number.

An iterative error reduction method using the method described above is formulated in equation (1).

- new_df=roundold_df/λc*2π+θab*λc

new_df :  estimated distance difference

old_df  : measured distance difference

θab      : measured phase difference

λc        : length of the wave used for the measurement

In the above equation (1), old_df represents the initial information of the distance difference between a gNB-a and a gNB-b to a UE. We assume old_df is measured and evaluated using a long wavelength signal. The old_df may also be provided by other means such as the Rel-17 TDoA method or GPS. λc denotes the wavelength of shorter subcarriers, and θab is the measured phase difference using the shorter subcarrier.

new_df represents a refined distance difference. In equation (1), round(old_df /λc) provides the estimated integer number. If this estimated integer number is correct, the residual error of the new_df is improved than the old_df. However, if the error deviation of old_df is far greater than the wavelength λc, the integer number estimation fails.

The equation (1) can be applied iteratively by measuring a shorter subcarrier and using the new_df as old_df for evaluating a new integer number. Accordingly, we may achieve centimeter level accuracy when we finally use the estimation result from the shortest subcarrier and the phase difference of the carrier wave.

Figure 2 (a) below shows the estimated position of the UE when the carrier wavelength (= 24.98cm) of 1.2 GHz frequency is used. Observing the CDF analysis in figure 2 (b), the carrier phase measurement method achieved a 1.2m error in the probability of 80%. This is slightly worse than what we expected because we later found there was some calibration mismatch that occurred in the experiment. We believe the result can be improved further in a future experiment.

| - Carrier phase (24.98cm wave) | - CDF (Phased v.s. TDoA) |
| --- | --- |

Figure 2. The result of carrier phase estimation with CDF graph

The CDF graph in figure 2 (b) above also shows a comparative analysis of the case where the TDoA method as defined in Rel-17 is applied. The TDoA curve is a simulation result by implementing the same environment. As shown in figure 2 (b), the carrier phase method far outperforms TDoA and even the subcarrier method is better than TDoA in some conditions.

The phase measurement method introduced here is effective only in FR1 bands. It is considered difficult to apply it to FR2 band signals because the carrier wavelength is only a few millimeters. More detailed descriptions and simulation results of the phase measurement method will be presented later.

2.2 Application to vertical positioning

When two adjacent antennas transmit carrier signals of identical frequency, the two signals create a regular pattern of radio interference. The interference pattern is very precise in that the shape doesn’t change even at a long distance. When the number of antenna elements is increased the interference pattern forms a radio beam.

The phase difference of the two carrier signals transmitted from adjacent antennas can be mathematically modeled as hyperbolic and asymptotes. According to the definition of the hyperbolic curve, the following mathematical relationship is established.

- θ12=r1-r2λc=2aλc

In the above equation (2), θ12 is the phase difference value of the two antenna signals measured by a UE. λc is the wavelength of the carrier. |r1-r2| denotes the distance difference from the UE to the dual antennas of remote gNB, and a denotes the major axis value of the hyperbolic curve. This major axis value a also becomes the slope value of the asymptote approaching the hyperbolic curve. Then the angle of departure, i.e., AoD, can be calculated by the equation (3). In this equation, Φ denotes the AoD value and 2F is the distance between the two antennas.

- ϕ=cos-1aF=cos-1λcθ122F

This method of measuring AoD using the information of carrier phase difference is already accepted in Bluetooth [5] and 802.11 standard [4]. It can achieve relatively finer results than the beam-sweeping method defined in Rel-17, but consumes much fewer resources. This proposed AoD measurement method can also be applied to a variety of use cases. In this contribution, we present an application to vertical positioning for public safety such as the E911 service. According to the E911 regulation, it is required to identify the altitude of the UE within several meters of error.

Figure 3 below shows the results of the vertical positioning experiment conducted in November 2020. In this experiment, vertically arranged three antennas with antenna spacing of 1m and 5m respectively, were used. Three sets of continuous PRS signals as described in R1-2203635 [6] are transmitted via the three vertical antennas. 1.2 GHz carrier frequency and 10 MHz sample rate were used for transmission of the signals.

| (a) vertical antenna tower installation | (b) UE elevation test |
| --- | --- |

Figure 3.  Vertical positioning experiment using carrier phase based DL-AoD method

To measure the vertical altitude, a UE box was lifted from the ground to top of the a seven-story building using a mini-crane. The vertical antenna tower was placed 150m to 500m away from the building. The UE measures the phase difference and AoD and converts the angular information to height. The altitude was displayed in the LED of the UE box and compared with the banner printed the vertical length.

As a result, we confirmed that the UE can accurately determine the altitude at a resolution of 1m or less even at a distance of 500m. This was sufficient performance to distinguish the number of floors of the building, which is the requirement by US E911 regulation. The whole experimental process can be viewed through a YouTube link (https://youtu.be/F0GrAait8Vo) [7].

In the experiment above, we used three vertically placed antennas in different spaces. Such an arrangement was devised to improve angular resolution and solve the angular ambiguity problem. The angular ambiguity is in principle similar to the integer ambiguity problem, and even the cause and solution are similar.

That is, in general, as the distance from the UE to antennas increases, the distance difference of |r1-r2| of the equation (2) becomes smaller. This means the angular resolution of θ12 is degraded and the AoD estimation becomes poor. One way to improve the angular resolution is by increasing the gap between the vertical antennas (i.e. antenna aperture), as a result, |r1-r2| value becomes larger. This gives an effect of increasing the dynamic range of angular variation higher than the noise floor. However, this in turn brings angular ambiguity problems explained below. The solution we present in this paper is using the three vertically spaced antennas as shown in figure 4.

| (a) Three vertical antenna arrangement | (b) Phase variation in antennas A and C |
| --- | --- |
|  | (c) Phase variation in antennas A and B |

Figure 4. Schematic view of the vertical positioning test installation

In figure 4 (a) above, the gap between the antennas A and C is relatively narrow. As a result, the phase variation between the two antennas, shown in figure 4 (b), is small. On the other hand, figure 4 (c) shows rapid phase change when it is measured using the wide-spaced antennas A and B. The graphs show frequent wrapping of phases from +π to -π. Because of this phenomenon, the UE cannot determine the exact AoD unless the accumulated number of phase wrappings from the ground is known.

One solution is using the approximate value of AoD measured using the narrow antennas A and C. Applying a method similar to equation (1), the UE may estimate the angular ambiguity number and recovers correct AoD and altitude information.

We confirmed through experiments that the degree of error improvement is almost proportional to the difference in antenna spacing. This method of AoD and altitude estimation using the carrier phase measurement can be applied in both the FR1 band carrier and the FR2 band carrier. More detailed mathematical analysis, field experiments, and simulation results will be covered later.

Mathematical description of the carrier-phase based measurement

This section briefly introduces some mathematical formulas for calculating carrier phase difference using a continuous sinusoidal subcarrier waveform.

In CP-OFDM, the tail portion of symbol samples is copied and placed in the Cyclic Prefix region. This makes it difficult to create a continuous waveform. However, when applying a special symbol sequence introduced in R1-2103635 [6], the subcarrier waveform becomes a continuous sinusoidal waveform form, as shown in Figure 6 (b). Using the continuous waveform, very precise phase measurement can be possible.

| - discontinued subcarrier wave | - continuous sinusoidal waveform |
| --- | --- |

Figure 6. Example of making discontinued subcarrier waveform to a sinusoidal wave

A positioning reference signal can be composed of a set of subcarriers spaced at equal distance in the frequency domain. The waveform can be represented as (4) below. In (4),  ωm denotes a subcarrier frequency.

(4)             A1eiω1t+A2eiω2t+…+Ameiωmt

A gNodeB-a modulates the positioning reference signal using a carrier frequency ωc  and transmits it periodically. The passband representation of the signal is (5)

(5)            Sat=A1eiω1t+A2eiω2t+…+Ameiωmt∙eiωct

Similarly, the reference signal transmitted from a neighboring gNB-b can be represented as (6) below. Note that gNB-b allocates the same subcarrier sets as used in gNB-a. This makes the conjugate operation of the reference signals simple in a receiving UE.

(6)        Sbt=B1eiω1t+B2eiω2t+…+Bmeiωmt∙eiωct=q=1mBqeiωc+ωqt

The reference signal sat  travels for a time a1 and arrives at a UE, and the signal sbt  travels for a time b1 and arrives at a UE. It can be represented as (7) below. For simplicity, we consider only the case of a line-of-sight path in this document. We assume the signals are transmitted at different moments to avoid a collision.

(7)    Sat-τa1=q=1mAqeiωc+ωqt-τa1=eiωct-τa1q=1mAqeiωqt-τa1

Sbt-τb1=q=1mBqeiωc+ωqt-τb1=eiωct-τb1q=1mBqeiωqt-τb1

The UE converts the passband signal into a baseband signal by multiplying the inverse carrier frequency (-ωc) as in (8). In Equation (8),  ϵ denotes a local clock error, which has to be removed later.

(8)    yat=e-iωct-ϵ∙Sat-τa1=e-iωcτa1-ϵ∙q=1.mAqeiωqt-τa1

ybt=e-iωct-ϵ∙Sbt-τb1=e-iωcτb1-ϵ∙q=1.mAqeiωqt-τb1

The sinusoidal waveforms included in (8) can be extracted using FFT / DFT or the filtering process. The detail of this process is out of scope. The decomposed subcarrier waveforms can be expressed in vector form as in (9). Again, ωc denotes a carrier frequency, ωm is a subcarrier frequency,  ϵ denotes a local clock error and, σa is the sum of error components that remained from the previous FFT or DFT process.

(9)                       YA=a1a2⋮am=N∙A1e-iωc+ω1τa1-ϵA2e-iωc+ω2τa1-ϵ⋮Ame-iωc+ωmτa1-ϵ+σA

YB=b1b2⋮bm=N∙B1e-iωc+ω1τb1-ϵB2e-iωc+ω2τb1-ϵ⋮Bme-iωc+ωmτb1-ϵ+σB

The UE calculates a conjugate product between the two phase vectors and obtains a carrier phase difference as in (10).

(10)

YAYB*=N∙&A1e-iωc+ω1τa1-ϵ&A2e-iωc+ω2τa1-ϵ&⋮&Ame-iωc+ωmτa1-ϵ∙N∙&B1*eiωc+ω1τb1-ϵ&B2*eiωc+ω2τb1-ϵ&⋮&Bm*eiωc+ωmτb1-ϵ=N2∙&A1B1*eiωc+ω1τb1-τa1&A2B2*eiωc+ω2τb1-τa1&⋮&AmBm*eiωc+ωmτb1-τa1

In equation (10) above, ωc denotes an angular frequency of a carrier.  ωc is a significantly greater value than ωm .  Therefore, (10) can be simplified as (11).

(11)

YAYB*=N2∙&A1B1*eiωc+ω1τb1-τa1&A2B2*eiωc+ω2τb1-τa1&⋮&AmBm*eiωc+ωmτb1-τa1≈N2∙&A1B1*eiωcτb1-τa1&A2B2*eiωcτb1-τa1&⋮&AmBm*eiωcτb1-τa1=N2∙eiωcτb1-τa1

The UE can calculate carrier phase difference by applying an angle function as below.

(12)                angleYAYB*=θab=ωcτb1-τa1

In (12), θab denotes phase difference between the two paths of gNB-a and gNB-b, and τb1-τa1 denotes the distance difference. Since the angle function is defined only in the range between -π ~ + π , the resulting angle value of (12) rapidly changes as UE moves a few centimeters, as shown in figure 2 (a).

The UE may also calculate a subcarrier phase difference by applying conjugate multiplication of the vector having the lower m-1 components of  YAYB*  , and the vector having the upper m-1 components of YAYB*, according to (13) below.

(13)

LowYAYB*∙conjHighYAYB*=&A1B1*eiωc+ω1τb1-τa1&A2B2*eiωc+ω2τb1-τa1&⋮&Am-1Bm-1*eiωc+ωm-1τb1-τa1∙&A2*B2eiωc+ω2τb1-τa1&A3*B3eiωc+ω3τb1-τa1&⋮&Am*Bmeiωc+ωmτb1-τa1

=N4∙A1A2*B1*B2eiω1τa1-τb1A2A3*B2*B3eiω1τa1-τb1⋮Am-1Am*Bm-1*Bmeiω1τa1-τb1

∴ angleLowYAYB*∙conjHighYAYB*=ω1τa1-τb1

In (13),  ω1 denotes an angular frequency corresponding to a subcarrier number 1, which is the longest wavelength subcarrier in NR. The phase value of (13) usually changes slowly in an area of hundreds of meters. Therefore (13) can be used for initial position estimation of UE, and for resolving integer ambiguity hidden in (12).

Conclusion

In conclusion, we propose following new aspects must be considered as a priority during the ongoing study phase of Rel-18 5G NR positioning WID [1].

Proposal 1: Study new reference signaling efficient for supporting phase-based measurement method

Proposal 2: Investigate ambiguity resolution methods and necessary impact on the 5G NR system

Proposal 3: Study phase based DL-AoD measurement method and necessary impact on 5G NR system

References

| [1] | RP-213588, Study on expanded and improved NR positioning, Intel, December 6-17, 2021 |
| --- | --- |
| [2] | 3GPP R1-2104880 , "Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning " , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [3] | 3GPP R1-2104844 , "Carrier Phase Based Downlink Angle of Departure Measurement" , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [4] | 3GPP R1-2101470 , " Potential Enhancements on DL-AoD positioning " , Qualcomm Incorporated , RAN WG1 #104-e e-Meeting, January 25th – February 5th, 2021 |
| [5] | Martin Wooley, "Bluetooth Direction Finding:A Technical Overview", Bluetooth SIG, Version 1.0.2, 22 Feb. 2021 |
| [6] | 3GPP R1-2203635, "Continuous PRS for improved carrier phase measurement" , Dankook University , RAN WG1 #109-e e-Meeting, May 9th – 20th, 2022 |
| [7] | https://youtu.be/F0GrAait8Vo |
| [8] | www.locaila.com |