3GPP TSG RAN WG1 Meeting #109-e	R1-2203635

e-Meeting, May 9th – 20th, 202

Agenda Item:	9.5.2.2  Improved accuracy based on NR carrier phase measurement

Source:	DanKook University, Locaila

Title:	Continuous PRS for improved carrier phase measurement

Document for:	Discussion & Decision

Introduction

Over the last years, GPS carrier phase technology has been successfully applied to a wide range of applications, providing centimeters of precision. In the case of GPS, extracting carrier phase information from different satellites required a complex process of restoring the carrier signal using the satellite's unique code. However, in the 5G NR system, it can be relatively simple to extract carrier phase information for each TX node because different subcarriers can be assigned to each TX node, and a simple FFT process provides the information on the amplitude and phase of the subcarriers [1].

In addition, solving the Integer Ambiguity problem has been a complicated issue because there are only two carrier signals, i.e. 1.2 GHz and 1.5 GHz, available in GPS. However, in 5G NR, different solutions for integer ambiguity can be devised using the plurality of frequency bands or bandwidth parts. Various lengths of pseudo carriers can be synthesized for resolving the integer ambiguity. Further, the use of additional subcarrier wavelengths that are much longer than carriers may also be a potential solution. It is necessary to develop a new carrier phase positioning method using the 5G NR subcarrier structure.

In this paper, we present some considerations for employing carrier phase-based positioning methods in 5G NR systems. In particular, we propose a new method of constructing positioning reference signals (PRS) that is efficient for phase measurement. The proposed PRS configuration method has superior phase measurement accuracy compared to the Release-17 staggered PRS pattern. We present some discussions and simulation results. In conclusion, we propose that the new positioning reference signal should be studied to achieve the improved positioning accuracy of the centimeter error range in 5G NR systems.

Continuous PRS pattern for efficient phase measurement

Extracting carrier phase information from ranging signals transmitted by multiple TX nodes can be achieved either by encoding signals using a CDMA method as employed by GPS, or by multiplexing signals in the time division method, as applied by the 802-series IEEE standard. However, in 5G NR systems, assigning different time slots to different TX nodes can be wasteful, so we do not recommend this method.

Rather, a frequency division method in which TX nodes are assigned different subcarrier groups and multiple TX nodes transmit PRS symbols simultaneously may be more suitable for 5G NR systems.  The receiving node can extract carrier phase and subcarrier phase information for each TX node through a simple FFT operation.

In principle, this approach can be implemented to some extent using the PRS signaling standardized in Release-17. However, the staggered PRS pattern is inefficient for applying to phase tracking and phase noise reduction because the subcarrier frequency is changing symbol by symbol.

For reliable and efficient phase tracking of an incoming signal, a block-type reference signal arrangement is more suitable because the subcarrier frequency is kept constant during the phase estimation period. The Phase Tracking Reference Symbol (PTRS), which was newly introduced for phase tracking in 5G NR, took this approach.

Figure 1 below shows the comparison of the Rel-17 staggered pattern and the proposed block type continuous PRS arrangement.

Figure 1 (a)  Rel-17 staggered PRS pattern               (b) Contiguous & continuous PRS

In figure 1 (b), the continuous PRS symbols are allocated in constant frequency on, contiguous time axis. As a result, the sinusoidal waveform consisting of the subcarriers can be continued smoothly across multiple symbols, and the receiving system can track the phase constantly. There is a symbol sequence that makes the symbol waveforms connected smoothly. It will be explained in section 4 below.

The other advantage of the continuous PRS configuration as in figure 1 (b) is that the phase measurement can be performed redundantly in any portion of the block period. In other words, the RF samples for FFT calculation can be captured in any part of the block samples. This effect is explained in detail using the illustration in figure 2 below.

Figure 2.  Sliding FFT computation using the block type C-PRS configuration

In figure 2, it is assumed two PRS signals (i.e. C-PRS set A and C-PRS set B) transmitted from neighboring gNB-A and gNB-B arrive simultaneously at a UE. It is also assumed that the gNB-A and gNB-B allocate their PRS symbols in different subcarriers to avoid a collision.

The receiving UE must decompose the carrier signal and measure the phase difference of the C-PRS A and C-PRS B. Usually, 2048 (or 4096) RF samples need to be collected to perform the FFT calculation. In the Rel-17 NR system, only N number of phase calculations are possible when the total number of PRS samples is no less than N*2048 (or N*4096).

However, employing the block type continuous PRS configuration, a far greater number of FFT calculations can be performed iteratively by sliding the FFT window. In the more detailed description, once the 2048 number of samples is collected and the FFT calculation is executed, we may add a few new samples to the FFT queue by removing the old samples and reusing the rest of the samples in the queue for another FFT calculation. This process is repeated until all samples in the C-PRS block are consumed.

Note that we assumed that the subcarrier waveforms consisting of the C-PRS symbols are smoothly connected throughout the block period. Provided that, the subcarriers can be modeled mathematically as continuous sinusoidal waveforms of which the frequencies correspond to the subcarrier number.

The phase difference between two sinusoidal waves of different frequencies is constantly spinning. However, if we know the frequency difference between the two waves, we may fix the spin and calculate the time-invariant phase difference. This value is subject to the distance difference regardless of the observation point in the block.

Therefore, we may obtain the identical value of phase difference no matter which portion of the block samples is captured in the FFT window. As a result, multiple phase calculations can be executed iteratively by moving the capturing window of the samples, and the result can be accumulated to improve the accuracy

3. Simulation result

Figure 4 below shows the effect of the sliding FFT operation explained above. In figure 4, the positioning accuracy of the carrier phase method is compared between the cases of figure 1 (a) and figure 1 (b).

For fair comparison, we assumed the Rel-17 staggered PRS pattern is also used for phase calculation. The TX nodes allocate the PRS symbols in different subcarriers and transmit them simultaneously. Perfect synchronization of the TX nodes is assumed. We do not consider the issue of Integer Ambiguity in this experiment. It is assumed the receiving UE knows the exact number of waves (i.e. the Integer Number) of the carrier signal and the subcarrier signals. Other assumptions of the simulation environment are summarized below:

| factor | value | factor | value |
| --- | --- | --- | --- |
| Size of the test site | 80m x 200m ground | Carrier frequency | 5.9 GHz |
| Channel environment | Rician | Sampling bandwidth | 10MHz |
| Number of TXs | 6 | Subcarrier spacing | 15 kHz |
| RX measurements | 4 lines (100m each) | Used subcarriers | 96 (=16 x 6 tx) |
| Measurement frequency | For every 0.25m (400 points in the 100m) | Used PRS symbols | 3 per frames |
| Antenna directivity | Omni | Algorithm | Modified Chan |
| Synchronization | perfect | SNR | -12 dBm |

Table 1 Assumptions of the simulation

In the simulation setting, 6 TX nodes, labeled from gNB-A ~ gNB-F, transmit 3 consecutive PRS symbols for every 10ms frame. 16 subcarriers are allocated for each TX node for composing the PRS symbols. Altogether, 96 subcarriers for each time slot are consumed in the 10MHz available bandwidth. 5.9GHz V2X frequency is used. A Rican channel with a strong LoS component is assumed, but we set the signal power to very weak to see the situation in which a relatively large phase noise occurs.

Figure 3 below shows the plots of estimated UE coordinates by applying the carrier phase positioning method based on the two different PRS patterns.

Figure 3 (a) Rel-17 Staggered PRS     (b) Continuous PRS

Figure 3 (a) on the left shows the result of carrier phase positioning when the Rel-17 staggered PRS symbol is used for phase measurement. In the simulation, the UE is moving 100m and estimated position along four straight lines on a ground of 80m x 200m size. gNBs transmit only three PRS symbols for every 10 msec. Therefore, the UE can execute only 3 FFT operations for each measurement point. The UE extracts carrier and subcarrier phase information using the FFT computation and calculates the complex conjugate between the phase information of gNBs to convert it into distance difference. Finally, Taylor or Chan algorithm is applied for evaluating the UE coordinates. Each plot in figure 3 (a) represents the estimated position while the dashed line in the background shows the true position for comparison.

Figure 3(b) on the right shows the result of carrier phase positioning using the continuous PRS proposed in this contribution.  By applying the sliding FFT method introduced in section 3, the UE can perform more than 2,000 times FFT operations with moving the FFT window one sample at a time.

Overall, it can be observed that the continuous PRS placement on the right in figure 3 (b) achieves more close estimation of true coordinates than with the Rel-17 staggered pattern on the left.

Figure 4

Figure 4 above is a CDF graph showing the accuracy of the position estimation from the experimental results of Figure 3 above. In Figure 4 above, the two curves below show the case where phase difference measurement and coordinate estimation were performed using a subcarrier with a wavelength of about 40m. In general, C-PRS showed an improvement of about 50% or more compared to Rel-17 PRS when the subcarrier wavelength is used.

The above two curves show the accuracy when 5.9 GHz carrier frequency with a wavelength of only 5 cm is used. If there is no integer ambiguity issue, it can be seen that the accuracy is significantly improved in the case of a short carrier wave compared to using a long-wavelength subcarrier.

Among the two above curves, the upper curve shows the precision of the C-PRS arrangement, and it can be seen that the error is improved by about 15% or more compared to the Rel-17 PRS. It is analyzed that the improvement of the C-PRS comes from the larger number of FFT calculations than the Rel-17 PRS, hence reducing the error variation.

4. Continuous PRS symbol sequence

Sections 2 and 3 show that the block-type continuous PRS structure is more efficient for carrier phase measurements than the staggered pattern in Rel-17. However, simply arranging PRS symbols in contiguous shape does not provide this improved performance. Not only do the subcarriers align in the block shape, but the waveforms should be connected smoothly.

One may think that simply assigning a constant value to a series of symbols may produce a plain sinusoidal wave. However, in actual passband signal, the waveform becomes irregularly discontinuous, as shown in figure 4 (a) below. The reason is that in CP-OFDM, the cyclic prefix is attached to the head of the symbol. Due to the CP portion, the symbol waveforms are further extended by the length of the attached CP. As a result, when these extended symbols are connected, the waveform becomes discontinuous.

Figure 4 (a) discontinuous wave in the symbol boundary, (b) continuous waveform by formula (1)

To ensure that the signals are smoothly connected, as shown in figure 4 (b), the phase of the following symbols must be turned by the amount of the CP length. The phase rotation sequence of these symbols can be expressed as formula (1) below.

Xlk=Xl-1k∙exp2πjk∙TCP,l-1Tu    (1)

In the formula (1) above, Xlk  denotes  ℓ th symbol in the k th  subcarrier. Tu denotes the length of the symbol duration except for the CP part. TCP,l-1 indicates the accumulated length of the CP up to ℓ-1 symbol.

The intention of the symbol sequence in formula (1) above is this: if the following symbols are pre-rotated as much as the phase rotated by the CP, the waveforms of the adjacent symbols will match exactly as shown in Figure 4(b) after the CP operation. Thus the passband waveform becomes a continuous sinusoidal signal, and it makes the receiving node easy for tracking the carrier phase.

Proposal 1. To improve the positioning performance of 5G NR by applying the carrier phase positioning method, a new PRS signal efficient for phase measurement must be studied.

Proposal 2: Further study on the benefit of the block type continuous PRS sequence as proposed in the formula (1)

Conclusion

In summary, we propose to include the followings in the scope of the current Rel-18 positioning work item.

Proposal 1. To improve the positioning performance of 5G NR by applying the carrier phase positioning method, a new PRS signal efficient for phase measurement must be studied.

Proposal 2: Further study on the benefit of the block type continuous PRS sequence as proposed in the formula (1)

References

| [1] | Han Dun, Christian C. J. M. Tiberius, Gerard J. M. Janssen, "Positioning in a Multipath Channel Using OFDM Signals With Carrier Phase Tracking", IEEE Access, Vol 8., 13. Jan. 2020 |
| --- | --- |

-----------------------------------------------------------------------------------------------------------

contact :   Hyeon Woo Lee     woojaa@dankook.ac.kr

Jaihyung Cho           jaihyung@locaila.com