3GPP TSG-RAN WG1 Meeting #109-e	R1-2203637

e-Meeting, 9th – 20th May 2022

Agenda Item:	8.17

Source:	Ericsson

Title:	Maintenance issues for NR small data transmissions in inactive state

Document for:	Discussion, Decision

# 1	Introduction

This contribution concerns the following remaining open issue associated with the Rel-17 WI on NR small data transmissions (SDT) in inactive state [1]:

- Validation rule when CG PUSCH and MsgA PUSCH occasions are overlapping

# 2	Discussion

The following FFS from RAN1#106bis-e has not yet been resolved in RAN1 (even after several rounds of discussions during RAN1#108-e) [2]:

Agreement A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion. - FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion

As can be seen from the agreement above, a CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion. Our preference is to have a unified validation rule for PRACH occasions and MsgA PUSCH occasions, both of which are cell-specifically configured. That is, a CG PUSCH occasion is not valid if it overlaps with valid MsgA PUSCH occasion.

For UEs supporting both CG-SDT and 2-step RACH, a CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH occasion.

| Reason for change: | To clarify that a CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH occasion. |
| --- | --- |
| Summary of change: | Changes to configured-grant based PUSCH transmission in Clause 19.1 of TS 38.213. |
| Consequences if not approved: | Unclear UE behaviour when there is an overlap between CG PUSCH and MsgA PUSCH occasions. |

A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1 and, for a UE supporting both configured-grant based PUSCH transmission as described in clause 19.1 and Type-2 random access procedure as described in clause 8.1A, with a valid msgA PUSCH occasion.

# 3	Conclusion

Based on the discussion in the previous section, we propose the following:

Proposal 1	For UEs supporting both CG-SDT and 2-step RACH, a CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH occasion.

# References

RP-212594, “Work Item on NR small data transmissions in INACTIVE state”, ZTE Corporation, 3GPP TSG RAN Meeting #93e, September 2021.

R1-220xxxx, “RAN1 agreements for Rel-17 WI on SDT”, Moderator (ZTE), 3GPP TSG RAN WG1 #108-e, November 2021.