3GPP TSG-RAN WG1 Meeting #109-e	R1-2203638

Online, May 9th – 20th, 2022

Agenda Item:	9.11.1

Source:	Ericsson

Title:	Discussion on power saving enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

In [1], a new SI on XR was agreed, with objectives covering three main areas, i.e. XR awareness in RAN, XR-specific power saving, and XR-specific capacity improvements. In this contribution we discuss study topics for UE power saving, related to the following objectives in [1]:

“Objectives on XR-specific Power Saving (RAN1, RAN2):

- Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

  - C-DRX enhancement.

  - PDCCH monitoring enhancement.”

# 2	Discussion on Power Saving Enhancements for XR

## 2.1 Existing power saving solutions and challenges with XR traffic

The SI on XR in [1] focuses on studying enhancements for two types of power saving solutions, namely C-DRX and PDCCH monitoring. We note that Rel-17 already supports a set of features for each of these two solution types, as follows. Standard C-DRX features comprise: (i) Long DRX cycles, (ii) Short DRX cycles, and (iii) the power saving signal DCP (i.e. DCI with CRC scrambled by PS-RNTI) based on DCI format 2_6, used for waking up the UE in individual Long DRX cycles. The standard PDCCH monitoring adaptation features for power saving are: (i) SSSG switching between dense and sparse PDCCH monitoring and (ii) PDCCH skipping for a given duration.

The challenge for these power saving solutions is to achieve a good tradeoff between the power saved at the UE and the additional traffic delay due to not monitoring the PDCCH continuously. This depends highly on the XR traffic characteristics and, in order to describe them, in this contribution we adopt the term “application packet”, consistent with the term “packet” in [2], where “a packet models the set of IP packets belong to the same video frame”. Nonetheless, an application packet refers also to other types XR flows and not only video.  Based on this terminology, the main XR traffic characteristics agreed in Rel-17 [2] are:

- non-integer video traffic period (i.e. application packet inter-arrival time), corresponding to 30, 60, 90, 120 fps video frame generation rate;

- multiple simultaneous DL/UL traffic flows, e.g. video, audio, pose, each periodic but with a different application packet periodicity;

- traffic jitter for video flows: typically ±4 ms (i.e. total jitter range of 8 ms), optionally ±5 ms. This is the jitter in application packet arrival time at RAN, which includes varying time effects at the application (varying frame encoding delay), as well as network transfer time [2].

- different and rather tight PDBs for each traffic type: 10 ms for VR/AR DL video traffic (optionally 5, 20 ms), 10 ms for VR/CG UL pose traffic, 15 ms for CG DL video traffic (optionally 10, 30 ms), 30 ms for AR UL video traffic (optionally 10, 15, 60 ms), and 30 ms for VR DL/AR UL audio traffic. Note that the PDB is the delay budget of the application packet for radio interface (i.e. “time budget for a packet to be transmitted over the air from a gNB to a UE” [2]).

Figure 1 shows an example of arrival moments of XR application packets at the RAN, for two DL flows with different periodicities.

Figure 1. Illustration of XR application packet arrivals of two DL XR flows with a different periodicity: a video flow (green) and an audio flow (magenta) with the periods of 16.67 ms and 10 ms, respectively. The video traffic experiences a random jitter of ±4 ms with a truncated Gaussian distribution around the mean traffic period.

## 2.2 System performance with standard C-DRX

The existing C-DRX features have several limitations when being configured for XR traffic with the characteristics above:

- The standard DRX cycle values are different than the non-integer period values of the video traffic (33.33, 16.67, 11.11, and 8.33 ms). Consequently, additional delay is incurred if video application packets arrive outside the DRX active time.

- The DRX features cannot always ensure a close match between the non-active time and the short and irregular sleep opportunities, as determined by the interleaved traffic pattern from multiple XR traffic flows (see Figure 1). This is even more difficult given that some flows are quasi-periodic and suffer from random jitter. As a result, the delay/UE power saving tradeoff may be suboptimal.

The impact of these limitations are exemplified in Figure 2, which shows the mean UE power consumption and fraction of satisfied UEs, for C-DRX with different existing features and different loads. Long DRX with the shortest possible cycle of 10 ms and a long value of 8 ms for the drx-onDurationTimer results in a marginal power saving gain of at most 5% vs. no DRX. On the other hand, it does not increase the delay significantly and thus achieves 7 UEs/cell, i.e. nearly the ideal capacity of no DRX. Here we use the capacity definition adopted in [2], i.e. the maximum number of users per cell with at least 90% of UEs being satisfied, where a UE is satisfied if all its flows meet their own PDB requirements (99% of the application packets transmitted within their PDB).

By contrast, the standard Short DRX configuration with 4 ms cycles saves up to 10% UE power vs. no DRX, but decreases the capacity by 1 UE/cell. This is due to the on-time covering 50% of the DRX cycle time and not being aligned with the video traffic arrival, which increases the delay. Consequently, standard DRX configurations cannot achieve a high capacity (by limiting the delay) and low UE power consumption at the same time. Thus, there is room to improve the delay/power consumption tradeoff and this could be achieved with C-DRX enhancements.

Note that the simulation setup used in this document is presented in Annex A: Simulation assumptions. Since the evaluation focuses on DRX, a fixed pattern is assumed for PDCCH monitoring. Specifically, it is assumed that the search space is configured such that the UE monitors the PDCCH in every slot of DRX active time. The UE power consumption model assumes the PDCCH occupies the first two symbols at the beginning of a slot.

- (b)

Figure 2. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for different existing C-DRX features: Long DRX with a cycle of 10 ms and drx-onDurationTimer of 8 ms (cyan), and Short DRX with a cycle of 4 ms and drx-onDurationTimer of 2 ms (green). Three flows are assumed: one DL video flow (60 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms for all configurations.

We observed that the power saving signal DCP achieves even lower power saving gains than Long DRX. This is due to the frequent application packet arrival from different flows, such that there is traffic in each Long DRX cycle (even for the shortest cycle duration of 10 ms) and thus the drx-onDurationTimer needs to be started in each cycle to avoid latency increases. Consequently, the DCP signal would never indicate to the UE to not start the drx-onDurationTimer, so sending such a DCP signal would not bring any benefit and would only increase the UE power consumption, by causing the UE to unnecessarily monitor DCP in non-active time. For this reason we do not show any results for DCP.

Existing C-DRX solutions do not achieve a good delay/power consumption tradeoff. Solutions that meet the delay requirements for marginal capacity decrease achieve only limited power saving gains of up to 5%, while solutions that save more power increase the delay and thus decrease capacity.

Based on our simulation studies and observations above, the following are proposed.

XR traffic characteristics, i.e. non-integer traffic period, multiple traffic flows, and traffic jitter range, should be jointly taken into account when configuring and evaluating C-DRX solutions.

Adopt enhancements to C-DRX to achieve a better delay/UE power consumption tradeoff.

## 2.3 C-DRX enhancements

In this section we discuss potential C-DRX enhancements to address the limitations above. Furthermore, the parameter values of the enhanced features are selected by the network assuming there is information about the XR traffic, e.g. from the application: application packet periodicity (generation rate) and periodicity changes for each application traffic flow; application packet periodicity start time per application flow, to understand where to align the DRX cycles; application packet jitter information, e.g. range, per application flow; and delay budget (PDB) of the application packet for radio interface, per application flow.

In order to enhance C-DRX power saving features, the network needs information about the XR traffic flows: application packet periodicity (generation rate) and periodicity changes for each application traffic flow; application packet periodicity start time per application flow, to understand where to align the DRX cycles; application packet jitter information, e.g. range, per application flow; delay budget (PDB) of the application packet for radio interface, per application flow.

### 2.3.1 Align DRX to non-integer traffic periodicity

The mismatch between the DRX cycle length and the non-integer video traffic periodicity can result in additional traffic delay and capacity decrease, especially if the value of the drx-onDurationTimer is short vs. the cycle length, as observed in Figure 2 for the standard Short DRX configuration. Due to this, it is beneficial to enhance DRX to match non-integer periodicities, such that the drx-onDurationTimer can be started when traffic is actually expected, in order to limit the delay. Additionally, the drx-onDurationTimer value could be set to cover only the time when traffic is expected, to make best use of the sleeping opportunities and save power.

Figure 3 shows results for a Long DRX cycle matched to the non-integer video period of 16.67 ms, as well as a standard DRX configuration with 10 ms cycle length. The results show that the matched DRX configuration decreases the UE power consumption by up to 5% and 9% compared to the existing DRX and no DRX, respectively, while maintaining the same (almost ideal) capacity of 7 UEs/cell. Consequently, matching DRX to the video traffic period is beneficial, since it maintains a short delay for video traffic and thus a high capacity, while saving UE power.

- (b)

Figure 3. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for DRX matched to the video traffic period of 16.67 ms with drx-onDurationTimer 10 ms (blue) and standard Long DRX with a cycle of 10 ms and drx-onDurationTimer 8 ms (cyan). Three flows are assumed: one DL video flow (60 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms for all configurations.

Matching the DRX cycle with the non-integer video application packet inter-arrival time achieves power saving gains of up to 5% w.r.t. standard DRX, while maintaining a high (almost ideal) capacity.

Adopt DRX enhancements to match various XR traffic periods, including non-integer (ms) video traffic periods.

#### 2.3.1.1 Solution to align DRX cycle with non-integer traffic period

A flexible solution to match the DRX cycle with the traffic period is to simply modify the existing formula for starting the drx-onDurationTimer for Long cycles. Two semi-static parameters (configured at the RRC) could be introduced in this formula:

- traffic_time_offset: indicates a fixed time shift for the start of drx-onDurationTimer;

- traffic_drx_offset: a number of cycles after which a new shift should be added.

With this solution, the DRX cycle length would be approximated to the next lower supported cycle value (from already existing values) w.r.t. the non-integer traffic periodicity. The formula to start the drx-onDurationTimer would then be modified to:

[(SFN × 10) + subframe number + (n × traffic_time_offset)] modulo (drx-LongCycle) =

(drx-StartOffset) + (n × traffic_time_offset) modulo (drx-LongCycle),

where n = n+1 whenever DRX_cycle_number modulo (traffic_drx_offset) = 0

and DRX_cycle_number = [(SFN × 10) + subframe number] / (drx-LongCycle).

Below we show an example for video traffic with generation rate 30 fps, traffic_drx_offset=4, traffic_time_offset=3, drx-LongCycle=32 ms, and drx-onDurationTimer value of 8 ms. The cells shaded in red indicate shifts between traffic arrival and DRX start that exceed the value of the drx-onDurationTimer, so the traffic arrives after the expiration of drx-onDurationTimer. Cells shaded in green show that the traffic arrives within the on-time of the DRX cycles.

| Application packet arrival [ms] |  | Problem with standard formula |  |  | Solution with enhanced formula |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DRX cycle start [ms] | Difference (packet arrival – DRX start) [ms] |  | DRX cycle start [ms] | Difference (packet arrival – DRX start) [ms] |
| 0.00 |  | 0 | 0.00 |  | 0 | 0.00 |
| 33.33 |  | 32 | 1.33 |  | 32 | 1.33 |
| 66.67 |  | 64 | 2.67 |  | 64 | 2.67 |
| 100.00 |  | 96 | 4.00 |  | 100 | 0.00 |
| 133.33 |  | 128 | 5.33 |  | 132 | 1.33 |
| 166.67 |  | 160 | 6.67 |  | 164 | 2.67 |
| 200.00 |  | 192 | 8.00 |  | 200 | 0.00 |
| 233.33 |  | 224 | 9.33 |  | 232 | 1.33 |
| 266.67 |  | 256 | 10.67 |  | 264 | 2.67 |
| 300.00 |  | 288 | 12.00 |  | 300 | 0.00 |
| 333.33 |  | 320 | 13.33 |  | 332 | 1.33 |
| 366.67 |  | 352 | 14.67 |  | 364 | 2.67 |
| 400.00 |  | 384 | 16.00 |  | 400 | 0.00 |
| 433.33 |  | 416 | 17.33 |  | 432 | 1.33 |
| 466.67 |  | 448 | 18.67 |  | 464 | 2.67 |
| 500.00 |  | 480 | 20.00 |  | 500 | 0.00 |
| 533.33 |  | 512 | 21.33 |  | 532 | 1.33 |
| 566.67 |  | 544 | 22.67 |  | 564 | 2.67 |
| 600.00 |  | 576 | 24.00 |  | 600 | 0.00 |
| 633.33 |  | 608 | 25.33 |  | 632 | 1.33 |

This solution to enhance the formula for Long DRX cycles has several advantages:

- Numerous cycle lengths and alignments can be supported by means of only two additional parameters.

- For choosing the values of the two new parameters, the network can take into account not only the traffic periodicity, but also other parameters, e.g. the value of the drx-onDurationTimer. Thus, if the drx-onDurationTimer value is long enough to cover small differences between a non-integer traffic periodicity and an integer DRX cycle, the network can minimize the number of new additional DRX cycle shifts, since such shifts are not needed in every consecutive cycle. This also results in overall fewer values that need to be supported for the two new parameters. By contrast, a different solution adding new supported Long cycle values at RRC for each non-integer periodicity could not solve the alignment problem in the long term, since these periodicity values need to be approximated to ms boundaries, resulting in accumulated offset in time. Additionally, this other solution is less future-proof, since the list of supported DRX cycle lengths at RRC would have to contain and be updated with all the exact traffic periodicities of any current/future XR application supported by the system.

- With the formula enhancement above, the two new parameters are semi-statically configured and do not require dynamic signaling, thus having a low signaling overhead.

### 2.3.2 Handling multiple traffic flows

The interleaved pattern of multiple (quasi-)periodic XR flows that are running in parallel creates irregular and short sleep opportunities. Current C-DRX solutions allow the NW to configure only one DRX configuration and, therefore, this single DRX configuration cannot really match the traffic characteristics of all XR flows. Thus, it cannot closely follow the short sleep opportunities and result in either low power saving gains, or in a long traffic delay. A solution to this issue is to support multiple simultaneous DRX configurations. We refer to this as multi-flow DRX and we present this solution as follows.

#### 2.3.2.1 Multi-flow DRX solution

In the multi-flow DRX solution, the NW provides multiple DRX configuration to a UE, all running in parallel. By doing so, the network could use XR traffic information such as traffic periodicity or jitter characteristics, provided e.g. by the application, to configure DRX in a way that the DRX parameters in each configuration match each individual (quasi-)periodic traffic flow. For instance, each DRX configuration can be configured with the traffic periodicity and the DRX cycle start can be aligned with the expected application packet arrival (or start of the jitter range) of one specific traffic flow.

Regardless of the DRX parameter values selected for each configuration, the multi-flow DRX solution works as follows:

- The UE monitors the PDCCH while the drx-onDurationTimer (or drx-InactivityTimer) is running in any of the DRX configurations, i.e. the overall active time is a logical ‘OR’ of the active times given by each DRX configuration.

- If a PDCCH is received for a new transmission, then any drx-InactivityTimer that is running at that time could be re-started.

An example illustration is shown in Figure 4, for two DRX configurations that are selected to match the parameter values (periodicity and alignment) of two traffic flows. The DL time slots in the TDD pattern in which the UE monitors the PDCCH are shaded in black. Thus, if configured with suitable parameter values, the multi-flow solution is able to follow the irregular time gaps due to the interleaved traffic flows and save UE power, while ensuring a low delay.

Figure 4. Illustration of multi-flow DRX with two configurations, each matching a DL traffic flow: the cycle and drx-onDurationTimer in green follow the video flow (16.67 ms period) and the DRX configuration in magenta follows the audio flow (10 ms period). The video traffic experiences a random jitter of ±4 ms with a truncated Gaussian distribution around the mean traffic period.

#### 2.3.2.2 Results for multi-flow DRX

Figure 5 shows example results for multi-flow DRX with two configurations and for a standard DRX configuration. We observe that multi-flow DRX decreases the UE power consumption significantly, by up to   12% and 9% w.r.t. no DRX and existing DRX, respectively. At the same time, it achieves the capacity of 5.2 UEs/cell, i.e. nearly ideal and higher than for the existing DRX.

- (b)

Figure 5. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for multi-flow DRX with two configurations (red) and a standard DRX configuration with a cycle of 10 ms (cyan). Three flows are assumed: one DL video flow (60 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms for all configurations. For multi-flow DRX, one DRX configuration matches the video period with drx-onDurationTimer 10 ms and the second configuration the audio period with drx-onDurationTimer 2 ms.

Finally, we note that having a second DRX configuration for the second traffic flow is useful especially when the period of the first DRX configuration is around or longer than the PDB of the second traffic flow. This is the case in the example above, where the first DRX cycle has a length of 33.33 ms (i.e. video periodicity), which is longer than the PDB of 30 ms for the audio flow. Consequently, the audio application packets may exceed the PDB often. It is thus useful to configure a second DRX cycle to match the audio period, in order to limit the audio traffic delay. For other cases where the first DRX cycle has a much shorter length, e.g. 16.67 ms for video with 60 fps, this single DRX configuration may be sufficient to ensure the relaxed PDB requirements for the audio flow (as observed in Figure 3).

Multiple simultaneous DRX configurations, each matching a traffic flow, achieve power saving gains 9% w.r.t. standard DRX, while maintaining a low delay and almost ideal capacity, especially if the PDBs are shorter than the DRX cycles.

Adopt multiple simultaneous DRX configurations for XR traffic with multiple (quasi-)periodic XR traffic flows.

### 2.3.3 Handling traffic jitter

In Release 17, RAN1 agreed that video traffic suffers from random jitter around its mean period (typically ±4 ms), covering a long range of 8 ms [2]. If this jitter range is not taken into account when configuring DRX, the video traffic may arrive randomly either before or after the on-time. This traffic would be subsequently buffered until the next DRX cycle, which would then increase the delay.

A simple solution using existing DRX functionality, is to configure the drx-onDurationTimer to run during the entire jitter range. However, given the long jitter range of 8 ms, the UE would monitor the PDCCH for a long time, which would lead to limited power saving gains, especially for video periods shorter than 16.67 ms. We thus propose to introduce a two-stage DRX solution as follows.

#### 2.3.3.1 Two-stage DRX solution

The two-stage DRX concept is illustrated in Figure 6. In principle, the UE does not need to continuously monitor PDCCH over the whole jitter range. It could be sufficient if the UE monitors a few short times during that range. In this case, existing DRX functionality could be used by configuring a long DRX cycle as stated above, with the difference that the UE would monitor PDCCH only intermittently during that “OnDuration” time.

To achieve this behavior, two-stage DRX introduces Inner DRX cycles within an Outer DRX cycle. The Outer DRX cycle can be matched to the traffic period and has an outer onDurationTimer. This outer onDurationTimer can be configured by the network to cover the jitter range, if information about the jitter range is known. The outer onDurationTimer determines the period over which shorter Inner DRX cycles (in the order of a few ms) run. In other words, the inner onDurationTimer is started (at the beginning of each Inner DRX cycle) only as long as the outer onDurationTimer is running; when the outer onDurationTimer expires, no subsequent Inner cycles are triggered. The UE monitors the PDCCH only during the inner onDuration time of the Inner cycles. Thus, the UE monitors the PDCCH with a short periodicity during the traffic jitter range, so it saves power during the short sleep durations, while at the same time the delay is not increased significantly.

Figure 6. Illustration of two-stage DRX.

Importantly, the two-stage DRX solution can be combined with multi-flow DRX, where the two-stage concept can be applied on top of one or multiple DRX configurations in multi-flow DRX. We emphasize that two-stage DRX is useful especially for the DRX configurations matching periodic XR flows with jitter, e.g. video. In order to illustrate how multi-flow and two-stage DRX can be combined, let’s consider the example in Figure 7, where there are two DRX configurations: the first one (green) also has two-stage DRX on top, while the second one (magenta) is a single-stage DRX configuration. In this case, the UE monitors the PDCCH whenever:

- the inner onDurationTimer (or drx-InactivityTimer) of the first DRX configuration (green) is running;

OR

- the drx-onDurationTimer (or drx-InactivityTimer) of the second DRX configuration (magenta) is running.

The resulting DL time slots in which the UE monitors the PDCCH are shaded in black. Furthermore, we note that the inner onDurationTimer of the first DRX configuration is triggered periodically only as long as the outer onDurationTimer of this configuration is running.

Figure 7. Illustration of multi-flow DRX with two configurations. The first configuration (green) is combined with two-stage DRX and is matching a video flow (16.67 ms period). The video traffic experiences a random jitter of ±4 ms with a truncated Gaussian distribution around the mean traffic period, where the outer onDurationTimer covers the jitter range. The second DRX configuration (magenta) follows an audio flow (10 ms period).

#### 2.3.3.2 Results for multi-flow with two-stage DRX

Figure 8 shows simulation results for the combined solutions of multiple DRX (one for video and one for audio traffic) with two-stage DRX for the configuration matching video traffic with jitter. For this example of video with 60 fps, multi-flow & two-stage DRX achieves a mean UE power saving gain of up to 16% w.r.t. no DRX, and 13% w.r.t. standard Long DRX. Importantly, multi-flow & two-stage DRX saves more power than the standard DRX configuration for the entire load range. Furthermore, the benefits of limiting the delay with multi-flow & two-stage DRX are reflected in the fraction of satisfied UEs at high loads, where multi-flow & two-stage DRX achieves almost the same capacity as standard DRX (i.e. only 0.5 UEs/cell lower). Consequently, multi-flow & two-stage DRX is superior to standard DRX in terms of power saving, with a negligible capacity loss.

- (b)

Figure 8. Results for (a) mean UE power consumption and (b) fraction of satisfied UEs, for multi-flow & two-stage DRX (magenta) and a standard DRX configuration with a Long cycle of 10 ms (cyan). Three flows are assumed: one DL video flow (30 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). The duration of drx-InactivityTimer is 2 ms in all cases. For multi-flow DRX, one DRX configuration matches the video traffic with two-stage DRX {outer DRX cycle equals video traffic period, 10 ms outer onDurationTimer, 4 ms inner DRX cycle, 2 ms inner onDurationTimer} and the second DRX configuration matches the audio period with drx-onDurationTimer 2 ms.

Another advantage of the two-stage DRX solution is that it is based on a semi-static RRC configuration for the Outer and Inner cycles. This reduces the need for dynamic signaling and thus the resulting overhead, as well as potential network-UE synchronization loss due to dynamic signaling misdetection.

It is necessary to enhance C-DRX to cope with traffic jitter, in order to improve the delay/UE power saving tradeoff.

When multiple XR flows are present and one of them suffers from jitter, two-stage DRX with multi-flow DRX saves 13% UE power w.r.t. the existing DRX, while achieving a nearly equal capacity.

Based on the simulation study and observations above, the following is proposed.

Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows.

## 2.4 Discussion on PDCCH monitoring adaptation

In Rel-17 study item phase, PDCCH monitoring adaptation features (i.e. SSSG switching and PDCCH skipping) were simulated by some companies in [2]. Promising results were shown, e.g. power saving gains of up to 20% for VR traffic with multiple flows and high load. We note that, due to the ongoing standardization of these features during the simulation time, the assumptions adopted in [2] may have been only partly compliant with the latest Rel-17 standardized version of PDCCH monitoring adaptation features. Nonetheless, we consider these simulations to be sufficient evidence of the good performance of existing Rel-17 PDCCH monitoring adaptation, suggesting that it may be difficult to discover major enhancements in this area.

Furthermore, although existing PDCCH monitoring adaptation features are promising for saving power, they could also introduce significant signaling overhead, since SSSG switching and PDCCH skipping are based on dynamic DCI signaling. C-DRX based solutions could manage with less signaling overhead, due to the overall (quasi-) deterministic characteristics of the XR traffic, for which semi-static PDCCH monitoring patterns with C-DRX are suitable.

Extensive power saving features exist for PDCCH monitoring in Rel-17.

Rel-17 PDCCH power saving features are baseline for Rel-18 enhancements.

Investigate potential enhancements to Rel-17 PDCCH monitoring, considering both cost (e.g., DCI overhead) and benefit (e.g., power saving gain).

# 3	Evaluation Methodology and Performance Metrics for UE Power Saving

For the purpose of investigating the potential enhancements for UE power saving, evaluation methodology and KPI need to be agreed for Rel-18 XR study item.

Since Rel-18 XR study item is a continuation of Rel-17 XR study item, in our view the simulation study can reuse as much as possible the assumptions in TR 38.838. That is,

- the XR traffic models (VR, CG, AR) in TR 38.838 section 5;

- the deployment scenarios in TR 38.838 section 6;

- the evaluation methodology in TR 38.838 section A.2;

- the simulation parameters presented in Table A.1-1 (for FR1) and Table A.1-2 (for FR2).

For power saving study, the KPI is power saving gain, where the UE power consumption calculation uses the model in TR 38.840.  As pointed out in TR 38.838, UE power saving tends to come at the cost of capacity loss. Thus for evaluating a method for power saving, the corresponding capacity results should be shown by the proponent company as well. A method that brings tangible power saving without sacrificing capacity is preferred.

For performance evaluation of candidate power saving methods, reuse as much as possible the evaluation methodology and simulation assumptions in TR 38.838.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Existing C-DRX solutions do not achieve a good delay/power consumption tradeoff. Solutions that meet the delay requirements for marginal capacity decrease achieve only limited power saving gains of up to 5%, while solutions that save more power increase the delay and thus decrease capacity.

Observation 2	In order to enhance C-DRX power saving features, the network needs information about the XR traffic flows: application packet periodicity (generation rate) and periodicity changes for each application traffic flow; application packet periodicity start time per application flow, to understand where to align the DRX cycles; application packet jitter information, e.g. range, per application flow; delay budget (PDB) of the application packet for radio interface, per application flow.

Observation 3	Matching the DRX cycle with the non-integer video application packet inter-arrival time achieves power saving gains of up to 5% w.r.t. standard DRX, while maintaining a high (almost ideal) capacity.

Observation 4	Multiple simultaneous DRX configurations, each matching a traffic flow, achieve power saving gains 9% w.r.t. standard DRX, while maintaining a low delay and almost ideal capacity, especially if the PDBs are shorter than the DRX cycles.

Observation 5	It is necessary to enhance C-DRX to cope with traffic jitter, in order to improve the delay/UE power saving tradeoff.

Observation 6	When multiple XR flows are present and one of them suffers from jitter, two-stage DRX with multi-flow DRX saves 13% UE power w.r.t. the existing DRX, while achieving a nearly equal capacity.

Observation 7	Extensive power saving features exist for PDCCH monitoring in Rel-17.

Based on the discussion in the previous sections we propose the following:

Proposal 1	XR traffic characteristics, i.e. non-integer traffic period, multiple traffic flows, and traffic jitter range, should be jointly taken into account when configuring and evaluating C-DRX solutions.

Proposal 2	Adopt enhancements to C-DRX to achieve a better delay/UE power consumption tradeoff.

Proposal 3	Adopt DRX enhancements to match various XR traffic periods, including non-integer (ms) video traffic periods.

Proposal 4	Adopt multiple simultaneous DRX configurations for XR traffic with multiple (quasi-)periodic XR traffic flows.

Proposal 5	Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows.

Proposal 6	Rel-17 PDCCH power saving features are baseline for Rel-18 enhancements.

Proposal 7	Investigate potential enhancements to Rel-17 PDCCH monitoring, considering both cost (e.g., DCI overhead) and benefit (e.g., power saving gain).

Proposal 8	For performance evaluation of candidate power saving methods, reuse as much as possible the evaluation methodology and simulation assumptions in TR 38.838.

# References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

3GPP, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, TR 38.838, V17.0.0, Dec. 2021.

3GPP, “Study on User Equipment (UE) power saving in NR”, TR 38.840, V16.0.0, June 2019.

# Annex A: Simulation assumptions

For all simulation results three flows are assumed: one DL video flow (60 or 30 fps, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). DG is configured for the two DL flows, whereas CG is configured for the UL pose flow, so this flow does not affect the DRX configuration.

The UE power consumption model is adopted from [3], with the additional UL power modelling in [2], namely linear interpolation method in linear scale for UE Tx power values other than 0 dBm and 23 dBm. Other parameters are summarized in Tables A.1 and A.2.

Table A.1: System simulation parameters

| Parameter | Deployment scenarios |
| --- | --- |
|  | Dense Urban (38.913 w/ following parameters) |
| Layout | 9cells with wraparound ISD: 200m |
| Channel model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25m |
| UE height | For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. UE height (hUT) in meters; general equation for UE height; hUT=3(nfl – 1) + 1.5. nfl for outdoor UEs; 1. nfl for indoor UEs; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8). |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Antenna Pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna Configuration | 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Pattern | Omni-directional, 0 dBi |
| UE Antenna Configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Down Tilt | 12 degrees |
| BS Transmit Power | 44 dBm per 20 MHz Note: For system BW larger than above, Tx power scales up accordingly. |
| UE max tx power | 23dBm |
| System Bandwidth | 100 MHz |

Table A.2: Other assumptions

| TDD Configuration | DDDSU |
| --- | --- |
| Scheduler | MU-MIMO PF scheduler |
| PHY processing delay | UE PDSCH processing Capability #1 DL NACK to retransmission delay 1.5ms |
| DMRS overhead | 1 DMRS symbol per PDSCH |
| Power control parameter | alpha: 0.8 |
| Transmission scheme | Reciprocity-based precoding |
| Configured Grant | On |