3GPP TSG-RAN WG1 Meeting #109-e	R1-2203640

Online, May 9th – 20th, 2022

Agenda Item:	9.11.3

Source:	Ericsson

Title:	Discussion on XR-Awareness

Document for:	Discussion, Decision

# 1	Introduction

- In [1], a new Study Item identifying three main areas of study, i.e., XR-Awareness in RAN, XR specific power savings and XR specific capacity improvements, was agreed. In this contribution we discuss the study topics related to the following objective in [1].

“Objectives on XR-awareness in RAN (RAN2):

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

- Study how the above information aids XR-specific traffic handling.”

We note that this objective is RAN2 only, however before discussing any potential enhancements in RAN1 it would be necessary to agree on what information can be potentially propagated from application and core to RAN. Once the list of potential parameters is captured, one can discuss whether it is beneficial for RAN to use those parameters. As RAN1 is a group traditionally doing all evaluations as well as considering power and capacity enhancement techniques, it is of importance to discuss it there as well.

# 2	Discussion

In TR 38.838, a model for XR traffic was agreed for evaluating capacity and power saving aspects in a 5G NR system. The model considers the multi-flow nature of XR traffic, which includes video, audio and pose flows, in DL and/or UL directions. The flows described in the TR have different periodicity and packet delay budget (PDB) constraints. Moreover, such flows are composed by application packets of different size (e.g., a video frame may be referred to as application packet, as better explained in the following). Therefore, to address the requirements of each XR flow in RAN, information about the flows and application packets therein is needed to enable enhancements for capacity and power saving.

As already done above, in this paper we consistently use the term "application packet", following the definition of “packet” as described in section 5.1.1.1 in TR 38.838 [2], to identify an entire application information unit, e.g., a video frame. This is particularly needed considering that individual IP packets within an XR application packet (e.g., a video frame) are dependent on each other and must be all received within the expected PDB to be of any use by the end user application.

The implication of application packet concept is that packets should no longer be treated independently in the RAN. To make it even more useful for RAN a new set of static and dynamic parameters describing the characteristics of the application packet is needed.

An application packet is used to identify a unit of application information, e.g., a video frame in RAN.

For XR services, it is inefficient for RAN to treat IP packets independently rather RAN should treat packets on application packet basis.

## 2.1 Dynamic and static information needed in RAN

To enable capacity and power saving enhancement discussed in [3] and [4], dynamic and static information about the application packet may be exposed to the RAN in different ways. In this context dynamic information would encompass per application packet information signaled for instance by the application in RTP extension headers in the IP packets associated to the application packet. Static information on the other hand would be provided once or infrequently. This information could be obtained by different means, for example, through an updated 5G Quality of Service (QoS) framework, learned by the network, or signaled by the application. SA2 and SA4 are discussing the means how to achieve this information. RAN1 and RAN2 should identify, however, what information is useful, how it can be used, and the benefits the information provides to the capacity or energy consumption. A short list of dynamic and static application information needed in RAN is presented in Table 1 below and further discussed in the upcoming sections.

Table 1 Dynamic & Static application information needed in RAN

| Dynamic information | Static information |
| --- | --- |
| Application packet size | Traffic flow periodicity |
| Association of IP packets to application packets | Jitter statistics, e.g., distribution, range |
| Number of IP packets in application packet | Application packet delay budget |
|  | Periodicity start time offset |
|  | Type of traffic e.g., video, pose, audio |

Two types of application information can be used in RAN, static and dynamic information

Dynamic information is needed per application packet, while static information is needed once or infrequently.

### 2.1.1	Application packet size and packets association/other information

For XR applications, the end user performance may depend on whether all IP packets belonging to a video frame or frame slice are successfully delivered. The fact that a group of IP packets is associated with an application packet and the size of the application packet can be used to assist the RAN such that it can perform efficient radio resource management committed to the QoS. With this information, for example, the scheduler can identify and treat all individual IP packets associated to the application packet in such a way that they are all delivered within the PDB. Knowing the application packet size also allows the scheduler to plan its resources as well as to decide whether it can meet the requirements for the given application packet. Hence characteristics describing the size of each application packet is needed, we note that information related to the application packet size may be conveyed in various ways e.g., number of bytes, number of IP packets together with the size of each individual IP packet within an application packet, an identifier for all IP packet associated to an application PDU. The important thing to note here is that this information needs be conveyed up-front for each application packet i.e., the arrival of the first IP packet within an application packet must convey the size of the entire application packet, with this information the scheduler may allocate resources faster and more efficient.

From the above discussion it follows that dynamic information keeping track of the IP packets constituting the application packet is needed using, for example, sequence numbering of packets within an application packet.

Section 2.6 in [3] shows the benefits of knowing the IP packets associated to an application packet and the application packet size. In this evaluation, the scheduler is able to increase the capacity by 10%.

Application packet size information is beneficial for the scheduler and by doing better prioritization, the network could increase capacity by 10%

### 2.1.2	Application packet delay budget

As already been described all IP packets belonging to an application packet is needed to be received within delay budget to satisfy the end user. The delay requirement provided by 5QI for delay critical flows is defined on IP packet basis and thus can’t be used directly for a XR traffic flow where reception of the whole application packet within delay budget is needed. Therefore, the latency requirement for whole application packets is information that can be useful, i.e., a static delay budget associated with each application packet traffic flow can be of potential benefit. Such PDB information can be utilized in different ways. For example, the scheduler may prioritize different users and different application data for a given user based on the PDB.  This falls under the concept of delay-based scheduling. While the scheduling approach is implementation specific, it is highly likely that having PDB information can help the scheduler to optimize its resources by prioritizing users and data in a more efficient manner. This is observed in section 2.5 in [3]. Another way the scheduler may utilize the PDB is to estimate if the application packet can be delivered within its latency requirement and if not decide to drop the entire application packet to free radio resources. These resources can then be assigned to other users or data which can still meet their PDB [3]. From a single user perspective, it could be argued that it is not good to drop packets. However, from a system point of view in which multiple users compete for the resources, it is more efficient to use available resources in a way which maximizes system capacity. This can be done for example by selectively discarding late application packets which, if not transmitted, can free up resources for more users. Furthermore, it is noted that some application packets which have passed the corresponding delay requirement may still be of use to the application. If this is the case, enough information must be signaled by the application to inform RAN as well as how late an application packet can be to still be considered valid for the application. This could preferably be taken into account in the PDB information which would make it easier because the scheduler can assume that any application packet which fails to meet the corresponding PDB, is no longer useful for the application.

Application packet delay budget information is beneficial for the scheduler and by doing proper prioritization, the network could increase capacity by 10%

Application packet delay budget information is beneficial for the scheduler and by dropping application packets not meeting the requirements, 10% more capacity can be obtained

In the uplink, this information is also relevant. A potential enhancement, that could benefit from this information, can be made to the buffer status report (BSR) feature. As application packet sizes grow so does the inaccuracies in the reported value of the UL transmit buffers in the BSR, the result of this is that the network over-provisions radio resources to the user, consequently the UE and network will waste power and capacity transmitting padding bits, also contributing to increased radio interference. With dynamic information of the application packet size and remaining application packet delay budget, enhancements to BSR can be made to decrease the padding and prioritize delay critical traffic. This enhancement is further discussed in our companion contribution [3].

### 2.1.3	Application packet periodicity

Enhancement to CDRX, DG, SPS and CG can all benefit from knowledge of application packet periodicity associated with the different traffic flows e.g., video, pose, audio. For example, enhanced features may use the knowledge of traffic flow periodicity to align CDRX on-activity, DG, SPS and configured grant occasions with incoming XR traffic arrival, however we note in [3] that SPS/CG may be more suitable for smaller XR flows like pose or audio when PDCCH signaling is an issue.  Furthermore, in the case the application changes its resolution or frame rate, for example, due to rate adaption, such changes may impact the periodicity of the traffic. In this case, updating the advertised traffic periodicity is required by the application.  Without knowledge of expected periodicity signaled from the application such information would have to be sensed in RAN over significant amount time. A long sensing period could potentially impact the performance of enhanced features such as CDRX which is relying on the traffic arrival being predictable.

From the discussion above we note that providing the static information of the application packet periodicity of each traffic flow present in the XR session may be used to enable enhancements for capacity and power savings. In addition, signaling the start time offset of the periodic traffic flow would enable quick initial configuration of CDRX.

Static information of the application packet periodicity of each traffic flow may be used to align scheduling and power saving features such as CDRX, DG, SPS and CG.

To enable a fast configuration of enhanced CDRX, static information of the start time offset of periodic XR traffic flow is needed.

### 2.1.4	Application packet jitter statistics

Jitter statistics e.g., distribution and jitter range, of the codec generating the video frames and the transport network is another example of application information of particular interest for scheduling enhancements. The traffic model in TR 38.838 [2], assumes a truncated gaussian distribution of the jitter for XR video traffic illustrated in Figure 1.

Figure 1 Special handling for the entire jitter range (red zone).

The jitter range, highlighted by the red zone, indicates the potential arrival time of an application packet carrying a video frame within a video traffic period. When considering enhancements addressing video traffic, we note that the jitter may impact the predictability of the traffic flow. Existing features, for example, SPS, CG and DRX relies on the periodicity of the traffic flow. If we suppose any of these features were configured to align with the video traffic period without knowledge of the jitter, the traffic arrival may arrive too late for the CG/SPS scheduling occasion or CDRX on-activity resulting in additional delay impacting the end user QoE. If we on the other hand assume knowledge of the jitter range, enhancement to said features may take the jitter into consideration limiting its impact. As an example, enhanced CDRX may be configured to cover the entire jitter range, however we note, that a long jitter range, resulting in equally long CDRX on-activity, may be suboptimal for power saving. Hence instead of covering the entire jitter range with CDRX on-activity, the on-activity may be split up into multiple parts that together cover the significant parts of the jitter range. Such configuration could limit the on-activity to conserve power. This enhancement is further discussed in section 2.3.3 in [4].

- Observation 10	Static application information on the expected jitter of the video flow can be used to enable enhancements.

## 2.2	Summary of the discussion

Based on the discussion in this document, we have identified what XR information is useful to be provided to RAN such that RAN can use the information to improve the capacity and power consumption while meeting the requirements for XR traffic.

Therefore, we propose the following to be considered:

The following characteristics of XR traffic are needed to be provided to RAN:

  - application packet size information (application packet size, association of IP packets to application packets, number of IP packets in application packet)

  - application packet periodicity (generation rate) and periodicity changes for each application traffic flow

  - application packet periodicity start time per application flow

  - application packet jitter information, e.g. range, per application flow

  - delay budget of the application packet for radio interface, per application flow.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	An application packet is used to identify a unit of application information, e.g., a video frame in RAN.

Observation 2	For XR services, it is inefficient for RAN to treat IP packets independently rather RAN should treat packets on application packet basis.

Observation 3	Two types of application information can be used in RAN, static and dynamic information

Observation 4	Dynamic information is needed per application packet, while static information is needed once or infrequently.

Observation 5	Application packet size information is beneficial for the scheduler and by doing better prioritization, the network could increase capacity by 10%

Observation 6	Application packet delay budget information is beneficial for the scheduler and by doing proper prioritization, the network could increase capacity by 10%

Observation 7	Application packet delay budget information is beneficial for the scheduler and by dropping application packets not meeting the requirements, 10% more capacity can be obtained

Observation 8	Static information of the application packet periodicity of each traffic flow may be used to align scheduling and power saving features such as CDRX, DG, SPS and CG.

Observation 9	To enable a fast configuration of enhanced CDRX, static information of the start time offset of periodic XR traffic flow is needed.

Observation 10	Static application information on the expected jitter of the video flow can be used to enable enhancements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The following characteristics of XR traffic are needed to be provided to RAN:

	application packet size information (application packet size, association of IP packets to application packets, number of IP packets in application packet)

	application packet periodicity (generation rate) and periodicity changes for each application traffic flow

	application packet periodicity start time per application flow

	application packet jitter information, e.g. range, per application flow

	delay budget of the application packet for radio interface, per application flow.

# References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

R1-2203639, “Discussion on capacity enhancements for XR”, Ericsson, RAN#109-e, May 2022.

R1-2203638, “Discussion on power saving enhancements for XR”, Ericsson, RAN#109-e, May 2022