3GPP TSG RAN WG1#109-e Meeting	                                               R1-2203650

e-Meeting, May 9th – 20th, 2022

Source:			 Southeast University (SEU)

Title:	Evaluation on AI-based CSI feedback

Agenda Item:	 9.2.2

Document for:	 Discussion and Decision

1.Introduction

With the recent rapid development of deep learning (DL) technology, many DL-based CSI feedback schemes have been developed [1,2,3]. DL can take full advantage of environment information to improve the feedback performance, which is difficult to achieve in codebook design.

Given the implicit feedback mechanism of current system, in this contribution, we propose a DL-based implicit feedback architecture to inherit the low-overhead characteristic, which uses NNs to replace the codebook-based precoding matrix encoding and decoding modules.

2.A DL-based implicit feedback architecture

Type I and Type II codebooks are designed based on discrete Fourier transform (DFT) vectors under the premise that antenna arrays are evenly arranged in horizontal and vertical dimensions and limits antenna design and optimization. Inspired by the successful applications of DL in explicit feedback, for specific antennas and complex scenarios where Type I and Type II codebooks are not applicable, DL can use simulated or measured data to train matched neural networks (NNs) [4].

Figure 1. Architecture of ImplicitNet

Figure 1 shows a DL-based implicit CSI feedback architecture named ImplicitNet. The feedback information Vstack is the eigenmatrix consists of the eigenvectors Vs extracted from Ns subbands, which can be written as:

Vstack=V1, V2,…,VNs

Where Vstack∈CNt*Ns, and Nt denotes the number of transmit antennas.

Specifically, the eigenvectors in the eigen-matrix Vstack are spliced in the vertical direction and then the real and imaginary parts are extracted and concatenated to form a real-valued vector. The Ns eigenvectors are compressed as a whole and the input dimension of the encoder is 2Nt×Ns×1.

The basic framework of ImplicitNet contains three modules: encoder, quantizer, and decoder. A three-layer bidirectional LSTM (bi-LSTM) network is used as the encoder for sufficient extraction. After feature extraction and compression by the three-layer bi-LSTM network, an L-dimensional compressed codeword is obtained and sent to the quantization module to generate the corresponding bitstream for feedback. B-bits uniform quantization in is adopted in the quantization module, and the total feedback overhead is L × B bits. Then the decoder at the BS recovers the eigenvector from the feedback bitstream.

Finally, the recovered vector is reshaped into matrix, denoted as Vstack. The performance metric of the network can be given by the cosine similarity:

ρ=1Nss=1Ns|vsHvs|||vs||2||vs||2

where vs is the column vector (eigenvector) that makes up the eigen-matrix Vstack.

The proposed DL-based architectures are built using Keras library, with Tensorflow as the backend, and all simulations are implemented on an NVIDIA DGX-1 workstation. The NNs are trained by supervised learning. The label of the input sample is set to the sample itself. The adaptive moment estimation (Adam) optimizer is used to update the weights of NNs in the training process. The batch size is set as 1,024, and the training epoch is 1,000. The initial learning rate is 0.001 and will be reduced by half if the validation loss does not decrease over 50 epochs.

3.Simulation Results

Based on the discussion in the above section, we conduct a numerical simulation to evaluate the performance of the proposed DL-based implicit feedback architecture.

The dataset we used is generated for link-level eigenvector-based channel state information (CSI) feedback research according to Table 7.7.1-1 and Table 7.7.1-3 in 3GPP TR 38.901.All the dataset we used can be downloaded freely on wireless intelligence (wireless-intelligence.com)

We generate 720,000 samples for each dataset. The samples are randomly shuffled and divided into training, validation, and testing sets containing 80%, 10%, and 10% samples, respectively.

Firstly, we tested the ImplicitNet on dataset CDLC30_3_100 and CDLC300_3_100 with assumption in Table 1&Table 2.

Table 1: Simulation assumption of dataset CDLC30_3_100

| Parameter | Value |
| --- | --- |
| Channel type | CDLC |
| Delay Spread | 30ns |
| UE speed | 3km/h |
| Interval sampling slots | 100 |
| Carrier frequency | 3.5GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antennas | 32T4R |
| Rank | 1 |
| Subband | 12 |
| Channel estimation | Ideal |
| UE number | 1000 |
| Slot number: | 100 |

Table 2: Simulation assumption of dataset CDLC300_3_100

| Parameter | Value |
| --- | --- |
| Channel type | CDLC |
| Delay Spread | 300ns |
| UE speed | 3km/h |
| Interval sampling slots | 100 |
| Carrier frequency | 3.5GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antennas | 32T4R |
| Rank | 1 |
| Subband | 12 |
| Channel estimation | Ideal |
| UE number | 1000 |
| Slot number: | 100 |

The results we obtained are shown in the table below, compared with type ll codebook in R15.

Table 3: Recover performance of ImplicitNet on dataset CDLC30_3_100

| Schemes | Feedback Bits | ρ |
| --- | --- | --- |
| Type ll | 327 | 0.940 |
| ImplicitNet | 48 | 0.991 |
|  | 72 | 0.993 |
|  | 96 | 0.996 |
|  | 144 | 0.997 |
|  | 192 | 0.997 |
|  | 288 | 0.997 |

Table 4: Recover performance of ImplicitNet on dataset CDLC300_3_100

| Schemes | Feedback Bits | ρ |
| --- | --- | --- |
| Type ll | 327 | 0.920 |
| ImplicitNet | 48 | 0.960 |
|  | 72 | 0.966 |
|  | 96 | 0.984 |
|  | 144 | 0.991 |
|  | 192 | 0.992 |
|  | 288 | 0.994 |

Observation 1: ImplicitNet still outperform type ll codebook when the feedback overhead is approximately reduced by 85%.

Then, we used dataset CDLA30_3_100(whose channel type is CDLA and other assumptions remain the same as CDLC30_3_100) to test the network trained on dataset CDLC30_3_100. The results are shown in Table 5.

Table 5: Recover performance of ImplicitNet on dataset CDLC30_3_100

| Schemes | Feedback Bits | ρ |
| --- | --- | --- |
| Type ll | 327 | 0.940 |
| ImplicitNet | 48 | 0.980 |
|  | 72 | 0.987 |
|  | 96 | 0.991 |
|  | 144 | 0.994 |
|  | 192 | 0.994 |
|  | 288 | 0.995 |

Observation 2: ImplicitNet model trained on CDLC30_3_100 performs nicely on dataset CDLA30_3_100, which shows the Generalization capabilities of ImplicitNet on CDL channel model.

After that, we tested the network’s generalization capabilities on different delay spread by dataset CDLC30_3_100 and CDLC300_3_100.

We firstly trained the network on a mixed dataset with 50% of CDLC30_3_100 samples and 50% of CDLC300_3_100 samples, and tested on both datasets. The results we obtained are shown in the table 6.

Table 6: Recover performance of ImplicitNet trained on mixed dataset

| Feedback Bits | ρ(CDLC30_3_100) | ρ(CDLC300_3_100) |
| --- | --- | --- |
| 48 | 0.984 | 0.966 |
| 72 | 0.990 | 0.971 |
| 96 | 0.9927 | 0.980 |
| 144 | 0.993 | 0.987 |
| 192 | 0.994 | 0.990 |
| 288 | 0.995 | 0.992 |

Then, we finetuned the network trained on dataset CDLC30_3_100 by 5000 samples of dataset CDLC300_3_100, and tested it on CDLC300_3_100. The results we obtained are shown in the table 7.

Table 7: Recover performance of ImplicitNet finetuned by 5000 samples in dataset CDLC300_3_100

| Feedback Bits | ρ |
| --- | --- |
| 48 | 0.951 |
| 72 | 0.959 |
| 96 | 0.960 |
| 144 | 0.972 |
| 192 | 0.974 |
| 288 | 0.986 |

Observation 3: ImplicitNet model can fit dataset with different delay spread by mixed training and finetuning.

4.Conclusion

In this contribution, we propose a DL-based implicit feedback architecture named ImplicitNet, and show the performance of the network. Simulation results show that ImplicitNet obviously outperform type ll codebook with lower feedback overhead, and the generalization capabilities on CDL model is also outstanding.

- Proposal 1: The AI based network’s performance need to be evaluated on different datasets.

- Proposal 2: The feedback overhead of AI based network needs to be taken into consideration.

- Proposal 3: Generalization capabilities of AI based network should be evaluated.

5.Reference

- C.-K. Wen, W.-T. Shih, and S. Jin, “Deep learning for massive MIMO CSI feedback,” IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748–751, 2018.

- J. Guo, X. Yang, C. Wen, S. Jin, and G. Y. Li, “Convolutional neural network-based multiple-rate compressive sensing for massive MIMO CSI feedback: Design, simulation, and analysis,” IEEE Transactions on Wireless Communications, vol. 19, no. 4, pp. 2827–2840, 2020.

- T. Wang, C.-K. Wen, S. Jin, and G. Y. Li, “Deep learning-based CSI feedback approach for time-varying massive MIMO channels,” IEEE Wireless Communications Letters, vol. 8, no. 2, pp. 416–419, 2019.

- M. Chen, J. Guo, C.-K. Wen, S. Jin, G. Y. Li, and A. Yang, “Deep learning-based implicit CSI feedback in massive MIMO,” IEEE Transactions on Communications, vol. 70, no. 2, pp. 935–950, 2022.