3GPP TSG RAN WG1 #109-e	R1-2203652

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.8.2

Source:	China Telecom

Title:	Remaining issues on joint channel estimation for PUSCH and PUCCH

Document for:		Discussion

Introduction

In RAN #90 e-meeting, a new Rel-17 work item on NR coverage enhancements was approved [1] and revised in [2]. The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD. The objectives include mechanism(s) to enable joint channel estimation.

- Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

  - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

    - Potential optimization of DMRS location/granularity in time domain is not precluded

  - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

Editors’ CRs on introduction of coverage enhancements in NR have been approved in RAN#94e [3] and RAN#95e [4]. In this contribution, we discuss the remaining issues on events and TPC commands for joint channel estimation for PUSCH and PUCCH.

Events

In RAN1#107bis-e [5] and RAN1#108-e [6], it was discussed extensively on the following two cases for UEs not capable of restarting DM-RS bundling.

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

Based the current specification, for UEs not capable of restarting DM-RS bundling, when PUSCH-Window-Restart or PUCCH-Window-Restart is enabled, it is not clear whether a new actual TDW is created for the above two cases, which has been acknowledged by the majority companies in RAN1#107bis-e and RAN1#108-e. In addition, companies acknowledged that there would be performance degradation if gNB and UE have different understandings. Therefore, it is necessary to clarify the specification.

Fig.1 Illustration of case 1

Fig.2 Illustration of case 2

The motivation of introducing UE capability of supporting restarting DMRS bundling is that UE may need to know the start of each actual TDW ahead of time. This issue is only relevant to dynamic events, since UE can know semi-static events beforehand. In this sense, a new actual TDW is created after a semi-static event no matter whether there are dynamic events before the semi-static event. For case 2, gNB should avoid the overlapping between semi-static events and dynamic events. In case the overlapping cannot be avoided, since the UE knows the semi-static events beforehand, a new actual TDW is created after the event.

Proposal 1: For UEs not capable of restarting DM-RS bundling,

- If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the triggered semi-static event.

- If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the triggered semi-static event.

- Note: No specification impact is expected.

TPC commands

In RAN1#107-e, the following working assumption was reached.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

In the above working assumption, there are four cases:

- Case 1: accumulate TPC commands for CG-PUSCH and semi-static PUCCH

- Case 2: absolute TPC commands for CG-PUSCH

- Case 3: accumulate TPC commands for DG-PUSCH and dynamic PUCCH

- Case 4: absolute TPC commands for DG-PUSCH

Companies have different understandings on Rel-15/16 power control procedure for Case 2/3/4.

Observations in [5]:

- Clarification on whether absolute TPC command is supported for group common TPC with DCI format 2_2 for Rel-15/16 is needed.

  - If supported, whether the timeline of absolute TPC command follows the accumulate TPC command.

- Clarification on the interpretation of the definition of KPUSCH(i)KPUSCH(i) for DG-PUSCH in TS 38.214 for Rel-15/16 is needed.

  - Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i. With this interpretation, value of KPUSCH(i)KPUSCH(i) for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB.

  - Interpretation 2: KPUSCH(i)KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the first PUSCH repetition for a TB. With this interpretation, value of KPUSCH(i)KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

In RAN1#108-e, the relevant issues were discussed in Rel-15 maintenance in [7]. However, no consensus has been reached and there would be no further discussion, which means that the ambiguous issue for Case 2/3/4 cannot be clarified by the specification anymore.

In RAN1#107bis-e [5] and RAN1#108-e [6], it was discussed extensively how to capture the working assumption. Different options have been proposed for Case 1.

- Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2a: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - Option 3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after are received between KPUSCHi1 symbols before transmission occasion i1 and no later than KPUSCHk symbols before transmission occasion k (i.e. including occasion k itself). - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n. - Combined Option 1&3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, KPUSCHi for PUSCH transmissions occasion i from K KPUSCHi1 symbols before the start a first symbol of the transmission occasion i1 before to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. - The definition of set for in Rel-15/16 TS 38.213 is reused. For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that are received between KPUSCHi1 symbols before transmission occasion i1 and KPUSCHk symbols before transmission occasion k. - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n.

In RAN1#108-e, it was further discussed how to proceed if finally no consensus can be reached on how to capture the working assumption. There are following alternatives.

- Alt 1: It’s up to Editor how to capture it into the specification. - Alt 2: The action of group common TPC commands with format 2_2 is regarded as an event. - Alt 3: UE does not expect to receive group common TPC commands with format 2_2 that would take effect during the actual TDWs.

Seen from [6], the discussion is painful. Even if the consensus can be achieved for Case 1, the problem still exists for Case 2/3/4 which cannot be handled anymore as discussed above. Therefore we think Alt 2 or Alt 3 makes the most sense.

Proposal 2: Down select one of the following two alternatives.

- Alt 2: The action of group common TPC commands with format 2_2 is regarded as an event.

- Alt 3: UE does not expect to receive group common TPC commands with format 2_2 that would take effect during the actual TDWs.

Conclusions

In this contribution, we discussed the remaining issues on events and TPC commands. We have following proposals:

Proposal 1: For UEs not capable of restarting DM-RS bundling,

- If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the triggered semi-static event.

- If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the triggered semi-static event.

- Note: No specification impact is expected.

Proposal 2: Down select one of the following two alternatives.

- Alt 2: The action of group common TPC commands with format 2_2 is regarded as an event.

- Alt 3: UE does not expect to receive group common TPC commands with format 2_2 that would take effect during the actual TDWs.

References

- 3GPP RP-202928, “New WID on NR coverage enhancements”, China Telecom, RAN#90e, December 7th – 11th, 2020.

- 3GPP RP-211566, “Revised WID on NR coverage enhancements”, China Telecom, RAN#92e, June 14th – 18th, 2021.

- 3GPP RP-212973, “Introduction of coverage enhancements in NR”, RAN1, RAN#94e, December 6th – 17th, 2021.

- 3GPP RP-220257, “Rel-17 maintenance of coverage enhancements in NR”, RAN1, RAN#95e, March 17th – 23rd, 2022.

- 3GPP R1-2200790, “Summary of email discussion on joint channel estimation for PUSCH”, Moderator (China Telecom), RAN1#107bis-e, January 17th – 25th, 2022.

- 3GPP R1-2202870, “Summary of email discussion on joint channel estimation for PUSCH”, Moderator (China Telecom), RAN1#108-e, February 21st – March 3rd, 2022.

- 3GPP R1-2202834, “Summary of [108-e-NR-CRs-03] Issue#4 Absolute TPC timeline and support of absolute TPC with DCI format 2_2”, Moderator (ZTE Corporation), RAN1#108-e, February 21st – March 3rd, 2022.