3GPP TSG RAN WG1 #109-e	R1-2203662

e-Meeting, May 9th – May 20th, 2022

Source:	China Telecom

Title:	Discussion on network energy saving performance evaluation methods

Agenda Item:	9.7.1

Document for:	Discussion

- Introduction

The new Rel-18 SI of network energy saving NR was approved in RAN#94e [1], and the following objectives are included.

1. Definition of a base station energy consumption model [RAN1] • Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. 2. Definition of an evaluation methodology and KPIs [RAN1] • The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. 3. Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: • How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] • Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

In this contribution, we focus on the performance evaluation methods of network energy saving, including the base state energy consumption model and KPIs.

- Considerations on the definition of the base station energy consumption model

2.1 Reference configurations for relative energy consumption model

The energy consumption of the base station including the static part and the dynamic part. The static energy consumption mainly corresponding to the circuit energy consumption of BBU and RRU, while the dynamic part comes from the energy consumption of RF. In the current network, the dynamic part domains total energy consumption. Since the massive MIMO is adapted in the base station, the dynamic energy consumption can be influenced by multiple factors, including TRX/beam number, bandwidth, component carrier number, PA efficiency, the period of common signals/channels, etc. For simplifying the work and focus on the influence of the factors on energy consumption, the framework of relative UE power consumption model in TR 38.840[2] is proposed to be referenced in the WI, and the reference configurations should be defined. Besides the factors mentioned in the TR 38.840, the system parameters corresponding to the RF should be considered for the base station energy consumption model.

Proposal 1: The reference configurations should be defined for the energy consumption model of base station. Besides the parameters in TR 38.840, the basic system parameters corresponding to the RF should be considered, at least including

•	Number of TRX/beam

•	Bandwidth

•	Number of PRB

•	Number of CC

2.2 Sleeping modes

For better evaluating and saving the network energy consumption, the sleeping modes of BS are needed. When the BS is in the sleeping mode, the energy consumption can be largely reduced. However, as shown in Fig 1, the lower power level of sleeping mode, the more time is needed for the BS to transit to the active mode. Therefore, multiple sleeping mode may be needed for different demand. The deep sleeping mode can be used for the scenarios without low-latency services, while the light sleeping mode can be more suitable for the symbol level switch on-off. But since the goal of defining the sleeping modes is to evaluation the energy consumption with/without the energy saving techniques, only typical sleeping modes are needed, but at least two sleeping modes should be defined. The specific number of the sleeping modes can be decided after the potential energy saving techniques have been confirmed.

Fig 1 the relation between transition time and power of sleeping mode

Proposal 2: The sleeping mode of BS should be defined for better evaluating the energy consumption of network.

- At least 2 sleeping modes should be defined.

  - Deep sleeping mode: with relative lower power level but longer transition time

  - Light sleeping mode: with relative higher power level but can finish the transition in symbol-level.

- Only typical sleeping modes are needed to be defined.

2.3 Evaluation scenarios and assumptions

The scenarios with massive MIMO should be evaluated with higher priority. And considering the current network construction, the urban micro in FR1 can be evaluated first.

Proposal 3:The scenario with massive MIMO should be evaluated with higher priority, the urban micro in FR1 can be a start point.

The moderator's summary in RAN#94-e[3] has confirmed that though not explicitly included in the objective of CA enhancement, the scenario of inter-band CA with SSB-less Scell should be considered in the network energy saving SI. So, we proposed also to evaluate the scenario of inter-band CA Sell with high priority.

Proposal 4:The scenario of inter-band CA Scell should be evaluated with higher priority.

The network energy consumption can be influenced directly by the traffic load, so the traffic mode and traffic load should be considered in the simulation assumptions.

Proposal 5:The simulation assumption should take the traffic mode and traffic load into consideration.

Since different techniques may be adapted in the FDD and TDD system, so both the FDD and TDD system should be simulated and evaluated.

Proposal 6: Both the FDD and TDD system should be simulated in the evaluation.

- Considerations on the KPIs of the network energy consumption

The network energy efficiency(EE) has been introduced in multiple specs in 3GPP and ITU-T for evaluating the energy consumption of NR system. However, the definition of the EE can be different in different specs. In 3GPP TR 38.913[4], the network EE is defined as

EEglobal=scenario KbKEEscenario K where EEScenario=load level 1a1V1EC1 bk refers to the weights of every deployment scenario where the network energy efficiency is evaluated. V1= Refers to the traffic per second served by a base station (in bits/s) EC1 = Refers to the power consumed by a base station to serve V1 (in Watt = Joule/s). a1 = Refers to the weight for each traffic load level.

This definition takes the different scenarios and traffic load level into consideration, the EE is a comprehensive consideration. While in this SI, the different scenarios and traffic loads are evaluated separated, so the EE can be simplified by the network throughput (in bps)/energy consumption (in Joule).

However, as pointed in the objective 2 of SI, the evaluation methodology should not focus on a single KPI, not only the capacity, but also the UPT/latency/coverage can also be considered to evaluate the performance. In the ITU-T L.1331[5], the evaluation methodology of EE can be referenced. The universal expression of EE can be given by

EEKPI=VKPIEC

Where the VKPI refers to the evaluated performance KPI (e.g. capacity, UPT, latency, coverage), and EC refers to the energy consumption (in Joule). With the universal expression of the EE, multiple KPIs can be evaluated in a general methodology.

Proposal 7: The universal definition of energy efficiency(EE) can be used for evaluating

- EEKPI=VKPIEC

  - VKPI refers to the focused evaluated performance KPI, which can be but not limited to

    - Capacity/system throughput (in bps)

    - UPT (in bps)

    - Latency (in ms)

    - Coverage (in m)

  - EC refers to the energy consumption (in Joule)

- Conclusion

In this paper, China Telecom’s views on the remain issues for PUSCH coverage enhancements are given.

Proposal 1:

The reference configurations should be defined for the energy consumption model of base station. Besides the parameters in TR 38.840, the basic system parameters corresponding to the RF should be considered, at least including

•	Number of TRX/beam

•	Bandwidth

•	Number of PRB

•	Number of CC

Proposal 2:

The sleeping mode of BS should be defined for better evaluating the energy consumption of network.

- At least 2 sleeping modes should be defined.

  - Deep sleeping mode: with relative lower power level but longer transition time

  - Light sleeping mode: with relative higher power level but can finish the transition in symbol-level.

- Only typical sleeping modes are needed to be defined..

.

Proposal 3:

The scenario with massive MIMO should be evaluated with higher priority, the urban micro in FR1 can be a start point.

Proposal 4:

The scenario of inter-band CA Scell should be evaluated with higher priority.

Proposal 5:

The simulation assumption should take the traffic mode and traffic load into consideration.

Proposal 6:

Both the FDD and TDD system should be simulated in the evaluation..

Proposal 7:

The universal definition of energy efficiency(EE) can be used for evaluating

- EEKPI=VKPIEC

  - VKPI refers to the focused evaluated performance KPI, which can be but not limited to

    - Capacity/system throughput (in bps)

    - UPT (in bps)

    - Latency (in ms)

    - Coverage (in m)

  - EC refers to the energy consumption (in Joule)

- References

3GPP RP-213554, “New SI: Study on network energy savings for NR”, Huawei, RAN#94-e, Dec 6th- 17th, 2021.

3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR(Release 16)”.

3GPP RP-213469, “Appendix 2 Detailed scope for potential R18 items for email discussion”, Chairman, RAN#94-e, Dec 6th- 17th, 2021.

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies (Release 16)”.

ITU-T L.1331, “Assessment of mobile network energy efficiency”