3GPP TSG RAN WG1 #109-e	R1-2203663

e-Meeting, May 9th – May 20th, 2022

Source:	China Telecom

Title:	Discussion on network energy saving techniques

Agenda Item:	9.7.2

Document for:	Discussion

- Introduction

The new Rel-18 SI of network energy saving NR was approved in RAN#94e [1], and the following objectives are included.

1. Definition of a base station energy consumption model [RAN1] • Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. 2. Definition of an evaluation methodology and KPIs [RAN1] • The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. 3. Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: • How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] • Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

In this contribution, we focus on the potential network energy saving techniques.

- Considerations on potential techniques

2.1 Potential techniques in time domain

According to the test results in current network, by symbol level switching off the TRX, the energy consumption can be reduced effectively for low load network with nearly no impact on performance. However, the transition time is needed for PA when switching-off the TRX, the symbol-level switching on-off frequently can lead to the latency, which may have influence on the network performance. To achieve the more efficient dynamic symbol on-off, we proposed Rel-18 to support a semi-persistent method of TRX symbol on-off. Several symbol on-off formats can be pre-configured, then be triggered by the UE with UCI or MAC CE or self-adapted adjusted by the gNB when the traffic load is low. With this mechanism, the symbol switch on-off can be more efficiently.

Proposal 1:

The semi-persistent symbol switch on-off should be studied for network energy saving.

2.2 Potential techniques in spatial domain

According to the test results in current work, by switching off the TRX from 64T to 32T, the energy consumption can be reduced effectively, especially for medium load network. However, the cell throughput descends about 15%-20% at the same time, which need to be further enhanced. For maintaining the better performance when saving the energy, the CSI reporting can be enhanced so that the channel can be better measured and deciding whether the TRX can be switch off and how to switch off the TRX can bring better performance.

Proposal 2:

The CSI reporting enhancement should be studied for better deciding the TRX switch on-off.

Since the TRX switch can be especially for the low and medium load network, the network can self-adapted decide to switch off the TRX according to the traffic load.

Proposal 3:

The network self-adapted switch-off the TRX according to the traffic load should be studied.

Besides, when switching off the TRX, the configured CSI-RS may not be suitable since the antenna ports have changed. Therefore, it is necessary to configure the CSI-RS if the TRX switch off is adopted. The CSI-RS should be configured adapted to the variation of the number of antenna ports.

Proposal 4:

The CSI-RS should be reconfigured when the TRX switch off is adopted.

2.3 Potential techniques in frequency domain

The moderator's summary in RAN#94-e[2] has confirmed that the scenario of inter-band CA with SSB-less Scell should be considered in the network energy saving SI. The SSB-less intra-band CA has been supported, we think the SSB-less inter-band CA should also be supported, so that an SSB in one band can provide DL timing/ frequency synchronization for a Scell in a different band, which bands are feasible and the related UE requirements should be identified.

Proposal 5:

The Scells without SSB in inter-band CA should be studied.

- Which bands are feasible and the related UE requirements should be further identified.

2.4 gNB DRX/DTX for energy saving

The UE DRX and WUS were introduced in the Rel-16 UE power saving[3]. When thes UE are configured with DRX, the DTX can be considered to be introduce for gNB. When the gNB is in DTX mode, the gNB can decide whether the DTX is cell specific or gNB specific. The DTX can be periodically or woke up by the WUS from UE with low layer signal (e.g. MAC CE, UCI). Besides, the gNB DRX mechanism can also be introduced when the traffic load is low. With these method, the gNB dormancy can be achieved and save the energy of network.

Proposal 6:

The gNB DRX/DTX should be studied for network energy saving.

- Conclusion

In this paper, China Telecom’s views on the remain issues for PUSCH coverage enhancements are given.

Proposal 1:

The semi-persistent symbol switch on-off can be supported in Rel-18.

Proposal 2:

The CSI reporting enhancement should be studied for better deciding the TRX switch on-off.

Proposal 3:

The network self-adapted switch-off the TRX according to the traffic load should be studied..

Proposal 4:

The CSI-RS should be reconfigured when the TRX switch off is adopted.

Proposal 5:

The Scells without SSB in inter-band CA should be studied.

- Which bands are feasible and the related UE requirements should be further identified.

Proposal 6:

The gNB DRX/DTX should be studied for network energy saving.

- References

3GPP RP-213554, “New SI: Study on network energy savings for NR”, Huawei, RAN#94-e, Dec 6th- 17th, 2021.

3GPP RP-213469, “Appendix 2 Detailed scope for potential R18 items for email discussion”, Chairman, RAN#94-e, Dec 6th- 17th, 2021.

3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR(Release 16)”.