3GPP TSG RAN WG1 #109			R1- 2203668

Online Meeting, 9th – 20th May 2022

Source:	Panasonic

Title: 	Discussion on Sidelink Co-channel Coexistence

Agenda Item:		9.4.2

Document for:	Discussion, Decision

# Introduction

This contribution is to discuss the mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink.

# Discussion

The following objective has been specified to be worked on for release 18 in the WID of sidelink evolution (RP-213634).

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In Release 16, the in-device coexistence of LTE and NR has been discussed for both long term and short term time-scale solutions. This is the case LTE and NR are used in the different bands.

It has been concluded in TR38.885 that: For long-term timescale coordination, coexistence is feasible from the physical-layer point of view by (pre-)configuring resource pools which are non-overlapping in the time-domain, with no need to modify LTE specifications. For short-term timescale coordination, coexistence is feasible between NR SL and LTE SL with SPS scheduling, when SL transmissions from both RATs overlap, or transmission from one overlaps with reception for the other, by prioritizing one of the RATs on each occurrence.

The following table is summarized (subject to processing time restriction) from TS38.213 section 16.2.4.1 - Simultaneous NR and E-UTRA transmission/reception on the short term time scale coordination.

| Overlaps of: Conditions: | LTE TX | LTE TX | LTE RX | LTE RX |
| --- | --- | --- | --- | --- |
|  | NR TX | NR RX | NR TX | NR RX |
| If packet priorities are known: | Transmit packet with higher priority | Transmit/Receive packet with higher priority | Transmit/Receive packet with higher priority | UE implementation |
| - In case same priority: | UE implementation | UE implementation | UE implementation | UE implementation |
| If packet priorities are not known: | UE implementation | UE implementation | UE implementation | UE implementation |

For the release 16 solutions, we observed that the short term time-scale coordination based on TDM and very much rely on UE implementation, except when packet priorities are known via either LTE or NR.  Considering lower spec impact, the priority rules defined in Release 16 could be reused as baseline for intra UE LTE/NR overlaps.

Proposal 1: The priority rules defined in Release 16 could be reused as baseline for intra-UE LTE/NR overlaps

For the intra UE LTE/NR overlap scenarios of [LTE TX, NR RX] and [LTE RX, NR TX], no concurrent Tx/RX could be performed as SL is half duplex only. For the overlap scenario of [LTE RX, NR RX], we think SL UEs should be capable to receive both LTE SL and NR SL as it has similar complexity with separate FDMed LTE and NR spectrums. Then for the overlap scenario of [LTE TX, NR TX], for SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing may also be considered.

Proposal 2: SL UEs with Co-channel Coexistence capability should be capable to receive both LTE SL and NR SL

Proposal 3: For SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing may also be considered

Other than to re-use the priority rules defined in Release 16 for intra-UE overlap scenarios, the inter-UE overlap scenarios of LTE and NR (which will be potential in-air collisions between UEs) also need to be considered. For mode-2 UE, as the information exchange are not efficient among SL UEs, we think at least some long term time-scale coordination should be considered. Some persistent or semi-persistent resource configurations can be allocated for LTE and NR respectively. Short term time-scale coordination also should be considered using control channels.

Proposal 4: Long term time-scale coordination should be considered for inter-UE LTE/NR overlap that some persistent or semi-persistent resource configurations can be allocated for LTE and NR respectively. Short term time-scale coordiantion using control channel should be also considered.

# Conclusion

In this contribution, followings observation and proposals are made:

Proposal 1: The priority rules defined in Release 16 could be reused as baseline for intra-UE LTE/NR overlaps

Proposal 2: SL UEs with Co-channel Coexistence capability should be capable to receive both LTE SL and NR SL

Proposal 3: For SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing may also be considered

Proposal 4: Long term time-scale coordination should be considered for inter-UE LTE/NR overlap that some persistent or semi-persistent resource configurations can be allocated for LTE and NR respectively. Short term time-scale coordiantion using control channel should be also considered.

# Reference

[1] RP-213634, “Work Item Description: NR sidelink evolution,” OPPO, LG Electronics, RAN#94-e

[2] TS38.213: Physical Layer Procedures for Control

[3] TR38.885: Study on NR Vehicle-to-Everything (V2X)