3GPP TSG RAN WG1 #109-e	R1-2203669

E-meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on NTN coverage enhancement

Agenda Item:		9.12.1

Document for:	Discussion and decision

Introduction

Rel.18 work item on Non-Terrestrial Network (NTN) [1] has been started. The scope is as follows.

- Coverage enhancement

- NR-NTN deployment in above 10 GHz bands

- Network verified UE location

- NTN-TN and NTN-NTN mobility and service continuity enhancements

On NTN coverage enhancement, the following work is to be conducted according to the WID [1].

Study phase until RAN#97 (September 2022)

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals

Specify coverage enhancement technique if issues are identified (after RAN#97)

The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec.

In this document, we discuss evaluation scenario, methodology and parameters for NTN coverage evaluation.

Evaluation of NTN coverage

According to WID [1], the target scenario for the coverage enhancement is voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0 dBi currently assumed for link budget analysis for NTN. The evaluation scenario, methodology and assumptions are discussed below.

Scenario

In NTN study item, 30 study cases with different satellite orbits, frequency bands, frequency reuse and so on, were defined and evaluated. In order to timely finalization of WID, target scenarios for the coverage evaluation should be limited. LEO would be the main target for smartphones while GEO would not be main target although the solution can be applied to GEO as well. We propose to limit the scenarios for the evaluation to LEO 1200 km because LEO 1200 km is more severe in terms of link budget than LEO 600 km and the evaluation result of LEO 1200 km could be applied to LEO 600 km. The main target frequency band should be S-band (2GHz) because target usage is VoIP and low data rate service i.e. narrow bands and lower frequency would be more suitable.

Therefore, good candidates for the evaluation with high priority would be the case 14 and case 29 (i.e. LEO1200km with satellite parameter set 1 and set 2, S-band, frequency reuse 1) shown in Table 1.

Table 1 Proposed high priority study cases for coverage enhancement

| Case | Satellite orbit | Satellite parameter set | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- |
| 14 | LEO-1200 | Set 1 | Handheld | S-band | Option 1 |
| 29 | LEO-1200 | Set 2 | Handheld | S-band | Option 1 |

In addition, handling of HAPS would be FFS. If HAPS is to be evaluated, related parameters need to be discussed. HAPS parameters in TR38.863 V030[4] on co-existence study in RAN4 would be a good starting point for the discussion.

Proposal 1: Deployment scenarios for coverage evaluation should be limited. High priority candidates would be study case 14 and 29 defined in TR38.821 (i.e. LEO 1200km with satellite parameter set 1 and set 2, S-band, frequency reuse 1)

Evaluation methodology

Link budget analysis shown in TR 38.821 section 6.1.3.1 should be used for the evaluation methodology. Table 2 shows the parameter list used for link budget calculation in the study item. Link budget calculation method in TR 38.821 is copied in Annex B in this document for convenience. The link budget provides the achievable SINR (or CINR) in each deployment scenario. The SINR calculated in the link budget should be compared with the required SINR of each physical channel (PUSCH, PUCCH, etc). The required SINR should be evaluated using LLS for each physical channel with Rel.17 coverage enhancement feature.

Table 2 Link budget template

| Parameters | Unit | Case 14 (LEO1200 set1) |  | Case 29 (LEO1200 set2) |  |
| --- | --- | --- | --- | --- | --- |
| Propagation |  | DL | UL | DL | UL |
| Frequency | GHz | 2 | 2 | 2 | 2 |
| Elevation angle | degree | 30 | 30 | 30 | 30 |
| Maximum distance | km | 1998.9 | 1998.9 | 1998.9 | 1998.9 |
| Free space path loss | dB | 164.5 | 164.5 | 164.5 | 164.5 |
| Atmospheric path loss | dB | 0.07 | 0.07 | 0.07 | 0.07 |
| Scintillation loss | dB | 2.2 | 2.2 | 2.2 | 2.2 |
| Shadowing margin | dB | 3 | 3 | 3 | 3 |
| Additional loss | dB | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |
| Bandwidth | MHz | 30 | 0.36 | 30 | 0.36 |
| Tx power | dBm | 54.8 | 23 | 54.8 | 23 |
| Antenna gain | dBi | 30 | TBD | 24 | TBD |
| EIRP | dBm | 84.8 | Depending on antenna gain | 78.8 | Depending on antenna gain |
| Receiver |  |  |  |  |  |
| antenna gain | dBi | TBD | 30 | TBD | 30 |
| number of antennas |  | 2 | 1 | 2 | 1 |
| polarization loss | dB | 3 | 3 | 3 | 3 |
| Noise Figure | dB | 7 | - | 7 | - |
| Ambient temperature | K | 290 | - | 290 | - |
| Antenna temperature | K | 290 | - | 290 | - |
| G/T | dB/K | Depending on antenna gain | 1.1 | Depending on antenna gain | -4.9 |
| Output |  |  |  |  |  |
| SNR | dB |  |  |  |  |
| SIR (based on geometry) | dB |  |  |  |  |
| SINR | dB |  |  |  |  |

Proposal 2: SINR derived from link budget analysis as in TR38.821 should be used as the target SINR for each deployment scenario. The required SINR evaluated by LLS for each physical channel with Rel.17 features should be compared with the target SINR.

Evaluation assumption

Satellite and UE parameters shown in TR 38.821 should be used as the baseline except antenna gain. -5 dBi which was proposed in e.g. [6] can be a starting point.

Satellite with circular polarization and UE with linear polarization should be assumed as a typical scenario. Therefore, 3 dB polarization loss should be considered.

For link level simulation, the following assumptions can be a starting point of the discussion. The same values as in TR 38.830 [5] on coverage enhancement are basically listed except NTN channel model. As Rel.17 coverage enhancement feature were specified, these are also enabled, i.e. TBoMS, DMRS bundling and so on.

Table A.1-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Packet size for VoIP | A packet size of 320 bits with 20ms data arriving interval is adopted. Size (bits). Payload; 256. CRC; 16 (TBS size lower than 3824 bits). MAC; 16 (with 12 bits SN size). RLC; 8 (with 6 bits SN size). PDCP; 16. RTP/UDP/IP; 24 (w RoHC). If applicable, companies report TB size assumed in evaluation. For SIP invite message - Payload of 1500 bytes can be a starting point. - The assumptions (TB size, time period etc.) are reported by companies. - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation - In addition, 1 second time period can also be considered. |
| Latency requirements for VoIP | Latency requirements assumed in VoIP evaluation are reported by companies. |
| Channel model for link-level simulation | Baseline NTN TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 30 deg. |
| UE velocity | 3 km/h |

Table A.1-2: Channel-specific parameters for PUSCH for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| SCS | 30 kHz |
| PUSCH duration | 14 OS |
| Repetitions | For VoIP, w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/MCS for VoIP | 4 PRBs for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Table A.1-3: Channel-specific parameters for PUCCH for FR1

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2 bits UCI. Format 3, 4 bits (3 bits A/N + 1 bit SR)/11/22 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| SCS | 30 kHz |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

Table A.1-4: Channel-specific parameters for PRACH for FR1

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Table A.1-5: Channel-specific parameters for PUSCH of Msg.3 for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| SCS | 30 kHz |
| HARQ configuration | For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Table A.1-6: Channel-specific parameters for PDSCH for FR1

| Parameter | Value |
| --- | --- |
| BLER | For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 or 4 for 2GHz |
| SCS | 30 kHz |
| HARQ configuration | For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS For PDSCH of Msg.4, 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |

Table A.1-7: Channel-specific parameters for PDCCH for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 or 4 for 2GHz |
| SCS | 30 kHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Table A.1-8: Channel-specific parameters for SSB for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 or 4 for 2 GHz |
| SCS | 30 kHz |
| Periodicity | 20 ms |
| Performance metric | Combination of 4 SSBs in 80 ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Proposal 3: For evaluation parameters of the respective physical channel, the same values as in TR38.830 on coverage enhancement except NTN channel model should be reused.

Conclusion

In this document, we discuss evaluation scenario, methodology and parameters for NTN coverage evaluation. The following proposals are made.

Proposal 1: Deployment scenarios for coverage evaluation should be limited. High priority candidates would be study case 14 and 29 defined in TR38.821 (i.e. LEO 1200km with satellite parameter set 1 and set 2, S-band, frequency reuse 1)

Proposal 2: SINR derived from link budget analysis as in TR38.821 should be used as the target SINR for each deployment scenario. The required SINR evaluated by LLS for each physical channel with Rel.17 features should be compared with the target SINR.

Proposal 3: For evaluation parameters of the respective physical channel, the same values as in TR38.830 on coverage enhancement except NTN channel model should be reused.

- Reference

- RP-220953, “WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- TR38.811 V15.4.0, “Study on New Radio (NR) to support non terrestrial networks”

- TR38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN)”

- TR38.863 V0.3.0, “Non-terrestrial networks (NTN) related RF and co-existence aspects”

- TR38.830 V17.0.0, “Study on NR coverage enhancements”

- RWS-210011, “On NR NTN Evolution”, Qualcomm

- Annex: Link budget calculation [3]

#### 6.1.3.1	Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB]	(6.1.3.1-1)

The formula for CNR calculation is

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]	(6.1.3.1-2)

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz, PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

Antenna-gain-to-noise-temperature GT can be derived by [2]

GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB]	(6.1.3.1-3)

where GR is receive antenna gain, Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. Receive antenna gain GR can be obtained by

GR dBi=GR,edBi+10log10NR,a-Lp[dB],  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna 	(6.1.3.1-4)

where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, Lp is polarization loss, η is the antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70), D is the equivalent antenna diameter, and λ is the wavelength.

EIRP can be calculated by

EIRP dBW=PT dBW-LC dB+GT [dBi]	(6.1.3.1-5)

where PT is antenna transmit power, LC is cable loss, and GT is transmit antenna gain and can be derived by

GT dBi=GT,e dBi+10log10NT,a,  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna	(6.1.3.1-6)

where GT,e is transmit antenna element gain and NT,a is the number of transmit antenna elements.

#### 6.1.3.2	Parameters

The parameter configuration for link budget analysis is listed in Table 6.1.3.2-1.

Table 6.1.3.2-1: Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band), 20 GHz for DL and 30 GHz for UL (Ka-band) |
| System bandwidth | 30 MHz (S-band), 400 MHz (Ka-band) |
| Channel bandwidth | DL: system bandwidth/ frequency reuse factor UL: UL in S-band (handheld UE): 360 kHz Otherwise: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be a challenge. |
| Satellite altitude | 600 km, 1200 km, 35786 km |
| Target elevation angle | 30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2) |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam based on logarithmic mean | Based on single satellite system-level calibration methodology, statistics for average CIR are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For DL calibration, CIR is computed by averaging CIR over UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-1 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment). For UL calibration, For Handheld device, the channel bandwidth is 360 kHz. For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. CIR is computed by averaging over 10 simultaneously transmitting UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-2 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment) The averaging should be performed over multiple realizations. |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | Ka-band: VSAT S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | The considerations of Section 6.1.1.1 on Polarization loss apply. |
| Outcome | CNIR |
| NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. dB |  |