3GPP TSG RAN WG1 #109-e										 		  R1-2203674

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.2

Source:	NEC

Title:	Discussion on remaining issues for multi-TRP deployment

Document for:	Discussion and Decision

- Introduction

Rel-17 enhancements multi-TRP deployment have been finished, while there are some remaining issues. In this contribution, we provided our views on some remaining issues for PUCCH and PUSCH.

- Discussion

TPC command value for PDCCH repetition

In Rel-17, PDCCH repetition was introduced based on multi-TRP, and it was agreed that each repetition has the same number of CCEs and coded bits, and corresponds to the same DCI payload.

While there is one thing missed during the discussion, which is accumulated TPC in case of PDCCH repetition. In case of accumulated TPC (i.e. if the UE is not provided tpc-Accumulation), sum of TPC command values within a duration is applied for PUSCH power control. So the issue is that in case of PDCCH repetition, whether one or both TPC command values of the two repetitions is applied. The main target for PDCCH repetition is to cope with blockage, as long as at least one of the two repetitions is decoded correctly, the DCI payload can be obtained. So based on this, it’s natural that only one TPC command value of the two repetitions is applied regardless of how many repetitions are decoded. So we propose that:

Proposal 1: In case of PDCCH repetition and accumulated TPC, only one TPC command value of the two repetitions is applied for PUSCH power control, and adopt the TP (1st TP in Appendix ).

TS38.213 h10 - 7.1.1 UE behaviour < Unchanged parts are omitted > - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i, if a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, as described in clause 10.1, only one TPC command value in the two PDCCH candidates is applied - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=1 if PO_UE_PUSCH,b,f,cj and αb,f,cj are provided by the second P0-PUSCH-AlphaSet in p0-AlphaSets; otherwise, l=0 - If j=1, - l is provided by the value of powerControlLoopToUse if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha in ConfiguredGrantConfig - l is provided by the value of powerControlLoopToUse2 if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 < Unchanged parts are omitted >

2nd TPMI field for PUSCH

For codebook based PUSCH repetition, two TPMI fields are applied, and the second TPMI field only indicates precoders corresponding to the number of layers indicated in the first TPMI field:

Agreement For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. For each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky KyTPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved. - How to describe/capture this in 38.212 is up to the editor.

And for full power transmission, subsets of precoders are applied based on the full power mode. While in current TS38.212, for full power transmission mode 1, more precoders can be indicated in the second TPMI field, in other words, more precoders can be applied in case of full power transmission mode 1, which is not correct and there is no such agreement. One example can be found as follows:

| Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; omitted; Bit field mapped to index; codebookSubset= nonCoherent. 0; 0; 1 layer: TPMI=0. 1; 1; 1 layer: TPMI=1. …; …; …. 3; 3; 1 layer: TPMI=3. 4; 4; 2 layers: TPMI=0. …; …; …. 9; 9; 2 layers: TPMI=5. 10; 10; 1 layer: TPMI=13. 11; 11; 2 layer: TPMI=6. 12; 12-15; Reserved. …. 20. 21. 22. 23. …. 29. 30-31. | Table 7.3.1.1.2-2D: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; omitted; Bit field mapped to index; codebookSubset= nonCoherent. 0; 0; 1 layer: TPMI=0. …; …; …. 15; 13; 1 layer: TPMI=13. 0; 14-15; 1 layer: reserved. …; 0; 2 layers: TPMI=0. 13; …; …. 14-15; 6; 2 layers: TPMI=6. 7-15; 2 layers: reserved. |
| --- | --- |

As shown above, in case of for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1, and for 1 layer transmission, it was agreed in Rel-16 (full power transmission) that TPMI {0,1,2,3,13} can be used, while for the second TPMI field, all of TPMIs {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13} are listed, we think it’s an incorrect capture.

And if the available TPMIs for full power transmission are retained, the number of bits for second TPMI field should also be updated. Still taking the above example, for second TPMI field, the number of TPMIs for 1 layer and 2 layers are 5 and 7, i.e. maximum value is 7, so 3 bits are enough for the second TPMI field.

So we propose:

Proposal 2: In case of PUSCH repetition and fullpowerMode1, update the TPMI index for the second TPMI field to capture correct available TPMIs for full power transmission, and adopt the TP (2nd TP in Appendix ).

TS38.212 h10 - 7.3.1.1.2 Format 0_1 < Unchanged parts are omitted > - Precoding information and number of layers – number of bits determined by the following: - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset; - 4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset; - 3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; - 2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent; - 1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used. For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field. - Second Precoding information – number of bits determined by the following: - 0 bits if SRS resource set indicator field is not present; - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset; - 3 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 3 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset; - 3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; - 1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent; - 1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; < Unchanged parts are omitted > Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. …; …; …; …. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 4; 2 layers: TPMI=0; 4; 2 layers: TPMI=0. …; …; …; …. 9; 2 layers: TPMI=5; 9; 2 layers: TPMI=5. 10; 1 layer: TPMI=13; 10; 1 layer: TPMI=13. 11; 2 layer: TPMI=6; 11; 2 layer: TPMI=6. 12; 1 layer: TPMI=4; 12-15; Reserved. …; …. 20; 1 layer: TPMI=12. 21; 1 layer: TPMI=14. 22; 1 layer: TPMI=15. 23; 2 layers: TPMI=7. …; …. 29; 2 layers: TPMI=13. 30-31; Reserved. Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. …; …; …; …. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 4; 2 layers: TPMI=0; 4; 2 layers: TPMI=0. …; …; …; …. 9; 2 layers: TPMI=5; 9; 2 layers: TPMI=5. 10; 3 layers: TPMI=0; 10; 3 layers: TPMI=0. 11; 4 layers: TPMI=0; 11; 4 layers: TPMI=0. 12; 1 layer: TPMI=13; 12; 1 layer: TPMI=13. 13; 2 layer: TPMI=6; 13; 2 layer: TPMI=6. 14; 3 layer: TPMI=1; 14; 3 layer: TPMI=1. 15; 1 layer: TPMI=4; 15; Reserved. …; …. 23; 1 layer: TPMI=12. 24; 1 layer: TPMI=14. 25; 1 layer: TPMI=15. 26; 2 layers: TPMI=7. …; …. 32; 2 layers: TPMI=13. 33; 3 layers: TPMI=2. 34; 4 layers: TPMI=1. 35; 4 layers: TPMI=2. 36-63; Reserved. Table 7.3.1.1.2-2C: Second precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …. 27; 1 layer: TPMI=27; 11; 1 layer: TPMI=11; 3; 1 layer: TPMI=3. 28-31; 1 layer: reserved; 12-15; 1 layer: reserved; 4-7; 1 layer: reserved. 0; 2 layers: TPMI=0; 0; 2 layers: TPMI=0; 0; 2 layers: TPMI=0. …; …; …; …; …; …. 21; 2 layers: TPMI=21; 13; 2 layers: TPMI=13; 5; 2 layers: TPMI=5. 22-31; 2 layers: reserved; 14-15; 2 layers: reserved; 6-7; 2 layers: reserved. 0; 3 layers: TPMI=0; 0; 3 layers: TPMI=0; 0; 3 layers: TPMI=0. …; …; …; …; 1-7; 3 layers: reserved. 6; 3 layers: TPMI=6; 2; 3 layers: TPMI=2; 0; 4 layers: TPMI=0. 7-31; 3 layers: reserved; 3-15; 3 layers: reserved; 1-7; 4 layers: reserved. 0; 4 layers: TPMI=0; 0; 4 layers: TPMI=0. …; …; …; …. 4; 4 layers: TPMI=4; 2; 4 layers: TPMI=2. 5-31; 4 layers: reserved; 3-15; 4 layers: reserved. Table 7.3.1.1.2-2D: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …. 3; 1 layer: TPMI=3. 15; 1 layer: TPMI=15; 4; 1 layer: TPMI=13. 0; 2 layers: TPMI=0; 5-7; 1 layer: reserved. …; …; 0; 2 layers: TPMI=0. 13; 2 layers: TPMI=13; …; …. 14-15; 2 layers: reserved; 6; 2 layers: TPMI=6. 7; 2 layers: reserved. Table 7.3.1.1.2-2E: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …. 3; 1 layer: TPMI=3. 15; 1 layer: TPMI=15; 4; 1 layer: TPMI=13. 0; 2 layers: TPMI=0; 5-7; 1 layer: reserved. …; …; 0; 2 layers: TPMI=0. 13; 2 layers: TPMI=13; …; …. 14-15; 2 layers: reserved; 6; 2 layers: TPMI=6. 0; 3 layers: TPMI=0; 7; 2 layers: reserved. …; …; 0; 3 layers: TPMI=0. 2; 3 layers: TPMI=2; 1; 3 layer: TPMI=1. 3-15; 3 layers: reserved; 2-7; 3 layers: reserved. 0; 4 layers: TPMI=0; 0; 4 layers: TPMI=0. …; …; 1-7; 4 layers: reserved. 2; 4 layers: TPMI=2. 3-15; 4 layers: reserved. < Unchanged parts are omitted > - 7.3.1.1.3 Format 0_2 < Unchanged parts are omitted > - Second Precoding information – number of bits determined by the following: - 0 bits if SRS resource set indicator field is not present; - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2, and codebookSubsetDCI-0-2; - 3 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2; - 3 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 and codebookSubsetDCI-0-2; < Unchanged parts are omitted >

RV sequence for PUSCH

Another issue is on RV sequence applied for PUSCH transmission towards the second TRP with the second TCI state, the design principle agreed is to apply RV pattern (0,2,3,1) separately for PUSCH repetition of different TRPs, as captured below in agreements.

Agreement For single DCI based PUSCH multi-TRP enhancements, support the following RV mapping for PUSCH repetition Type A, - DCI indicates the first RV for the first PUSCH repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the second TRP (The same method as PDSCH scheme 4) - FFS: Reuse of the same method for PUSCH repetition Type B. Agreement For single DCI based M-TRP PUSCH repetition Type B, support the following RV mapping, - DCI indicates the first RV for the first PUSCH actual repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH actual repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the first actual repetition towards second TRP (The same method as PDSCH scheme 4).

However, in current TS, the RV sequence applied to second TRP is not a cyclic shifted version of legacy RV sequence (0 2 3 1) but with an offset per each element in this sequence. One of the possible issue is that, some new RV sequences are introduced with non-self-decodable RVs transmitted consecutively and in the first two transmission, as shown in the analysis below, RV sequence (2 1 3 0) or (1 2 0 3) may be applied to PUSCH transmission to the second TRP.

According to Table 5.1.2.1-3 of TS 38.214, when rvs=1 rvid indicated by the DCI scheduling the PDSCH; rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 3; 0; 2. 2; 3; 0; 2; 1. 3; 0; 2; 1; 3. 1; 2; 1; 3; 0. when rvs=3 rvid indicated by the DCI scheduling the PDSCH; rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 2; 0. 2; 1; 2; 0; 3. 3; 2; 0; 3; 1. 1; 0; 3; 1; 2.

To accurately reflect agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, we proposal the following TP.

Proposal 3: To reflect the agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, adopting the TP (3rd TP in Appendix ).

TS 38.214 < Unchanged parts are omitted > The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state. Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present 0; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4. 2; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4. 3; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4. 1; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4. rvid indicated by the DCI scheduling the PDSCH; rvs=1, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 2; 3; 1; 0. 2; 3; 1; 0; 2. 3; 1; 0; 2; 3. 1; 0; 2; 3; 1. rvid indicated by the DCI scheduling the PDSCH; rvs=2, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 0; 2. 2; 1; 0; 2; 3. 3; 0; 2; 3; 1. 1; 2; 3; 1; 0. rvid indicated by the DCI scheduling the PDSCH; rvs=3, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 0; 2; 3. 2; 0; 2; 3; 1. 3; 2; 3; 1; 0. 1; 3; 1; 0; 2.

- Conclusion

In this contribution, we provided our views on some remaining issues on PDCCH and PUSCH based on multi-TRP transmission, and we proposed that:

Proposal 1: In case of PDCCH repetition and accumulated TPC, only one TPC command value of the two repetitions is applied for PUSCH power control, and adopt the TP (1st TP in Appendix).

Proposal 2: In case of PUSCH repetition and fullpowerMode1, update the TPMI index for the second TPMI field to capture correct available TPMIs for full power transmission, and adopt the TP (2nd TP in Appendix).

Proposal 3: To reflect the agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, adopting the TP (3rd TP in Appendix).

- References

- TS 38.213 V17.1.0

- TS 38.214 V17.1.0

- TS 38.212 V17.1.0

Appendix (TPs corresponding to proposals)

1st TP (TPC value)

TS38.213 h10 - 7.1.1 UE behaviour < Unchanged parts are omitted > - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i, if a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, as described in clause 10.1, only one TPC command value of payloads in the two PDCCH candidates is applied - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=1 if PO_UE_PUSCH,b,f,cj and αb,f,cj are provided by the second P0-PUSCH-AlphaSet in p0-AlphaSets; otherwise, l=0 - If j=1, - l is provided by the value of powerControlLoopToUse if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha in ConfiguredGrantConfig - l is provided by the value of powerControlLoopToUse2 if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 < Unchanged parts are omitted >

2nd TP (2nd TPMI field)

TS38.212 h10 - 7.3.1.1.2 Format 0_1 < Unchanged parts are omitted > - Precoding information and number of layers – number of bits determined by the following: - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset; - 4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset; - 3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; - 2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent; - 1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used. For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field. - Second Precoding information – number of bits determined by the following: - 0 bits if SRS resource set indicator field is not present; - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset; - 3 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 3 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset; - 3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; - 1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent; - 1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset; - 2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset; < Unchanged parts are omitted > Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. …; …; …; …. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 4; 2 layers: TPMI=0; 4; 2 layers: TPMI=0. …; …; …; …. 9; 2 layers: TPMI=5; 9; 2 layers: TPMI=5. 10; 1 layer: TPMI=13; 10; 1 layer: TPMI=13. 11; 2 layer: TPMI=6; 11; 2 layer: TPMI=6. 12; 1 layer: TPMI=4; 12-15; Reserved. …; …. 20; 1 layer: TPMI=12. 21; 1 layer: TPMI=14. 22; 1 layer: TPMI=15. 23; 2 layers: TPMI=7. …; …. 29; 2 layers: TPMI=13. 30-31; Reserved. Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. …; …; …; …. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 4; 2 layers: TPMI=0; 4; 2 layers: TPMI=0. …; …; …; …. 9; 2 layers: TPMI=5; 9; 2 layers: TPMI=5. 10; 3 layers: TPMI=0; 10; 3 layers: TPMI=0. 11; 4 layers: TPMI=0; 11; 4 layers: TPMI=0. 12; 1 layer: TPMI=13; 12; 1 layer: TPMI=13. 13; 2 layer: TPMI=6; 13; 2 layer: TPMI=6. 14; 3 layer: TPMI=1; 14; 3 layer: TPMI=1. 15; 1 layer: TPMI=4; 15; Reserved. …; …. 23; 1 layer: TPMI=12. 24; 1 layer: TPMI=14. 25; 1 layer: TPMI=15. 26; 2 layers: TPMI=7. …; …. 32; 2 layers: TPMI=13. 33; 3 layers: TPMI=2. 34; 4 layers: TPMI=1. 35; 4 layers: TPMI=2. 36-63; Reserved. Table 7.3.1.1.2-2C: Second precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …. 27; 1 layer: TPMI=27; 11; 1 layer: TPMI=11; 3; 1 layer: TPMI=3. 28-31; 1 layer: reserved; 12-15; 1 layer: reserved; 4-7; 1 layer: reserved. 0; 2 layers: TPMI=0; 0; 2 layers: TPMI=0; 0; 2 layers: TPMI=0. …; …; …; …; …; …. 21; 2 layers: TPMI=21; 13; 2 layers: TPMI=13; 5; 2 layers: TPMI=5. 22-31; 2 layers: reserved; 14-15; 2 layers: reserved; 6-7; 2 layers: reserved. 0; 3 layers: TPMI=0; 0; 3 layers: TPMI=0; 0; 3 layers: TPMI=0. …; …; …; …; 1-7; 3 layers: reserved. 6; 3 layers: TPMI=6; 2; 3 layers: TPMI=2; 0; 4 layers: TPMI=0. 7-31; 3 layers: reserved; 3-15; 3 layers: reserved; 1-7; 4 layers: reserved. 0; 4 layers: TPMI=0; 0; 4 layers: TPMI=0. …; …; …; …. 4; 4 layers: TPMI=4; 2; 4 layers: TPMI=2. 5-31; 4 layers: reserved; 3-15; 4 layers: reserved. Table 7.3.1.1.2-2D: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …. 3; 1 layer: TPMI=3. 15; 1 layer: TPMI=15; 4; 1 layer: TPMI=13. 0; 2 layers: TPMI=0; 5-7; 1 layer: reserved. …; …; 0; 2 layers: TPMI=0. 13; 2 layers: TPMI=13; …; …. 14-15; 2 layers: reserved; 6; 2 layers: TPMI=6. 7; 2 layers: reserved. Table 7.3.1.1.2-2E: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset = partialAndNonCoherent; Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …. 3; 1 layer: TPMI=3. 15; 1 layer: TPMI=15; 4; 1 layer: TPMI=13. 0; 2 layers: TPMI=0; 5-7; 1 layer: reserved. …; …; 0; 2 layers: TPMI=0. 13; 2 layers: TPMI=13; …; …. 14-15; 2 layers: reserved; 6; 2 layers: TPMI=6. 0; 3 layers: TPMI=0; 7; 2 layers: reserved. …; …; 0; 3 layers: TPMI=0. 2; 3 layers: TPMI=2; 1; 3 layer: TPMI=1. 3-15; 3 layers: reserved; 2-7; 3 layers: reserved. 0; 4 layers: TPMI=0; 0; 4 layers: TPMI=0. …; …; 1-7; 4 layers: reserved. 2; 4 layers: TPMI=2. 3-15; 4 layers: reserved. < Unchanged parts are omitted > - 7.3.1.1.3 Format 0_2 < Unchanged parts are omitted > - Second Precoding information – number of bits determined by the following: - 0 bits if SRS resource set indicator field is not present; - 0 bits if the higher layer parameter txConfig = nonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook; - 3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2, and codebookSubsetDCI-0-2; - 3 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2; - 3 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 and codebookSubsetDCI-0-2; < Unchanged parts are omitted >

3rd TP (RV sequence)

TS 38.214 h10 < Unchanged parts are omitted > The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state. Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present 0; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4. 2; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4. 3; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4. 1; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4. rvid indicated by the DCI scheduling the PDSCH; rvs=1, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 2; 3; 1; 0. 2; 3; 1; 0; 2. 3; 1; 0; 2; 3. 1; 0; 2; 3; 1. rvid indicated by the DCI scheduling the PDSCH; rvs=2, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 0; 2. 2; 1; 0; 2; 3. 3; 0; 2; 3; 1. 1; 2; 3; 1; 0. rvid indicated by the DCI scheduling the PDSCH; rvs=3, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 0; 2; 3. 2; 0; 2; 3; 1. 3; 2; 3; 1; 0. 1; 3; 1; 0; 2.