3GPP TSG RAN WG1 #109-e										 		  R1-2203675

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.3

Source:	NEC

Title:               Discussion remaining issues on SRS enhancement

Document for:	Discussion and Decision

- Introduction

Rel-17 SRS enhancements have been finished, while there are some remaining issues. In this contribution, we provided our views on some remaining issues.

- Discussion

In RAN1#108e meeting, the maximum number of CS for RPFS was discussed, which will impact on the SRS capacity. In current spec, the maximum number of CS is determined based on value of KTC, the reason is that in Rel-15/16 SRS framework, the number of RBs for SRS is a multiple of 4, so maximum number of CS can be represented as a function of only KTC. But to be accurate, the final factor to determine the maximum number of CS is actually the length of SRS sequence.

Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC.

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

In Rel-15/16, based on number of RBs for SRS a multiple of 4, the SRS sequence length and maximum number of cyclic shift values are actually as follows:

| KTC | nSRScs,max | SRS sequence length Msc,bSRS=mSRS,bNscRBKTC |
| --- | --- | --- |
| 2 | 8 | Actually a multiple of 24 |
| 4 | 12 | Actually a multiple of 12 |
| 8 | 6 | Actually a multiple of 6 |

Based on the conclusion, there is no further restriction on the number of RBs (1PFmSRS, BSRS) for RPFS, as long as there is no new sequence length. While this will cause some values of number of RBs for SRS, for example, when the number of RBs configured for SRS is mSRS, BSRS = 20, and if PF=4, then the actual number of RBs for the SRS is actually 20/4=5. If comb value KTC is configured to be 2, the final sequence length for SRS is Msc,bSRS=mSRS,bNscRBKTCPF =30, this sequence length exists in legacy TS, so this configuration satisfies requirement. While there is an issue, that is if based on current spec, the maximum number of CS for this configuration is 8 (based on the function of KTC=2), but the actually maximum number of CS for this SRS configuration can not support 8, as the length of SRS sequence is 30, only a multiple of 6. So actually, the maximum number of CS supported for this SRS configuartion is only 6, i.e. nSRScs,max should be 6. While based on current function, nSRScs,max=8, only if the cyclic shift value αi=2πnSRScs,inSRScs,max is 0 or 4, i.e. only if αi=2π08 (same as αi=2π06) and αi=2π48 (same as αi=2π36), the SRS ports can be orthogonal. In other words, up to 2 orthogonal ports can be supported based on different CS. Even based on network configuration to guarantee the orthogonality is not a good way, as discussed in last meeting, capacity of SRS is significantly reduced and 4-port SRS can only be supported with restriction.

And the key issue exists when KTC=2 (with current TS, nSRScs,max=8), while with PF configured, the sequence length is not always a multiple of 8, then only 2 orthogonal ports can be supported based on CS, which significantly reduce the usability for RPFS, and this is not a corner case, especially in case of frequency hopping, which companies think the typical use case for RPFS. As shown in Table 1, we listed the sequence length for different SRS bandwidth configuration, in case of KTC = 2 and PF=2. Based on current maximum number of CS determination scheme, nSRScs,max=8, and sequences can be orthogonal if the sequence length is a multiple of 8, while we can see all yellow marked sequence length is not a multiple of 8, i.e. based on nSRScs,max=8, orthogonal ports will be significantly reduced, and 4-port SRS can not be supported, especially for frequency hopping case, there are quite limited case available in last and second last column.

Table 1. SRS sequence length (mSRS,bNscRBKTCPF ) in case of KTC = 2 and PF=2

|  | mSRS,0PF | Sequence length | mSRS,1PF | Sequence length | mSRS,2PF | Sequence length | mSRS,3PF | Sequence length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 1 | 8/2=4 | 24 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 2 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 3 | 16/2=8 | 48 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 4 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 | 4/2=2 | 12 |
| 5 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 6 | 24/2=12 | 72 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 7 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 |
| 8 | 28/2=14 | 84 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 9 | 32/2=16 | 96 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 10 | 36/2=18 | 108 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 |
| 11 | 40/2=20 | 120 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 |
| 12 | 48/2=24 | 144 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 13 | 48/2=24 | 144 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 14 | 52/2=26 | 156 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 15 | 56/2=28 | 168 | 28/2=14 | 84 | 4/2=2 | 12 | 4/2=2 | 12 |
| 16 | 60/2=30 | 180 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 |
| 17 | 64/2=32 | 192 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 18 | 72/2=36 | 216 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 19 | 72/2=36 | 216 | 36/2=18 | 108 | 12/2=6 | 36 | 4/2=2 | 12 |
| 20 | 76/2=38 | 228 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 21 | 80/2=40 | 240 | 40/2=20 | 120 | 20/2=10 | 60 | 4/2=2 | 12 |
| 22 | 88/2=44 | 264 | 44/2=22 | 132 | 4/2=2 | 12 | 4/2=2 | 12 |
| 23 | 96/2=48 | 288 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 24 | 96/2=48 | 288 | 48/2=24 | 144 | 24/2=12 | 72 | 4/2=2 | 12 |
| 25 | 104/2=52 | 312 | 52/2=26 | 156 | 4/2=2 | 12 | 4/2=2 | 12 |
| 26 | 112/2=56 | 336 | 56/2=28 | 168 | 28/2=14 | 84 | 4/2=2 | 12 |
| 27 | 120/2=60 | 360 | 60/2=30 | 180 | 20/2=10 | 60 | 4/2=2 | 12 |
| 28 | 120/2=60 | 360 | 40/2=20 | 120 | 8/2=4 | 24 | 4/2=2 | 12 |
| 29 | 120/2=60 | 360 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 30 | 128/2=64 | 384 | 64/2=32 | 192 | 32/2=16 | 96 | 4/2=2 | 12 |
| 31 | 128/2=64 | 384 | 64/2=32 | 192 | 16/2=8 | 48 | 4/2=2 | 12 |
| 32 | 128/2=64 | 384 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 33 | 132/2=66 | 396 | 44/2=22 | 132 | 4/2=2 | 12 | 4/2=2 | 12 |
| 34 | 136/2=68 | 408 | 68/2=34 | 204 | 4/2=2 | 12 | 4/2=2 | 12 |
| 35 | 144/2=72 | 432 | 72/2=36 | 216 | 36/2=18 | 108 | 4/2=2 | 12 |
| 36 | 144/2=72 | 432 | 48/2=24 | 144 | 24/2=12 | 72 | 12/2=6 | 36 |
| 37 | 144/2=72 | 432 | 48/2=24 | 144 | 16/2=8 | 48 | 4/2=2 | 12 |
| 38 | 144/2=72 | 432 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 39 | 152/2=76 | 456 | 76/2=38 | 228 | 4/2=2 | 12 | 4/2=2 | 12 |
| 40 | 160/2=80 | 480 | 80/2=40 | 240 | 40/2=20 | 120 | 4/2=2 | 12 |
| 41 | 160/2=80 | 480 | 80/2=40 | 240 | 20/2=10 | 60 | 4/2=2 | 12 |
| 42 | 160/2=80 | 480 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 43 | 168/2=84 | 504 | 84/2=42 | 252 | 28/2=14 | 84 | 4/2=2 | 12 |
| 44 | 176/2=88 | 528 | 88/2=44 | 264 | 44/2=22 | 132 | 4/2=2 | 12 |
| 45 | 184/2=92 | 552 | 92/2=46 | 276 | 4/2=2 | 12 | 4/2=2 | 12 |
| 46 | 192/2=96 | 576 | 96/2=48 | 288 | 48/2=24 | 144 | 4/2=2 | 12 |
| 47 | 192/2=96 | 576 | 96/2=48 | 288 | 24/2=12 | 72 | 4/2=2 | 12 |
| 48 | 192/2=96 | 576 | 64/2=32 | 192 | 16/2=8 | 48 | 4/2=2 | 12 |
| 49 | 192/2=96 | 576 | 24/2=12 | 72 | 8/2=4 | 24 | 4/2=2 | 12 |
| 50 | 208/2=104 | 624 | 104/2=52 | 312 | 52/2=26 | 156 | 4/2=2 | 12 |
| 51 | 216/2=108 | 548 | 108/2=54 | 324 | 36/2=18 | 108 | 4/2=2 | 12 |
| 52 | 224/2=112 | 672 | 112/2=56 | 336 | 56/2=28 | 168 | 4/2=2 | 12 |
| 53 | 240/2=120 | 720 | 120/2=60 | 360 | 60/2=30 | 180 | 4/2=2 | 12 |
| 54 | 240/2=120 | 720 | 80/2=40 | 240 | 20/2=10 | 60 | 4/2=2 | 12 |
| 55 | 240/2=120 | 720 | 48/2=24 | 144 | 16/2=8 | 48 | 8/2=4 | 24 |
| 56 | 240/2=120 | 720 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 57 | 256/2=128 | 768 | 128/2=64 | 384 | 64/2=32 | 192 | 4/2=2 | 12 |
| 58 | 256/2=128 | 768 | 128/2=64 | 384 | 32/2=16 | 96 | 4/2=2 | 12 |
| 59 | 256/2=128 | 768 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 60 | 264/2=132 | 792 | 132/2=66 | 396 | 44/2=22 | 132 | 4/2=2 | 12 |
| 61 | 272/2=136 | 816 | 136/2=68 | 408 | 68/2=34 | 204 | 4/2=2 | 12 |
| 62 | 272/2=136 | 816 | 68/2=34 | 204 | 4/2=2 | 12 | 4/2=2 | 12 |
| 63 | 272/2=136 | 816 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |

In case of PF is configured, the SRS sequence mapping to REs is actually changed, still taking the discussed example, (mSRS, BSRS = 20, PF=4, KTC = 2), the sequence length is 30, and the sequence is mapped to REs with comb 2, while for legacy UE, if SRS sequence length is 30, the configuration is actually mSRS, BSRS = 20, and  KTC = 8, i.e. the 30-length sequence is mapped to REs with comb 8, which is totally different mapping structure. So we are wondering with the new SRS sequence mapping structure, why sticking to legacy nSRScs,max rules and reducing the usability? The highlighted configurations (sequence length not a multiple of 8) are actually with values of subband not a multiple of 4 (different from legacy), in other words, even keeping nSRScs,max to be 8, the RPFS UE can not be orthogonal with legacy UE based on CS, as same sequence can not be fully overlapped between RPFS UE and legacy UE, and actually the non-highlighted configurations can be achieved with legacy configuration, that’s why we have concern on the usability of RPFS. If based on implementation to use non impacted configurations (i.e. the configurations not highlighted in above table), our concern is that these can be achieved with legacy configurations already, which also leading to RPFS useless.

The key issue exists when K_TC = 2 (i.e. nSRScs,max=8, needing sequence length a multiple of 8, not like 6 is a divisor of 12), so we think the determination of nSRScs,max should be updated, otherwise, the SRS enhancement is meaningless.

Proposal: When P_F = 2 or 4 and K_TC = 2, if Msc,bSRS/8 is not an integer, nSRScs,max=12, otherwise, nSRScs,max=8.

- Conclusion

In this contribution, we provided our views on SRS enhancement, and we proposed that:

Proposal: When P_F = 2 or 4 and K_TC = 2, if Msc,bSRS/8 is not an integer, nSRScs,max=12, otherwise, nSRScs,max=8.

- References

- Chairman’s notes, RAN1#108-e, February 21st – March 3rd, 2022.