3GPP TSG RAN WG1 #109                                              R1- 2203677

e-Meeting, May 9th – 20th, 2022

Source:	NEC

Title:	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs

Agenda Item:	8.12.1

Document for: 	Discussion and Decision

- Introduction

The work items for NR support of multicast and broadcast service were approved in RAN #86. After several RAN1 e-meetings involved the discussion about reliability improvement for UEs in RRC_CONNECTED state, the basic functions for it can be achieved, but some remaining issues are left which will be solved in Rel-17 maintenance stage.

According to the agreements and conclusions achieved in RAN1 #108 e-meeting and previous e-meetings, the following two issues will be discussed in this contribution:

- Overlapping between NACK-only and SR with same priority.

- More than one NACK-only based feedbacks.

- Overlapping between NACK-only and SR with same priority

Regarding overlapping between PUCCH for NACK-only and SR, the following agreement was agreed in the RAN1 #106b e-meeting about the multiplexing between NACK-only based feedback for multicast and PUCCH transmissions for HARQ-ACK feedback/CSI or PUSCH transmission for unicast with FFS for the case of SR[1].

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS the case of PUCCH for SR.

The multiplexing between NACK-only and SR has been discussed for a long time. During RAN1 #107b e-meeting, the moderator proposed that take Alt1 (same as Proposal 3.2.1.4-1 above) as baseline and solicited opinions from other companies. Due to it is different to converge after one round of discussion, this issue was postponed [2] to the subsequent meetings. After several rounds of discussion in the last meeting, there are two proposals listed by the moderator for down-selection.

#### Proposal 3.2.1.4-1 (GTW) When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16. - Alt1-1: PUCCH resources configured for unicast will be used. - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16. - the PUCCH resource is determined by the PRI in the last DCI for multicast; - Alt1-2: the PUCCH resource is determined by RRC configuration; - Alt1-3: the PUCCH resource for NACK-only feedback transmission is SR resource. - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported: - Alt2-1: SR is dropped - Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16. - Alt2-3: NACK-only is dropped #### Proposal 3.2.1.4-2 (if proposal 3.2.1.4-1 is not agreeable) Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17, - It is up to UE implementation when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

Obviously, the main issue of multiplexing between NACK-only and SR from above proposals is how to select the PUCCH resource used for transmitting SR and NACK-only.

For the case of up to two HARQ-ACK bits transformed from NACK-only based feedback, there are three alternatives given in Proposal 3.2.1.4-1. Alt 1-1 select the PUCCH resource by legacy procedure of Rel-15/16, which is in line with RAN1 #106 e-meeting agreements that determining the PUCCH resources for transmitting the multiplexing of ACK/NACK-based HARQ-ACK feedback for multicast and unicast HARQ-ACK is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast. In detail, UE selects the PUCCH resource within PUCCH-Config/PUCCH-ConfigurationList configured for unicast by the PRI indicated in the UE specific DCI.

One of the benefits of Alt 1-1 is it has little impact on standardization work. For Alt 1-2 and Alt 1-3, additional specification works are needed to determine the PUCCH resource, e.g., configure the RRC parameter of PUCCH resource. In detail, for Alt 1-2, how to define the PUCCH resource may be controversial, e.g., a new PUCCH resource set or not. For Alt 1-3, the group-common resource of NACK-only is not suitable for multiplexing SR which is UE a specific UL transmission. Maybe some improvement can be introduced but it is not the best choice in maintaining phase.

The second bullet is the case of more than two TBs with NACK-only based feedback. Both Alt 2-1(drop SR) and Alt 2-3(drop NACK-only) will cause the dis-alignment between UE and gNB. Alt 2-2 is preferred and align with the case of up to two HARQ-ACK bits for NACK-only feedback (the first bullet). Moreover, Alt 2-2 has little standardization effect per previous agreements.

The moderator also mentions that proposal 3.2.1.4-2 will be agreed if proposal 3.2.1.4-1 is not agreeable. We also think proposal 3.2.1.4-2 will undermine the usefulness of NACK-only based feedback. We think the UE can achieve the multiplexing between multicast and unicast just like the multiplexing between unicasts.

Proposal 1:

- When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority,

- For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

  - PUCCH resources configured for unicast will be used.

- For the case of more than two TBs of NACK-only, NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

- More than one NACK-only based feedbacks

Regarding more than one NACK-only based feedbacks that is available for transmission in the same PUCCH slot, after several times e-meeting discussions, this controversial issue was agreed as follows[2]:

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between numbers of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

Since it is difficult to down select from Alt 1 and Alt 4, it is agreed to define RRC configuration to configure both of two alternatives by RRC configuration.

For Alt 4, the mapping rules is the most important issue to be solved. It has been agreed that the NACK-only can be transmitted by PUCCH format 0 and PUCCH format 1 per previous agreements.

- For PUCCH format 0, According to TS 38.213[3], the information is carried by different 𝑚cs, and multiple UEs can be multiplexed in orthogonal PUCCH resource by configuring with different UE-specific 𝑚0. For multicast service with NACK-only based feedback, due to there is no need to ensure the orthogonality among a group of UEs scrambled by the same G-RNTI, the orthogonal PUCCH resources can represent the different combinations of TBs with NACK-only feedback. For the selection of PUCCH resource, the PUCCH resource ID can be associated with the order of TB reception. For example, as shown in the below Fig.1, if UE1 and UE2 are configured with NACK-only based feedback, for the three TBs receptions, the different combinations of TBs can be represented by different initialCyclicShift.

Fig. 1 Receive the same TBs in a MBS group by different UEs

In detail, for any two of UEs in a same MBS group, the PUCCH resource for the different TB combination with NACK-only can refer to Table 1. The UE1 and UE2 will transmit different sequences by PUCCH format 0, and the gNB can decode the received sequence to determine which TBs will be retransmitted.

Table 1 Relationship between different TB combination and shift cyclic

| NACK-only for | m0+mcs | NACK-only for | m0+mcs |
| --- | --- | --- | --- |
| TB1 | 0 | TB1 and TB3 | 6 |
| TB2 | 1 | TB2 and TB3 | 8 |
| TB3 | 2 | TB1, TB2 and TB3 | 10 |
| TB1 and TB2 | 4 |  |  |

- For PUCCH format 1, according to TS 38.211[4], the orthogonal resource can be defined by RRC parameter timeDomainOCC. For a group of UEs scrambled by a same G-RNTI, if any one of the UEs will use the PUCCH format 1 to transmit the NACK-only base feedback for the combination of multiple TBs, the different orthogonal PUCCH resources configured by RRC parameter initialCyclicShift and timeDomainOCC can represent the different combinations of TBs with NACK-only feedback. For example, as shown in below Fig.2, if UE1 and UE2 are configured with NACK-only based feedback, for the six TBs receptions, the different combinations of TBs can be represented by different initialCyclicShift and timeDomainOCC.

Fig. 2 Receive the same TBs in a MBS group by different UEs

In detail, for any two of UEs in a same MBS group, the PUCCH resource for the different TB combination with NACK-only can refer to Table 2. The UE1 and UE2 will use different PUCCH resource using the combinations of sequences and OCC value by PUCCH format 1, and the gNB can decode the received PUCCH resource to determine which TBs will be retransmitted.

Table 2 Relationship between different TB combination and shift cyclic

| NACK-only for | m0+mcs | timeDomainOCC |
| --- | --- | --- |
| TB1 | 0 | 0 |
| … | … | … |
| TB6 | 0 | 5 |
| TB1 and TB2 | 0 | 6 |
| … | … | … |
| TB5 and TB6 | 1 | 6 |
| … | … | … |
| All TBs | 11 | 6 |

Proposal 2:

- When no more than 4 NACK-only feedbacks are available for transmission in the same PUCCH slot, the UE selects orthogonal PUCCH resources according to combinations of TBs with NACK-only feedback. The orthogonal PUCCH resources can be defined by different shift cyclic and timeDomainOCC (for PUCCH format 1).

- Conclusion

In this contribution, we discussed the remaining issues for reliability improvements for RRC_CONNECTED UEs. Observations and proposals are summarized as following:

Proposal 1:

- When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority,

- For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

  - PUCCH resources configured for unicast will be used.

- For the case of more than two TBs of NACK-only, NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

Proposal 2:

- When no more than 4 NACK-only feedbacks are available for transmission in the same PUCCH slot, the UE selects orthogonal PUCCH resources according to combinations of TBs with NACK-only feedback. The orthogonal PUCCH resources can be defined by different shift cyclic and timeDomainOCC (for PUCCH format 1).

References

- R1-2202883, FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs, Moderator, RAN1 #108.

- 3GPP RAN1 #108 e-meeting, chairman’s notes.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”.

- 3GPP TS 38.211, “NR; Physical channels and modulation”.