3GPP TSG RAN WG1 #109-e			                                                 R1-2203678

e-Meeting, May 9th – 20th, 2022

Source:	NEC

Title:	Remaining issues of PDSCH/PUSCH enhancements for 52.6 to 71GHz

Agenda Item:	8.2.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss one remaining issue of PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz.

- Discussion

## 2.1 Clarification on priority determination for multiple PXSCHs scheduled by a single DCI

In RAN1 106-e meeting, whether and how to apply URLLC related fields for multiple PUSCHs/PDSCHs scheduled by a single DCI was discussed. And following agreement was achieved [1].

Agreement: - For a DCI that can schedule multiple PUSCHs, - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs. - For a DCI that can schedule multiple PDSCHs, - Priority indicator field is applied to all of scheduled PDSCHs.

However, it seems that the priority determination for multiple PUSCHs/PDSCHs scheduled by a single DCI with a priority indicator field has not been captured in current specification below [2].

9 UE procedure for reporting control information [….] If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule a PUSCH transmission of any priority, or a PDSCH reception and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. [….]

Therefore, we give the following TP to capture priority determination for multiple PUSCHs/PDSCHs scheduled by a single DCI with a priority indicator field.

Proposal 1:

- Adopt following text change for clause 9 in TS 38.213.

9 UE procedure for reporting control information [….] If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule PUSCH transmission(s) of any priority, or PDSCH reception(s) and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. [….]

- Conclusion

In this contribution, we discussed one remaining issue for PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz. Proposals are summarized as following:

Proposal 1:

- Adopt following text change for clause 9 in TS 38.213.

9 UE procedure for reporting control information [….] If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule PUSCH transmission(s) of any priority, or PDSCH reception(s) and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. [….]

References

- 3GPP RAN1 NR #106-e meeting, chairman’s notes.

- 3GPP TS 38.213 V17.1.0, NR Physical layer procedures for control