3GPP TSG RAN WG1 #109-e										 	R1-2203682

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.1.2

Source:	NEC

Title:               	Discussion on two TAs for multi-DCI

Document for:	Discussion and Decision

- Introduction

Rel-18 MIMO evolution for downlink and uplink [1] will study and specify two TAs for multi-DCI for MTRP operation. In this paper, we discuss the possible impacts of two TAs for multi-DCI.

Objective of WI [1] - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Discussion

  - Discussion on signaling of two TAs

The need of supporting an additional TA is based on the scenario as shown below, that TTA,1 measured based on the propagation between TRP1 and UE is not the same with the TTA,2 measured based on the propagation between TRP2 and UE. It is known that this assumption is only reasonable when the difference between TTA,1 and TTA,2 is larger than a length that UE can handle.

Observation 1: Two TAs enhancements are needed only when |TTA,2 - TTA,1| is larger than a threshold.

Current specification supports one TA value per TAG, which can be calculated at UE based on signalling like TA command and RRC configuration or default value of TimingAdvanceOffset. To be more specific, the formula TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc is used. Then if multiple TAs are going to be supported in one TAG, it requires some updates on the signalling to provide additional information about the extra TA value, for example, NTA,1, NTA,2 or NTA,offset, 1, NTA, offset, 2.

Observation 2: Additional signaling is needed for UE to calculate multiple TAs per TAG.

Figure 1 illustration of possible two TAs

Furthermore, as defined in WID, that in Rel-18 multiple TAs are not addressed for simultaneous multi-panel transmission, which means that applied TA switching is needed if multiple TAs are maintained at UE. To avoid frequent TA command to update applied TA value, associations between TA values and TRPs are needed and TRP switching can implicitly indicate the applied TA switching. Considering that unified TCI state will be enhanced for MTRP case, TCI state update can also provide the same indication for TA update.

Proposal 1: Support associations between TAs and TRPs, and TRP switching indicates applied TA switching.

  - Discussion on application of two TAs

When two TAs are introduced for UL transmissions, some procedures may need to consider the impact of TA switching. For example, if multiple TAs are applied in an alternating way for PUSCH repetition as shown below, the possible overlapping may need to be solved. To be more specific, if the transmission with a larger applied TA value is in the later one of consecutive slots, the start of it may be overlapped in time with the end of the UL transmissions with a smaller applied TA. Since UE may not be able to transmit simultaneously to two TRPs (as decided in WID, even UE is capable, we are not going to discuss it), it may cause dropping of the second transmission. To avoid this overlapping, a time gap can be reserved and the length of the gap should be larger than |TTA,2 - TTA,1|.

Figure 2 illustration of possible overlapping

Observation 3: There might be overlapping issue if multiple TAs are applied in consecutive slots.

Proposal 2: Support a time gap between consecutive slots applying different TAs.

TA value switching may also be needed in BFR procedure. In MTRP case, q(0,0) and q(0,1) are two BFD-RS sets for two TRPs respectively, q(1,0) and q(1,1) are two CBD-RS sets for two TRPs respectively, q(0,0) is associated with q(1,0), q(0,1) is associated with q(1,1). According to discussions in the first section on the associations between TAs and TRPs, the associations can be extended to TAs, BFD-RS sets, CBD-RS sets, as in the table below.

Table 1 associations between TAs, TRPs, BFD-RS sets, CBD-RS sets

| TTA,1 | TRP1 | BFD-RS set q(0,0) | CBD-RS set q(1,0) |
| --- | --- | --- | --- |
| TTA,2 | TRP2 | BFD-RS set q(0,1) | CBD-RS set q(1,1) |

If UE assesses that TRP 1 link is worse than BFD threshold, it may temperately stop the application of TTA,1. Meanwhile, if UE finds the available new beam qnew , TA value associated with qnew can be assumed for transmission of BFRQ. After gNB’s response to BFRQ, TA value associated with qnew can be assumed for the follow-up UL transmissions including PUCCH and PUSCH and possibly SRS.

Figure 3 illustration of BFR and TA switching

Proposal 3: For MTRP BFR, support the application of TA value associated with qnew for UL transmissions after BFD.

- Conclusion

In this contribution, we provided our views on two TAs for multi-DCI, and we have the following observations and proposals:

Observation 1: Two TAs enhancements are needed only when |TTA,2 - TTA,1| is larger than a threshold.

Observation 2: Additional signaling is needed for UE to calculate multiple TAs per TAG.

Observation 3: There might be overlapping issue if multiple TAs are applied in consecutive slots.

Proposal 1: Support associations between TAs and TRPs, and TRP switching indicates applied TA switching.

Proposal 2: Support a time gap between consecutive slots applying different TAs.

Proposal 3: For MTRP BFR, support the application of TA value associated with qnew for UL transmissions after BFD.

- References

- RP-213598, WID: MIMO Evolution for Downlink and Uplink