3GPP TSG RAN WG1 #109-e										 	R1-2203686

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.4.1

Source:	NEC

Title:               	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

- Introduction

Rel-18 MIMO evolution for downlink and uplink [1] will study and specify methods to support simultaneous multi-panel UL transmission. In this paper, we discuss UE panel management and UL precoding indication for multi-panel transmission.

Objective of WI [1] - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Discussion

  - Discussion on the report of panel information

In Rel-17, UE could provide, via UE capability reporting, its panel ‘type’ information to gNB by reporting capability value set which contains the maximum supported number of SRS antenna ports. For example, UE can report that it is equipped with one 2-port panel and one 4-port panel. Meanwhile, UE could report capability value set index along with beam report to indicate its panel ‘status’. For example, UE can report 4-port panel index along with CRI-RSRP report to let gNB be aware that 4-port panel is currently in use or to be used and is associated with reported CRI. Since no further details have been specified in Rel-17, panel management including panel switch might be heavily based on mutual understanding and implementations of gNB and UE.

To support simultaneous multi-panel transmission, the corresponding panel information should be aligned between gNB and UE as well. Starting from what has been specified, two possible options can be considered as follows.

- Option 1: simultaneous multi-panel can be treated as one special panel ‘type’, e.g., as one of capability value set contains the maximum supported number of SRS ports when multiple panels are involved in transmission at the same time. To be more specific, for UE with two 2-port panels, it can report capability value set 1: 2-port, capability value set 2: 4-port.

- Option 2: simultaneous multi-panel can be treated as a combination of multiple independent panels. To be more specific, for UE with two 2-port panels, it can report capacity value set 1: 2-port and a multiplication factor 2 to indicate that two panels can be used for transmission simultaneously.

Proposal 1: Support using UE capability value set to report panel information corresponds to simultaneous multi-panel.

  - Discussion on SRS and PUSCH configurations

To support codebook and non-codebook based PUSCH transmission, SRS resource sets are configured for precoding matrix calculation and further indications are provided in DCI for grant-based PUSCH about the selected SRS resource sets, SRS resources and so on. With simultaneous multi-panel, both CB-based and NCB-based PUSCH transmissions should be supported, and the transmission schemes including joint transmissions like (e.g., coherent transmissions, non-coherent transmissions), SDM repetition can be studied for a higher capacity and a better reliability, respectively.

Proposal 2: Support both codebook-based and noncodebook-based simultaneous multi-panel PUSCH transmission. Study and support both joint PUSCH transmission and SDM PUSCH repetition across simultaneous multi-panel.

In previous releases, there are several rules on the configurations and indications of SRS resources/resource sets/ports and PUSCH layers/ports. A brief summary is provided in the next table.

Table 1 Supported CB/NCB based PUSCH and corresponding SRS configuration/indication

| Configuration/indication | Codebook | Non codebook |
| --- | --- | --- |
| SRS resource set configuration | one or two SRS resource sets with usage set to 'codebook'; same number of SRS resources in the two SRS resource sets; | one or two SRS resource sets with usage set to 'nonCodebook'; same number of SRS resources in the two SRS resource sets; associated CSI-RS per set, if applicable; |
| SRS resource configuration | Same nrofSRS-Ports for all SRS resources in a set, except fullpowerMode2 | one SRS port for each SRS resource |
| SRS resource set indicator in DCI | 2 bits if two SRS resource sets indicate s-TRP mode: (TRP1, TRP2) and m-TRP repetition mode: (TRP1-TRP2 order, TRP2-TRP1 order) |  |
| SRS resource indicator (SRI) in DCI | one or two SRI(s); nrofSRS-Ports for the two indicated SRS resources to be the same; 1st SRI: ; 2nd SRI: log2(NSRS); | one or two SRI(s); 1st SRI: ; 2nd SRI: log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk) |
| Precoding information and number of layers (TPMI) indication in DCI | one or two TPMI(s) the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports | - |
| codebookSubset | fullyAndPartialAndNonCoherent partialAndNonCoherent nonCoherent | - |
| FullPowerTransmission | fullpower fullpowerMode1 fullpowerMode2 | - |
| PUSCH layers | same number of layers for first and second TPMI; Each TPMI, based on indicated codepoint of SRS Resource Set indicator, is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set; | same number of layers for first and second SRS resource indicators; one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) and their corresponding PUSCH layers {0 … ν-1}; |
| PUSCH port | same antenna port(s) as the SRS port(s) in the SRS resource indicated; | same antenna port(s) as the SRS port(s) in the SRS resource indicated, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as ; |

It can be observed that some of those configurations, indications and restrictions might be not suitable to support simultaneous multi-panel transmissions, for example, we may need more SRS resource sets to support multi-panel transmissions, and some requirements like same number of layers indicated with 1st and 2nd SRS resource indicator/TPMI might be too restricted considering different UE panel capability and transmission schemes different from TDM repetition.

In our views, before going into more details, one important question to answer is how many SRS resource sets are needed to support simultaneous multi-panel transmissions. At least the following two options can be considered.

- Option 1: a dedicated SRS resource set is configured for simultaneous multi-panel.

- Option 2: two SRS resource sets are configured, corresponds to two UE panels, when SRS resources from different SRS resource sets are transmitted simultaneously, it can be used for precoder search for simultaneous multi-panel.

The SRS configurations discussed above might impact the design of DCI, including the bitwidth of SRS resource set indicator, the required number and bitwidth of SRIs/TPMIs in DCI, etc.

Proposal 3: Study the required SRS configurations and specify impacts on DCI indication to support simultaneous multi-panel.

For CB-based simultaneous multi-panel transmission, coherent type and full power mode may need to be also studied to support various UE implementations, since both have impacts on codebook subset determination.

For coherent type, there might be at least two different assumptions as follows.

- Assumption 1: antennas across panels cannot be coherent and antennas within a panel can be full coherent.

- Assumption 2: antennas across panels can be full coherent.

For full power discussion, there might be at least two different assumptions as follows.

- Assumption 1: per UE total power constraint. Same total power is assumed for both single-panel and simultaneous multi-panel transmission. In this case, transmit power scaling may be needed at both panel-level and port-level.

- Assumption 2: per panel power constraint. More power consumption is expected for simultaneous multi-panel transmission. In this case, transmit power scaling may be done within a panel.

To simplify discussions, down-selection may be needed to keep the most reasonable assumptions.

Proposal 4: Study the impacts of the coherent type and full power mode and specify the required improvements to support simultaneous multi-panel CB-based PUSCH.

- Conclusion

In this contribution, we provided our views on UL precoding indication for multi-panel transmission, and we have the following proposals:

Proposal 1: Support using UE capability value set to report panel information corresponds to simultaneous multi-panel.

Proposal 2: Support both codebook-based and noncodebook-based simultaneous multi-panel PUSCH transmission. Study and support both joint PUSCH transmission and SDM PUSCH repetition across simultaneous multi-panel.

Proposal 3: Study the required SRS configurations and specify impacts on DCI indication to support simultaneous multi-panel.

Proposal 4: Study the impacts of the coherent type and full power mode and specify the required improvements to support simultaneous multi-panel CB-based PUSCH.

- References

- RP-213598, WID: MIMO Evolution for Downlink and Uplink