3GPP TSG RAN WG1 #109-e										 	      R1-2203687

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.4.2

Source:	NEC

Title:               	Discussion on SRI/TPMI enhancement

Document for:	Discussion and Decision

- Introduction

Rel-18 MIMO evolution for downlink and uplink [1] will specify enhancements to support 8 Tx UL operation. In this contribution, we provided our views on TPMI enhancement.

Objective of WI [1] - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Discussion

Firstly, regarding the description “to support 4 and more layers” in WID, we’d like to clarify how to support 1-3 layers. One way is to support 1-3 layers based on enhancements with 8Tx, while this seems not the main target of the WID, which is to increase the data rate, and for transmission supporting 4 or more layers, the propagation environment is typically stable and the channel is suitable for higher rank transmission. And supporting 1-3 layers needs much larger overhead, especially for TPMI and DMRS indication. Another way is to support 1-3 layers based on 4-port SRS (i.e. legacy scheme), while this needs signalling enhancement for switching between 4 Tx and 8 Tx.

Proposal 1: Clarifying whether 1-3 layer transmission is based on enhancements with 8Tx or not.

Codebook based uplink transmission is typically applied in commercial, and similar as Rel-15 design, full, partial and non coherent codebooks should be supported. Considering different structures of antenna deployment for variant devices, similar as current spec, from UE perspective, reporting capability of full, partial and non coherent is sufficient. Codebook subsets can be configured from network device.

Proposal 2: From UE perspective, reporting capability of full, partial and non coherent is sufficient.

For full coherent codebook, similar as Rel-15 discussion, downlink codebook can be reused for uplink transmission with 8Tx, and there are several downlink codebooks, Type I single-panel, Type I multi-panel, Type II, e-Type II and Fe-Type II, and only Type I single-panel codebook supports >4 layers, so we think this should be reused for uplink codebook enhancement.

Proposal 3: Type I single-panel codebook is reused for uplink full coherent codebook enhancement.

Regarding partial coherent and non coherent codebook, antenna selection codebook should be supported. As the increased number of antenna ports, free selection of antenna ports will cause large overhead and unnecessary, for example, antenna port grouping can be introduced, and antenna ports within a group can be indicated for partial and non coherent codebooks.

Proposal 4: Overhead reduction for partial and non coherent codebook should be studied.

- Conclusion

In this contribution, we provided our views on increased number of orthogonal DMRS ports, and we have the following observations and proposals:

Proposal 1: Clarifying whether 1-3 layer transmission is based on enhancements with 8Tx or not.

Proposal 2: From UE perspective, reporting capability of full, partial and non coherent is sufficient.

Proposal 3: Type I single-panel codebook is reused for uplink full coherent codebook enhancement.

Proposal 4: Overhead reduction for partial and non coherent codebook should be studied.

- References

- RP-213598, WID: MIMO Evolution for Downlink and Uplink