3GPP TSG RAN WG # 109-e										   	   R1-2203693

E-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1.1

Source: 	NEC

Title:	Channel Access of Sidelink on Unlicensed Spectrum

Document for:	Discussion/Decision

- Introduction

According to the WID of Rel-18 sidelink evolution in [1], sidelink on unlicensed spectrum should be discussed and the channel access of SL-U should be considered based on NR-U scheme. In this contribution, analyses of preliminary schemes for channel access of SL-U are presented.

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

- Discussion

  - General Issues

As described in the WI, the channel access mechanisms from NR-U shall be reused for sidelink unlicensed operations.

Overall, there are two types of channel access operations in NR-U, i.e., LBE and FBE, and it should be first discussed and determined whether both of the operations need to be studied for SL-U. On one hand, LBE operation covers most of the key features and procedures used in unlicensed spectrum communication and provides broad applicability. On the other hand, FBE operation is heavily dependent on spectrum regulation policy and network deployment. Considering the workload of the WI, it may be more reasonable to focus on LBE operation first, and then if time allowed, carry out further study on FBE operation for SL-U.

- The study of the channel access mechanism for SL-U should focus on LBE operation first, and FBE operation should be with low priority.

According to the schemes of NR-U, there are some differences between DL and UL channel access procedures as described in [2]. In general, the study of sidelink is usually based on UL schemes which may benefit UE implementation and simplify the standardization progress. A similar principle should be used for the study of SL-U. Besides, reusing UL channel access procedure for SL-U may provide a better foundation for further discussion of mode 1 scheduling scheme. Considering the above factors, it is proposed to study the channel access procedure for SL by reusing UL channel access as a baseline.

Furthermore, the types of channel access procedures used in UL, i.e., Type 1 and Type 2A/2B/2C channel access procedures, should also be reused for SL-U.

- The channel access procedure for SL-U should be designed based on UL channel access procedure in NR-U.

- Based on UL channel access procedure, both Type 1 and Type 2 (2A/2B/2C) channel access procedures should be supported for SL-U.

To support the channel access procedures, some relevant parameters should be discussed and determined for SL-U.

The channel access priority class (CAPC) for sidelink should be defined as it is an essential factor for determining the types of channel access procedures for corresponding sidelink signals and determining the available length of contention window and the maximum channel occupancy time (COT) in Type 1 channel access procedure. Similarly, the CAPC for UL should be used as a baseline for SL-U.

For a channel occupancy in NR-U, in order to maintain a required time gap between two adjacent transmissions, a CP extension (CPE) may be applied before an actual signal. In general, the CPE scheme of UL channel access procedure should be reused for SL-U. Considering that the sidelink transmissions have to align with the available sidelink transmission starting points according to the resource pool configuration, the details of using CPE for SL-U may need further discussion and be determined according to the requirement of sidelink.

- The CAPC for SL should be determined by using CAPC for UL as a baseline.

- The details of using CP extension should be discussed and determined according to the features and requirements of SL-U.

  - Impacts of LBT Failure

Compared to sidelink in licensed spectrum, to perform sidelink communication in unlicensed spectrum, a key issue to be solved is the impact of potential LBT failure.

    - Mode 1

As described in WID, the Uu operation in mode 1 scheme is limited to licensed spectrum for SL-U. It means that gNB should schedule unlicensed spectrum resources for sidelink while gNB itself may not work on the unlicensed spectrum.

According to the scheduling of gNB, UE should firstly perform a channel access procedure for the scheduled sidelink resources, and then transmit sidelink signals on the assigned resources if channel access succeeds. In the case that the LBT procedure fails, UE cannot perform sidelink transmission, and the LBT failure cannot be known by gNB. Accordingly, gNB is unable to follow up on the sidelink situation and cannot provide proper scheduling of unlicensed spectrum resources.

In a word, LBT failure may cause invalid scheduling in mode 1 for SL-U, and potential improvement for this case should be studied and discussed.

In mode 1, LBT failure may lead to invalid scheduling of the sidelink resource in unlicensed spectrum.

- To discuss whether and how to improve the invalid scheduling caused by LBT failure in mode 1 for SL-U.

    - Mode 2

In Rel-16, the mode 2 sensing and resource selection procedure is defined based on dedicated sidelink resource pools, and it can mitigate or avoid potential resource collisions among sidelink UEs. By using the mode 2 procedure, a UE can reserve resources for further transmissions and other UEs may exclude the reserved resources from the candidate resource set.

In the scenario of SL-U, channel resources may be occupied by not only sidelink UEs but also devices of other types of RATs, and then the resource collisions that are not caused by sidelink UEs cannot be identified and solved by using the legacy mode 2 procedure. Besides, the devices of other RATs cannot receive and understand the SCI, i.e., the resource reservation of a sidelink UE is invalid for other devices. Therefore, other types of devices may try to occupy the channel which is overlapped with the reserved resources. It may lead to LBT failure of the sidelink UE and decrease the efficiency of using the legacy mode 2 procedure in SL-U.

Considering the potential impact of LBT failure, it is necessary to discuss and determine whether and how to improve the legacy mode 2 procedure for SL-U.

The legacy mode 2 sensing and resource selection procedure is based on a dedicated sidelink resource pool in which potential resource collisions are only considered among sidelink UEs.

LBT failure may cause the resources selected by using the legacy mode 2 procedure cannot be used by the Tx UE, and further lead to lower resource efficiency of SL-U.

- Considering the potential impact of LBT failure, it should be discussed whether and how to improve the legacy mode 2 procedure for SL-U.

    - SL Channels

With the assumption that resource structure and channel types in Rel-16 sidelink are reused in SL-U, before a sidelink signal transmission, including PSBCH, PSCCH/PSSCH, and PSFCH, a channel access procedure should be performed. In that way, LBT failure would lead to the relevant sidelink channels/signals cannot be transmitted.

As the information carried on the channels may have different priorities, the impacts of corresponding information unavailable caused by LBT failure are different. Then, the requirement and potential improvement for different sidelink channels should be further studied and determined, some relevant preliminary analyses are presented in [3].

  - Contention Window Adjustment in SL-U

When using Type 1 channel access procedure, a contention window should be determined. In NR-U, the contention window is determined based on HARQ-ACK feedback. Accordingly, a similar procedure should be used for the contention window adjustment in SL-U. But there is a special issue to be considered for SL-U contention window determining, i.e., sidelink HARQ-ACK feedback is optional and may be unavailable. For example, in the cases that no PSFCH resource is configured in a resource pool, HARQ-ACK is not required by Tx UE, and for the sidelink broadcast transmissions, sidelink HARQ-ACK feedback is unavailable for contention window adjustment procedure in SL-U.

Therefore, to carry out Type 1 channel access procedure, the contention window adjustment for SL-U should be discussed, especially in the case that sidelink HARQ-ACK feedback is unavailable.

Sidelink HARQ-ACK feedback is optional based on configuration, i.e. sidelink A/N may be unavailable for contention window adjustment in SL-U.

- For contention window adjustment in SL-U, it should be discussed:

  - Whether sidelink HARQ-ACK feedback should be used for contention window adjustment in SL-U;

  - In the case that sidelink HARQ-ACK feedback is unavailable, how to implement contention window adjustment;

  - COT Sharing in SL-U

The channel occupancy timing (COT) sharing is supported between gNB and UEs in NR-U, which can improve the success rate of channel access and resource efficiency. For SL-U, if each sidelink UE performs a channel access procedure for its sidelink signal transmission independently, the performance of the whole sidelink system may be uncertain and with low efficiency.

Considering the features and requirements of SL-U, it is reasonable to discuss and support COT sharing between sidelink UEs. As the sidelink cast types lead to different relationships among sidelink UEs, including a couple of UEs with unicast, UEs belong to the same group, and sidelink UEs with broadcast, the sidelink COT sharing in each case may require a different operation. Before discussing the detailed scheme, the potential cases which may support COT sharing in SL-U should be discussed and determined.

- To improve sidelink performance, COT sharing among sidelink UEs should be supported.

- For sidelink COT sharing among UEs, the following cases should be further discussed and determined:

  - case 1: COT sharing between Tx UE and Rx UE with sidelink unicast;

  - case 2: COT sharing among UEs which belong to the same sidelink group;

  - case 3: COT sharing among all sidelink UEs;

- Conclusion

In this contribution, some issues related to the channel access procedure for Rel-18 SL-U are discussed, and the relevant observations and proposals are listed as follows:

Observation 1:	In mode 1, LBT failure may lead to invalid scheduling of the sidelink resource in unlicensed spectrum.

Observation 2:	The legacy mode 2 sensing and resource selection procedure is based on a dedicated sidelink resource pool in which potential resource collisions are only considered among sidelink UEs.

Observation 3:	LBT failure may cause the resources selected by using the legacy mode 2 procedure cannot be used by the Tx UE, and further lead to lower resource efficiency of SL-U.

Observation 4:	Sidelink HARQ-ACK feedback is optional based on configuration, i.e. sidelink A/N may be unavailable for contention window adjustment in SL-U.

Proposal 1:	The study of the channel access mechanism for SL-U should focus on LBE operation first, and FBE operation should be with low priority.

Proposal 2:	The channel access procedure for SL-U should be designed based on UL channel access procedure in NR-U.

Proposal 3:	Based on UL channel access procedure, both Type 1 and Type 2 (2A/2B/2C) channel access procedures should be supported for SL-U.

Proposal 4:	The CAPC for SL should be determined by using CAPC for UL as a baseline.

Proposal 5:	The details of using CP extension should be discussed and determined according to the features and requirements of SL-U.

Proposal 6:	To discuss whether and how to improve the invalid scheduling caused by LBT failure in mode 1 for SL-U.

Proposal 7:	Considering the potential impact of LBT failure, it should be discussed whether and how to improve the legacy mode 2 procedure for SL-U.

Proposal 8:	For contention window adjustment in SL-U, it should be discussed:

−	Whether sidelink HARQ-ACK feedback should be used for contention window adjustment in SL-U;

−	In the case that sidelink HARQ-ACK feedback is unavailable, how to implement contention window adjustment;

Proposal 9:	To improve sidelink performance, COT sharing among sidelink UEs should be supported.

Proposal 10:	For sidelink COT sharing among UEs, the following cases should be further discussed and determined:

−	case 1: COT sharing between Tx UE and Rx UE with sidelink unicast;

−	case 2: COT sharing among UEs which belong to the same sidelink group;

−	case 3: COT sharing among all sidelink UEs;

- Reference

- RP-220300, WID revision: NR sidelink evolution

- TS 37.213, Physical layer procedures for shared spectrum channel access

- R1-2203694, Discussion on physical channel design framework, NEC