3GPP TSG RAN WG # 109-e											   R1-2203695

E-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.2

Source: 	NEC

Title:	Co-existence between LTE Sidelink and NR Sidelink

Document for:	Discussion/Decision

- Introduction

According to the WID of Rel-18 sidelink evolution in [1], co-channel co-existence of LTE sidelink and NR sidelink should be discussed and evaluated, and the scheme should be designed based on Rel-16 in-device coexistence framework. In this contribution, analyses of requirements for sidelink co-channel co-existence and preliminary schemes are presented.

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Discussion

  - Scenario and Requirement

With the progress of the V2X industrialization, different network deployments and spectrum allocation among countries and regions are implemented, and it further leads to the requirement and discussion on co-existence of LTE sidelink and NR sidelink. In other words, spectrum sharing between the two RATs has become an issue to be solved.

Several scenarios that may require to support co-channel co-existence of LTE sidelink and NR sidelink, including regions with dedicated and scarce spectrum resources for sidelink, the network transition progress from LTE sidelink to NR sidelink, and a system with flexible and interactivity requirements of V2X services, etc.

Considering the scenario and requirements of the V2X network, the co-channel co-existence of LTE sidelink and NR sidelink needs to be discussed and determined.

According to the requirements of different regions and network deployments, the co-existence of LTE sidelink and NR sidelink needs to be discussed.

Furthermore, according to the practical scenarios and requirements, it should be carefully analyzed and determined what level of flexibility and timeliness should be achieved through co-existence scheme of the two RATs. Besides, the compromise between flexibility and complexity should also be considered in the study of co-existence.

Taking an example of the V2X network transition progress from LTE sidelink to NR sidelink, it may require a non-real-time reconfiguration of the spectrum resource for the two RATs, i.e., semi-static reconfiguration of resource pools for each of the RATs can provide gradually transition from LTE sidelink to NR sidelink. For other scenarios of co-existence, a semi-static spectrum resource division and reconfiguration scheme may be sufficient to satisfy the requirements.

A semi-static spectrum resource division and reconfiguration scheme may be sufficient for most scenarios and requirements of co-channel co-existence of LTE sidelink and NR sidelink.

The compromise between flexibility and complexity should be considered in the study of co-existence schemes.

  - Potential Co-existence Schemes

As mentioned in [2], there are two alternatives for co-channel co-existence of LTE sidelink and NR sidelink:

  - Alt 1: Resource pool separation between the two RATs.

  - Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

In Alt 1, the semi-static TDM resource pools are used for LTE sidelink and NR sidelink, and it is involved in Rel-16 sidelink features. By using dedicated resource pools for the two RATs respectively, there is no impact on LTE sidelink. Meanwhile, it introduces limited standard work of NR sidelink and most of the relevant issues can be solved by UE implementation.

By using Alt 1 for the co-existence of the two RATs, i.e., with pre-configuration or reconfiguration of the resource pools for LTE sidelink and NR sidelink, it can provide semi-static flexibility of resource sharing of the two RATs, and requires less standard effort and no impact on both the two RATs.

The resource pool separation scheme for the co-existence of LTE sidelink and NR sidelink can provide configurable spectrum usage with semi-static flexibility.

In Alt 2, overlapping resource pools are used for LTE sidelink and NR sidelink. It means that no impact on LTE sidelink should be introduced while NR sidelink UEs need to avoid the resources which are used by LTE sidelink UEs. From the perspective of NR sidelink UEs, some essential modifications of resource assignments should be discussed and standardized.

For mode 1 resource scheme, as the overlapping sidelink resources may be scheduled by eNB and gNB respectively without information exchange, potential resource collision may be introduced. While using mode 2 resource scheme, NR sidelink UEs have to not only perform Rel-16 mode 2 procedure to select resources, but also need to carry out sensing and avoiding the resources reserved by LTE sidelink UEs. Accordingly, by using Alt 2, the actual efficiency of resource sharing between the two RATs may need further study and assessment.

For both mode 1 and mode 2 resource schemes, dynamic sharing between LTE sidelink and NR sidelink with overlapping resource pools needs more discussion and may introduce lots of specification impact.

It needs more discussion and assessment on dynamic resource sharing with overlapping resource pools between LTE sidelink and NR sidelink.

- Conclusion

In this contribution, requirements and preliminary schemes for sidelink co-channel co-existence are discussed, and the relevant observations and proposals are listed as follows:

Observation 1:	According to the requirements of different regions and network deployments, the co-existence of LTE sidelink and NR sidelink needs to be discussed.

Observation 2:	A semi-static spectrum resource division and reconfiguration scheme may be sufficient for most scenarios and requirements of co-channel co-existence of LTE sidelink and NR sidelink.

Observation 3:	The resource pool separation scheme for the co-existence of LTE sidelink and NR sidelink can provide configurable spectrum usage with semi-static flexibility.

Observation 4:	For both mode 1 and mode 2 resource schemes, dynamic sharing between LTE sidelink and NR sidelink with overlapping resource pools needs more discussion and may introduce lots of specification impact.

Proposal 1:	The compromise between flexibility and complexity should be considered in the study of co-existence schemes.

Proposal 2:	It needs more discussion and assessment on dynamic resource sharing with overlapping resource pools between LTE sidelink and NR sidelink.

- Reference

- RP-220300, WID revision: NR sidelink evolution

- RP-212664, Moderator’s summary for discussion on [RAN94e-R18Prep-04] Sidelink enhancements (excluding positioning and relaying)