3GPP TSG RAN WG1 #109-e		      R1-2203696

e-Meeting, May 9th – May 20th, 2022

Source:	NEC

Title:	Discussion on positioning support for RedCap UEs

Agenda Item:	9.5.3

Document for:	Discussion and Decision

Introduction

In RAN#94, a revised SID on NR Positioning Enhancements was approved [1]. The objectives of the positioning support for RedCap UEs are:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we provide our initial views on positioning support for RedCap UEs.

Discussion

## 2.1   RedCap UEs Capability limitation

Through the Rel-17 NR RedCap work item, 3GPP has established a framework for enabling reduced capability NR devices suitable for a range of use cases, including the industrial sensors, video surveillance, and wearables use cases, with requirements on low UE complexity and sometimes also on low UE power consumption. The following UE complexity reduction features are supported in Rel-17:

- Reduced maximum UE bandwidth:

  - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz.

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

- Reduced minimum number of Rx branches

- Maximum number of DL MIMO layers

- Relaxed maximum modulation order

- Duplex operation

Various existing positioning methods, e.g. RTT, TDOA, AOA/D, could be performed by RedCap UEs, but as the evaluation results shown in [2], performance of timing-based positioning methods depend on the bandwidth of the PRS, the limited reference signal transmission bandwidth has a great impact on the accuracy of the positioning. In Rel-18, in order to further reduce RedCap UE complexity, the UE bandwidth would be reduced to 5MHz in FR1, it will be a bigger challenge to accuracy of the positioning. So it’s necessary to study the impact on the accuracy of the positioning with a very limited bandwidth.

In current DL-AoD positioning method, when the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same DL PRS Rx beam index have been performed using the same spatial domain filter for reception. For the reduced minimum number of Rx branches, the impact on performance of the DL-AoD positioning needs to be further estimated.

Proposal 1: Study the impact on the accuracy of the positioning with a bandwidth reduction and reduced Rx branches.

## 2.2   Positioning support for RedCap UEs

In previous RAN discussions, some companies mentioned to use frequency hopping to benefit from frequency diversity and increase the effective bandwidth of PRS, and LTE has introduced PRS hopping in FeMTC. In FeMTC, the supported PRS bandwidth for frequency hopping is 6 PRB, and frequency hopping interval is one PRS occasion. The hopping configuration is transmitted to UE including the number of PRS frequency hopping bands, and the first sub-band is located at the centre of the system bandwidth.

For NR REDCAP UEs, the PRS hopping pattern in LTE can be a start point, and we support to study PRS frequency hopping and suggest to estimate the accuracy enhancement gain from the PRS frequency hopping. If frequency hopping is enable, the hopping configuration from higher layer should contain at least the number of subbands and the frequency and time information of each subband. In order to meet the similar accuracy requirement as normal UEs, several measurement periods may be needed, accordingly the measurement reporting delay is longer, so there is a balance between the accuracy and delay.

In current PRS reception procedure [3], the UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap, and UE is not expected to process DL PRS without configuration of measurement gap. If frequency hopping is enable for NR REDCAP UEs, there will be a RF tuning between each subband hopping, so need to consider whether the location of hopping subband in time domain should be aligned with a measurement gap.

Proposal 2: For NR REDCAP UEs, study the PRS frequency hopping scheme and consider the time and frequency allocation for each hopping subband

If frequency hopping is enable, there may be several subbands in different time and frequency resources. However, the frequency resources of a whole PRS subband transmission from a cell may be occupied temporarily by other cells.  Therefore, for better utilizing the frequency resources, a muting mechanism for disabling one or some of the frequency hopping subbands should be further studied.

Proposal 3: For NR REDCAP UEs, if frequency hopping is enable, study the muting mechanism for subbands.

Conclusion

In this contribution, we share our views on positioning support for RedCap UEs, and the following proposals are made.

Proposal 1: Study the impact on the accuracy of the positioning with a bandwidth reduction and reduced Rx branches.

Proposal 2: For NR REDCAP UEs, study the PRS frequency hopping scheme and consider the time and frequency allocation for each hopping subband

Proposal 3: For NR REDCAP UEs, if frequency hopping is enable, study the muting mechanism for subbands.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel

- TR 38.855 V16.0.0

- 3GPP TS 38.214 V16.7.0 (2021-09)