3GPP TSG RAN WG1 #109-e	R1-2203698

Electronic Meeting, May 9th – 20th, 2022

Source: 	NEC

Title: 	Discussion on signaling for side control information

Agenda item:	9.8.2

Document for:	Discussion and Decision

Introduction

The study aspects of network-controlled repeater (taken place of “smart repeater” in offline discussion) are further discussed in RAN#94-e meeting, and the discussions are summarized in [1]. Based on the discussions, a related SID [2] was approved, whose objective is shown as follows:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide views on L1/L2 signaling for side control information of network-controlled repeaters.

Discussion

Treating network-controlled repeaters as a network-like node or a UE-like device is discussed in RAN#94-e meeting. If a network-like node is approved, there will be a high probability to influence the core network design, which contradicts with the intention of low cost for enhancing coverage by introducing network-controlled repeater. Therefore, treating it as a UE-like device is more reasonable. And the communication of side control information between network-controlled repeater and gNB can be achieved with the same procedure and mechanism as those between normal UE and gNB.

Observation 1: It’s more rational to treat network-controlled repeater as a UE-like device.

For transmitting the side control information, a simplest way is applying the whole UE protocol stack directly for network-controlled repeater. However, it is too complex and too redundant for network-controlled repeater. As the justification descripted in the SID [2], network-controlled repeater is an enhancement of RF repeater. On the one hand, it simply amplifies and forwards the signal according to the side control information, without decoding the received signal to suppress interference. On the other hand, its requirement on communication or information interaction with gNB is much lower than a normal UE. Then in general, the flexible signaling based on the whole protocol stack for a normal UE is unnecessary.

Observation 2: Whole UE protocol stack is too complex and too redundant for a network-controlled repeater.

From the scenarios and assumption listed in the related SID [2] , we known that the network-controlled repeater is stationary. Based on this assumption, the link between gNB and network-controlled repeater is expected to have a stable quality. Thus, some side control information such as beam information of the link between gNB and network-controlled repeater doesn’t need to train and update frequently. And resource for side control information transmission doesn’t need to be allocated dynamically to match the time variability of channel. Then a less flexible signaling with large updating period can be used for side control information transmission.

Moreover, the link quality between network-controlled repeater and UE is time-varying due to the mobility of UE. So the side control information related to this link should be updated according to the mobility, and the exist procedure, signaling and configuration can be applied directly. For example, information of transmitting beam for network-controlled repeater in DL transmission should be trained and updated frequently, to guarantee the probability of radio link failure lower than a given value.

Proposal 1: Support combined static or semi-static signalling and dynamic signaling as exist ones defined for normal UE for side control information transmission of network-controlled repeater.

Conclusions

In this contribution, we discuss smart repeaters, and have the following observations and proposal:

Observation 1: It’s more rational to treat network-controlled repeater as a UE-like device.

Observation 2: Whole UE protocol stack is too complex and too redundant for a network-controlled repeater.

Proposal 1: Support combined static or semi-static signalling and dynamic signaling as exist ones defined for normal UE for side control information transmission of network-controlled repeater.

References

- RP-212663, 3GPP TSG RAN Meeting #94e, Moderator’s summary for discussion on [RAN94e-R18Prep-03] Additional topological improvements - smart repeater.

- RP-213700, 3GPP TSG RAN Meeting #94e, New SI: Study on NR Network-controlled Repeaters.