3GPP TSG RAN WG1 #109-e                                                                                  R1-2203699

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.12.1

Source:	Lenovo

Title:	Remaining issues on reliability improvement for RRC_CONNECTED UEs

Document for:	Discussion

# Introduction

The contribution is focused on reliability improvement issues for RRC-CONNECTED UEs in Rel-17 NR MBS.

In RAN1#108-e meeting, regarding NACK-only based HARQ-ACK feedback and HARQ-ACK codebook determination, the corresponding agreements are made as below:

Agreement

For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements):

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - FFS: how to determine PUCCH resource

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast.

    - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID.

      - FFS mapping details.

      - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook.

    - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs

  - Alt4 is not supported for more than 4 TBs

- FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR

- FFS: UE capability

Agreement

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the following option 1 (from the previous agreement) is adopted:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Agreement

When NACK-only based HARQ-ACK feedback is used for multicast SPS PDSCH without PDCCH scheduling, the UE determines a priority index from the HARQ-ACK codebook index in the configuration for SPS multicast, using the same method with the one for ACK/NACK based feedback.

Agreement

When UE is configured with unicast SPS and multicast SPS with NACK-only based feedback for multiplexing on the same PUCCH for the same priority case, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK.

- For NACK only based HARQ-ACK transformed to ACK/NACK based HARQ-ACK, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH and the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast, as agreed for ACK/NACK based feedback.

Agreement

Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

Agreement

If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast:

- If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast

- Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs.

  - FFS: additional restrictions

Agreement

When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit.

- Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others.

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Agreement

When UE is configured with enhanced Type-2 codebook for unicast and when the UE is scheduled to multiplex enhanced Type-2 HARQ-ACK for unicast with Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot,

- UE generates separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook.

Agreement

- For the following two cases, the fallback operation for the Type-1 HARQ-ACK codebook for multicast is applied to G-RNTI/G-CS-RNTI configured with HARQ-ACK enabled only,

  - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1,

  - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell,

  - SPS PDSCH reception(s) associated with G-CS-RNTIs.

- Note: If the UE receives two SPS PDSCH releases for two respective G-CS-RNTIs with DAI=1, or two PDSCHs by DCI 4_1 with DAI=1 for two respective G-RNTIs on the PCell, there is no fallback.

Agreement

For Type-1 codebook generation,

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, FFS between the 3 alternatives below:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

  - Other alternatives are not precluded

Agreement

If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

  - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

Agreement

When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

  - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

  - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

- FFS: details of the UL DAI field

Agreement

For NACK-only feedback for PUCCH format 0 or format 1, only 1 cyclic shift is used.

- The sequence cyclic shift for NACK-only is mcs=0.

- For PF1 NACK-only, set b0=0.

- Note: the initialCyclicShift is configured for PUCCH-format0 and PUCCH-format1 as legacy.

Agreement

For multiplexing NACK-only with HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority or ACK/NACK based HARQ-ACK feedback for multicast with another G-RNTI for the same priority, the multiplexing does not depend on the decoding result.

- Note: when multiplexing the NACK-only is transformed into ACK/NACK as agreed.

Agreement

For Type-2 codebook generation, UE reports HARQ-ACK bits only for TBs with enabled HARQ-ACK by RRC or DCI.

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the following Alt2 (from the previous agreement) is adopted:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Agreement

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

However, there are still some FFS issues which need to be resolved in maintenance phase. Hence, in this contribution, more detailed views on remaining issues are present from our side.

# Discussion

## HARQ-ACK enabling/disabling

According to latest RAN1 agreement, for a given G-RNTI, when RRC configures presence of HARQ-ACK feedback enabling/disabling indication (HEI) in the DCI format with CRC scrambled by the given G-RNTI, this enabling/disabling HARQ-ACK feedback indication is only included in DCI format 4-2 with CRC scrambled by the given G-RNTI. In detail, this enabling/disabling HARQ-ACK feedback indication is not included in DCI format 4-1 with CRC scrambled by the given G-RNTI.

There are several open issues:

- One problem is when UE receives a DCI format 4-1 with CRC scrambled by the G-RNTI, should the UE assume HARQ-ACK feedback is enabled or disabled for the scheduled GC-PDSCH?

- Another problem is when the DCI format 4-1 is missed by UE, UE can’t know whether this missed DCI format is DCI format 4-1 or DCI format 4-2 and whether the missed DCI enables or disables HARQ-ACK feedback.

- Similarly, when one DCI format 4-1 is missed by UE, UE can’t know whether this missed DCI format is DCI format 4-1 or DCI format 4-1 and whether the missed DCI enables or disables HARQ-ACK feedback.

One example is shown in Figure 1, where four group-common DCIs, DCI 1~4, schedule four group-common PDSCHs for a given G-RNTI, respectively. DCI 1, DCI 3 and DCI 4 are DCI format 4-2 while DCI 2 is DCI format 4-1. For the given G-RNTI, RRC signaling configures the presence of enabling/disabling HARQ-ACK feedback indicator in DCI format 4-2 with CRC scrambled by the G-RNTI. HARQ-ACK feedback is enabled in DCI 1, DCI 3 and DCI 4 with HEI (enabling/disabling HARQ-ACK feedback indicator) setting to “1” and corresponding HARQ-ACK feedback is indicated to be transmitted in same slot. Since DCI 2 does not include enabling/disabling HARQ-ACK feedback indicator, upon reception of DCI 2, how can UE determine the HARQ-ACK feedback is enabled or disabled?

Furthermore, as shown in Figure 1, assuming DCI 2 is missed by UE, according to DAI, UE can identify there is a DCI missed between DCI 1 and DCI 3. However, UE can’t know whether the missed DCI is DCI format 4-1 or DCI format 4-2 as well as HARQ-ACK feedback for PDSCH 2 is enabled or not. In addition, assuming DCI 1 is missed by UE, according to DAI, UE can identify there is a DCI missed before DCI 2. However, UE can’t know whether the missed DCI is DCI format 4-1 or DCI format 4-2 as well as whether HARQ-ACK feedback for PDSCH 1 is enabled or not.

As a result, UE can’t determine HARQ-ACK codebook for the given G-RNTI and can’t multiplex HARQ-ACK feedback for the given G-RNTI with unicast HARQ-ACK feedback and HARQ-ACK feedback for other multicast services in same HARQ-ACK codebook.

Figure 1: HARQ-ACK codebook determination for multicast DCI

To simplify the UE behavior and avoid any misunderstanding between UE and gNB on HARQ-ACK codebook size, 1-bit HEI should be included in both DCI format 4-1 and DCI format 4-2. Considering each field in DCI format 4-1 has fixed size, this 1-bit HEI is always included in DCI format 4-1. In detail, if RRC signaling configures presence of enabling/disabling HARQ-ACK feedback indication (HEI) in multicast DCI format for a given G-RNTI, one-bit HEI is included in the multicast DCI format 4-1 indicating enabled or disabled HARQ-ACK feedback for group-common PDSCH scheduled by the multicast DCI format 4-1; otherwise, this bit is reserved. This follows same design principle of 1-bit UL/SUL indicator in fallback DCI format 1-0. For DCI format 4-1, one of the reserved three bits can be used as 1-bit HEI. The presence of this bit is always there and not configurable by RRC signaling so as to guarantee each field in the multicast DCI format 4-1 is of fixed size.

Based on above discussion, we have below proposals:

Proposal 1: Support one-bit enabling/disabling HARQ-ACK feedback indication in multicast DCI format 4-1.

## UL DAI details

Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded Agreement When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. - The UL-DAI for multicast is 1-bit for Type-1 codebook - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs. - The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded - FFS: details of the UL DAI field

For Rel-17 MBS, when HARQ-ACK feedback for unicast PDSCHs and HARQ-ACK feedback for multicast PDSCHs carrying one or more multicast services overlaps with PUSCH, UE should multiplex the HARQ-ACK feedback for unicast and the one or more multicasts on the PUSCH as legacy. Multiple HARQ-ACK sub-codebooks are generated with separate DAIs used for unicast and multiple multicast services, e.g., a first sub-codebook comprises HARQ-ACK information bits for unicast, a second sub-codebook comprises HARQ-ACK information bits for PDSCHs carrying multicast 1 and a third sub-codebook comprises HARQ-ACK information bits for PDSCHs carrying multicast 2, …, until the last sub-codebook comprises HARQ-ACK information bits for PDSCHs carrying multicast with the largest G-RNTI value of the supported multiple multicast services, wherein the sub-codebooks for multicast are ordered in increasing order of G-RNTI value. In this way, when the HARQ-ACK feedback for unicast PDSCHs and HARQ-ACK feedback for multicast PDSCHs carrying one or more multicast services are multiplexed on the PUSCH, the best way is to included multiple UL DAIs in the UL grant with each UL DAI corresponding to one sub-codebook for the unicast or one of the multiple multicast services, which can avoid any misunderstanding of the final HARQ-ACK codebook size between UE and gNB. This is aligned with the principles of CBG-based retransmission where two UL DAIs are included in the UL grant with one UL DAI corresponding to TB based sub-codebook and another UL DAI corresponding to CBG-based sub-codebook.

For Type-2 HARQ-ACK codebook determination, in order to indicate UL-DAI for the unicast HARQ-ACK sub-codebook and multiple HARQ-ACK sub-codebooks, one option is to indicate the separate UL DAIs for unicast and each of the one or more multicast services in the UL grant, which has below drawbacks: (1) UE can’t determine the payload size of the UL grant; (2) too much overhead may be caused especially when UE is configured with CBG-based transmission; (3) it is not feasible to include too many UL DAIs in the UL grant considering the limit of DCI payload size.

As shown in above agreement, if Type-2 codebook is configured for unicast and multicast, RAN1 has agreed that 2 UL DAIs are included in DCI scheduling PUSCH with one UL DAI used for unicast and another UL DAI used for multicast. When the UE is configured with multiple G-RNTIs, the 2-bit UL DAI for multicast is applied for all the configured G-RNTIs. One problem is how to indicate the UL DAI for the multiple multicast services considering different multicast services may have different associated UL DAIs.

To address the problem, the straightforward way is to indicate the largest DL DAI value among the configured G-RNTIs as mentioned in Alt 1-3. As a result, the multiple HARQ-ACK sub-codebooks for the multiple multicast services can have same sub-codebook size according to the UL DAI for multicast. In this way, upon reception of the UL grant, UE shall generate the first sub-codebook for unicast according to the first UL DAI as legacy and generate each of the multiple multicast sub-codebooks according to the second UL DAI. There is no misunderstanding between UE and gNB on the unicast sub-codebook size and multicast sub-codebook size and only one additional UL DAI is included in the UL grant which can guarantee the UL DCI has a reasonable payload size.

Based on above discussion, we have below proposals:

Proposal 2: For Type-2 HARQ-ACK codebook for both unicast and multicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported.

The above way can be easily extended to the case when the unicast and multicast HARQ-ACK codebooks are configured with different types. Similar to same Type-1 or Type-2 HARQ-ACK codebook configured for unicast and multicast, separate UL DAIs are included in the UL grant, wherein a first UL DAI is applied for unicast HARQ-ACK feedback and a second UL DAI is applied for multicast HARQ-ACK feedback for all the configured multicast services as mentioned above for Type-1 or Type-2 HARQ-ACK codebook determination.

- in case of Type-2 HARQ-ACK codebook configured for unicast and Type-1 HARQ-ACK codebook configured for multicast, separate UL DAIs are included in the UL grant, wherein the first UL DAI indicates the codebook size for the unicast HARQ-ACK feedback and the second UL DAI indicates whether the multicast HARQ-ACK feedback is to be multiplexed on the scheduled.

- In case of Type-1 HARQ-ACK codebook configured for unicast and Type-2 HARQ-ACK codebook configured for multicast, separate UL DAIs are included in the UL grant, wherein the first UL DAI indicates whether the unicast HARQ-ACK feedback is to be multiplexed on the scheduled PUSCH and the second UL DAI indicates the largest DL DAI values among the configured G-RNTIs.

Based on above discussion, we have below proposals:

Proposal 3: For Type-2 HARQ-ACK codebook configured for multicast and Type-1 HARQ-ACK codebook configured for unicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported.

Regarding UL DAI in DCI format 0-1 or 0-2, we have agreed that unicast and multicast have separate UL DAIs. So it is better not to merge the two UL DAIs together. Furthermore, since when UE is configured with CBG-based retransmission for unicast, 1st UL DAI and 2nd UL DAI have been used for two sub-codebooks for unicast. In that case, it is not perfect to say 2nd UL DAI is used for multicast. Our understanding is 1st DAI and 2nd DAI are kept unchanged for unicast and 3rd DAI can be introduced for multicast. With 3rd UL DAI for multicast, the standard change is easier.

Regarding how to capture UL DAI details in DCI format 0-1 or 0-2, we have below TP:

**********************<irrelevant part omitted>************************************************

-    1st downlink assignment index – 1, 2 or 4 bits:

-     1 bit for semi-static HARQ-ACK codebook;

-     2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured;

-     4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st  downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-    2nd downlink assignment index – 0, 2 or 4 bits:

-     2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured;

-     4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true;

-     0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-    3rd downlink assignment index – 0, 1, or 2 bits:

-     0 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast is not configured, or if pdsch-HARQ-ACK-Codebook-Multicast = semiStatic is configured and the higher layer parameter fdmed-Reception-Multicast is not configured;

-     1 bit if semi-static HARQ-ACK codebook is configured for multicast;

-     2 bits if dynamic HARQ-ACK codebook is configured for multicast.

**********************<irrelevant part omitted>************************************************

Proposal 4: Support the above TP to capture UL DAI details for unicast and multicast.

## Type-1 HARQ-ACK codebook determination

For Type-1 HARQ-ACK codebook determination, fallback operation has been agreed for multicast in Rel-17 for reducing the HARQ-ACK overhead due to semi-statically determined HARQ-ACK codebook size. One remaining issue is how to generate the Type-1 HARQ-ACK codebook for all configured G-RNTIs if at least one G-RNTI is configured with enabled HARQ-ACK feedback.

Considering the main motivation of using RRC signaling to configure the enabled or disabled HARQ-ACK feedback is the configuration flexibility, it does not make sense to add the restriction that UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTIs.

Furthermore, since the HARQ-ACK feedback enabled or disabled is configured by RRC signaling, there should be no misunderstanding between UE and gNB on the Type-1 HARQ-ACK codebook size. For a G-RNTI configured with disabled HARQ-ACK feedback, it does make sense that UE shall not report any HARQ-ACK information for the G-RNTI.

Proposal 5: For Type-1 HARQ-ACK codebook determination, UE does not report any HARQ-ACK information for a G-RNTI configured with disabled HARQ-ACK feedback.

If RRC configures the presence of 1-bit HEI in DCI, for Type-1 HARQ-ACK codebook determination, each candidate PDSCH reception occasion is mapped to one HARQ-ACK information bit. If the DCI is detected, then one-bit ACK or NACK is generated regardless of whether HARQ-ACK feedback enabled or disabled; If the DCI is not detected, then NACK is generated.

To resolve the issue, Type-1 HARQ-ACK codebook and presence of 1-bit HEI in DCI should not be configured simultaneously. Hence, UE doesn’t expect to be configured with Type-1 HARQ-ACK codebook with presence of 1 bit HEI in DCI.

Proposal 6: Type-1 HARQ-ACK codebook and presence of 1-bit HEI in DCI should not be configured simultaneously for a UE.

## Type -2 HARQ-ACK codebook determination

For a given G-RNTI, when RRC configures presence of HARQ-ACK feedback enabling/disabling indication (HEI) in the DCI format with CRC scrambled by the given G-RNTI, if the DCI format is missed by UE, UE can’t know whether HEI indicates enabled HARQ-ACK feedback or disabled HARQ-ACK feedback and can’t determine a HARQ-ACK codebook for the given G-RNTI. Needless to say, when HARQ-ACK feedback for unicast or for multicast associated with another G-RNTI, HARQ-ACK codebook determination is one problem at UE side. Misunderstanding on HARQ-ACK codebook size between UE and gNB easily occurs.

One example is shown in Figure 2, where four group-common DCIs, DCI 1~4, schedule four group-common PDSCHs for a given G-RNTI, respectively. At gNB side, HARQ-ACK feedback is enabled in DCI 1 and DCI 4 with HEI setting to “1” and corresponding HARQ-ACK feedback is indicated to be transmitted in same slot. HARQ-ACK feedback is disabled in DCI 2 and DCI 3 with HEI setting to “0”, i.e., UE is supposed not to transmit HARQ-ACK feedback for two PDSCHs scheduled by DCI 2 and DCI 3. When DCI 2 is missed by UE, according to DAI, UE can identify there is a DCI missed between DCI 1 and DCI 3. However, UE can’t know whether HARQ-ACK feedback for PDSCH 2 is enabled or not. As a result, UE can’t determine HARQ-ACK codebook for the given G-RNTI and can’t multiplex HARQ-ACK feedback for the given G-RNTI with unicast HARQ-ACK feedback and HARQ-ACK feedback for other multicast services in same HARQ-ACK codebook.

Figure 2: HARQ-ACK codebook ambiguity caused in case of missed DCI

To resolve the above issue, when HARQ-ACK feedback is disabled by a DCI, the DAI in the DCI should not be updated. Hence, even if the DCI is missed, UE can’t identify it and has no ambiguity whether the DCI enabled or disabled HARQ-ACK feedback so that it has no impact on HARQ-ACK codebook generation. In detail, for a DCI with HEI disabled, DAI in the DCI indicates same value as that in a previous DCI with HEI enabled.

One example is shown in Figure 3. Assuming both DCI 2 and DCI 3 indicate disabled HARQ-ACK feedback via HEI, then the value of DAI in both DCI 2 and DCI 3 is not updated and indicates same value as DAI in DCI 1 which enables HARQ-ACK feedback, i.e., DAI=1 in both DCI 2 and DCI 3 to follow DAI value in DCI 1. In case DCI 2 is missed, since DAI in DCI 2 is not updated, UE can’t identify DCI 2 is missed and won’t generate a bit for PDSCH 2. In case only DCI 1 is missed, since DAI in DCI 4 is equal to 2, UE can identify one DCI which requires HARQ-ACK feedback is missed and shall generate NACK bit for PDSCH 1. In that sense, there is no misunderstanding between UE and gNB on the HARQ-ACK codebook size for the multicast service.

Figure 3: HARQ-ACK codebook ambiguity solved

Based on above discussion, we have below proposals:

Proposal 7: For a DCI with HEI disabled, DAI in the DCI indicates same value as that in a previous DCI with HEI enabled.

## Single NACK-only based feedback multiplexing with SR

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS: The case of PUCCH for SR.

As mentioned above, there is one problem needs to be resolved when NACK-only based HARQ-ACK feedback is to be transmitted in a slot where a positive SR is also to be transmitted. According to legacy behavior of multiplexing HARQ-ACK feedback with SR, UE should transmit a PUCCH format 0 on PRB configured for NACK-only based HARQ-ACK feedback with different value to indicate the positive SR. In detail, for only NACK-only based HARQ-ACK feedback to be transmitted in PUCCH format 0,  is provided by initialcyclicshift of PUCCH-format0, and mcs=0 for indicating NACK; when the NACK-only based HARQ-ACK feedback needs to be multiplexed with positive SR in PUCCH format 0,   is provided by initialcyclicshift of PUCCH-format0, and mcs=6 for indicating NACK and positive SR. In that sense, two different cyclic shifts are used to generate two different sequences for PUCCH format 0, one for indicating only NACK-only based HARQ-ACK feedback and another for indicating positive SR and NACK-only based HARQ-ACK feedback.

However, the above mechanism can’t work for Rel-17 MBS as a group-common PUCCH resource may be shared by UEs in the same group when NACK-only based HARQ-ACK feedback is adopted. When two or more UEs in the same group should transmit positive SR and NACK-only based HARQ-ACK feedback in one slot, above mechanism will inevitably cause PUCCH resource collision since the PRB for PUCCH format 0,  provided by initialcyclicshift of PUCCH-format0, and mcs=6 for indicating NACK and positive SR, are totally shared by the group of UEs. In that sense, upon reception of the sequence generated by mcs=6, gNB can only know at least one UE transmits NACK corresponding to PTM transmission while can’t differentiate which UE sends the positive SR. As a result, gNB can’t send UL grant to the UE which really needs UL resource.

Consequently, multiplexing of NACK-only based feedback and positive SR has to be transmitted in one PUCCH in the UE-specific PUCCH resource. Considering SR resource is UE-specifically configured by RRC signaling, multiplexing of NACK-only based feedback and positive SR seems the only option. To differentiate positive SR only without NACK-only based feedback (e.g., when UE correctly decodes the group-common PDSCH, UE doesn’t need to transmit ACK to gNB), different cyclic shift should be used for positive SR only and positive SR multiplexing with NACK-only based feedback. In case that negative SR and NACK-only based feedback, UE shall transmit the NACK on the group shared PUCCH resource and don’t need transmit negative SR; In case that negative SR and ACK for group-common PDSCH, UE doesn’t transmit any feedback to gNB.

Based on above discussion, we have below proposals:

Proposal 8: For SR multiplexing with single NACK-only based feedback, positive SR multiplexing with NACK-only feedback is transmitted in SR resource.

Proposal 9: For SR multiplexing with single NACK-only based feedback, different cyclic shifts are used to differentiate positive SR only and positive SR multiplexing with NACK-only feedback.

## Multiple NACK-only based feedback multiplexing

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR FFS: UE capability

As shown in above, for Alt 4, there are several remaining issues on determining PUCCH slot, PUCCH resource and mapping details. The PUCCH slot can be directly determined based on k1 in the last DCI scheduling multicast among DCIs with NACK-only based HARQ-ACK feedback multiplexing in one slot.

One open issue is how to determine the number of TBs according to Type-1 HARQ-ACK codebook or Type-2 HARQ-ACK codebook. Type-1 HARQ-ACK codebook leads to too much overhead which is not consistent with the motivation of using NACK-only based HARQ-ACK feedback. Type-2 HARQ-ACK codebook can minimize the HARQ-ACK feedback overhead. Existing Type-2 HARQ-ACK codebook generation for one or multiple configured G-RNTIs can be used to determine the HARQ-ACK feedback combination for the up to 4 TBs.

In addition, the PUCCH resource for NACK-only based feedback is configured by RRC signaling or dynamically by DCI is also undefined. Considering the DCI may be missed, using DCI to dynamically indicate the PUCCH resource is not reliable as RRC signaling. A mapping table is defined between the HARQ-ACK feedback combination for the up to 4 TBs and a specific PUCCH resource of a set of PUCCH resources. The set of PUCCH resources is configured by RRC signaling. The design principle of the mapping table is to establish a linkage from the HARQ-ACK information bits for the one or multiple multicast services to a specific PUCCH resource and ensure different HARQ-ACK feedback combinations for the up to 4 TBs corresponding to orthogonal PUCCH resources.

Furthermore, when defining the mapping table, the PUCCH format should be considered. In case of PUCCH format 0, up to 15 sequences are defined as the set of PUCCH resources and different HARQ-ACK feedback combinations for the up to 4 TBs correspond to different sequences; in case of PUCCH format 1, since each PUCCH format 1 can carry 1 or 2 bits, the number of required PUCCH resources can be reduced compared with that for PUCCH format 0. E.g., when each PUCCH format 1 carries a single bit, each bit can have two values, 0 or 1, then up to 8 PUCCH resources are enough for up to 4 TBs to cover up to 15 HARQ-ACK feedback combination; when each PUCCH format 1 carries two bits, each bit can have two values, 0 or 1, then up to 4 PUCCH resources are enough for up to 4 TBs to cover up to 15 HARQ-ACK feedback combination.

The last open issue is whether the mapping table method is applied to single G-RNTI or multiple G-RNTIs. We think it is applied to both. The key point is how to generate the HARQ-ACK feedback combination for the up to 4 TBs. Existing Type-2 HARQ-ACK codebook generation for one or multiple configured G-RNTIs can be used to determine the HARQ-ACK feedback combination. Then UE can determine a PUCCH resource according to the generated HARQ-ACK feedback combination and the defined mapping table. If the up to 15 PUCCH resources are applied for a single G-RNTI, then too much PUCCH resources are required for linking one HARQ-ACK feedback combination for multiple G-RNTIs to one PUCCH resource. This is not reasonable.

Based on above discussion, we have below proposals:

Proposal 10: The PUCCH slot is determined based on K1 value indicated by the last multicast DCI.

Proposal 11: A set of PUCCH resources for mapping multiple NACK-only feedback multiplexing is configured by RRC signaling.

Proposal 12: The HARQ-ACK feedback combination is determined based on Type-2 HARQ-ACK codebook.

Proposal 13: The PUCCH resource for transmission is determined according to the HARQ-ACK feedback combination and the PUCCH mapping table.

Proposal 14: The PUCCH mapping table is defined based on the number of TBs and the PUCCH format.

Proposal 15: The PUCCH mapping table is applied to both single G-RNTI case and multiple G-RNTI case.

# Conclusion

In this contribution, we focus on the reliability improvement for MBS transmission and have below proposals:

Proposal 1: Support one-bit enabling/disabling HARQ-ACK feedback indication in multicast DCI format 4-1.

Proposal 2: For Type-2 HARQ-ACK codebook for both unicast and multicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported.

Proposal 3: For Type-2 HARQ-ACK codebook configured for multicast and Type-1 HARQ-ACK codebook configured for unicast, UL DAI for multicast indicates the largest DL DAI values among the configured G-RNTIs, i.e., Alt 1-3 is supported.

Proposal 4: Support the below TP to capture UL DAI details for unicast and multicast.

**********************<irrelevant part omitted>************************************************

-    1st downlink assignment index – 1, 2 or 4 bits:

-     1 bit for semi-static HARQ-ACK codebook;

-     2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured;

-     4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st  downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-    2nd downlink assignment index – 0, 2 or 4 bits:

-     2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured;

-     4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true;

-     0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-    3rd downlink assignment index – 0, 1, or 2 bits:

-     0 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast is not configured, or if pdsch-HARQ-ACK-Codebook-Multicast = semiStatic is configured and the higher layer parameter fdmed-Reception-Multicast is not configured;

-     1 bit if semi-static HARQ-ACK codebook is configured for multicast;

-     2 bits if dynamic HARQ-ACK codebook is configured for multicast.

**********************<irrelevant part omitted>************************************************

Proposal 5: For Type-1 HARQ-ACK codebook determination, UE does not report any HARQ-ACK information for a G-RNTI configured with disabled HARQ-ACK feedback.

Proposal 6: Type-1 HARQ-ACK codebook and presence of 1-bit HEI in DCI should not be configured simultaneously for a UE.

Proposal 7: For a DCI with HEI disabled, DAI in the DCI indicates same value as that in a previous DCI with HEI enabled.

Proposal 8: For SR multiplexing with single NACK-only based feedback, positive SR multiplexing with NACK-only feedback is transmitted in SR resource.

Proposal 9: For SR multiplexing with single NACK-only based feedback, different cyclic shifts are used to differentiate positive SR only and positive SR multiplexing with NACK-only feedback.

Proposal 10: The PUCCH slot is determined based on K1 value indicated by the last multicast DCI.

Proposal 11: A set of PUCCH resources for mapping multiple NACK-only feedback multiplexing is configured by RRC signaling.

Proposal 12: The HARQ-ACK feedback combination is determined based on Type-2 HARQ-ACK codebook.

Proposal 13: The PUCCH resource for transmission is determined according to the HARQ-ACK feedback combination and the PUCCH mapping table.

Proposal 14: The PUCCH mapping table is defined based on the number of TBs and the PUCCH format.

Proposal 15: The PUCCH mapping table is applied to both single G-RNTI case and multiple G-RNTI case.

# References

Chairman’s notes on RAN1#108-e meeting

RP-201038, “New Work Item on NR support of Multicast and Broadcast Services”, Huawei

R1-2203670, “On group scheduling for RRC-CONNECTED UEs”, Lenovo, Motorola Mobility

R1-2202883, “FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs”, Moderator (Huawei)