3GPP TSG RAN WG1 #109-e                                                                                   R1-2203703

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1.1

Source:	Lenovo

Title:	Channel access mechanism for sidelink on FR1 unlicensed spectrum

Document for:	Discussion

# Introduction

The Rel-18 NR Sidelink evolution WID was agreed upon during the RAN#94-e meeting [1], where one of the objectives is targeted to study the support of sidelink transmission on unlicensed spectrum. The detailed objectives in the WID are listed below:

- Study the support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Evaluation methodology for sidelink operation on unlicensed spectrum - Sidelink channel access mechanism for unlicensed spectrum based on regional regulation requirement and use the existing channel success schemes from NR-U as a starting point - Reuse Rel-16 resource allocation mechanism as much as possible - Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - No specific optimizations for existing NR SL feature - Frequency bands for the unlicensed spectrum in FR1 are 5GHz and 6GHz - No specific optimizations for FR2 unlicensed spectrum

For unlicensed spectrum, based on the learning from Rel-16 NR-U, the main target is to study the channel access mechanism in order to not only meet the regulation requirements but also guarantee the fair coexistence on unlicensed spectrum. In this contribution, we present our views on channel access mechanism and procedure for sidelink transmission on FR1 unlicensed spectrum.

# Discussion

## Frame based equipment channel access mechanism

Wireless signal transmission on unlicensed spectrum should meet the requirements of regulation subject to the management of the located country/region. According to the regulation on Frame based equipment (FBE) channel access mechanism, a device where the transmit/receive structure has a periodic timing with a declared periodicity equal to the Fixed Frame Period (FFP) between 1 and 10ms, and a single CCA slot, i.e., one-shot CCA with at least 9μs sensing interval. Transmissions can start only at the beginning of the FFP immediately following a successful one-shot CCA. If the FBE device finds the channel occupied, no transmission occurs on that channel during the following FFP. The channel occupancy time (COT) cannot be greater than 95% of the FFP and has to be followed by an Idle Period until the start of the next FFP such that the Idle Period is at least 5% of the COT, with a minimum of 100μs. An FBE structure with a fixed frame period is shown in Figure 1. Additionally, within each COT, the regulation allows multiple DL-to-UL transmission switching subject to a one-shot CCA. There is no requirement of gap between transmission bursts. UEs served by the gNB that wins the contention can transmit within the COT, where the winning gNB is the initiating device and the served UEs are responding devices in FBE. An FBE initiating device may change its FFP at most every 200ms.

Figure 1: Fixed Frame Period Structure for FBE

In Rel-16 NR-U, FBE based channel access is specified and named as semi-static channel acess. Even in Rel-17 URLLC, FBE is supported for IIoT use cases where the operator (the factory) may have the capability to clear the environment to make sure there is no other RAT deployed in the same carrier.

When unlicensed spectrum is used for sidelink transmission, FBE should be considered as one channel access mechanism for sidelink transmission so as to exploit the benefit of avoidance of inter-UE blocking.

Based on above analysis, we have below proposals:

Proposal 1: FBE-based channel access is supported for sidelink transmissions in FR1 unlicensed spectrum.

## Load based equipment channel access mechanism

Compared with FBE, load-based equipment (LBE) channel access procedure is more complicated, such as generation of random backoff counter, contention window size update, channel access priority class and maximum channel occupancy. In Rel-16 NR-U, several types of channel access procedures are specified: Type-1, Type-2A/2B/2C. Different channel access types can be applied for different cases. The detailed description is listed in TS37.213 and not repeated here.

In TS37.213, there are two CAPC tables defined separately for DL and UL with differences on the values of , , , and allowed sizes. When gNB intends to initiate a COT for DL transmission, it shall determine a CAPC value according to Table 4.1.1-1 before performing Type-1 channel access procedure; correspondingly, when a UE intends to initiate a COT for UL transmission, it shall determine a CAPC value according to Table 4.2.1-1 before performing Type-1 channel access procedure.

Table 4.1.1-1: Channel Access Priority Class

| Channel Access Priority Class () |  |  |  |  | allowed sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

Table 4.2.1-1: Channel Access Priority Class

| Channel Access Priority Class () |  |  |  |  | allowed sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For , =10ms if the higher layer parameter 'absenceOfAnyOtherTechnology-r14' indicates TRUE, otherwise, =6ms. NOTE 2: When =6ms it may be increased to 8 ms by inserting one or more gaps. The minimum duration of a gap shall be 100 µs. The maximum duration before including any such gap shall be 6 ms. |  |  |  |  |  |

When unlicensed spectrum is used for sidelink transmission, it is straightforward to reuse both Type-1 and Type 2A/2B/2C channel access types for sidelink transmissions. For a UE intending to initiate a COT for sidelink transmission over unlicensed spectrum, a CAPC should be determined before performing Type-1 channel access procedure. Since there are two tables specified in TS37.213 as shown above, Table 4.2.1-1 defined for UL transmission can be reused for sidelink transmission to determine a CAPC.

Based on above analysis, we have below proposals:

Proposal 2: Table 4.2.1-1 in TS37.213 is reused for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 3: Both Type-1 and Type 2A/2B/2C are reused for sidelink transmissions in FR1 unlicensed spectrum.

## Baseline LBT and deployment scenarios

In Rel-16 NR-U, LBT sub-band size defines the LBT bandwidth e.g., 20 MHz to be performed by the device in an unlicensed carrier and many LBT sub-bands could be configured inside an unlicensed carrier. The transmission and reception in sidelink are performed within a resource pool which is configured inside a bandwidth part. Hence the relationship between the LBT sub-band and resource pool which includes how the LBT sub-band is configured. The LBT sub-band could be configured as part of the resource pool configuration if the LBT sub-band and the resource pool size are same, however the LBT sub-band may span across more than one resource pool if the resource pool size is smaller than the LBT sub-band size.

The slots that are part of a resource pool can be (pre-)configured with a bitmap and the length of the bitmap can be equal to 10, 11, …, 160 and usually there are more than one resource pool configured inside a bandwidth part. The channel occupancy duration may occupy more than one resource pool in time domain based on the sidelink slots configuration for a resource pool which is quite different from the Rel16 NR-U configuration.

Sidelink device accessing the channel on which sidelink transmission is to be performed should set the energy detection threshold less than or equal to the maximum energy detection threshold defined. Such energy detection threshold adaptation procedure for sidelink should reuse the procedure defined in uplink.

Proposal 4: RAN1 support the LBT sub-band size i.e., LBT bandwidth configuration of 20 MHz, LBT energy detection threshold for a sidelink device operating in an unlicensed carrier

NR Sidelink unlicensed operation may coexist with other Wifi, LTE and NR unlicensed technologies and following deployment scenarios could be considered:

- Unlicensed NR-U carrier scheduling data transmission on SL-unlicensed carrier

- Licensed NR carrier scheduling data transmission on SL-unlicensed carrier

- LTE licensed carrier scheduling data transmission on SL-unlicensed carrier

- Standalone SL-unlicensed carrier i.e., out of coverage

Among the listed deployment scenarios, the new WID on sidelink unlicensed prioritizes the study on the Licensed carrier scheduling data transmission on SL-unlicensed carrier and standalone SL-unlicensed carrier i.e., out of coverage deployment.  LTE-eNB scheduling NR Sidelink was supported in NR Rel16 and this support should be extended for the unlicensed sidelink carrier.

Proposal 5: RAN1 could prioritize the study of mode 1 -- NR licensed carrier (gNB) scheduling data transmission on SL-unlicensed carrier and LTE licensed carrier (eNB) scheduling data transmission on SL-unlicensed carrier

## Burst-based sidelink transmissions

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. Different to SCI decoding-based sensing mechanism, LBT procedure is performed based on energy detection in each sensing slot. In detail, if the detected energy on one channel in one sensing slot is lower than the energy detection threshold, then the channel is deemed as empty or clear or available in that sensing slot; otherwise, the channel is deemed as occupied or non-available in that sensing slot.

For unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT). According to Table 4.2.1-1 in TS37.213, the MCOT is related to channel access priority class (CAPC) value. Consequently, when a UE performs a successful channel access Type 1, the UE can occupy the channel with maximum 2ms, 4ms, 6ms or 10ms, depending on the corresponding CAPC value which is determined based on the traffic priority. The higher the CAPC value, the longer the MCOT on the unlicensed spectrum. As shown in Table 4.2.1-1 in TS37.213, for CAPC value of 3 or 4, the MCOT can be 10ms, which is equivalent to 10 slots in case of 15kHz subcarrier spacing, 20 slots in case of 30kHz subcarrier spacing, 40 slots in case of 60kHz subcarrier spacing.

To avoid any risk of losing the channel, similar to DL burst and UL burst in TS37.213, a sidelink burst can be defined as a set of transmissions from a UE without any gaps greater than 16us. Multiple consecutive slots can be included in the sidelink burst. Each PSCCH/PSSCH starts from symbol 0 of a slot to avoid complicated designs at both Tx UE side and Rx UE side. One example is shown in Figure 2, where a UE initiates a COT via LBT Cat.4 using CAPC 2, then it can contiguously transmit 4 PSCCH/PSSCHs in the four consecutive slots without sensing. So the sidelink burst can include the four slots from symbol 0 of slot n to symbol 9 of slot n+3.

Figure 2: Burst based transmission for sidelink

Proposal 6: Burst-based sidelink transmission is defined for sidelink transmissions in FR1 unlicensed spectrum.

## Channel access types for sidelink transmissions

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. It does make sense that a Tx UE initiated COT can be shared to one or more Rx UEs when the requirements of COT sharing is satisfied. One example is shown in Figure 4. After initiating a COT by performing Type 1 channel access procedure, Tx UE, named UE1 for simplicity, performs contiguous sidelink transmissions without any gap larger than 16us in time domain, and then can transmit COT sharing information to share some slots within the COT to Rx UE (e.g., UE2) for transmitting PSCCH and associated PSSCH. Upon reception of the COT sharing information from UE1, UE2 can perform Type 2A or Type 2B channel access procedure and transmit PSCCH/PSSCH within the COT if the Type 2A/2B channel access procedure is successful.

Furthermore, if HARQ-ACK feedback is enabled, then PSFCH transmission can be contained within the COT so that an Rx UE can perform Type 2A or Type 2B channel access procedure and transmit PSFCH within the COT if the Type 2A/2B channel access procedure is successful. To leave the gap for Tx/Rx switching and performing Type 2A/2B channel access, one symbol can be reserved as gap before each PSFCH transmission.

Figure 3: COT sharing for PSCCH/PSSCH and PSFCH transmissions

On the other hand, considering UE processing capability, it may be not possible for a Rx UE to decode a PSSCH and transmit the corresponding PSFCH within the COT. One example is shown in Figure 4, where CAPAC value of 2 is used for initiating the COT so the MCOT can have 4 slots. Two slots are assumed as the processing delay for PSFCH transmission. When HARQ-ACK feedback is enabled, then PSFCHs corresponding to PSSCHs in slot n and slot n+1 can be transmitted within the COT so that an Rx UE can perform Type 2A or Type 2B channel access procedure for transmitting the PSFCHs within the COT. However, for PSSCHs in both slot n+2 and slot n+3, due to the processing delay, the Rx UE can’t transmit PSFCHs for both PSSCHs within the COT and has to perform Type-1 channel access procedure for transmitting the PSFCHs outside of current COT. For quickly accessing the unlicensed channel, Type-1 channel access procedure with CAPC value of 1 can be adopted for transmitting a PSFCH outside of COT.

Figure 4: PSFCH transmissions inside/outside of COT

Based on above analysis, we have below proposals:

Proposal 7: A COT initiated by a Tx UE via Type 1 channel access can be shared to Rx UEs for transmitting PSFCH(s) inside of the COT.

Proposal 8: A COT initiated by a Tx UE via Type 1 channel access can be shared to other UEs for transmitting PSCCH/PSSCH(s) inside of the COT subject to regulation requirements on COT sharing.

Proposal 9: Type 1 channel access with lowest CAPC value can be performed for transmitting a PSFCH outside of a COT where the corresponding PSSCH is transmitted.

## UE-to-UE COT sharing operation

Up to and including NR Release 17, NR-U has provided mechanisms to operate NR-RAT in unlicensed spectrum to share a channel occupancy time (COT) with another device. The benefit of such COT sharing is that a device initiating a COT can grant another device the right to use the initiated COT without requiring the target device to initiate its own COT. Initiating a COT usually involves a certain sensing time before transmissions are allowed, while starting transmission in a shared COT requires only a fraction of the sensing time required for initiating a COT.

A COT is characterized by one or more of the following properties:

- COT Initiator: The node initiating a COT, e.g. following a channel access procedure as specififed in 3GPP TS 37.213

- Channel Access Priority Class: One or more classes that imply e.g. required sensing duration when initiating a COT, MCOT, e.g. as defined in 3GPP TS 37.213.

- MCOT: The maximum COT duration of a channel occupancy, counted or measured from the first transmission after initiation of the COT.

In the traditional cellular topology with a centralized scheduler, usually a gNB initiates a COT and may share this COT with one or more UEs; or a UE initiates a COT and could share this COT with a gNB. The sharing of a COT is indicated by a specific field in downlink control information or uplink control information. The COT sharing indicator may include means to indicate the maximum remaining duration of the COT being shared.

The following definitions are used for the purpose of sidelink channel access mechanisms:

Tx UE: A UE that transmits a COT sharing indicator via a sidelink connection

Rx UE: A UE that receives a COT sharing indicator via a sidelink connection

COT Initiator: A sidelink device that initiated a channel occupancy e.g., Tx UE

COT Donor: A sidelink device that transmits a COT sharing indicator, e.g. a Tx UE. The COT Donor may be identical to the COT Initiator

COT Recipient: A sidelink device that receives a COT sharing indicator, e.g. a Rx UE

The UE-to-UE COT sharing principles should take into consideration that the COT initiator which is the Tx UE may have connection with multiple Rx UEs or destinations as shown in Figure 1. The COT initiator or the Tx UE could share its initiated COT with multiple Rx UEs or destinations. The COT recipient acting as Rx UEs after receiving the COT sharing indicator could transmit data or feedback in the remaining channel duration.

Figure 5: UE to UE COT sharing

The UE-to-UE COT sharing operation requires new SCI format to incorporate COT sharing indicator similar to DCI format 2_0 and further discussions are necessary how to transmit the COT sharing indicator to multiple Rx UEs or destination ids. The COT sharing indictor should include the remaining duration in terms of number of physical slots, channel access priority value, destination id of the COT recipient. If there are more than one COT recipients, then RAN1 should discuss how to signal more than one destination ids along with the COT sharing indicator.

Proposal 10: Support UE to UE COT sharing

Proposal 11: Study mechanism to indicate UE to UE COT sharing via sidelink to one or multiple UEs or destinations

COT initiator which is the Tx UE should have the flexibility to transmit unicast, groupcast or broadcast data in the initiated COT and if there are no more data to be transmitted within the COT may share the initiated COT to multiple Rx UEs or destination ids. COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data and hence how to define the cast type of the SCI format carrying the COT sharing indicator should be discussed in RAN1.

Proposal 12: COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data and hence RAN1 should need to discuss mechanism to define the cast type of the SCI information carrying the COT sharing indicator

COT recipient could perform PSCCH/PSSCH, PSFCH transmission in the shared COT to any UEs or destination ids unless the one of the transmissions is made to UE or source-destination id that provided the COT sharing indicator in that shared COT

Proposal 13: COT recipient could use the shared COT to make PSSCH, PSFCH transmissions to any UEs or destination ids with a restriction that atleast one transmissions is to be made to the UE or source-destination id that provided the COT sharing indicator

In a sidelink environment, a UE may have sidelink connections to and from multiple UEs. Consequently, a UE may receive COT sharing indicators from many COT donor UEs. Since each of the indicated shared COTs results from a different donor device that may have e.g. remaining COT durations, priority class. COT recipient needs to select which of the indicated shared COTs to use and to abide by the rules pertaining to the selected COT.

Proposal 14: RAN1 needs to study mechanism for COT recipient to select one COT sharing indicator/COT donor

## Short control signalling exemption

The short control signalling exemption is defined in NR-U to exempt transmission by certain physical channel or reference signal from performing any LBT procedures. However, there are rules defined that any such transmission performed under the short control signalling exemption should not exceed 5ms duration in a 100ms window for 5GHz carrier. RAN1 should discuss the physical channels under this rule such as SL-SSB transmission by the Sync Ref UE, PSFCH transmission etc.,

Proposal 15: RAN1 support short control signalling exemption in sidelink unlicensed operation  for S-SSB, PSFCH

## Contention window size update

The type-1 channel access describing contention window size (CWS) adaptation procedure described in 3GPP TS 37.213 for NR-U specifically designed for unicast PDSCH, PUSCH transmission. The NR-U contention window size adaptation procedure considers amount of HARQ feedback reception from TB, CBG based transmission within the reference duration. However, such CWS adjustment procedure should be studied for sidelink channel access mechanism considering unicast, groupcast and broadcast type of PSCCH/PSSCH transmission, transmission in a reference duration to multiple Rx UEs/destination ids and different HARQ feedback types especially for groupcast type e.g., Groupcast HARQ feedback option 1 (common NACK), Groupcast HARQ feedback option 2(dedicated ACK/NACK) defined in NR Rel16 Sidelink.

Proposal 16: RAN1 to study on the contention window size update procedure considering different cast type, transmission/reception from multiple Rx UEs or destinations and different HARQ feedback types

## One shot SL HARQ feedback

In Rel-16, as part of the NR-U feature one shot HARQ feedback report transmission with UE transmitting a snapshot of HARQ-ACK report of all HARQ process using PUCCH or PUSCH when a UE receives DCI format 1_1 contain one bit to request the transmission of one shot HARQ feedback.

However, in sidelink the one shot HARQ feedback reporting might need to be associated to a destination, cast types and moreover there is only a limited number of HARQ process between a source-destination pair of UEs as shown in the below Figure 2. Hence the procedure of transmission of SL one shot HARQ feedback should consider different source-destination ids, cast types such as unicast, groupcast, broadcast, HARQ feedback options and blind retransmissions etc., Also reporting one shot HARQ feedback report from Tx UE to gNB for all SL HARQ process should be studied which could be useful when due to LBT failures there could be delay in sidelink HARQ feedback reporting to gNB.

Figure 6: Showing active HARQ processes between UE1, UE2 and UE3

The sidelink feedback transmission/reception by a UE could be affected due to LBT failures and there exists NR-U features such as one-shot HARQ feedback, non-numerical HARQ feedback timing indicator to counter this problem. Such feature could be studied and requires adaptation for sidelink unlicensed operation.

Proposal 17: RAN1 to study the benefit of introducing the one-shot HARQ feedback, non-numerical HARQ feedback timing indicator features for sidelink unlicensed operation

## Non-numerical feedback timing indicator

Non-numerical value as feedback timing was introduced which eventually postponed the transmission of the generated HARQ feedback while a subsequentHARQ HA transmission of numerical value as part of the DL DCI triggers the transmission of the previously generated HARQ feedback for those HARQ processes that was transmitted using non-numerical value indication. The Rel16 sidelink HARQ feedback reporting may get delayed due to LBT failures and channel occupancy limitation. Such delay in the sidelink HARQ feedback reception at the Tx UE may also affect the reporting of SL HARQ feedback to gNB.

Currently the feedback timing for sidelink transmission is (pre)configured in a resource pool and not dynamically signalled and there should be some signalling enhancement needed to dynamically indicate the non-numerical feedback timing value to the Rx UEs or destinations to delay the transmission of SL HARQ feedback reporting. Further a trigger needs to be subsequently transmitted to request the transmission of SL HARQ feedback of those SL HARQ process previously indicated with non-numerical HARQ feedback timing value.  Such mechanism may also beneficial for gNB to delay the HARQ feedback transmission from a Tx UE using the non-numerical HARQ feedback timing value in DCI format 3_0.

Proposal 18: RAN1 to study the benefit of delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for an unlicensed spectrum

## Other PSFCH enhancements

In many geographical and/or frequency regions, a device is required to pass a clear channel assessment (CCA) procedure, such as LBT, before being allowed to transmit on the unlicensed carrier (shared spectrum). In case the CCA fails for a given transmission instance, the device is not allowed to transmit. Consequently this may lead in not being able to transmit a PSFCH in a specific PSFCH period. This leads to potentially unnecessary retransmissions of packets, as the original data transmitter expecting ACK/NACK feedback on PSFCH may trigger a retransmission in case of missing ACK/NACK feedback.

An additional problem occurs in option 1 (where a UE reports feedback on the PSFCH for the data only if the feedback is NACK) for groupcast transmissions, where a missing feedback due to a failed CCA may be interpreted as an ACK by the original data transmitter, even though the feedback transmitter may have intended to convey a NACK to request a retransmission; in this case, the original data transmitter may be unaware that a retransmission of the data is necessary, leading to undetected data loss.

Up to Release 17, a PSFCH occasion is always periodic and occurs in the last slot of a PSFCH period, i.e., there is one occasion per PSFCH period, N = {1, 2, 4} configured in a resource pool.  This is sufficient for a licensed carrier scenario especially in a scheduled mode but imposes severe limitations in case sidelink transmission occur on a sidelink unlicensed carrier.

Proposal 19: RAN1 could further study the PSFCH enhancement to mitigate problems arising due to delayed sidelink HARQ feedback reception for an unlicensed spectrum

# Conclusion

In this contribution, we focus on the channel access mechanism and procedure for sidelink transmission on FR1 unlicensed spectrum and have below observations and proposals:

Proposal 1: FBE-based channel access is supported for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 2: Table 4.2.1-1 in TS37.213 is reused for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 3: Both Type-1 and Type 2A/2B/2C are reused for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 4: RAN1 support the LBT sub-band size i.e., LBT bandwidth configuration of 20 MHz, LBT energy detection threshold for a sidelink device operating in an unlicensed carrier

Proposal 5: RAN1 could prioritize the study of mode 1 -- NR licensed carrier (gNB) scheduling data transmission on SL-unlicensed carrier and LTE licensed carrier (eNB) scheduling data transmission on SL-unlicensed carrier

Proposal 6: Burst-based sidelink transmission is defined for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 7: A COT initiated by a Tx UE via Type 1 channel access can be shared to Rx UEs for transmitting PSFCH(s) inside of the COT.

Proposal 8: A COT initiated by a Tx UE via Type 1 channel access can be shared to other UEs for transmitting PSCCH/PSSCH(s) inside of the COT subject to regulation requirements on COT sharing.

Proposal 9: Type 1 channel access with lowest CAPC value can be performed for transmitting a PSFCH outside of a COT where the corresponding PSSCH is transmitted.

Proposal 10: Support UE to UE COT sharing

Proposal 11: Study mechanism to indicate UE to UE COT sharing via sidelink to one or multiple UEs or destinations

Proposal 12: COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data and hence RAN1 should need to discuss mechanism to define the cast type of the SCI information carrying the COT sharing indicator

Proposal 13: COT recipient could use the shared COT to make PSSCH, PSFCH transmissions to any UEs or destination ids with a restriction that atleast one transmissions is to be made to the UE or source-destination id that provided the COT sharing indicator

Proposal 14: RAN1 needs to study mechanism for COT recipient to select one COT sharing indicator/COT donor

Proposal 15: RAN1 support short control signalling exemption in sidelink unlicensed operation  for S-SSB, PSFCH

Proposal 16: RAN1 to study on the contention window size update procedure considering different cast type, transmission/reception from multiple Rx UEs or destinations and different HARQ feedback types

Proposal 17: RAN1 to study the benefit of introducing the one-shot HARQ feedback, non-numerical HARQ feedback timing indicator features for sidelink unlicensed operation

Proposal 18: RAN1 to study the benefit of delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for an unlicensed spectrum

Proposal 19: RAN1 could further study the PSFCH enhancement to mitigate problems arising due to delayed sidelink HARQ feedback reception for an unlicensed spectrum

# References

RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.