3GPP TSG RAN WG1 #109-e                                                                                   R1-2203704

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1.2

Source:	Lenovo

Title:	Physical layer design framework for sidelink on FR1 unlicensed spectrum

Document for:	Discussion

# Introduction

The Rel-18 NR Sidelink evolution WID was agreed upon during the RAN#94-e meeting [1], where one of the objectives is targeted to study the support of sidelink transmission on unlicensed spectrum. The detailed objectives in the WID are listed below:

- Study the support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Evaluation methodology for sidelink operation on unlicensed spectrum - Sidelink channel access mechanism for unlicensed spectrum based on regional regulation requirement and use the existing channel success schemes from NR-U as a starting point - Reuse Rel-16 resource allocation mechanism as much as possible - Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - No specific optimizations for existing NR SL feature - Frequency bands for the unlicensed spectrum in FR1 are 5GHz and 6GHz - No specific optimizations for FR2 unlicensed spectrum

For unlicensed spectrum, based on the learning from Rel-16 NR-U, the main target is to design physical layer channels and signals to meet the regulation requirements. In this contribution, we present our views on sidelink physical layer structure and procedure design so as to not only meet regulation requirements but also improve spectrum efficiency for unlicensed spectrum.

# Discussion

## Interlace-based structure

Wireless signal transmission on unlicensed spectrum should meet the requirements of regulation subject to the management of the located country/region. Hence, the design of uplink waveform for NR-U PUSCH/PUCCH should meet the regulation requirements on unlicensed spectrum.

The requirements mainly include two aspects:

- Occupied channel bandwidth (OCB): the bandwidth containing 99% of the power of the signal, shall be between 80% and 100% of declared Nominal Channel Bandwidth;

- Maximum PSD (power spectrum density) with a resolution bandwidth of 1MHz, e.g., 10dBm/MHz.

These two requirements dictate that a signal which occupies a small portion of the channel bandwidth cannot be transmitted at the maximum available power at the UE due to the PSD and OCB constraints.

In Rel-16 NR over unlicensed spectrum (NR-U), interlace-based waveform is adopted as uplink waveform for unlicensed spectrum. Each interlace is equally distributed within the whole carrier bandwidth and spans more than 80% carrier bandwidth. The regulation requirements of OCB can be met if a signal or channel can occupy at least one interlace. Moreover, two adjacent PRBs of one interlace are separated by above 1 MHz distance and power boosting can be adopted for each PRB of one interlace. E.g., for 20MHz bandwidth and 15kHz SCS, all the available 106 PRBs are partitioned into 10 interlaces. Each interlace has 11 or 10 PRBs and is equally distributed within the 20MHz bandwidth. In this way, each interlace spans more than 80% system bandwidth so that the regulation requirements of OCB can be met. Moreover, 10 PRBs of one interlace are equally spaced in frequency so that two adjacent PRBs of one interlace are separated by 1.8MHz distance. Hence, one PRB within one interlace can be transmitted with 10dBm maximum power.

Figure 1: Interlace-based structure for 15kHz SCS

Different to Rel-16 NR-U where the minimum resource allocation unit is a interlace, a concept of sub-channel is defined in existing NR sidelink as the minimum resource allocation unit, where a subchannel comprises of multiple contiguous PRBs. Sidelink transmission/reception happens in a resource pool which can be configured with ‘M’ sub-channels with each subchannel including ‘N’ contiguous PRBs. The possible sub-channel size can be configured within the set of {n10, n12, n15, n20, n25, n50, n75, n100}. A resource pool for sidelink does not span across entire sidelink carrier or LBT sub-band. Furthermore, each sub-channel is a self-contained structure containing PSCCH, PSSCH and PSFCH resources.

Therefore, there are two options to meet the regulatory requirements for sidelink transmission on unlicensed spectrum:

- Option 1: reuse Rel-16 NR-U design with PRB-based interlace structure for sidelink channels, e.g., PSCCH, PSSCH and PSFCH, to meet regulation requirements on OCB (at least 80% frequency spanning) and PSD (e.g., 10dBm/MHz).

- Option 2: A new interlacing structure based on sub-channel wise interlacing by considering the traditional sidelink design as much as possible i.e., PSCCH, PSSCH and PSFCH, as shown in below Table 1. For other subchannel size than 10, there are some left overs PRBs that might constitute a full subchannel. Depending on the numerology decision i.e.., SCS, sub-channel size this could be further studied and optimized.

Table 1: Sub-channel wise interlacing

| Subchannel size (PRBs) | No of sub-channels in a resource pool with 20MHz BW (100PRBs @15KHz SCS) | Interlacing structure examples considering 15kHz SCS | Percentage of occupied channel bandwidth per interlace |
| --- | --- | --- | --- |
| 10 | 10 | M= 2, N= 5 Interlace 0 SC #0, 2, 4, 6, 8 Interlace 1 SC #1, 3, 5, 7, 9 | 90% |

In addition, there are multiple numerologies defined for NR including 15, 30, 60 kHz for FR1. Rel-16 NR-U has agreed the number of interlaces is determined according to the subcarrier spacing, i.e., 10 interlaces with 10 or 11 PRBs per interlace for 15 kHz subcarrier spacing and 5 interlaces with 10 or 11 PRBs per interlace for 30kHz subcarrier spacing. For 60 kHz subcarrier spacing, it is not specified in Rel-16 NR-U for interlace-based structure.

Table 2: Max transmission bandwidth configuration NRB for FR1 (450 – 7125 MHz)

For the combination of 20MHz bandwidth and 60 kHz subcarrier spacing, candidate PRB-based interlace designs are listed in Table 2, wherein M is the number of interlaces and N is the number of PRBs per interlace. Generally, the combination of 4 interlaces and 6 PRBs per interlace can’t meet the regulation requirement of at least 80% occupancy ratio. The combination of 3 interlaces and 8 PRBs per interlace can nearly meet the occupancy ratio requirement. Instead, the combination of 2 interlaces and 12 PRBs per interlace can obviously meet the requirement. The drawback is the frequency separation is 1.44MHz between two adjacent RBs of one interlace so the power boosting target for 10dBm per PRB can’t be met due to the regulation requirements of PSD requirement of 10dBm/MHz. However, the two interlaces provides some scheduling flexibility on two-UE multiplexing.

Table 2: Candidate interlace structure for 20MHz bandwidth and 60 kHz SCS

| SCS | Candidate design | Occupied channel bandwidth per interlace | Percentage of occupied channel bandwidth per interlace over declared channel bandwidth |
| --- | --- | --- | --- |
| 60 kHz (total 24 PRBs) | M = 4, N = 6 | 15.12MHz (21 PRBs) | 75.6% |
|  | M = 3, N = 8 | 15.84MHz (22 PRBs) | 79.2% |
|  | M = 2, N = 12 | 16.56MHz (23 PRBs) | 82.8% |

Hence, for FR1 unlicensed spectrum, all the subcarrier spacings can be supported, i.e., 10 interlaces for 15kHz subcarrier spacing, 5 interlaces for 30kHz subcarrier spacing and 2 interlaces for 60kHz subcarrier spacing.

Based on above analysis, we have below proposals:

Proposal 1: Interlace-based structure is supported for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 2: Subcarrier spacing of 60kHz is supported for sidelink transmission in FR1 unlicensed spectrum in addition to 15kHz and 30kHz.

Proposal 3: RAN1 should discuss new design for interlacing structure for sidelink unlicensed operation by considering the design of sub-channels and resource pool

## Frame structure

When unlicensed spectrum is used for sidelink transmission, needless to say, the channel access procedure, also named LBT, is required before any sidelink transmission. Different to SCI decoding-based sensing mechanism, LBT procedure is performed based on energy detection in each sensing slot. In detail, if the detected energy on one channel in one sensing slot is lower than the energy detection threshold, then the channel is deemed as empty or clear or available in that sensing slot; otherwise, the channel is deemed as occupied or non-available in that sensing slot.

For unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT). According to Table 4.2.1-1 in TS37.213, the MCOT is related to channel access priority class (CAPC) value. Consequently, when a UE performs a successful channel access Type 1, the UE can occupy the channel with maximum 2ms, 4ms, 6ms or 10ms, depending on the corresponding CAPC value which is determined based on the traffic priority. The higher the CAPC value, the longer the MCOT on the unlicensed spectrum. As shown in Table 4.2.1-1 in TS37.213, for CAPC value of 3 or 4, the MCOT can be 10ms, which is equivalent to 10 slots in case of 15kHz subcarrier spacing, 20 slots in case of 30kHz subcarrier spacing, 40 slots in case of 60kHz subcarrier spacing.

To avoid any risk of losing the channel, similar to DL burst and UL burst in TS37.213, a sidelink burst can be defined as a set of transmissions from a UE without any gaps greater than 16us. Multiple consecutive slots can be included in the sidelink burst. Each PSCCH/PSSCH starts from symbol 0 of a slot to avoid complicated designs at both Tx UE side and Rx UE side. One example is shown in Figure 2, where a UE initiates a COT via LBT Cat.4 using CAPC 2, then it can contiguously transmit 4 PSCCH/PSSCHs in the four consecutive slots without sensing. So the sidelink burst can include the four slots from symbol 0 of slot n to symbol 9 of slot n+3.

Figure 2: Burst based transmission for sidelink

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. It does make sense that a Tx UE initiated COT can be shared to one or more Rx UEs when the requirements of COT sharing is satisfied. One example is shown in Figure 3. After initiating a COT by performing Type 1 channel access procedure, Tx UE, named UE1 for simplicity, performs contiguous sidelink transmissions without any gap larger than 16us in time domain, and then can share some slots within the COT to Rx UE (e.g., UE2) for transmitting PSCCH and associated PSSCH. There are one or more symbols which may be used as gap between two consecutive PSSCHs. If HARQ-ACK feedback is enabled, then PSFCH transmission occasions should be contained within the COT with at least one symbol as gap before or after each PSFCH transmission. Therefore, the structure of the COT is needed for a Rx UE which intends to share the COT for transmitting PSCCH/PSSCH and/or PSFCH.

Figure 3: UE-to-UE COT sharing for sidelink transmission

Based on above analysis, we have below proposals:

Proposal 4: Burst-based sidelink transmission is defined for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 5: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot.

Proposal 6: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

## HARQ

For sidelink transmission on unlicensed spectrum, if channel access procedure is successful, a Tx UE transmits a PSSCH to a Rx UE and waits for the reception of PSFCH from the Rx UE. Without such HARQ-ACK feedback, the Tx UE has to retransmit the PSSCH even though the Rx UE has correctly decoded the PSSCH. There are two risks that the Tx UE may not receive the PSFCH from the Rx UE:

- LBT risk: Due to unpredicted LBT outcome at Rx UE side, the sidelink HARQ-ACK feedback may not be transmitted if LBT at Rx UE side is failed.

- Hidden node interference: If LBT at Rx UE side is successful, the Rx UE transmits PSFCH to Tx UE. Due to hidden node interference on the shared unlicensed spectrum, the Tx UE may not correctly decode the PSFCH.

Therefore, when one sidelink HARQ-ACK feedback is to be transmitted on an unlicensed carrier, the sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities (TOs) and transmitted in the first transmission opportunity with LBT success. One example is shown in Figure 4, where one PSSCH transmitted in slot 1 can have 4 transmission opportunities in slot 4, 8, 12 and 16 with PSFCH periodicity of 4 slots.

Figure 4: Multiple transmission opportunities for one sidelink HARQ-ACK feedback

Proposal 7: sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities in FR1 unlicensed spectrum.

## Resource pool structure

In Rel-16/17 NR sidelink communication, higher layer signaling configuration, synchronization procedure and physical channel structure are designed and specified based on resource pool. The data transmission/reception and resource allocation for both mode 1 and mode 2 are assigned within a resource pool. So, to consider less standardization workload, resource pool relevant design can be reused for sidelink on FR1 unlicensed spectrum.

Proposal 8: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum

## Physical channel design

In Rel-16/17 NR sidelink communication, potential PSCCH is assigned in each subchannel. When scheduling multiple subchannels, PSCCH is allocated in the lowest subchannel in frequency domain. The size of PSCCH in frequency domain can be {10, 12, 15, 20, 25} RBs. In time domain, the PSCCH occupies 2 to 3 symbols.  For Interlace-based structure, potential PSCCH can be still designed to assign in each interlace and to occupy 2 to 3 symbols.

Proposal 9: To study the location of PSCCH in frequency/time domain for interlace-based structure.

In Rel-16/17 NR sidelink communication, to consider the difference usage scenario, the number of columns of DMRS can be various, i.e., 2 ,3 or 4 columns in one time slot. The time domain location of DMRS can be reused. The frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 10: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

In Rel-16/17 NR sidelink communication, unicast, groupcast opiton1/2 and broadcast were supported. For the sidelink communication on unlicensed band, each cast type should be supported.

Proposal 11: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum

For unicast and groupcast opiton1/2, the feedback information is transmitted on PSFCH channel. The PSFCH region is assigned the last second and third symbols at one time slot (the last symbol and the fourth symbol from last symbol are GP). This kind of design can be reused for unlicensed spectrum. In addition, each HARQ feedback should be transmitted on one interlace but not on RB on unlicensed band, the PSFCH resource may be insufficient. For example, if a UE performs a groupcast option 2 transmission on 20MHz bandwidth, the 20MHz bandwidth may include 10 interlaces in case of 15KHz SCS, that is only 10 PSFCH resources can be provided to maximum10 number UEs in this group. So besides legacy PSFCH resources, to study increasing the PSFCH resource for unlicensed spectrum.

Proposal 12: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum

## Resource allocation mode 1 and mode 2

### Mode 1

In mode 1, as mentioned in WID objective, Uu operation for mode 1 is limited to licensed spectrum only. In this way, legacy scheduling and feedback mechanism on Uu link for mode 1 can be reused. The difference between legacy resource allocation and resource allocation on unlicensed band is that the scheduled UE needs to perform a LBT operation before its scheduled sidelink transmission. The assistance information for LBT operation at UE side may be signaled from network and/or based on UE determination. The content of this assistance information should be studied. In addition, the LBT operation may be failure and the UE behavior and potential solution should be studied.

Proposal 13: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

The sidelink grants are allocated by gNB using DCI format 3_0 and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. DCI field could indicate the channel access priority value together with the sidelink grant and the behavior of Tx UE as a COT initiator could be different in the following ways:

Tx UE after receiving the sidelink grant could autonomously choose the channel access priority class value to perform Cat 4 LBT and accordingly initiate channel occupancy duration. In this method, the selection of channel access priority class value by the Tx UE could be based on the priority/latency of the data in the buffer which is in accordance with the existing LCP rule. In the second method, if the DCI indicates the channel access priority value, then the Tx UE could perform the Cat 4 LBT and accordingly initiate channel occupancy duration.

Proposal 14: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

The sidelink grants are allocated by gNB using DCI and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. When LBT fails on all the indicated resources more sidelink resources are required from gNB. How the Tx UE request more resource from gNB using the physical layer signalling should be discussed.

Proposal 15:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB

### Mode 2

In mode 2, UE may perform autonomous resource selection for SL transmission. The LBT failure issue is also existing. When the transmission is transmitted on reserved resource within same COT, e.g., for retransmission of the same TB, LBT type 2 is performed for the retransmission, the LBT failure probability may be reduced. But to consider the long period traffic, i.e., the reserved resource is out of an initial COT, the UE should re-initial a COT, the LBT failure probability may be increased. So, the assistance information of a UE performing LBT operation is also needed to be signaled on sidelink. The assistance information can be used by the UE who performs LBT operation, or it can be used by the UE who performs autonomous resource selection.

Proposal 16: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 17: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

## Multi-slot scheduling

A sidelink device is required to pass a clear channel assessment (CCA) procedure, such as Cat 4 LBT to initiate a channel occupancy in an unlicensed carrier (shared spectrum). The sidelink transmission are allowed to be performed within the maximum channel occupancy duration defined by the respective channel access priority class which is counted or measured from the first transmission after initiation of the COT. Consecutive transmission within he occupied channel duration without gaps or gap less than 25µs is beneficial so that initiator device does not need to perform Cat 2 LBT to reaffirm the channel. In the current sidelink physical layer design upto three non-consecutive sidelink resources could be allocated or reserved within a boundary of 32 slots using the sidelink grant provided by DCI format 3_0 to the Tx UE and PSCCH transmission to the Rx UE while the same upto three resources could be reserved using mode 2. The current resource scheduling mechanism based on configured grant, dynamic scheduling using mode 1 and candidate resource selection, reservation mechanism using mode 2 requires changes to perform consecutive transmission which could be based on multi-slot scheduling.

Proposal 18: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling

## S-SSB design

In Rel-16/17 NR sidelink communication, the sidelink synchronization information is carried in a S-SSB that consists of S-PSS and S-SSS and PSBCH. The S-SSB spans one slot in time and uses the same numerology as the one (pre-)configured in the SL BWP. The S-SSB spans 11 common RBs in frequency and the frequency location of an S-SSB is (pre-)configured within the SL BWP. Therefore, a UE does not need to perform blind detection in the frequency domain to find an S-SSB. The S-SSBs are transmitted periodically with a period of 160ms. Within each period, the distribution of S-SSB slots is constrained by two parameters: one is offset of the first S-SSB relative to the boundary of the period, the other is the interval between two adjacent S-SSB slots.

In unlicensed band, the challenges that arise in S-SSB design and transmission come from the following aspects. The first is related to the S-SSB design in meeting OCB requirement for large channel bandwidth in unlicensed band. The second is related to the transmission of S-SSB and LBT requirement. As a result, S-SSB need to be redesigned in the unlicensed band.

Proposal 19: Study S-SSB design and transmission in unlicensed band.

# Conclusion

In this contribution, we focus on the physical layer framework for sidelink transmission on FR1 unlicensed spectrum and have below observations and proposals:

Proposal 1: Interlace-based structure is supported for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 2: Subcarrier spacing of 60kHz is supported for sidelink transmission in FR1 unlicensed spectrum in addition to 15kHz and 30kHz.

Proposal 3: RAN1 should discuss new design for interlacing structure for sidelink unlicensed operation by considering the design of sub-channels and resource pool

Proposal 4: Burst-based sidelink transmission is defined for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 5: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot.

Proposal 6: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 7: sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities in FR1 unlicensed spectrum.

Proposal 8: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum

Proposal 9: To study the location of PSCCH in frequency/time domain for interlace-based structure.

Proposal 10: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 11: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum

Proposal 12: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum

Proposal 13: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

Proposal 14: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

Proposal 15:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB

Proposal 16: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 17: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

Proposal 18: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling

Proposal 19: Study S-SSB design and transmission in unlicensed band.

# References

- RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.