3GPP TSG RAN WG1 #109-e	R1-2203708

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.3

Source:		Fujitsu

Title:	Remaining issues of multi-PDSCH/PUSCH scheduling via a single DCI

Document for:	Discussion/Decision

- Introduction

In this contribution, we would like to present some discussions about some remaining issues of multi-PDSCH/PUSCH scheduling via a single DCI.

- Discussion

2.1 SPS/CG activation/retransmission based on DCI format 1_1 with CS-RNTI

In RAN1#107b-e meeting, the following agreement was made for SPS/CG activation/release.

Agreement A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

In RAN1#108-e meeting，the following agreement was made for SPS/CG retransmission.

Agreement - When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV. - When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

As per the agreements, when DCI format 1_1/0_1 is provided with a TDRA table in which one or more TDRA rows include multiple SLIVs, it could be used for SPS/CG activation/release and retransmission when indicating a TDRA row with only one SLIV.

For SPS activation/retransmission, in TS 38.214 h10 as below, the yellow states that pdsch-AggregationFactor in pdsch-config may apply to SPS activation/retransmission with DCI format 1_1, while the green says that pdsch-AggregationFactor in pdsch-config would not apply to DCI format 1_1 when DCI format 1_1 is provided with a TDRA table in which one or more TDRA rows include multiple SLIVs.

When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the same symbol allocation is applied across the pdsch-AggregationFactor, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, consecutive slots.

When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

For CG retransmission, there is a similar situation according to TS 38.214 h10 as below.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. - the number of slots used for TBS determination N is equal to 1. For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in clause 6.1.4.1 and transformPrecoder described in clause 6.1.3.

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17.

According to the above, when TimeDomainAllocationListForMultiPDSCH-r17/TimeDomainAllocationListForMultiPUSCH-r17 is configured, it is ambiguous whether pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config would be applicable for SPS/CG activation/retransmission with DCI format 1_1/0_1. In order to ensure common understanding between gNB and UEs, it is necessary to clarify it.

Proposal 1: In order to avoid ambiguity, clarify whether pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config would be applicable for SPS/CG activation/retransmission with DCI format 1_1/0_1 when TimeDomainAllocationListForMultiPDSCH-r17/TimeDomainAllocationListForMultiPUSCH-r17 is configured.

As aforementioned, DCI format 1_1/0_1 for SPS/CG activation/retransmission should indicate a TDRA row with only one SLIV, which is same as the legacy. For the same spirit, the legacy rule for applying pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config to SPS/CG activation/retransmission can be directly reused.

Proposal 2: pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config should be applicable for SPS/CG activation/retransmission with DCI format 1_1/0_1 when TimeDomainAllocationListForMultiPDSCH-r17/TimeDomainAllocationListForMultiPUSCH-r17 is configured.

2.2 Type-1 HARQ-ACK codebook with time domain bundling

Time domain bundling was agreed to be supported for Type-1 HARQ-ACK codebook. In RAN1#107b-e meeting, the following agreement was achieved.

Agreement in RAN1107bis-e: - Update the previous agreement made in RAN1#107-e, as follows: Agreement (RAN1#107-e) For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell. - If the RRC parameter enables time domain bundling operation, - To determine the set of candidate PDSCH reception occasions, - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16) - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index. - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case. - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

However, there are still many remaining issues to be discussed and clarified for the case of time domain bundling.

Issue 1: How to perform logical AND operation

According to the agreement, for a determined candidate PDSCH reception occasion, it is possible that two or more valid PDSCHs are associated with the occasion and the valid PDSCHs are scheduled by different DCIs. Meanwhile, the agreement also states that “Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion”. Therefore, per agreement, it means that logical AND operation is applied across all valid PDSCHs, regardless of how the valid PDSCHs are scheduled.

For example, for HARQ-ACK feedback in slot n, assume

- two TDRA rows: row 0 with SLIV 0_0, row 1 with SLIV 1_0, 1_1 and 1_2.

- two configured K1 values: K1= 1, 2.

As shown in Figure 1, corresponding to candidate PDSCH reception occasion 1, there is one PDCCH scheduling a valid PDSCH via indicating TDRA row 0, and one PDCCH scheduling a valid PDSCH via indicating TDRA row 1. In such case, the HARQ-ACK information for candidate PDSCH reception occasion 1 should be equal to logical AND operation applied across the two valid PDSCHs.

Figure 1. example of more than one valid PDSCH scheduled by more than one DCI for an occasion

However, in TS 38.213 h10, it is ambiguous whether the Logical AND operation is aligned with the agreement above.

Based on the agreement and relevant specification, we have the following observation and proposal.

Observation 1: According to the agreement made in RAN#107b-e meeting for type-1 HARQ-ACK codebook with time domain bundling, logical AND operation may be applied to PDSCHs scheduled by different DCIs (up to gNB’s scheduling). However, the current specification (TS 38.213 h10) seems not aligned with the agreement (the specification seems to limit logical AND operation to the case where the associated valid PDSCHs are scheduled by a single DCI).

Proposal 3: For Type-1 HARQ-ACK codebook with time domain bundling, clarify the following in TS 38.213:

- Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by one or more DCIs.

Issue 2: How to support SPS PDSCH

According to discussion so far, when time domain bundling is supported, it is possible that for a determined candidate PDSCH reception occasion, the associated PDSCHs include both PDSCH scheduled by DCI and PDSCH scheduled by SPS (SPS PDSCH). For example, as shown in Figure 2, for occasion 1, one associated valid PDSCH is scheduled by a DCI, and one associated valid PDSCH is scheduled by SPS.

Figure 2. example of one valid PDSCH scheduled by one DCI and one valid PDSCH scheduled by SPS for an occasion

However, so far, there is still no discussion about how to handle the case where the PDSCHs associated with a determined candidate PDSCH reception occasion include both PDSCH scheduled by DCI and PDSCH scheduled by SPS (SPS PDSCH).

In our view, for such case, the following options can be considered.

- Option 1: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by DCI and SPS.

- Option 2: For a determined candidate PDSCH reception occasion, if UE receives a DCI which schedule a PDSCH associated with the occasion, UE will not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH.

Observation 2: For Type-1 HARQ-ACK codebook with time domain bundling, it has not been discussed how to support SPS PDSCH.

Proposal 4: For Type-1 HARQ-ACK codebook with time domain bundling, discuss how to support SPS PDSCH and consider the following two options.

- Option 1: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by DCI and SPS.

- Option 2: For a determined candidate PDSCH reception occasion, if the UE receives a DCI which schedules a PDSCH associated with the occasion, the UE would not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH.

Issue 3: how to support HARQ-ACK information feedback for a single valid PDSCH which is scheduled by a DCI which indicate a TDRA row including more than one SLIV.

According to discussions so far, when time domain bundling is supported, it is possible that for a determined candidate PDSCH reception occasion, only a single valid PDSCH, which is scheduled by a DCI which indicate a TDRA row including more than one SLIV, is associated with the occasion.

For example, as shown in Figure 3, only one valid PDSCH is associated with occasion 1, and the DCI scheduling the valid PDSCH indicates row 1 which includes more than one SLIV. It should be straight forward that the HARQ-ACK information bit for occasion 1 should be equal to the HARQ-ACK information bit for the single valid PDSCH.

Figure 3. example of one valid PDSCH scheduled by one DCI indicating a TDRA row with more than one SLIV for an occasion

However, in the current pseudo-code in TS 38.213 h10. how to generate the HARQ-ACK information bit for such case is not correct.

In the current pseudo-code, if‘a PDSCH’ in the yellow part means a configured PDSCH (regardless of whether it is valid or not), the cyan part would be meaningless, because anyway there would be a configurd PDSCH associated with the last SLIV. Therefore, we assume‘a PDSCH’ in the yellow part means a valid PDSCH.

Based on the assumption regarding‘a PDSCH’ in the yellow part, for the case as shown in Figure 3, where the single valid PDSCH is not associated with the last SLIV, NACK would be set for the HARQ-ACK information bit according to the cyan part, regardless of whether the PDSCH is successfully received. Meanwhile, for a case where a single valid PDSCH is associated with the last SLIV, logical AND operation would be applied according to the cyan part. The operations for both cases are obviously incorrect.

while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; end if j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = NACK; end if j=j+1; else if the PDSCH is associated with the last SLIV in the TDRA row; ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c else ojACK = NACK; end if j=j+1; end if else …

Observation 3：According to TS38.213 h10, for Type-1 HARQ-ACK codebook with time domain bundling, for a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row including more than one SLIV, the corresponding HARQ-ACK information bit is not correctly set.

Proposal 5: For Type-1 HARQ-ACK codebook with time domain bundling, update the pseudo-code in TS 38.213 to clarify the following:

- For a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row with more than one SLIV, the corresponding HARQ-ACK information bit is equal to the HARQ-ACK information bit for the single valid PDSCH.

Note that Issue 1-3 are related to the same part of pseudo-code in TS 38.213. How to update the specification should depend on outcome of discussions about all these issues.

2.3 Indication of an inapplicable value of k1 in a DCI scheduling multiple PDSCHs

In Rel-16, when the PDSCH-to-HARQ_feedback timing indicator filed of a first DCI indicates k1=-1, the corresponding HARQ-ACK information should be fed back in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed of a second DCI, if some timeline requirements are fulfilled. For some cases, the timeline requirements consider the time domain location of PDSCH scheduled by the first DCI. For example, the slot indicated by the PDSCH-to-HARQ_feedback timing indicator of the second DCI is no later than a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format, or the second DCI is later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format.

In Rel-17, for operation with 480 kHz and/or 960 kHz SCS, it was agreed in RAN1#106bis-e meeting that the value range of k1 indicated in RRC is -1 ~ 127 for DCI format 1_1. That is, similar to Rel-16, in Rel-17, DCI format 1_1 may provide an inapplicable value of k1 in PDSCH-to-HARQ_feedback timing indicator field, regardless of SCS.

Agreement: For NR operation with 480 kHz and/or 960 kHz SCS, the value range of k1 indicated in RRC is -1 ~ 127 for DCI format 1_1 and 0 ~ 127 for DCI format 1_2. - Note: this does not imply that DCI format 1_2 supports multi-PDSCH scheduling

In Rel-17, DCI format 1_1 may be configured with multi-PDSCH scheduling. As aforementioned for Rel-16, in case of SPS PDSCH receptions being configured/activated, to multiplex HARQ-ACK information corresponding to the first DCI into the PUCCH/PUSCH in the slot indicated by the second DCI, the corresponding timeline requirements would be based on the time domain location of PDSCH scheduled by the first DCI. However, the legacy timeline requirements do not consider the case of multi-PDSCH scheduling, where there may be invalid PDSCHs which are colliding with semi-static UL symbols. So, it is necessary to further the timeline requirements for the new case.

In our views, if indication of k1=-1 in DCI format 1_1 is supported when multi-PDSCH scheduling is configured, following the spirit of Rel-16, only the valid PDSCHs need to be considered for the corresponding timeline requirements. For example, the slot indicated by the PDSCH-to-HARQ_feedback timing indicator of the second DCI is no later than a SPS PDSCH reception received after the last valid PDSCH scheduled by the first DCI format, or the second DCI is later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the last valid PDSCH scheduled by the first DCI format.

Proposal 6:  For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

- TP#1 can be considered.

2.4 HARQ process ID

In RAN1#108-e meeting, the following agreement was made for multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism.

Agreement For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism, - If at least one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH (i.e., both repetitions) is considered as invalid. - Note: No specification impact on Type-1 HARQ-ACK codebook construction is expected, as a consequence of this agreement. - Note: This is not applied for the case when the multi-PDSCH DCI schedules only a single PDSCH.

However, in TS 38.214 as below, it seems still not clear whether the PDSCH considered as invalid according to the agreement is allocated with HARQ process ID or not, although we think it should be straight forward to be not since UE would not receive the PDSCH.

When the UE is scheduled with multiple PDSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.

Proposal 7: For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism, clarify HARQ process ID allocation.

- Option 1: If at least one of the repetitions of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID.

- Option 2: If the 1st repetition of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID.

- Conclusion

According to the discussions above, we have the following observations and proposals for multi-PDSCH/PUSCH scheduling via a single DCI.

SPS/CG

Proposal 1: In order to avoid ambiguity, clarify whether pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config would be applicable for SPS/CG activation/retransmission with DCI format 1_1/0_1 when TimeDomainAllocationListForMultiPDSCH-r17/TimeDomainAllocationListForMultiPUSCH-r17 is configured.

Proposal 2: pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config should be applicable for SPS/CG activation/retransmission with DCI format 1_1/0_1 when TimeDomainAllocationListForMultiPDSCH-r17/TimeDomainAllocationListForMultiPUSCH-r17 is configured.

Type-1 HARQ-ACK codebook with time domain bundling

Observation 1: According to the agreement made in RAN#107b-e meeting for type-1 HARQ-ACK codebook with time domain bundling, logical AND operation may be applied to PDSCHs scheduled by different DCIs (up to gNB’s scheduling). However, the current specification (TS 38.213 h10) seems not aligned with the agreement (the specification seems to limit logical AND operation to the case where the associated valid PDSCHs are scheduled by a single DCI).

Proposal 3: For Type-1 HARQ-ACK codebook with time domain bundling, clarify the following in TS 38.213:

- Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by one or more DCIs.

Observation 2: For Type-1 HARQ-ACK codebook with time domain bundling, it has not been discussed how to support SPS PDSCH.

Proposal 4: For Type-1 HARQ-ACK codebook with time domain bundling, discuss how to support SPS PDSCH and consider the following two options.

- Option 1: Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, and the valid PDSCHs may be scheduled by DCI and SPS.

- Option 2: For a determined candidate PDSCH reception occasion, if the UE receives a DCI which schedules a PDSCH associated with the occasion, the UE would not receive SPS PDSCH (if any) associated with the same occasion and not report HARQ-ACK information corresponding to the SPS PDSCH.

Observation 3：According to TS38.213 h10, for Type-1 HARQ-ACK codebook with time domain bundling, for a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row including more than one SLIV, the corresponding HARQ-ACK information bit is not correctly set.

Proposal 5: For Type-1 HARQ-ACK codebook with time domain bundling, update the pseudo-code in TS 38.213 to clarify the following:

- For a candidate PDSCH reception occasion associated with a single valid PDSCH scheduled by a DCI which indicates a TDRA row with more than one SLIV, the corresponding HARQ-ACK information bit is equal to the HARQ-ACK information bit for the single valid PDSCH.

Inapplicable value for HARQ feedback

Proposal 6:  For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

- TP#1 can be considered.

HARQ process ID

Proposal 7: For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism, clarify HARQ process ID allocation.

- Option 1: If at least one of the repetitions of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID.

- Option 2: If the 1st repetition of a PDSCH collides with semi-static UL symbols, the PDSCH (i.e., both repetitions) is not allocated with HARQ process ID.

TP#1

------------------------------- Text Proposal for 38.213, v17.0.0, Clause 9.1.3 --------------------------------

*** Unchanged text omitted ***

If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16,

-	if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where

-	if the UE is not provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the last valid PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the last valid PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the last valid PDSCH scheduled by the first DCI format, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback, the first DCI format does not  have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the last valid PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the first DCI format does not have associated HARQ-ACK information without scheduling a PDSCH reception or TCI state update, and the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the last valid PDSCH scheduled by the first DCI format, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4.

-	otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission.

*** Unchanged text omitted ***

------------------------------------------------- End Text Proposal -------------------------------------------------