3GPP TSG RAN WG1#109e	        	R1-2203726

E-meeting, 25 May – 5 May 2022

Agenda Item:					    9.1.4.1, UL precoding indication for multi-panel transmission

Source:	Sony

Title:	Considerations on UL precoding indication for multi-panel transmission

Document for:	Discussion

# Introduction

WI objective 6

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

- UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

  - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

- UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

  - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

Features for UL panel selection was introduced in 3GPP Rel-17 [1]. Multi UL transmissions influence UL coverage, throughput, and robustness and various beam configurations may be optimal at different conditions.

# Discussion

Based on the objective 6 in [1] up to four layers are to be supported, based on up to two codewords and two TRPs. How to associate two code-words (CW) to multiple streams that reach different TRPs is an interesting topic.

In general, there are multiple approaches one can take which influence both latency (need for joint decoding), robustness (CW length) and complexity (e.g., in case of different CW rates).

- If a CW is associated with a single TRP the consequence is that two CWs are needed for dual TRP transmission.

- If CW can be associated with two TRPs each layer may be limited to carry a single CW or

- all the CWs may be interlaced within the four layers.

The pros and cons with the different approaches may vary depending on channel conditions and TRP implementations (i.e,. back haul capacity) and scenarios and should be investigated.

In Rel 16, TS38.211, table 7.3.1.3-1: “Codeword-to-layer mapping for spatial multiplexing”, supports a single CW can be mapped to up to four layers (case 3 above), while two CWs indicate that at least 5 layers are needed.

To this end, if deemed beneficial, different combinations of CW to layer mapping are needed.

Proposal 1: Study, and if beneficial define new CW to layer and TRP mapping alternatives.

Furthermore, Similar to FR1, the number of simultaneous UL and DL channels are typically limited by the number of BB channels which are limited to 4 in the objective. In FR2, the BB channels are then shared by multiple panels. To allow for maximum degree of design freedom, all of:

- the number of panels,

- the number of Rx BB channels,

- the number of Tx BB channels,

are up to UE implementation. Furthermore, panel capabilities are not declared or well defined in 3GPP, e.g., Rx only, Tx only, polarization limitations etc. Such properties may have implications in UL panel selection. It is also a fact that power consumption is high in FR2 operation, based on which non used panels are typically turned off. This means that it is way more power efficient with two-layer transmission from a single panel compared to single-layer transmissions from two panels.

Based on the above concerns, it may be beneficial if the UE could share panel capabilities. Where relevant panel properties are associated with panel IDs. This would enable the gNB to schedule multi TRP operation in a more power optimal way, from a UE perspective.

Observation 1: gNB awareness of UE panel properties would enable more power efficient scheduling from a UE perspective.

Proposal 2: Study, and if beneficial consider UE panel property sharing.

# Conclusions

Here we list the Observations and Proposals made in this contribution

Proposal 1: Study, and if beneficial define new CW to layer and TRP mapping alternatives.

Observation 1: gNB awareness of UE panel properties would enable more power efficient scheduling from a UE perspective.

Proposal 2: Study, and if beneficial consider UE panel property sharing.

# References

[1]			 RP-213598