3GPP TSG RAN WG1 #109-e								R1-2203728

e-Meeting, May 9th – 20th, 2022

Agenda Item 	:	9.2.1

Source 	:	Sony

Title 	:	Consideration on common AI/ML framework

Document for 	:	Discussion and decision

- Introduction

According to the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface SID [1], the SI contains the following objectives:

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In this contribution, we provide our views on common AI/ML framework.

- Issue of AI/ML common framework

In this section, we discuss about potential issue of common AI/ML framework. We raise four issues to introduce AI/ML models in the air interface.

<Who and how train AI/ML model>

First issue is who and how to train the AI/ML model. We show examples in figure 1. Considering applying AI/ML models to transmitter and receiver, there would be two types of training, one is a collaboration scenario between transmitter and receiver, and the other is a no-collaboration scenario.

For the without-collaboration scenario, the transmitter and receiver train their AI/ML models using their own implementation (Type 1). This scenario doesn’t consider any signalling and transfer of AI/ML model, therefore there are only implementation issues.

For the with-collaboration scenario, the transmitter and receiver train their AI/ML models using some signalling information, training data or use transferred trained AI/ML model (Type 2&3). This scenario has some specification impact to introduce the collaboration, so RAN1 should mainly study this scenario to identify issues and solutions for transmitter and receiver collaboration.

Figure 1: Collaboration for AI/ML training between gNB and UE

Proposal 1: RAN1 should prioritise the study collaboration scenarios between transmitter and receiver to identify issues and solutions.

Considering collaboration scenarios, there are three types of scenario. We show examples in figure 2. One scenario entails the AI/ML model being trained by the gNB only (Type 1), the second scenario entails the AI/ML model being trained by the UE only (Type 2), and the final scenario is one where the AI/ML model is trained by both of gNB and UE with/without transfer of the trained AI/ML model (Type 3).

First one and second one is to train AI/ML model by one side. Because AI/ML model need to be deployed to encode or decode at both of side, these scenarios need to transfer trained AI/ML model to other side. Third one needs to train AI/ML model at both of side, so providing training data and some signalling for AI/ML model management would be needed. In addition, AI/ML model transferring also can be applied for this scenario. In these scenarios, issue is to study that what signalling information would be needed for training and how to transfer AI/ML model.

Figure 2: Collaboration scenarios for AI/ML training

Proposal 2: RAN1 should study what signalling information would be needed for training and how to transfer an AI/ML model.

<How to keep valid AI/ML model and update process>

After training and deployment of an AI/ML model at both of the transmitter and receiver, the AI/ML model should be kept valid for reliable communication. If the deployed AI/ML model becomes invalid due to the passage of time or due to changes in the channel environment, the result decoded by the AI/ML model would not be reliable. In this case, the AI/ML model needs to be updated at the transmitter and receiver.

To keep the AI/ML model valid, firstly the gNB or UE needs to monitor whether the AI/ML model is valid or not. For example, if the gNB fails to decode uplink data which is encoded using the AI/ML model, the gNB needs to ascertain whether the AI/ML model is valid or not. If the result of verification is that the model is invalid, the gNB also needs to indicate to the UE that the model is invalid and then AI/ML model update process should be started. This is one example of this issue. We think that monitoring, validation, and the update process of the AI/ML model should be studied in this work.

Proposal 3: RAN1 should study monitoring, validating and the update process of the AI/ML model.

<Which function block should be enhanced and how to apply AI/ML model>

Conventional PHY layer signal processing is composed of some functions, for example, channel coding, scrambling, modulation, precoding and others. To introduce AI/ML model signal processing, we should consider which function block should be enhanced by AI/ML.

We show examples in figure 3. We think there are five possibilities. The first one is to enhance one function block (Type 1), the second one is to enhance some combined function blocks (Type 2), the third one is to enhance all function blocks (Type 3) and the last one is to add a new function block which is composed of an AI/ML model (Type 4). In addition, we should also consider how to enhance these function blocks. For example, one method is to replace or add one or some function blocks with AI/ML based function blocks as above mentioned (Type 1~4). Another method is to infer appropriate signal processing parameters using AI/ML models (Type 5). This method uses conventional function blocks and adds an AI/ML based parameter control function to apply the appropriate PHY parameters.

As shown above, there are some possibilities about how to enhance current signal processing by using AI/ML models. We think that replacing or adding AI/ML based function blocks has large specification impact, but these types has large potential to enhance performance. On the other hand, controlling current function blocks using AI/ML models might not have such large specification impact. We think all types have pros and cons to enhance signal processing, so we propose that RAN1 should study both of signal processing replacement by AI/ML based function blocks and the control of conventional function blocks by AI/ML based parameter control functions.

Figure 3: Enhanced function blocks with AI/ML

Proposal 4: RAN1 should study both of signal processing replacement by AI/ML based function blocks and the control of conventional function blocks by AI/ML based parameter control functions.

<How to switch between AI/ML model signal processing and conventional signal processing>

To introduce AI/ML model signal processing, how to switch between the AI/ML model-based signal processing and conventional signal processing is one of the issues that needs to be addressed. If the AI/ML model is always a valid model, the switching mechanism might not be needed. However, we think the AI/ML model sometimes becomes an invalid model due to the passage of time or due to changes in the channel environment. When the AI/ML model is invalid, the AI/ML model needs to be updated and during this period, the AI/ML model signal processing cannot be applied. In this case, a fallback mechanism of using conventional signal processing would be needed. Therefore, RAN1 should study the switching mechanism between AI/ML model signal processing and conventional signal processing.

Figure 4: Switching between AI/ML based signal processing and conventional signal processing

Proposal 5: RAN1 should study the switching mechanism between AI/ML model-based signal processing and conventional signal processing.

- Conclusion

In this contribution, we discussed our views on common AI/ML framework. We proposed as follows:

Proposal 1: RAN1 should prioritise the study collaboration scenarios between transmitter and receiver to identify issues and solutions.

Proposal 2: RAN1 should study what signalling information would be needed for training and how to transfer an AI/ML model.

Proposal 3: RAN1 should study monitoring, validation and the update process of the AI/ML model.

Proposal 4: RAN1 should study both of signal processing replacement by AI/ML based function blocks and the control of conventional function blocks by AI/ML based parameter control functions.

Proposal 5: RAN1 should study the switching mechanism between AI/ML model-based signal processing and conventional signal processing.

References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, Dec. 2021.