3GPP TSG RAN WG1 #109-e								R1-2203729

e-Meeting, May 9th – 20th, 2022

Agenda Item 	:	9.2.2.2

Source 	:	Sony

Title 	:	Considerations on CSI measurement enhancements via AI/ML

Document for 	:	Discussion and decision

# Introduction

According to the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface SID [1], the SI contains the following objectives:

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In this contribution, we provide our views on CSI measurement enhancements via AI/ML.

# Use case of Compression of CSI feedback

In this section, we discuss about compression of CSI feedback which is one of the use cases of CSI feedback enhancement. We think there are mainly two types of sub use cases for compression of CSI feedback. One is to reduce usage of physical resources to feedback CSI report. Another is to enhance granularity of CSI feedback information.

For both of these cases, transmission data is compressed by AI/ML encode processing in the transmitter and the receiver needs to decode the compressed data by AI/ML decode processing. The mechanisms are the same between both use cases, but the purposes are different.

We show example about sub use cases in figure 1. Purpose of first case is to reduce usage of physical resources, so this case could improve frequency usage efficiency if AI/ML encoder and decoder work fine. In this case, CSI feedback table which is same as without AI/ML processing can be used. On the other hand, purpose of second case is to enhance granularity of CSI feedback information. This case needs to use enhanced CSI feedback table which has finer CSI feedback information.

Considering these cases, RAN1 should study how to reduce the transmission data size and how to introduce finer CSI feedback.

Figure 1: CSI feedback sub use cases

Proposal 1: RAN1 should study how to reduce the transmission data size and how to introduce finer CSI feedback.

# CSI Measurement and Feedback

In this section we analyse two sources of inaccuracy of CSI feedback

  - Timeliness of channel measurements

In usual operation, the UE carries out CSI measurements in one subframe or slot, the results of these measurements are sent to the network in the form of CSI reports in a later UL slot/subframe and the gNB uses these measurements to select and schedule resources for future transmissions to and from the UE in another slot/subframe. The degree to which the channel quality measurements taken from a time before the actual transmission time reflect the channel quality at the time in which the actual physical channel transmissions take place depends on how far apart the two times are but also on the relative mobility of the UE. For a non-stationary UE, radio propagation conditions can change between the two times concerned. It is desirable to take the channel measurement close to the actual physical channel transmission slot or subframe. Failing this, CSI measurements taken some time in the past can also be extrapolated to the likely physical channel transmission time and then used to determine the allocation of transmission resources. AI/ML can help in finding a good model for this extrapolation or prediction of channel measurements from previous and likely coarse measurements.

Proposal 2: RAN1 study paradigms for CSI measurement prediction to closer to the transmission resources allocated.

  - Frequency and time granularity of channel measurements

CSI measurements are taken on configured CSI resources. Configured CSI resources represent an overhead as data cannot be transmitted on the CSI resources. For these reasons, the maximum configurable density for CSI resources is 3 i.e. 3 resource elements per PRB. This maximum density determines the maximum amount of information that can be transmitted in a measurement report. It is desirable to increase CSI resource density as this gives better channel measurement accuracy and granularity. However, an increase in density, as well as depleting DL transmission resources will also require more reporting resources in the UL. By post-processing sparse CSI measurements by for example interpolation, the frequency (and time) granularity of CSI measurements can be improved. This improved granularity can be used to make better resource allocation decisions. Post-processed CSI measurements, in the limit, could have a CQI value for every resource block. To transmit this to the gNB, further compression is needed to reduce the number of bits needed.

The post-processing of CSI measurements can be subjected to ML to derive a good model that can be applied to extend the coarse channel measurements into a full band measurement with granularity approaching a channel measurement per resource element. Compressing this full-band channel measurement can also be subjected to ML training to produce a concise transmissible format that when exploded at the gNB will provide higher granularity CSI measurements for better resource allocation and choice of MCS etc.

Proposal 3: RAN1 study paradigms for increasing CSI measurement granularity in both time and frequency for more accurate resource and MCS allocation.

With denser CSI measurements and their efficient reporting, there may be a need for new methods of resource allocation and associated transport channel processing that exploit the denser channel measurements.

Proposal 4: RAN1 study new methods of resource allocation and transport channel processing based on higher CSI measurement granularity.

# Conclusion

In this contribution, we discussed our views on CSI measurement enhancements via AI/ML. We observed and proposed as follows:

Proposal 1: RAN1 should study how to reduce the transmission data size and how to introduce finer CSI feedback.

Proposal 2: RAN1 study paradigms for CSI measurement prediction to closer to the transmission resources allocated.

Proposal 3: RAN1 study paradigms for increasing CSI measurement granularity in both time and frequency for more accurate resource and MCS allocation.

Proposal 4: RAN1 study new methods of resource allocation and transport channel processing based on higher CSI measurement granularity.

References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, Dec. 2021.