3GPP TSG RAN WG1 #109-e			R1-2203731

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	9.2.4.2

Source:		Sony

Title:			Considerations on AI/ML for positioning accuracy enhancement

Document for:	Discussion & Decision

# Introduction

A new study item on Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in  RANP#94e [1]. The objective of the study item is shown below:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

This contribution provides our initial views on AI/ML for positioning, including the proposed sub use-cases, scenarios and also some considerations on AI/ML for positioning accuracy enhancement purposes.

# Discussions on AI/ML for positioning

## Scenarios for evaluation

One of the objectives of using AI/ML for positioning as discussed in [1] is to mitigate the NLOS effect, which may significantly degrade the positioning accuracy. Particularly, this issue is severe in a scenario with dense obstacles surrounding the UE, such as indoor hotspot  mix-office scenario (InH-MO) and indoor factory with dense high clusters (InF-DH). In downlink-based positioning, a UE may receive the multipath components. For the positioning measurement purpose, the UE typically performs cross corelation between the received signal and the self-generated reference signal to obtain a power delay profile (PDP), which is an estimation of the delay domain of the channel. Due to the fact that the light-of-sight (LOS) signal arrives first, the UE normally would identify the first arrival path in the PDP as the LOS path. However, LOS signal strength can be very weak in a channel where NLOS components are dominant. In such a case, UE may detect a false LOS path, since the ‘first arrival path’ observed in the PDP can be a strong reflection or just a noise signal. This could lead to significantly degraded positioning estimation.

The LOS condition in term of LOS probability for different indoor scenarios is shown in Figure 1. The LOS probability model is as described in [3]. From Figure 1, we can observe that the LOS probability is inversely related with the distance from UE to gNB. Among various scenarios, the InF-DL scenario has the lowest LOS probability in which the channel can barely obtain LOS path if the distance from UE to gNB is larger than 20 m. Besides, InH-MO, InF-SL and InF-DH also have comparatively low LOS probability as the distance increases.

Figure 1: : LOS probability in various indoor scenarios

Observation 1: Among various indoor scenarios, InF-DL gives lowest LOS probability. InH-MO, InF-SL and InF-DH have also comparatively low LOS probability as increasing the distance.

During NR positioning study in Rel-17, InF-SH and InF-DH had been considered for evaluation purposes. We consider a more challenging channel should be considered in NR positioning Rel-18. The usage of AI/ML can be expected to improve positioning accuracy by alleviating the issue due to NLOS components. Hence, for AI/ML for positioning study in Rel-18, the representative scenarios for evaluation should be the channel model with rich NLOS components. This could be InF-SL (indoor factory with sparse clutter and low base station height) and InH-MO (indoor hotspot mix office). Even a challenging scenario using InF-DL (indoor factory with dense clutter and low base station height) can be further discussed.

Proposal 1: Consider the scenarios with channel model with rich NLOS components (e.g., InF-SL, InH-MO, InF-DL) for positioning accuracy enhancement evaluation in AI/ML topic.

## NLOS mitigation using AI/ML

Some agreements on multiple paths reporting were made in Rel.17 in RAN#107e [2] and it has been captured as part of the specifications. For DL-TDOA and Multi-RTT  positioning, UE is capable of measuring and reporting a maximum of 8 additional paths. In the measurement report, the indices of the additional paths and the associated relative path power measurement (PRS-RSRPP) for each path could be reported. Likewise in UL case, the gNB could also report multipath measurements to LMF.

- Agreement - For enhanced multipath reporting support N=8 for the value of maximum number of additional paths. - Define a UE capability for the UE to report its supported value of maximum number of additional paths (no larger than 8) - Agreement - Support the LMF to request DL PRS-RSRPP together with timing measurement as part of DL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths. - Support the LMF to request UL SRS-RSRPP together with timing measurement as part of UL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths.

By allowing UE/TRP to report the received power of the multipath components, the LMF has the chance to process and measure in the channel delay domain. As of now, it is up to LMF how this information is utilized (i.e., implementation aspect of LMF). We consider the usage of AI/ML to process such information, including the collected path measurements from UE/TRP, would be beneficial, especially to improve the positioning accuracy.

Multipath measurement and reporting open up a new opportunity for AI/ML deployment, since LMF can collect more detailed channel information from UE/TRP. For example, in the legacy, LMF would only obtain one timing measurement per measurement report. The accuracy of identifying LOS/NLOS mainly relies on the UE. But now the LMF is capable of acquiring multiple path information. Having the path information including relative signal power (PRS-RSRPP) and relative delay of multiple propagation path, LMF can make its own decision on the LOS path identification.

Observation 2: The multiple paths reporting from UE/TRP to LMF as a feature in rel-17 could assist LMF to make its own decision on LOS path selection.

In principle, LMF can collect a huge amount of positioning measurement data which could be from different UEs/gNBs. It enables LMF to further create and train an AI/ML model using the collected data.  This method can be also used for NLOS mitigation.

Proposal 2: Consider to support LMF to create and train AI/ML model for NLOS mitigation.

The usage of Machine learning can be briefly divided into three phases:

- Data collection phase

- Model training and updating phase

- Model deployment phase

Figure 2 summarizes these three phases and the involved communication units. In the data collection phase, channel quality and multipath information are measured and collected for further analysis. This procedure is performed by UE or gNB,  particularly for the UE with a fixed-location and can provide an exact location. The sources of the data can differ in DL and UL cases. In DL, UE receives the reference signal and processes the data, while in UL, the gNB performs this procedure. After the measurement is performed, LMF collects the results, such as the path information, quality matrix and possible its own location. It should be noted that in this phase, LMF also needs to process and validate the data to further transfer it to a training set as an input to the model training.

Once LMF collects a certain amount of data, LMF can perform ML model training. Model training step may include model selection, training, validation and testing. Lastly, at the output from the training phase, an inference model will be deployed in UE or gNB or LMF to handle specific inference operation, e.g., NLOS detection.

Data Collection with processing and validation

(LMF or reference UE/gNB)

Model Training and Updating

(LMF)

Model deployment

(UE/gNB/LMF)

Figure 2: Procedure of Machine learning utilization for positioning.

Observation 3: The procedure of ML for positioning can be at least divided in three steps:

- Data collection with data processing and validation,

- Model Training and updating,

- Model deployment.

In terms of specification impact, two aspects may typically be relevant. The first aspect is the signalling for training data collection. ML model training relies on the known properties learnt from the training data, which contains measurements from the UE or gNB (as the input to the ML model) and the ground truth value (as the expected output from the ML model given specific input set). Only by feeding the ML model with a sufficiently large input set and the expected output set, LMF can obtain a robust inference model. To fulfil the above requirement, there should be some specific reporting from UE/gNB to LMF to allow the LMF to collect the right information for model training. Therefore, potentially there could be some new specification impact on the reported content dedicated for model training and which has not been supported in Rel.17.

For example, to train an AI/ML model for NLOS identification, LMF needs to be aware of the true LOS/NLOS labels of the measurements in the training set. These labels should be informed by a reference UE or reference gNB which knows its own location.

The second aspect is about signalling the trained model in the model deployment phase. After the model training, the LMF can utilize the model and apply it on the new measurements. But the model may not necessarily be used only in LMF. The inference model can also be deployed in the other units. For example, LMF can also report the model to UE/gNB and allow them to perform ML inference by themselves. This can potentially avoid heavy computation in LMF.

Proposal 3: Consider the specification impact on these two aspects:

- Reporting positioning measurements to LMF for model training (e.g., the ground truth output of the model.)

- Signalling and deploying the inference model in UE/gNB.

# Summary

In this contribution, we have discussed our view on usage of AI/ML for positioning. Our observation are listed below:

Observation 1: Among various indoor scenarios, InF-DL gives lowest LOS probability. InH-MO, InF-SL and InF-DH have also comparatively low LOS probability as increasing the distance.

Observation 2: The multiple paths reporting from UE/TRP to LMF as a feature in rel-17 could assist LMF to make its own decision on LOS path selection.

Observation 3: The procedure of ML for positioning can be at least divided in three steps:

- Data collection with data processing and validation,

- Model Training and updating,

- Model deployment.

We have also the following proposals:

Proposal 1: Consider the scenarios with channel model with rich NLOS components (e.g., InF-SL, InH-MO, InF-DL) for positioning accuracy enhancement evaluation in AI/ML topic.

Proposal 2: Consider to support LMF to create and train AI/ML model for NLOS mitigation.

Proposal 3: Consider the specification impact on these two aspects:

- Reporting positioning measurements to LMF for model training (e.g., the ground truth output of the model.)

- Signalling and deploying the inference model in UE/gNB.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #107-e	, e-Meeting, November 11th – 19th, 2021

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)