3GPP TSG RAN WG1 #109-e		R1-2203733

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	9.3.3

Source: 	Sony

Title: 	Enhancements to dynamic/flexible TDD for Full Duplex operation

Document for: 	Discussion / decision

# Introduction

An SI on Duplex Evolution is agreed in RAN#94-e to identify solutions for full duplex TDD.  One of the objectives is [1]:

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD

  - Identify possible schemes and evaluate their feasibility and performances

This contribution discusses some considerations on full duplex TDD operations.

# Discussions

## Full Duplex TDD

Currently, TDD operates in Half Duplex (HD) mode where the gNB or UE can either transmit or receive but not at the same time.  In TDD, the time resources are split between UL and DL where typically more time resources are allocated in the DL, which faces the following drawbacks:

- Latency: Since a UE cannot transmit in DL resources, a UE that has uplink transmission during a DL time resource (slot or OFDM symbols), has to delay its uplink transmission until an UL time resource is available, thereby introducing latency.

- Resource utilization: Since the entire frequency bandwidth can only be DL or UL in a cell, if a link, e.g. DL, has very little traffic, the resource would be underutilized as it cannot be used to alleviate congestion in the UL traffic.

- UL coverage: A typical DL:UL time resource split is DL heavy thereby causing sparse UL resource.  Coverage enhancement schemes that use repetitions in the UL may not have sufficient UL resource for the UL repetitions within an acceptable latency. Hence this may have an impact to UL coverage.

Recognizing these drawbacks in HD-TDD, an SI on Duplex Evolution to study solutions for Full Duplex TDD (FD-TDD) is agreed [1].  FD-TDD enables simultaneous UL & DL transmissions in the same frequency band, and this would reduce the drawbacks identified in HD-TDD.  In this SI, we assume that only the gNB operates in FD-TDD whilst the UE operates in HD-TDD, that is, the complexity to implement FD-TDD in Rel-18 is mostly at the gNB.

## Slot Format

In existing dynamic/flexible HD-TDD, the Slot Format can be semi-statically and dynamically configured.  The OFDM symbols (OS) in a slot can be semi-statically configured to be DL, UL or Flexible (FL), where semi-static FL symbols can be further indicated to be either UL or DL dynamically using the SFI carried by a GC-DCI Format 2_0 and any remaining FL symbols that are not changed by SFI can be further changed by UL or DL grants.

FD-TDD can be implemented using Flexible (FL) OFDM symbols (OS), by scheduling DL and UL transmissions to different UEs to use the same time resources but in different frequency resources during the FL symbols.  An example is shown in Figure 1, where at time t1, UE#1 receives DL Grant DG#1 which schedules a PDSCH between time t4 to t7 and corresponding PUCCH between time t11 and t13.  At time t5, UE#2 receives an UL Grant UG#2 scheduling a PUSCH between time t6 to t8.  DG#1 implicitly indicates to UE#1 that semi-static FL symbols between time t6 to t7 are DL symbols whilst UG#2 implicitly indicates to UE#2 that these semi-static FL symbols are UL symbols.  Hence by indicating different slot formats in these semi-static FL symbols for UE#1 and UE#2 the gNB is able to operate FD-TDD between time t6 to t7.  Similarly, DG#3 schedules UE#3 with PDSCH between time t10 to t12, which implicitly indicates that semi-static FL symbols between time t11 to t12 are DL symbols whilst UE#1 is implicitly indicated by DG#1 that these semi-static FL symbols are UL symbols.  In this way, the gNB is able to operate in FD-TDD with UE#1 and UE#3 between time t11 to t12.  This method of using existing FL symbols enables an FD-TDD gNB to support FD-TDD operation for legacy HD-TDD UEs.

Figure 1: FD-TDD using flexible OFDM symbols

Observation 1: FD-TDD can be supported for legacy UEs by scheduling DL & UL transmissions for different UEs within the same Flexible OFDM symbols but in different frequency resources.

Since FD-TDD can be supported for legacy UEs in FL symbols, to maximise FD-TDD operation, the slot format needs to be configured with lots of FL symbols.  However, this configuration may not be practical because there are backward compatibility issues such as SSB configurations, Common PDCCH search spaces and PRACH resources that are Cell Common semi-statically configured as DL and UL symbols for legacy UEs and are therefore difficult to reconfigure as FL symbols.  It is also typically difficult for TDD network operators to reconfigure their slot formats since they are configured based on traffic and capacity and reconfiguration would require extensive re-dimensioning of the network, which would disrupt legacy operations.  That is, reconfiguring the slot format to have heavy FL symbols would have a major impact on legacy UEs’ operations.

Observation 2: Reconfiguring slot format with heavy FL symbols for support of FD-TDD would have major impacts on legacy operations.

In the legacy system, semi-statically configured DL and UL OFDM symbols cannot be changed dynamically.  This imposes a restriction for FD-TDD operation as the entire OFDM symbol can only be used for either DL or UL.  Hence, this restriction needs to be removed for Rel-18 UEs that are capable of FD-TDD.  One way to do this is to enable the gNB to indicate to these FD-TDD capable UEs whether to overwrite DL symbols to UL symbols and vice-versa, that is, indicate whether an OFDM symbol that has been originally configured as DL is to be used for UL transmission and whether an OFDM symbol that has been originally configured as UL is to be used for DL transmission.  An example is shown in Figure 2, where a 1 bit overwrite indicator is implemented in DL & UL grants.  Here UE#1 receives UL Grant UG#1 scheduling a Type B PUSCH repetition with repetition factor of 2, that is the UE is to repeat this PUSCH, back to back, twice.  In UG#1, the overwrite indicator is set to disable and hence the UE is not allowed to overwrite DL (and UL) symbols.  The 1st repetition R1 is transmitted fully between time t6 to t8 since it doesn’t overlap with any semi-static DL symbols.  However, the 2nd PUSCH repetition R2, overlaps with semi-static DL symbols between time t8 to t10 and consequently it is segmented (as per legacy procedures) to a smaller actual PUSCH repetition that is two symbols long between time t10 to t11.  At time t5, UE#2 receives DL Grant DG#2 scheduling PDSCH and a corresponding PUCCH.  DG#2 indicates that overwriting is enabled and so the scheduled PDSCH occupying time t5 to t8 overwrites semi-static UL symbols between time t7 and t8.  Similarly, the corresponding PUCCH occupying time t14 to t17 overwrites semi-static DL symbols between time t14 to t15.  This method also allows both new Rel-18 UEs and legacy UEs to jointly operate in FD-TDD from the network perspective, where new Rel-18 UEs that are capable of FD-TDD can fully utilise all OFDM symbols for FD-TDD whilst legacy UEs can use only the flexible symbols for FD-TDD.

Figure 2: Overwrite indicator

Observation 3: To enable Rel-18 UEs to fully utilize FD-TDD from the network perspective, allow UE to overwrite DL OFDM symbols to UL OFDM symbols and vice-versa.

Proposal 1: Allow gNB to indicate in the DL & UL grants whether semi-static/SFI indicated DL and UL OFDM symbols can be overwritten for UL and DL transmissions respectively.

# Conclusion

In this contribution, we discuss some considerations on FD-TDD, and we observe the following:

Observation 1: FD-TDD can be supported for legacy UEs by scheduling DL & UL transmissions for different UEs within the same Flexible OFDM symbols but in different frequency resources.

Observation 2: Reconfiguring slot format with heavy FL symbols for support of FD-TDD would have major impacts on legacy operations.

Observation 3: To enable Rel-18 UEs to fully utilize FD-TDD from the network perspective, allow UE to overwrite DL OFDM symbols to UL OFDM symbols and vice-versa.

We therefore propose the following:

Proposal 1: Allow gNB to indicate in the DL & UL grants whether semi-static/SFI indicated DL and UL OFDM symbols can be overwritten for UL and DL transmissions respectively.

# References

[1] RP-220633, “Revised SID: Study on evolution of NR duplex operation,” CMCC, RAN#95-e