3GPP TSG RAN WG1#109-e	R1-2203736

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.4.2

Source: 	Sony

Title:	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion

Introduction

In RAN#95-e meeting, a new WID on the evolution of sidelink had been approved. Sidelink co-channel coexistence for LTE sidelink and NR sidelink to satisfy the requirements of limited spectrum resource has been described in [1].

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

We will analysis and discuss our views on sidelink co-channel coexistence in this contribution.

Discussion

For the dedicated ITS spectrum in 5.9GHz band, the amount of available bandwidth for cellular-V2X services is very scarce. In order to share this very limited spectrum resource to support both LTE V2X basic safety transmission and advanced NR V2X service, coexistence mechanism between LTE sidelink and NR sidelink has been agreed to study and identify.

In Rel-16 IDC (In-device coexistence) mechanism, semi static resource pool separation between two RATs and priority regulations have been approved for non co-channel coexistence. In Rel-18, for supporting automotive industry, 5GAA has proposed co-channel co-existence as high priority topic. Traffic collision will frequently happen when LTE sidelink service and NR sidelink service happened in the co-channel, the previous dropping behaviour will seriously impact the sidelink service performance. And for more and more NR sidelink vehicles on the road, sharing the LTE V2X channels can increase the spectrum usage efficiency and enhance reliability of NR sidelink.

Dynamic coexistence scheme to dynamically allocate non-overlapped resource to LTE sidelink and NR sidelink can solve those co-existence issues and achieve better performance. We will discuss the TDM dynamic coexistence scheme in chapter 3.

Proposal 1: Dynamic coexistence scheme for co-channel coexistence between LTE sidelink and NR sidelink can be supported.

TDM Dynamic Co-existence Scheme

In Rel-16 IDC (In-device coexistence) TDM mechanism, long term time-scale coordination has been agreed. Non-overlapping (in time domain) resource pools are (pre-)configured for NR sidelink and LTE sidelink, there is no information is exchanged between LTE and NR sidelink within UE. Although this solution is feasible, it may have impact on latency, reliability and data rate requirement in some applications. Then short term time-scale coordination has been approved which in the context of LTE sidelink and NR sidelink can exchange information (e.g., packet priority) prior to the time of transmission and reception. When collision happened, UE will use priority regulations to determine which RAT’s packet can be transmitted/received and drop the low priority RAT’s packet.

In Rel-18 sidelink, short term time-scale is more suitable for co-channel coexistence and some improvements are needed to decrease the impact caused by dropping behaviour. Dynamic coexistence scheme can try to avoid the transmission or reception overlapping in advance. In general case, the following two scenarios need to be analysis for dynamic coexistence scheme:

Scenario 1: LTE sidelink and NR sidelink exchange coordination information without network

Scenario 2: LTE sidelink and NR sidelink exchange coordination information with network

Scenario 1: LTE sidelink and NR sidelink have coordination information exchange procedure without network

When sidelink UE is out of coverage, information exchange between LTE RAT and NR RAT can assist sidelink UE to allocate non-overlapping resource to LTE sidelink and NR sidelink. Once LTE sidelink and NR sidelink co-exist in the same channel, UE’s NR sidelink RAT need to know the resource allocation form its LTE RAT and other Sidelink UE’s NR RAT.

As illustrated in figure 1, UE’s LTE RAT will send its own LTE sidelink resource allocation to its NR RAT using coordination message 1. Besides coordination message1, when LTE sidelink and NR sidelink can coexist in the co-channel, UE1 needs to know all the other LTE sidelink and NR sidelink resource usage for its own NR sidelink resource (re-)selection through coordination message2. Though UE1 knows part of the other UE’s LTE resource allocation information form its own LTE RAT’s sensing behaviour, half-duplex and hidden node, exposed node problem are still exist and will cause UE1 can’t obtain enough LTE channel resource usage information. As NR sidelink UE will occupy LTE sidelink channel, we need very carefully assign LTE sidelink channel to NR sidelink service. Rel-17 sidelink supports inter-UE coordination scheme for resource allocation transmission to overcome the half-duplex, hidden node, exposed node problem, we can reuse this inter-UE coordination scheme to transmit coordination message2. Coordination message2 contains other UE’s LTE sidelink and NR sidelink resource allocation information.

Figure1 Coordination information exchange between UE(s)

Proposal 2: Reuse inter-UE coordination scheme to exchange LTE/NR sidelink resource allocation information.

After UE1 obtains the resource usage from its LTE RAT and other UE’s coordination information. UE1 exclude those resource from its NR sidelink candidate resource set in resource (re-)selection procedure. The candidate resource set can be constructed by LTE dedicated sidelink channel and NR dedicated sidelink channel.

As illustrated in figure2, UE1 has LTE dedicated resource pool and NR dedicated resource pool. UE1’s LTE RAT will reports its LTE sidelink resource allocation information to its NR RAT dynamically or periodically. When NR sidelink packet arrive at UE1, UE1 will select resource for TB2 at first on NR dedicated channel. Once UE1’s LTE sidelink and NR sidelink resource allocation collision happened, and no non-overlapping resource can be found in NR dedicated resource pool. UE1 will select resource for TB2 from the candidate resource set contained LTE dedicated resource pool. As UE1 knows the other UE’s LTE sidelink resource allocation, UE1 can avoid the collision with its own LTE sidelink and other UE’s LTE sidelink in LTE dedicated sidelink resource pool simultaneously.

Figure2 Resource selection between LTE and NR sidelink channels

Observation1: Exchange information dynamically and periodically between UE’s LTE RAT and NR RAT is supported for co-channel coexistence.

Proposal 3: NR sidelink will use its dedicated resource pool at first and then (re-)select resource in LTE sidelink dedicated resource pool when collision happened.

Scenario 2: LTE sidelink and NR sidelink have coordination information exchange procedure with network

When the sidelink UE is in the coverage, network involved dynamic coexistence scheme can coordinate and allocate non-overlapping resource for sidelink UE’s LTE RAT and NR RAT. There are two deployment scenarios:

- UE in the coverage of LTE base station

When UE is in the coverage of LTE base station, eNB knows the LTE sidelink resource usage and assists to allocate usable resource in LTE sidelink channels for NR sidelink to LTE RAT. LTE RAT will transmit this resource allocation information to its NR RAT. When NR RAT find transmission and/or reception overlap with LTE RAT, NR sidelink can use this resource allocation grant.

- UE in the coverage of NR base station

When UE is in the coverage of NR base station, UE can report coexistence issues and/or its LTE sidelink resource allocation and other NR UEs’ LTE sidelink resource allocation to gNB. gNB will allocate non-overlapping resource to NR sidelink UE.

Observation2: NR sidelink UE can get non-overlapping resource allocation information from its LTE RAT.

Proposal 4: NR Sidelink UE can inform coexistence issues to network and provide assistant information to network.

Conclusion

Observation1: Exchange information dynamically and periodically between UE’s LTE RAT and NR RAT is supported for co-channel coexistence.

Observation2: NR sidelink UE can get non-overlapping resource allocation information from its LTE RAT.

Proposal 1: Dynamic coexistence scheme for co-channel coexistence between LTE sidelink and NR sidelink can be supported.

Proposal 2: Reuse inter-UE coordination scheme to exchange LTE/NR sidelink resource allocation information.

Proposal 3: NR sidelink will use its dedicated resource pool at first and then (re-)select resource in LTE sidelink dedicated resource pool when collision happened.

Proposal 4: NR Sidelink UE can inform coexistence issues to network and provide assistant information to network.

Reference

[1] RP-220300, “WID revision: NR sidelink evolution,” 3GPP TSG RAN Meeting #95e.