3GPP TSG RAN WG1 #109-e	                         			  R1-2203737

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	9.5.1.1

Source: 	Sony

Title: 	Considerations on SL positioning scenarios and requirements

Document for: 	Discussion & Decision

# Introduction

In RANP#94e, a new study item on expanded and improved NR positioning was approved [1]. One of the objectives in the study item solutions for sidelink positioning:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution discusses our initial views on our preference in term of use-cases, scenarios for the evaluation of the proposed solutions for sidelink positioning. Furthermore, we also discuss the positioning requirements of V2X use-cases.

# Discussion

## 2.1 Scenarios for Sidelink Positioning

According to the SID, numerous use-cases have been considered, such as V2X, public-safety, commercial, and Industrial IoT (IIoT). We consider these use-cases are still quite diverse. During the study item, we could still discuss but we would prefer to prioritize at least one of those use-cases.

Among the use-cases listed in the SID, we prefer to prioritize V2X use case for positioning evaluation. V2X and public safety are the use-cases that were discussed in RAN level as a study item in Rel-17. The other use-cases were discussed in SA level. Furthermore, the market demand on V2X positioning is very strong considering 5G Automotive Association (5GAA) requests 3GPP to identify the solutions to support V2X positioning. Other possible use-case can be the public-safety use-case. In our view, the identified positioning solutions in V2X positioning can also be generally applied in other use-cases.

For the coverage scenarios, we tend to firstly investigate the in-coverage scenario. We consider in-coverage scenario is the most common scenario in V2X positioning (i.e., in comparison to partial coverage and out-of-coverage). In this scenario, both Uu-interface and PC5-interface are available. It is expected the resources related to sidelink positioning are allocated by gNB. This is similar to Mode-1 in legacy V2X communication. PC5-link will be used for sidelink positioning, such as obtaining relative positioning between UEs. Uu-interface has been commonly used in legacy NR positioning, such as obtaining an absolute positioning of a UE. Hence, in-coverage scenario would enable us to evaluate hybrid positioning (jointly use of Uu- and PC5-interfaces for positioning).

Proposal 1: Prioritize the study on V2X positioning use cases with in-coverage scenario in SL positioning Study item.

For the evaluation methodology, we prefer to reuse the method provided in TR 37.885 [2], in which urban grid scenario highway scenario were identified as the baseline scenarios for evaluation of V2X. A detailed simulation assumption regarding scenarios below 6Ghz (FR1) and above 6GHz (FR2) can be found below. Note: For the evaluation purpose, we can prioritize FR1 over FR2 considering FR2 for V2X is also part of Rel-18 (in parallel with Positioning Rel-18).

Parameters regarding evaluation scenarios below 6 GHz are given in the following table:

Table 1: Evaluation scenarios for FR1

| Parameters | Urban grid for V2X | Highway for V2X |
| --- | --- | --- |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 6 GHz Micro BS to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 2 GHz or 4GHz Between vehicle/pedestrian UE: 6 GHz Micro BS to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| Aggregated system bandwidth | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) |
| Simulation bandwidth | 20 or 40 MHz (DL+UL) 10 and 20 MHz (baseline for SL) 100 MHz (optional for SL) | 20 or 40 MHz (DL+UL) 10 and 20 MHz (baseline for SL) 100 MHz (optional for SL) |
| BS Tx power | Macro BS: 49dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm Micro BS: 24dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Note: 33dBm for RSU is not precluded | Macro BS: 49dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm Micro BS: 24dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Note: 33dBm for RSU is not precluded |
| UE Tx power | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded |
| BS receiver noise figure | 5dB | 5dB |
| UE receiver noise figure | 9 dB |  |

Parameters regarding evaluation scenarios above 6 GHz are given in the following table:

Table 6.1.1-2: Evaluation scenarios above 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 30 GHz Between vehicle/pedestrian UE: 30 or 63 GHz Micro BS to/from vehicle/pedestrian UE : 30 GHz UE-type-RSU to/from vehicle/pedestrian UE: 30 or 63 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 30 GHz Between vehicle/pedestrian UE: 30 or 63 GHz Micro BS to/from vehicle/pedestrian UE : 30 GHz UE-type-RSU to/from vehicle/pedestrian UE: 30 or 63 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| Aggregated system bandwidth | Up to 1 GHz (DL+UL) Up to 1 GHz (SL) | Up to 1 GHz (DL+UL) Up to 1 GHz (SL) |
| Simulation bandwidth | 200 MHz (DL+UL) 200 MHz (SL) | 200 MHz (DL+UL) 200 MHz (SL) |
| BS Tx power | Macro BS: 43dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 43dBm. EIRP should not exceed 78 dBm and is also subject to appropriate scaling | Macro BS: 43dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 43dBm. EIRP should not exceed 78 dBm and is also subject to appropriate scaling |
| UE Tx power | Vehicle/pedestrian UE or UE type RSU: 23 dBm for 30 GHz, 21 dB baseline for 63 GHz, 27 dBm optional for 63 GHz. For both 30 and 63 GHz, EIRP should not exceed 43 dBm. | Vehicle/pedestrian UE or UE type RSU: 23 dBm for 30 GHz, 21 dB baseline for 63 GHz, 27 dBm optional for 63 GHz. For both 30 and 63 GHz, EIRP should not exceed 43 dBm. |
| BS receiver noise figure | 7 dB | 7 dB |
| UE receiver noise figure | 13 dB (baseline), 10 dB (optional) |  |

Proposal 2: Consider urban grid scenario and highway scenario and the evaluation methodology in TR 37.885 for the evaluation of V2X positioning.

## 2.2 Requirements for Sidelink Positioning

Regarding the sidelink positioning requirements, particularly for V2X positioning, we consider the requirements described in TR 38.845 [3] as the baseline. Here, the set of requirements are obtained by incorporating various requirements in 3 different sources, including TS 22.261[4], TS 22.186[5] and one from 5GAA[6]. Furthermore, the positioning requirements are categorized based on the service level. The requirements as in are [3]:

- Set 1: 10 – 50 m with 68 – 95 % confidence level.

- Set 2: 1 – 3 m with 95 – 99 % confidence level.

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

Among these requirements, Set 3 gives the most stringent positioning requirement which only allows 0.1m - 0.5 m positioning error for 95% - 99% of the use cases. In terms of SL positioning evaluation. we tend to select Set 3 as the baseline requirement for the evaluation purpose. This requirement is very relevant for one of the important scenarios in V2X, such as VRU collision avoidance.

Besides accuracy, latency is also another important requirement in V2X positioning. For V2X positioning, we consider reusing the requirement for IIoT use case in [7], in which physical layer latency for position estimation of UE should be less than [10] ms and the End-to-end latency should be less than [100]ms. We consider end-to-end latency is equivalent to service level latency as described in [6].

Proposal 3: Consider both positioning accuracy and positioning latency for SL positioning requirement.

Proposal 4: Consider the following requirements for the evaluation of V2X positioning:

- For the absolute/relative position, [0.1 – 0.5] m horizontal accuracy with [95 – 99] % confidence level.

- End-to-end or service level latency for position estimation of UE (< [100] ms)

- Physical layer latency for position estimation of UE (< [10] ms)

# Conclusion

In this contribution, we discuss the potential scenario and requirement for SL positioning study. Our proposals are given below:

Proposal 1: Prioritize the study on V2X positioning use cases with in-coverage scenario in SL positioning Study item.

Proposal 2: Consider urban grid scenario and highway scenario and the evaluation methodology in TR 37.885 for the evaluation of V2X positioning.

Proposal 3: Consider both positioning accuracy and positioning latency for SL positioning requirement.

Proposal 4: Consider the following requirements for the evaluation of V2X positioning:

- For the absolute/relative position, [0.1 – 0.5] m horizontal accuracy with [95 – 99] % confidence level.

- End-to-end or service level latency for position estimation of UE (< [100] ms)

- Physical layer latency for position estimation of UE (< [10] ms)

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- 3GPP TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR

- 3GPP TR 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases, V17.0.0 (2021-09)

- 3GPP TS 22.261: "Service requirements for the 5G system".

- 3GPP TS 22.186 v16.2.0: "Enhancement of 3GPP support for V2X scenarios".

- 3GPP RP-210040: "Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases," (source: 5GAA).

- 3GPP TR 38.857, Study on NR Positioning Enhancements; (Release 17)