3GPP TSG RAN WG1 #109-e	                         			  R1-2203738

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	9.5.1.3

Source: 	Sony

Title: 	Considerations on potential solutions for SL positioning

Document for: 	Discussion & Decision

# Introduction

A new release-18 study item on expanded and improved NR positioning was approved in RANP#94e meeting in December 2021 [1]. The study item to develop sidelink positioning is related to the outcome of the previous study/report,  the study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" which focussed on V2X and public safety use cases with the outcome being captured in TR 38.845. Additionally, SA1 has developed requirements for "Ranging based services" in TS 22.261, and  positioning accuracy requirements for IIoT uses cases in out-of-coverage scenarios in TS22.104.

The particular objective related to the sidelink positioning in Rel-18 is given below [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution provides our initial views / considerations on potential solutions for SL positioning.

# Discussion

Studying of sidelink reference signals for positioning purposes is part of the study item. In the existing NR sidelink, the following reference signals have been used:

- Demodulation reference signal (DMRS): for channel estimation purpose to be used for demodulation.

- Sidelink PSS/SSS: for synchronization purpose

- Sidelink Channel State Information (CSI): for channel measurement

- Sidelink Phase-tracking reference signal (PTRS): for the mitigation of the effect of phase noise.

The aforementioned reference signals are not designed for positioning purpose. However, positioning reference signals have been introduced in Uu-interface, such as:

- DL-Positioning reference signal (PRS): for positioning purpose in downlink direction based on Gold sequence.

- UL-Sounding reference signal (SRS) for positioning: for positioning purpose in uplink direction and based on Zadoff-Chu sequence.

During study item phase of NR positioning rel-16, extensive study had been carried out and resulting in the selection of the above reference signal for positioning using Uu interface. UL-SRS which is based on Zadoff-Chu sequence with the main property of low-PAPR. The signal with low-PAPR is beneficial for the UE to obtain high power amplifier efficiency, hence, improved UE power consumption. Based on this reason, we consider sidelink positioning reference signal is similar to UL-SRS for positioning in Uu-link. Further modifications may be needed to customize the property of sidelink transmission.

Proposal 1: Consider UL-SRS as the sidelink reference signal for positioning, as the baseline during the evaluation.

In V2X positioning or any other sidelink positioning use-cases, a UE may need to determine the distance to surrounding UEs. For example, a moving vehicle needs to determine whether there are surrounding VRUs and the relative distance to those VRUs. The determination operation should be performed in a quick manner. Hence, in case one or more VRUs are detected to be very close, the moving vehicle and/or VRUs can perform the necessary action to avoid potential collision. The aforementioned determination of surrounding UEs (VRUs) can be facilitated by allowing concurrent transmission of SL-PRS from multiple UEs. For example, multiple VRU(s) can transmit “SL-PRS” concurrently and/or within the same set of resources. The transmitted SL-PRS can be received by moving vehicle(s), VRU(s), or RSU(s). The concurrent transmission of “SL-PRS” can be arranged by allowing multiplexing of SL-PRS from multiple UEs within a given set of resources (e.g., time-division multiplexing (TDM), frequency-division multiplexing (FDM), and/or Code-division Multiplexing (CDM))

Proposal 2: Consider supporting multiplexing of sidelink positioning reference signal from several UEs within a given set of resources.

The UE is expected to perform positioning measurement based on the received “SL-PRS”. The relative positioning estimation (i.e, relative distance) is obtained based on the collected positioning measurement. We consider sidelink positioning should consider the legacy positioning measurement, such as received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA). In addition to the relative distance between UEs, other parameters, such as angle of arrival (AoA), angle of departure (AoD), LOS indication, can be obtained based on those positioning measurements.

Proposal 3: Relative positioning is estimated based on received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA).

NR Positioning based on direct-link (Uu) has been well established since Rel-16. NR positioning is designed to support commercial requirements that have stringent positioning accuracy as required for certain Industrial IoT use-cases. The required accuracy as introduced in Rel-17 is in the order of 20 cm which is quite challenging. The study of sidelink positioning can be used to improve the overall positioning accuracy. For example, if one of the UE is RSU in which the location is known and the RSU has LOS component towards the VRU or moving vehicle. On the other hand, the LOS component from gNB in direct-link can be limited due to buildings and obstructions. In this particular example, it would be beneficial to jointly utilize direct-link and sidelink for positioning estimation purpose.

Proposal 4: Support hybrid positioning where the UE receives reference signal for positioning from both direct-link (Uu) and sidelink (PC5) and jointly utilize for positioning estimation purpose.

One of the challenging scenarios for positioning in V2X use-cases is to handle Vulnerable Road User (VRU). This is one of the scenarios that has been identified by 5GAA as shown in Figure 1.

Figure 1: Illustration of VRU, ref. 5GAA Technical report on C-V2X Use Cases and Service Level Requirements Volume I

In Figure 1, the main purpose is to alert vehicle (Host Vehicle - HV) of VRUs approaching the road or crossing an intersection and warn of any risk of collision. Hence, to provide safety, typically in the road or crossing, there is a road side unit (RSU) that can be in a form of road sign or traffic light. In V2X context, HV, VRU and RSU are equipped with communication module that can support both Uu and PC5 interfaces. In order to avoid the collision, the position of VRU and HV should be known, at least the relative distance  position between VRU and HV. If the relative distance is relatively close then a warning on potential collision message can be provided. RSU can be used to assist the position of VRU and HV. The positioning estimation can be performed in VRU, HV, RSU, or LMF, depending on the context / purposes.

Observation 1: In V2X scenario, some UE(s) may be stationary (e.g., a UE in RSU). This type of UE can be used to facilitate positioning procedure, such as to perform as reference UE for positioning.

The radio resources for the SL-PRS transmissions are limited. Furthermore, it should also consider the following circumstances:

- The need to perform positioning measurement and estimation in a timely manner. Hence, the transmission, in some cases, may require low latency and high accuracy.

- UEs (VRUs) distribution that can be significantly changed. In one particular time, it can be many UEs (rush hours) and another particular time, it can be a few UEs (midnight time).

The usage of radio resources and the SL-PRS transmission should be controlled (e.g., by gNB) so that sidelink-positioning can be used, particularly to support V2X use-cases (e.g., collision avoidance) where the positioning estimation shall be obtained in a timely manner. Network-controlled radio resources have been supported in legacy sidelink for communication and known as Mode-1. In a junction scenario and during rush hour, the number of VRUs are expected to be high. There should be a mechanism to control which UEs that are allowed to transmit. For example, the RSUs can be configured to transmit SL-PRS and the VRUs are configured to receive SL-PRS. Outside of rush-hours, there could be the opposite case where VRUs are configured to transmit SL-PRS. In this example, UE-types classification for V2X positioning (e.g., RSU, VRU, Car) can be beneficial for V2X positioning. RAN1 can consider sending LS to RAN2 or SA2 with respect to this.

Proposal 5: Consider to support UE-types classification for V2X positioning (e.g., RSU, VRU, Car).

The concept of region / zone has been commonly used in sidelink for communication. In a given zone, a UE is expected to perform communication using a specific carrier frequency. Similar concept of region / zone can be applied for positioning purpose, such as to adapt positioning procedure based on the region/zone of the UE. For example, if a UE is in a cross-junction, the UE is expected to perform frequent positioning procedure (e.g., positioning measurement / estimation), particularly to detect and avoid potential collision.

Proposal 6: Consider to adapt positioning procedure based on the region/zone of the UE.

Sidelink positioning relies on the interaction, such as transmission and reception, between UEs. For example, a UE-A transmits SL-PRS and another UE-B receives it and performs a positioning measurement. From the positioning measurement, the UE-B is expected to be able to measure the relative position (distance), such as UE-A is X meters away from UE-B. UE-B would still not be able to know the direction of UE-A relative from UE-B.

Figure 2: Illustration of relative positioning without angle information

Obtaining the relative positioning and relative angle are needed in certain use-cases. One of the use-cases is the use-cases identified in ranging based services technical report [5]. Furthermore, the work in SA2 [6], has identified a key issue on supporting ranging service operation procedure with the assistance of another UE. The assistance of another UE can be used to estimate both relative positioning (distance) and angle as illustrated in Figure 3.

Figure 3: Illustration of ranging between host and target UEs, with help form an assistant UE.

We consider RAN1 should also study this aspect, particularly how the physical layer aspects and the interation with higher layer can support this procedure.

Proposal 7: Consider supporting positioning procedure with the assistance of another UE for the estimation of relative positioning and relative angle.

# Conclusion

In this contribution, we provide our initial view on potential solution for sidelink positioning. The observations and proposals are listed below.

Observation 1: In V2X scenario, some UE(s) may be stationary (e.g., a UE in RSU). This type of UE can be used to facilitate positioning procedure, such as to perform as reference UE for positioning.

Proposal 1: Consider UL-SRS as the sidelink reference signal for positioning, as the baseline during the evaluation.

Proposal 2: Consider supporting multiplexing of sidelink positioning reference signal from several UEs within a given set of resources.

Proposal 3: Relative positioning is estimated based on received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA).

Proposal 4: Support hybrid positioning where the UE receives reference signal for positioning from both direct-link (Uu) and sidelink (PC5) and jointly utilize for positioning estimation purpose.

Proposal 5: Consider to support UE-types classification for V2X positioning (e.g., RSU, VRU, Car).

Proposal 6: Consider to adapt positioning procedure based on the region/zone of the UE.

Proposal 7: Consider supporting positioning procedure with the assistance of another UE for the estimation of relative positioning and relative angle.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021.

- TR38.845

- TS22.261

- TS22.104

- TR 22.855 Study on Ranging-based Services

- TR 23.700-86