3GPP TSG RAN WG1#109-e	        	R1-2203742

E-meeting, May 9th – 20th, 2022

Agenda Item:					    9.8.2

Source:	Sony

Title:	Considerations on L1/L2 signaling for side control information

Document for:	Discussion and decision

# Justification and objective

RAN#94-e approved a Rel-18 SI on network-controlled repeaters (NCRs). The justification and objectives are given in the SID [1] and reproduced below. In [2], we provided our initial views on what side control signaling is essential for adopting NCRs on top of Rel-17 3GPP specifications. This contribution considers mechanisms to enable L1/L2 signaling of the necessary side control information, based on our findings in [2].

## Justification

Coverage is a fundamental aspect of cellular network deployments. Mobile operators rely on different types of network nodes to offer blanket coverage in their deployments. Deployment of regular full-stack cells is one option, but it may not always be possible (e.g., no availability of backhaul) or economically viable.

As a result, new types of network nodes have been considered to increase mobile operators’ flexibility for their network deployments. For example, Integrated Access and Backhaul (IAB) was introduced in Rel-16 and enhanced in Rel-17 as a new type of network node not requiring a wired backhaul. Another type of network node is the RF repeater which simply amplify-and-forwards any signal that it receives. RF repeaters have seen a wide range of deployments in 2G, 3G and 4G to supplement the coverage provided by regular full-stack cells. In Rel-17, RAN4 specified RF and EMC requirements for such RF repeaters for NR targeting both FR1 and FR2.

While an RF repeater presents a cost-effective means of extending network coverage, it has its limitations. An RF repeater simply does an amplify-and-forward operation without being able to take into account various factors that could improve performance. Such factors may include information on semi-static and/or dynamic downlink/uplink configuration, adaptive transmitter/receiver spatial beamforming, ON-OFF status, etc.

A smart repeater is an enhancement over conventional RF repeaters with the capability to receive and process side control information from the network. Side control information could allow a smart repeater to perform its amplify-and-forward operation in a more efficient manner. Potential benefits could include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial directivity, and simplified network integration.

## Objective

The study on NR smart repeaters is to focus on the following scenarios and assumptions [the numbering is ours]:

- [AS-1] Smart repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- [AS-2] For only single hop stationary smart repeaters

- [AS-3] Smart repeaters are transparent to UEs

- [AS-4] Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

NOTE 1: Cost efficiency is a key consideration point for smart repeaters.

Study and identify which side control information below is necessary for smart repeaters including assumption of max transmission power [RAN1] [the numbering is ours]

- [SC-1] Beamforming information

- [SC-2] Timing information to align transmission / reception boundaries of smart repeater

- [SC-3] Information on UL-DL TDD configuration

- [SC-4] ON-OFF information for efficient interference management and improved energy efficiency

- [SC-5] Power control information for efficient interference management (as the 2nd priority)

Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Study the following aspects of smart repeater management

- Identification and authorization of smart repeaters [RAN2, RAN3]

NOTE 2: Coordination with SA3 may be needed.

NOTE 3: It has the possibility to rename the SI title in RAN#95e.

# Introduction

Figure 1 shows a schematic view of a gNB, an NCR, and a UE, as part of a 5G NR communications network. We assume that an NCR consists of two logical components: a mobile terminal (MT)-NCR component and a radio unit (RU)-NCR component. Generally speaking, the NCR-MT component takes care of the control plane; it exchanges configuration messages with the gNB and configures the NCR-RU component accordingly. Once configured, the NCR-RU provides a transparent amplify-and-forward function between the gNB and a target UE.

Figure 1. Schematic view of a gNB, an NCR, a UE, and the associated signaling paths.

Our contribution [2] discusses possible architectures and protocol stacks for the system in Figure 1. In this document, we are concerned with the signaling aspects.

# L1/L2 signaling

In the following, we list elements of information that we believe need to be signaled between the gNB and the NCR to transport side control information. The signaling itself can be implemented as L1 signaling (i.e., DCI), L2 signaling (i.e., MAC CE), or a combination. The precise implementation of the preceding, that is, the precise DCI and MAC CE messages needed, should be discussed during RAN1#109-e.

## Beamforming information

In [2], we argued that NCR-MT front-haul (FH) and access links are suited for different types of beamforming information. While signals on the NCR-RU FH link can be typeD-QCLed with those on the NCR-MT link, e.g., SS/PBCH and CSI-RS, NCR-RU access link signals have no obvious QCL sources. Therefore, the spatial properties of the NCR-RU access link may need to be discovered, e.g., by configuring UEs to measure on access beams. The preceding suggests that the gNB should be able to address beamforming information to two different recipients: the NCR-RU FH link and the NCR-RU access link.

- The gNB should be able to send beamforming information to the NCR-RU FH and access links separately. For example, beamforming information for the NCR-RU FH link can include typed QCL indications referring to NCR-MT link signals, such as SS/PBCH and CSI-RS signals. On the other hand, RAN1 should study how to use the typed QCL principle to signal beamforming information to the NCR-RU access link.

MAC CE and DCI Rel-17 mechanisms can be used as a starting point. In particular, a set of TCI states can be pre-configured via L2/MAC CE semi-static signaling. Then, at a later time, desired TCI states can be dynamically activated using L1/DCI messages.

- The BS can pre-configure the NCR-RU with a set of TCI states via L2/MAC CE semi-static signaling for side control beamforming information. Then, at a later time, desired TCI states can be dynamically activated using L1/DCI messages.

## Timing information

We found in [2] that the timing of the NCR-MT and -RU components may need to be set independently. While [2] suggested that using NCR-MT as the time reference of the NCR-RU component, adjustments to accommodate for delay spreads in the propagation channel of certain UEs may be needed. We, therefore, make the following proposal.

- For side control timing information, consider a timing adjust command for adjusting the timing of the NCR-RU concerning the NCR-MT to accommodate transmissions from UEs with different timing needs.

Given that this adjustment is only needed occasionally, MAC CE signaling appears adequate.

## UL/DL TDD configuration information

Similar to the preceding subsection, in [2], we argued that the gNB might configure the NCR-MT and -RU components with different UL/DL TDD patterns. In addition, we also found that a combination of L1 and L2 signaling can accommodate the signaling needs of this side control information.

- The gNB should be able to configure the NCR-MT and NCR-RU with independent UL/DL TDD patterns.

- For side control UL/DL TDD information, consider L2/MAC CE signaling to configure an UL/DL TDD pattern compatible with cell-specific UL/DL TDD configuration. Also, consider L1/DCI signaling to configure UL/DL TDD patterns to enforce UE-specific UL/DL TDD configurations.

## ON/OFF information

Regarding side control ON/OFF information, in [2], we proposed introducing so-called activity patterns, which superimpose already configured UL/DL TDD patterns and can be used to switch the NCR-RU ON and OFF on a slot or OFDM-symbol basis. UL-only, DL-only, or UL/DL activity patterns can be considered for increased efficient interference management and energy efficiency. See [2] for further details.

- For side control ON/OFF information, consider the signaling of

- periodic activity patterns, superimposed to UL/DL TDD patterns, with a granularity of slots or OFDM symbols, to combat periodic interference patterns;

- aperiodic ON/OFF activation/deactivation commands, to address episodic interference situations.

- For side control ON/OFF information, both L1/DCI and L2/MAC CE signaling can be considered. For example, MAC CE commands can configure and activate periodic activity patterns and pre-configure a set of aperiodic activity patterns. DCI signaling can activate aperiodic activity patterns at specific slots or OFDM symbols.

# Conclusions

This document considered L1/L2 signaling to transport essential side control information from a gNB to a network-controlled repeater (NCR). We made the following observations and proposals:

- The gNB should be able to send beamforming information to the NCR-RU FH and access links separately. For example, beamforming information for the NCR-RU FH link can include TypeD QCL indications referring to NCR-MT link signals, such as SS/PBCH and CSI-RS signals. On the other hand, RAN1 should study how to use the typeD QCL principle to signal beamforming information to the NCR-RU access link.

- The BS can pre-configure the NCR-RU with a set of TCI states via L2/MAC CE semi-static signaling for side control beamforming information. Then, at a later time, desired TCI states can be dynamically activated using L1/DCI messages.

- For side control timing information, consider a timing adjust command for adjusting the timing of the NCR-RU concerning the NCR-MT to accommodate transmissions from UEs with different timing needs.

- The gNB should be able to configure the NCR-MT and NCR-RU with independent UL/DL TDD patterns.

- For side control UL/DL TDD information, consider L2/MAC CE signaling to configure an UL/DL TDD pattern compatible with cell-specific UL/DL TDD configuration. Also, consider L1/DCI signaling to configure UL/DL TDD patterns to enforce UE-specific UL/DL TDD configurations.

- For side control ON/OFF information, consider the signaling of

- periodic activity patterns, superimposed to UL/DL TDD patterns, with a granularity of slots or OFDM symbols, to combat periodic interference patterns;

- aperiodic ON/OFF activation/deactivation commands, to address episodic interference situations.

- For side control ON/OFF information, both L1/DCI and L2/MAC CE signaling can be considered. For example, MAC CE commands can configure and activate periodic activity patterns and pre-configure a set of aperiodic activity patterns. DCI signaling can activate aperiodic activity patterns at specific slots or OFDM symbols.

# References

- 3GPP RP-213592, “Study on smart repeaters,” 3GPP TSG RAN Meeting #94e, ZTE Corporation.

- 3GPP R1-2203741, “Considerations on side control information to enable NR network-controlled repeaters,” Sony.